# PLANT DESIGN FOR THE PRODUCTION OF POLY-ALUMINIUM CHLORIDE FROM GHANAIAN BAUXITE

A Plant Design Project Report

Presented to the

Department of Chemical Engineering

Faculty of Mechanical and Chemical Engineering

College of Engineering

Kwame Nkrumah University of Science and Technology, Kumasi

By

AMOAH, KELVIN

DARKO, MELISSA MAKU AFRIYIE

GYASI-DENTEH, PRINCE SUGAR

OSABUTEY, SYLVESTER NII ABBEY

In Partial Fulfilment of the Requirements

For the Degree of

Bachelor of Science (HONS)

Chemical Engineering

February 2026

©

Table of Contents

1	INTRODUCTION	3

1.1	Background	3

1.2	Problem Statement	6

1.3	Justification	7

1.3.1	Municipal Water Treatment: The Barekese Case Study	7

1.3.2	Macro-Economic Drivers and 2025 Stabilisation	8

1.3.3	The GIADEC Framework and Value Addition Incentives	9

1.3.4	Regional Demand for PAC in West Africa	9

2	LITERATURE REVIEW	11

2.1	OVERVIEW OF COAGULATION IN WATER TREATMENT	11

2.1.1	Classification of Coagulants and Technical Comparison	12

2.1.2	Advantages of PAC over Conventional Coagulants	15

2.2	CHEMISTRY AND STRUCTURE OF POLYALUMINIUM CHLORIDE (PAC)	17

2.3	RAW MATERIAL CHARACTERISATION: AWASO BAUXITE	19

2.4	COMMINUTION OF BAUXITE ORE	23

2.4.1	Comminution Theory: Energy–Size Relationships	24

2.4.2	Primary Crushing: Jaw Crushers	25

2.4.3	Secondary Comminution: Wet Ball Milling	26

2.4.4	Wet Classification: Hydro-cyclones	27

2.5	KINETIC CONSIDERATIONS FOR ACID LEACHING	28

2.5.1	Primary Reaction Kinetic Modelling (Power Law)	29

2.5.2	Competing Iron Dissolution Kinetics	30

2.6	REVIEW OF PRODUCTION TECHNOLOGIES	31

2.6.1	The Alkaline (Bayer) Route	31

2.6.2	Direct Acid Leaching Route	32

2.6.3	The Polymerisation Process: Basicity Adjustment and the Role of the Al₁₃ Polycation	33

2.6.4	Calcination: Effect of Temperature on Alumina Solubility	37

3	PROCESS SELECTION	40

3.1	Process Route Selection	40

3.1.1	Feed Preparation and Beneficiation	41

3.1.2	Milling and Slurry Formation	42

3.1.3	Digester Kinetics and Thermodynamics	43

3.1.4	Solid-Liquid Separation	43

3.1.5	Speciation Control via Basicity (OH/Al Ratio)	43

3.1.6	Ageing (Maturation) of PAC	45

3.2	Quality Standards for PAC	48

4	MATERIAL AND ENERGY BALANCE	50

4.1	Material Balance	50

4.1.1	Assumptions	50

4.1.2	Unit Operation Material Balance Descriptions	52

4.1.3	Overall Material Balance Table	85

4.2	Energy Balance	133

4.2.1	Energy Balance Framework and Sign Convention	133

4.2.2	Per-Unit Energy Balance	134

4.2.3	Plant-Wide Energy Balance Summary	155

5	Equipment specification	157

5.1	Solid Handling and Comminution Infrastructure	157

5.1.1	Primary Jaw Crusher (Block: CRK-201)	157

5.1.2	Solid Handling Hopper and Mixer (Block: H-301)	159

5.2	High-Temperature Pyrometallurgical Pre-Treatment	159

5.2.1	High-Temperature Fabric Filter Baghouse (Block: BH-101)	159

5.3	Hydrometallurgical Reaction and Leaching Systems	162

5.3.1	Polymerisation and Maturation Tank (Block: R-702)	162

5.4	Separation and Clarification Equipment	163

5.4.1	Liquid-Solid Hydrocyclone Array (Block: HC-501)	163

5.4.2	Mechanical Filter Press (Block: FP-601)	164

5.4.3	High-Intensity Magnetic Separator (Block: MS-401)	166

5.5	HEAT EXCHANGER NETWORK (HEN)	166

5.5.1	Air and Flue Gas Recovery Exchangers (Blocks: CL-201, HX-102, HX-103)	166

5.5.2	Acidic and Corrosive Heat Exchangers (Blocks: HX-201, HX-601, HX-203)	168

5.6	Fluid Transport: Pumping Infrastructure	169

5.7	Fluid Transport: Piping Network and Rheological Guidelines	171

5.8	Auxiliary Fluid Storage, Mixing, and Flow Control	173

6	DESIGN OF SPECIFIC EQUIPMENT	175

6.1	ONE-DIMENSIONAL STEADY-STATE SIMULATION OF A COUNTER-CURRENT ROTARY KILN FOR BAUXITE REDUCTIVE ROASTING – Gyasi-Denteh, Prince Sugar	175

6.1.1	Abstract	175

6.1.2	INTRODUCTION	175

6.1.3	LITERATURE REVIEW	178

6.1.4	CHEMICAL ENGINEERING CALCULATIONS	187

6.1.5	SIMULATION RESULTS AND DISCUSSION	201

6.1.6	MECHANICAL ENGINEERING CALCULATIONS	208

6.2	ALUMINA ACID DIGESTION REACTOR DESIGN – OSABUTEY, SYLVESTER NII ABBEY	213

6.2.1	Problem Statement	213

6.2.2	Introduction	213

6.2.3	Equipment Selection and Justification	214

6.2.4	Process and Equipment Description	215

6.2.5	Chemical Engineering Design	217

6.2.6	Mechanical Design	233

6.3	DESIGN OF A BALL MILL – AMOAH, Kelvin	251

6.3.1	INTRODUCTION	251

6.3.2	LITERATURE REVIEW	253

6.3.3	CHEMICAL ENGINEERING DESIGN CALCULATIONS	260

6.3.4	MECHANICAL ENGINEERING CALCULATIONS	267

6.4	DESIGN OF A BASICITY ADJUSTMENT REACTOR	276

6.4.1	INTRODUCTION	276

6.4.2	PROBLEM STATEMENT	277

6.4.3	AIM	277

6.4.4	OBJECTIVES	277

6.4.5	LITERATURE REVIEW	278

6.4.6	REACTION MECHANISMS OF BASICITY ADJUSTMENT	282

6.4.7	JUSTIFICATION OF THE METHOD	285

6.4.8	CHEMICAL ENGINEERING DESIGN CALCULATIONS	286

6.4.9	MECHANICAL DESIGN	295

7	PLANT LOCATION AND LAYOUT	301

7.1	Plant Location	301

7.1.1	Availability of Land	301

7.1.2	Raw Material Supply	302

7.1.3	Nearness to Market	303

7.1.4	Availability of Labour	303

7.1.5	Availability of Utilities	304

7.1.6	Waste Disposal	304

7.1.7	Energy Availability	305

7.1.8	Transportation and Access	306

7.1.9	Climate	308

7.2	Choice of Location	308

7.3	Plant Layout	309

7.3.1	Factors Considered in the Plant Layout	310

8	PROCESS CONTROL AND INSTRUMENTATION	313

8.1	Introduction	313

8.2	Process Control Philosophy	313

8.3	Control Strategy for the Acid Leaching Reactor Train (R-601)	316

8.3.1	Reactor Temperature Control	317

8.3.2	Acid-to-Bauxite Ratio Control	318

8.3.3	Reactor Level Control	318

8.3.4	Reactor Pressure Control	318

8.3.5	Agitation Monitoring	319

8.4	Summary of Process Control Strategies for Major Process Units	320

9	SAFETY AND POLLUTION CONTROL	324

9.1	INTRODUCTION	324

9.2	KEY HAZARDS ON THE PLANT	324

9.2.1	Mechanical Hazards	324

9.2.2	Chemical Hazards	325

9.2.3	Thermal Hazards	325

9.2.4	Environmental Hazards	326

9.3	SAFETY MEASURES	326

9.3.1	Worker Safety	326

9.3.2	Equipment Safety	327

9.3.3	Product Safety	327

9.4	SAFETY AND POLLUTION CONTROL	328

9.4.1	Area 100 – Raw Material Handling and Preparation	328

9.4.2	Area 200 – Pyro-Processing (Rotary Kiln)	328

9.4.3	Area 300 – Flue Gas Treatment	328

9.4.4	Area 400 – Dry Magnetic Separation	329

9.4.5	Area 500 – Wet Grinding and Classification	329

9.4.6	Area 600 – Leaching and Solid-Liquid Separation	329

9.4.7	Area 700 – Basification, Polymerization and Purification	330

9.5	HAZARD AND OPERATABILITY (HAZOP) ANALYSIS	330

9.5.1	RISK MATRIX METHODOLGY	330

9.5.2	HAZOP Analysis and Risk Ranking for Area 100 – Raw Material Handling	331

9.5.3	HAZOP Analysis and Risk Ranking for Area 200 – Pyro-Processing	332

9.5.4	HAZOP Analysis and Risk Ranking for Area 300 – Flue-Gas Treatment	333

9.5.5	HAZOP Analysis and Risk Ranking for Area 400 – Dry Magnetic Separation	334

9.5.6	HAZOP Analysis and Risk Ranking for Area 500 – Wet Grinding	335

9.5.7	HAZOP Analysis and Risk Ranking for Area 600 – Leaching	336

9.5.8	HAZOP Analysis and Risk Ranking for Area 700 – Basification and Polymerisation	337

9.6	POLLUTION CONTROL SUMMARY	337

10	PLANT ECONOMIC ANALYSIS	339

10.1	Executive summary	339

10.2	Introduction and plant overview	339

10.2.1	Key Plant Parameters and Basis of Estimate	340

10.3	Capital Expenditure (CAPEX)	341

10.3.1	Costing Methodology	341

10.3.2	Equipment Cost Estimate	342

10.3.3	Total Capital Investment Summary	348

10.4	Operating Expenditure (OPEX)	350

10.4.1	Raw Material Costs	350

10.4.2	Utility Costs	351

10.4.3	Labour Costs	352

10.4.4	Maintenance	353

10.4.5	Insurance	354

10.4.6	Plant Overheads	354

10.4.7	Logistics and Distribution	354

10.4.8	Other Variable Costs	355

10.4.9	Fixed Costs: CSR, Property Tax and Permits	355

10.4.10	Total OPEX Summary	355

10.5	Revenue and Market Analysis	356

10.5.1	Product Specification and Pricing	356

10.5.2	Market Size and Penetration Strategy	357

10.5.3	By-Product Revenue	358

10.5.4	Total Revenue Projection	358

10.6	Profitability and Discounted Cash Flow Analysis	359

10.6.1	Financing Structure and Cost of Capital	359

10.6.2	Depreciation and Taxation	361

10.6.3	Key Economic Performance Indicators	363

11	CONCLUSIONS	366

11.1	Process Design and Technical Viability	366

11.2	Plant Location and Layout	366

11.3	Process Control and Instrumentation	367

11.4	Health, Safety and Environmental Management	367

11.5	Plant Economic Analysis	368

12	RECOMMENDATIONS	369

12.1	Secure a Long-Term Offtake Agreement with GWCL	369

12.2	Pursue Concessionary Development Finance	369

12.3	Commission a Bankable Feasibility Study and FEED	370

12.4	Derisk the Hydrochloric Acid Supply Chain	370

12.5	Validate Process Chemistry at Pilot Scale	370

12.6	Establish a Formal Product Quality Management System	371

12.7	Develop an ECOWAS Export Market Strategy	371

12.8	Strengthen the Pollution Control and Waste Management Framework	372

12.9	Optimise Financing Structure Through GIADEC Incentives	372

12.10	National Policy Advocacy	373

13	REFERENCES	374

14	APPENDICES	387

14.1	Rotary kiln matlab simulation	387

14.2	Acid leaching reactor matlab simulation	410

14.2.1	SCM_RTD_SOLVER Size two equal-volume CSTRs using the SCM-RTD model	410

14.2.2	optimize_cstr_series	418

14.3	Appendix: P&ID Nomenclature & Tagging Legend	423

LIST OF FIGURES

Figure 1-1 Barekese raw water turbidity from 2013 to 2018. (Source: Barekese Water Quality Laboratory, 2018)	7

Figure 2-1 Technical Comparison of Principal Coagulant Classes for Water Treatment	15

Figure 2-2 Ball and Stick model of the Keggin polycation	18

Figure 2-3 The Geological and Mineral Resource map of Ghana, highlighting the regional distribution of bauxite deposits.	21

Figure 3-1 Process Flow Diagram	40

Figure 3-2 Schematic representation of the magnetic separation of magnetite from the roasted ore.	42

Figure 4-1 Streams 101, 102, 103, 104, 201, 202, 203	94

Figure 4-2 Streams 204, 301, 303B, 304, 304B, 401, 501	103

Figure 4-3 Streams 502, 503, 504, 505, 506, 506B, 601	112

Figure 4-4 Streams 602, 603, 701, BH-AIR, BH-DUST, CAAL2O4, CAAL2O4B	121

Figure 4-5 Streams PW-2, SIO2	132

Figure 5-1 Design summary for primary jaw crusher, CRK-201	158

Figure 5-2 Particle size distribution of the input and output of the Jaw Crusher, CRK-201	158

Figure 5-3 Design summary for Mechanical Filter Press, FP-601	165

Figure 5-4 Design summary for high temperature Gas Exchangers	167

Figure 5-5 Design summary for Acidic and corrosive heat exchangers	168

Figure 5-6 Comprehensive Pump Performance Specifications	170

Figure 5-7 Piping Specification and Fluid Dynamics	172

Figure 6-1 Labelled longitudinal section of a charged rotary kiln; Source: (Tscheng & Watkznson, 1979)	182

Figure 6-2 Phase diagram for Al(OH)3; Source: (Ioelovich, 2017)	184

Figure 6-3 Cross-section of a charged rotary kiln; Source: (Tscheng & Watkznson, 1979)	186

Figure 6-4 Variation of bauxite bulk density along kiln length	190

Figure 6-5 Temperature profiles of Freeboard, solid bed and wall	202

Figure 6-6 Heat duty profiles	202

Figure 6-7 Thermal conductivity profiles	203

Figure 6-8 Molar flow rate profiles for gibbsite dehydration	204

Figure 6-9 Solid molar flow rates of Fe2O3 and Fe3O4	204

Figure 6-10 Gas/Vapour molar flow rates	205

Figure 6-11 Gas phase partial pressures	206

Figure 6-12 Reaction Rate Profiles	206

Figure 6-13 Dynamic fill degree (f) and bulk density profiles	207

Figure 6-14 Schematic diagram illustrating the shrinking core model - product layer diffusion	219

Figure 6-15 Residence distribution of single CSTR vs. Two equal CSTRs in series	223

Figure 6-16 Mean alumina conversion vs. residence time per reactor (Two-Stage CSTR system)	225

Figure 6-17 Conversion vs Total Residence Time — Single CSTR and Two Equal CSTRs in series	226

Figure 6-18 Motion of charge in a tumbling mill	258

Figure 6-19Matlab codes for the simulation of a kinetic plot	291

Figure 6-20 A graph of conversion against time	292

Figure 7-1 Road and rail network linking Takoradi Port and the Awaso bauxite mine to the proposed plant site. Schematic corridor map; distances approximate.	307

Figure 8-1Control System Architecture of the PAC Production Plant	316

Figure 11-1 Key Plant Design and Economic Parameters	341

Figure 11-2 Year-by-Year Cash Flow Projection (USD)	363

LIST OF TABLES

Table 1-1 Comparative Study of Ghana’s Macroeconomic Baseline (2022–2024) and Projected Fiscal Policy Shifts (2025–2026) Impacting PAC Plant Viability. Sources: Bank of Ghana (2025); Ministry of Finance (2026); ECOTIS (2025)	8

Table 1-2 Estimated Annual PAC Demand in Selected West African Countries. Sources: WHO (2023); ECOTIS (2025); industry estimates.	10

Table 2-1 Profile Stratigraphy and mineralogical distribution of minerals at the Awaso bauxite deposit	21

Table 2-2  Comparative Evaluation of the Alkaline (Bayer) Route and the Direct Acid Leaching Route for PAC Production	33

Table 6-1 Counter-current flow arrangement in Unit RK-201	187

Table 6-2 Solid feed composition and mass flow rates, Unit RK-201.	187

Table 6-3 Gas feed composition and mass flow rates, Unit RK-201	188

Table 6-4 Chemical reactions and standard enthalpies.	188

Table 6-5 Kiln geometric and kinematic parameters	189

Table 6-6 Per-unit-length interface areas	191

Table 6-7 Gas-phase heat capacity polynomials (NIST WebBook)	192

Table 6-8 Solid-phase heat capacity expressions	192

Table 6-9 Parameters for gas-to-exposed-wall heat transfer coefficient.	193

Table 6-10 Wall thermal property values.	195

Table 6-11 Solid-phase mass balance ODEs.	198

Table 6-12 Gas-phase mass balance ODEs.	199

Table 6-13 Nine boundary conditions for the two-point BVP.	200

Table 6-14 bvp4c solver options.	200

Table 6-15 Key performance metrics computed in post-processing.	200

Table 6-16 Wall Thickness Specification	208

Table 6-17 Summary of mechanical design parameters, Unit RK-201.	212

Table 6-18 Design basis and criteria for the acid digestion reactor	217

Table 6-19 Adopted kinetic parameters	221

Table 6-20 Residence time optimisation results	225

Table 6-21 Component mass flowrates and densities	226

Table 6-22 Reactor heat duties from Aspen Plus® simulation	228

Table 6-23 Effect of aspect ratio on reactor dimensions	233

Table 6-24 Pressure vessel design criteria	235

Table 6-25 Summary of nozzle sizes	239

Table 6-26 Physical properties for agitation design	240

Table 6-27 Agitator shaft design summary	245

Table 6-28 Lug support geometry	247

Table 6-29 Chemical Design Input Parameters	261

Table 6-30 A summary of the chemical engineering calculations.	266

Table 6-31 Mechanical Design Input Parameters	267

Table 6-32 A summary of the mechanical engineering calculations.	274

Table 6-33 Reactor Specification	286

Table 6-34 Base Specification	288

Table 6-35 Summary of key design results	298

Table 6-36 Mechanical engineering calculations	299

Table 7-1 Transport routes serving the site.	307

Table 8-1 Process control objective for the PAC Production Plant	315

Table 8-2 Summary of Control Strategy for the Acid Leaching Reactor Train (R-601)	319

Table 10-1 Key plant design and economic parameters	340

Table 10-2 Area 100 Equipment Cost Estimate (2026, Ghana Basis)	343

Table 10-3 Area 200 Equipment Cost Estimate (2026, Ghana Basis)	343

Table 10-4 Area 300 Equipment Cost Estimate (2026, Ghana Basis)	344

Table 10-5 Area 400 Equipment Cost Estimate (2026, Ghana Basis)	344

Table 10-6 Area 500 Equipment Cost Estimate (2026, Ghana Basis)	345

Table 10-7 Area 600 Equipment Cost Estimate (2026, Ghana Basis)	345

Table 10-8  Area 700 Equipment Cost Estimate (2026, Ghana Basis)	346

Table 10-9 HEN Equipment Cost Estimate (2026, Ghana Basis)	347

Table 10-10 Ancillary Equipment Cost Estimate (2026, Ghana Basis)	347

Table 10-11 Total Capital Investment Summary	348

Table 10-12 CAPEX Phasing by Construction Year	349

Table 10-13 Annual Raw Material Costs (100% Capacity Utilisation)	350

Table 10-14 Annual Utility Costs (PURC 2026 Tariff Basis, 100% Utilisation)	352

Table 10-15 Shift Labour Staffing and Annual Cost	352

Table 10-16 Day & Administrative Labour Staffing and Annual Cost	353

Table 10-17 Total Annual OPEX Summary (100% Utilisation, Year 1 Real Prices)	356

Table 10-18 PAC Product Pricing Structure	357

Table 10-19 Annual Revenue Projection (Years 1–20)	359

Table 10-20 WACC Calculation Parameters	361

Table 10-21 Summary of Key Economic Performance Indicators	364

Table 14-1 Material Specifications & Piping Classes	423

Table 14-2 Fluid & Process Service Codes	424

Table 14-3 Equipment Class Prefixes	426

Table 14-4 Instrumentation & Control Valves (ISA-5.1)	428

# ABSTRACT

The severe contamination of Ghana's water bodies, driven predominantly by illegal small-scale mining ("Galamsey"), has escalated raw water turbidity in major river basins to levels exceeding 14,000 NTU. These unprecedented levels far exceed the treatment capabilities of conventional coagulants such as aluminium sulphate (alum). To address this critical water security crisis and eliminate Ghana's estimated annual USD 3.1 million expenditure on imported coagulants, this project presents a comprehensive whole-plant design for the domestic production of liquid Poly-Aluminium Chloride (PAC) utilizing indigenous Ghanaian bauxite.The proposed facility leverages locally sourced, high-quality gibbsitic bauxite from the Awaso deposit. The process begins with beneficiation, where haematite is converted to magnetite and subsequently removed via magnetic separation. The upgraded ore undergoes direct acid leaching with hydrochloric acid at 110°C under atmospheric pressure. Following digestion and rigorous solid-liquid separation to eliminate iron oxide and silica-rich residues, controlled basicity adjustment is achieved through the addition of calcium aluminate. This step, conducted at 75°C, targets a basicity of 65–75% (OH/Al ratio of 2.0–2.4) to promote the formation of the highly active Al13 Keggin polycation Al13O4OH24H2O127+. The product is then aged to ensure full polymerization and PAC stabilization. Quality control protocols ensure the final liquid PAC satisfies the stringent indices of the Chinese National Standard GB 15892-2009 (≥29.0 wt.% Al₂O₃, basicity 40–90%, pH 3.5–5.0) prior to storage and distribution.

Rigorous process modelling and simulation were conducted in Aspen Plus, employing the ENRTL-RK thermodynamic property method to accurately capture the complex electrolyte chemistry of the system. Economically, the plant's design is underpinned by a favourable macroeconomic environment, decisively improving the competitiveness of domestic manufacturing. Strategically positioned within the Ghana Integrated Aluminium Development Corporation (GIADEC) framework for downstream value addition, the facility boasts an estimated production capacity to serve both domestic municipal water treatment demand and the broader West African regional market (over 80,000 tonnes per annum). Ultimately, this plant design provides a technically robust, economically viable, and strategically vital solution to Ghana's water treatment challenges while advancing the nation's industrialization goals.

# INTRODUCTION

## Background

Access to safe drinking water is a keystone of public health and sustainable development. The World Health Organisation (WHO) estimates that approximately 2.2 billion people globally still lack safely managed drinking water services, with Sub-Saharan Africa bearing the heaviest burden. (WHO/UNICEF JOINT MONITORING PROGRAMME FOR WATER SUPPLY, SANITATION AND HYGIENE, n.d.). Coagulation, a physico-chemical process that destabilises suspended colloidal matter by adding a coagulant chemical, enabling aggregation (flocculation), sedimentation, and filtration to produce potable water of acceptable quality, is at the heart of conventional surface water treatment.

Coagulants are broadly classified into inorganic metal salts, inorganic polymeric coagulants, and natural/synthetic organic polymers. The most widely applied inorganic coagulants are aluminium-based compounds, notably aluminium sulphate (alum, Al₂(SO₄)₃·nH₂O) and ferric chloride (FeCl₃), with alum historically dominating the global market on account of its low unit cost, established supply chains and well-understood chemistry. Iron-based coagulants (ferric sulphate and ferric chloride) offer stronger performance in cold, low-turbidity waters but introduce colour issues. Poly-aluminium Chloride (PAC) represents the most essential advancement in inorganic polymeric coagulants; it incorporates pre-formed polynuclear hydroxo-aluminium species that exert superior charge-neutralisation and sweep-flocculation mechanisms, in comparison with conventional alum. (Daud et al., 2009; Wang et al., 2004).

Each coagulant type has advantages and limitations. Alum is inexpensive but demands strict pH adjustment (optimum 6.0–7.5), produces large volumes of sludge, and yields elevated residual aluminium in treated water. Ferric salts are effective across a wider pH range (5.0–9.0) and handle high turbidity well but can impart colour and are more corrosive. Natural coagulants such as Moringa oleifera seed extract are promising for rural, low-cost settings but lack the predictable performance and regulatory acceptance required for large municipal plants. PAC, by contrast, is effective across pH 5.0-8.5, produces 30-50% less sludge than alum, requires a lower dosage, and preserves performance in chilly.

Poly-aluminium Chloride (PAC) has emerged as the preferred advanced coagulant for municipal drinking water production globally. Its active constituent is the Al₁₃ Keggin polycation, Al13O4OH24H2O127+, a highly charged polynuclear species that delivers rapid and thorough charge neutralisation of negatively charged colloids, even at the extreme turbidity levels now regularly recorded in Ghana’s principal river basins (Zouboulis & Tzoupanos, 2010).

Adopting effective coagulants in Ghana has become urgent. The country is battling a severe water quality crisis driven by rapid urbanisation and, most critically, illegal small-scale gold mining, locally known as “Galamsey.” These activities have deposited heavy metals (mercury and lead) into major water bodies and have driven turbidity in the Pra, Ankobra and Offin rivers to levels as high as 14,000 NTU (Aikins, 2024).

This project proposes a whole-plant design for the domestic production of PAC, using locally sourced gibbsitic bauxite from the Awaso deposit in the Western-North Region of Ghana. The production pathway uses a direct acid leaching route, reacting beneficiated bauxite with hydrochloric acid at optimised conditions (90–120°C, atmospheric pressure) to achieve greater than 90% aluminium extraction, followed by controlled basicity adjustment and polymerisation to yield a liquid PAC product meeting municipal drinking water treatment standards (Zouboulis & Tzoupanos, 2010). The facility is designed within the strategic framework of the Ghana Integrated Aluminium Development Corporation (GIADEC) to add downstream value to the nation’s bauxite resources.

Liquid-form PAC is the preferred commercial product specification for water-based coagulant production primarily because the active polymeric aluminium species, particularly the highly effective Al₁₃ tridecameric complex (Al₁₃O₄(OH)₂₄⁷⁺ ), exist naturally in aqueous solution and are preserved in their optimal coagulation-active state without the need for further processing. Among the polymeric Al species, Al₁₃ is regarded as the most effective and stable species in water and wastewater treatment, and the production of PAC with a high Al₁₃ concentration is considered the primary aim of PAC research and industrial production (Jiang and Graham, 1998). Unlike solid PAC, liquid PAC does not require energy-intensive concentration and high-temperature drying operations. This is because liquid PAC is generally obtained first as the direct process product and then subjected to evaporation and drying to obtain solid PAC, which are additional unit operations that significantly increase both capital and operating costs. Furthermore, liquid PAC is convenient for metering in industrial applications and is well-suited to continuous dosing processes, integrating directly into the metering pump and dosing infrastructure standard at municipal and industrial water treatment facilities without any prior dissolution or preparation steps. Its liquid form facilitates automation and precise dosing, which is advantageous in large-scale municipal treatment plants compared to the manual handling required for solid alternatives, and liquid PAC typically generates up to 30–50% less sludge volume compared to traditional aluminium sulphate, with faster floc formation and settling rates improving clarifier throughput and filter run times (Richardson, 2026). These combined technical and operational advantages make liquid PAC the dominant commercial form in global water treatment markets, and the most technically rational product specification when proximity to end-users and adequate liquid storage infrastructure are present.

## Problem Statement

Pollution of Ghana’s water bodies has reached crisis levels. Illegal small-scale mining has contaminated approximately 60% of the country’s water sources, spiking turbidity levels in major rivers such as the Pra, Ankobra and Offin to as high as 14,000 NTU (Aikins, 2024). Ghana’s rivers are its most critical raw water source; the Ankobra and Pra basins alone supply water to approximately six million people in the Western and Central Regions through a network of treatment plants operated by the Ghana Water Company Limited (GWCL).

The GWCL has reported that traditional aluminium sulphate (alum) is no longer effective against such extreme turbidity loads. As a result, the cost of water purification in affected regions has increased by up to 60% because larger doses of chemicals are required to achieve potability. (My JoyOnline, 2023). Experts indicate that without a decisive shift in treatment strategy or pollution control, Ghana faces the existential threat of being unable to supply adequate potable water to its urban population by 2030 (Aikins, 2024).

Compounding this technical challenge is an economic one: Ghana currently imports its entire coagulant requirement, primarily from China and Europe, at an estimated annual cost exceeding USD 3.1 million. These imports are denominated in foreign currency, making the cost of water treatment extremely sensitive to exchange rate volatility. There is no domestic manufacturing facility that converts Ghana’s abundant, high-quality bauxite into this high-value, strategically critical chemical.

There is, therefore, a compelling and urgent need to establish domestic PAC production from Ghanaian bauxite. Such a facility would simultaneously address water security, reduce foreign exchange expenditure, and provide a platform for higher-value industrial processing of the nation’s mineral resources.

## Justification

The case for domestic PAC production in Ghana rests on four mutually reinforcing pillars: the performance imperative for advanced coagulation, sound macroeconomic fundamentals, the GIADEC strategic framework, and the growing regional demand for PAC in West Africa.

### Municipal Water Treatment: The Barekese Case Study

The Barekese Water Treatment Plant (BWTP) serves the Kumasi Metropolitan Area, and its operational data provides a compelling, data-driven argument for PAC adoption. A study by (Nti et al., 2021) analysing plant performance data from 2013 to 2018 demonstrated that raw water turbidity at Barekese ranged from 8.0 to 84.9 NTU, while colour ranged from 130 to 654 Pt-Co units.

Figure 1-1 Barekese raw water turbidity from 2013 to 2018. (Source: Barekese Water Quality Laboratory, 2018)

Under these conditions, alum dosages of 56.7 to 96.8 mg/L were required, resulting in pH depression and elevated sludge volumes. PAC, by contrast, achieved equivalent or superior treated water quality at dosages of only 14.1 to 43.2 mg/L, with a sludge volume reduction of 30–50% and without the pH destabilisation associated with alum. Given the post-2019 escalation of Galamsey-driven turbidity, the performance gap between alum and PAC has almost certainly widened considerably beyond that documented in the 2013–2018 study period.

The study by Nti et al. (2021) concluded that an ideal PAC dosage of 15 mg/L yielded superior turbidity and colour removal compared with alum, with a 30-50% reduction in sludge volume. This operational evidence from a major Ghanaian treatment plant provides a definitive, empirically grounded case for transitioning to PAC as the primary coagulant for municipal water treatment in Ghana.

### Macro-Economic Drivers and 2025 Stabilisation

The macroeconomic environment in Ghana has undergone a significant positive inflexion since mid-2024. The Ghanaian cedi appreciated by approximately 30% against the US dollar over 2025 (Bank of Ghana, 2025), substantially reducing the local-currency cost of imported capital equipment for plant construction. Annual consumer price inflation has decelerated to approximately 5.4% (Ministry of Finance, 2026), down from a peak of over 54% recorded in December 2022, making long-term project financial modelling more dependable. The Ministry of Finance has abolished VAT on mining exploration activities, reducing pre-project certification costs. Meanwhile, ECOWAS common external tariff adjustments have increased import duties on coagulant chemicals sourced from China from 10% to 20%, directly improving the price competitiveness of a domestically manufactured alternative.

Taken together, these macroeconomic shifts present a uniquely favourable window for the commissioning of a PAC production facility. The economics of local manufacturing versus continued imports have tilted decisively in favour of investment.

Table 1-1 Comparative Study of Ghana’s Macroeconomic Baseline (2022–2024) and Projected Fiscal Policy Shifts (2025–2026) Impacting PAC Plant Viability. Sources: Bank of Ghana (2025); Ministry of Finance (2026); ECOTIS (2025)

| Economic Indicator | 2022–2024 Baseline | 2025–2026 Shift | Impact on PAC Project |
| --- | --- | --- | --- |
| Cedi/USD Performance | Chronic Depreciation | ~30% Appreciation | Lowered cost of imported machinery and spares. |
| Annual Inflation Rate | >23.8% | ~5.4% (Ministry of Finance, 2026) | Predictability in labour and power cost planning. |
| VAT on Exploration | 15.0% | 0.0% (Abolished) | Reduced upfront costs for resource certification. |
| Import Tariffs (China) | 10.0% | 20.0% (Increased; (ECOTIS, 2025)) | Higher protection for local manufacturers. |

### The GIADEC Framework and Value Addition Incentives

The Ghana Integrated Aluminium Development Corporation (GIADEC), established under Act 976 of 2018, was created to manage Ghana’s interests across the full aluminium value chain. The GIADEC Master Plan explicitly identifies the production of aluminium-based chemicals, including PAC, as a priority downstream product for value addition to Ghanaian bauxite. By locating a PAC plant within the GIADEC industrial framework, this project qualifies for a range of fiscal incentives, including preferential utility tariffs, import duty exemptions on capital equipment and accelerated depreciation allowances.

Furthermore, the GIADEC framework creates a pathway for the proposed plant to secure a long-term offtake agreement with the GWCL, providing the revenue certainty required to underpin project financing. The integration of this PAC facility into the national aluminium development strategy represents not merely a single industrial investment, but a foundational step in building a diversified, high-value minerals processing sector.

### Regional Demand for PAC in West Africa

The market for PAC extends well beyond Ghana’s borders. Across West Africa, rapid urbanisation, deteriorating raw water quality, and the expansion of formal water supply infrastructure are driving substantial and growing demand for advanced coagulants. Ghana’s strategic location at the economic heart of the ECOWAS region, its relatively well-developed port infrastructure at Takoradi and Tema, and its improving investment climate position it as a natural hub for PAC manufacturing to serve the sub-regional market.

Table 1-2 Estimated Annual PAC Demand in Selected West African Countries. Sources: WHO (2023); ECOTIS (2025); industry estimates.

| Country | Population (millions, 2024) | Urban Access to Safe Water (%) | Estimated Annual PAC Demand (tonnes) | Primary Import Source |
| --- | --- | --- | --- | --- |
| Ghana | 34.1 | 78% | 4,000 – 6,000 | China, Europe |
| Côte d’Ivoire | 28.2 | 68% | 3,500 – 5,000 | France, China |
| Burkina Faso | 23.3 | 49% | 1,500 – 2,500 | China |
| Nigeria | 224.0 | 72% | 35,000 – 50,000 | China, Europe |
| Togo | 9.3 | 56% | 800 – 1,200 | China |
| Benin | 13.7 | 55% | 900 – 1,400 | China |
| Senegal | 17.8 | 76% | 1,500 – 2,500 | France, China |
| Sub-Saharan Total (est.) | >400 | ≈66% | >80,000 | Predominantly China |

The data in table above indicate a combined sub-regional PAC demand exceeding 80,000 tonnes per annum, virtually all of which is currently imported from China and Europe. Even capturing a modest 5–10% market share in the ECOWAS region represents a commercially significant opportunity. Nigeria alone, with its 224 million population and substantial GWCL-equivalent utility infrastructure, represents a market for PAC an order of magnitude larger than Ghana’s domestic requirement. The proposed plant, once its domestic supply obligations are met, is strategically positioned to serve this regional export demand.

# LITERATURE REVIEW

## OVERVIEW OF COAGULATION IN WATER TREATMENT

Coagulation is the physicochemical process by which colloidal and finely suspended particles in water are destabilised and subsequently made to aggregate into settleable or filterable flocs. Natural surface waters contain a complex matrix of impurities, including inorganic particles (clay minerals, silica), humic and fulvic acids, algal cells, and microbial matter. Most of these species carry a net negative surface charge at neutral pH, typically −15 to −30 mV, which generates an electrostatic repulsion known as the electrical double layer (Derjaguin–Landau–Verwey–Overbeek, or DLVO, theory). This repulsive energy barrier prevents particle aggregation despite the attractive Van der Waals forces that exist between particles at short range(Daud et al., 2009).

Coagulation proceeds through two distinct but complementary mechanisms. The first is charge neutralisation, in which positively charged metal hydroxo species from the coagulant adsorb onto the surface of negatively charged colloids, compressing the electrical double layer and reducing the zeta potential to near zero. At this point, the repulsive barrier collapses, and particles may approach closely enough for the Van der Waals attraction to initiate aggregation. The second is sweep flocculation, in which an amorphous metal hydroxide precipitate forms and physically enmeshes colloidal particles as it settles through the suspension. In practice, both mechanisms operate simultaneously; the dominant pathway is determined by coagulant type, dosage, pH, and water chemistry.

The coagulant dose must be carefully optimised. Under-dosing leaves the zeta potential insufficiently neutralised, while over-dosing can re-stabilise colloids through charge reversal, leading to re-dispersion. This optimal dosage window is considerably wider for PAC than for simple metal salts, a property of significant practical value in treating raw water of fluctuating quality.

Following coagulation, flocculation is induced by gentle mixing, which promotes orthokinetic collisions between destabilised particles. The resulting flocs are removed by sedimentation and rapid sand or multi-media filtration. In conventional plants treating moderately turbid water, this sequence reliably produces water meeting WHO turbidity guidelines (<1 NTU). However, at the extreme turbidity levels associated with Galamsey-contaminated rivers in Ghana, the performance of conventional alum coagulation degrades markedly, necessitating the use of high-performance coagulants such as PAC.

### Classification of Coagulants and Technical Comparison

Coagulants used in water and wastewater treatment are broadly classified into three principal categories: inorganic metal salt coagulants, inorganic polymeric coagulants, and natural or synthetic organic polymer coagulants (Sillanpää et al., 2018; Kumar et al., 2022). Each class operates through distinct physicochemical mechanisms and carries characteristic performance advantages and limitations that determine its suitability for specific water quality conditions and treatment objectives.

(a) Inorganic Metal Salt Coagulants

The most widely applied inorganic metal salt coagulants are aluminium sulphate (alum, Al₂(SO₄)₃·nH₂O) and the ferric salts — ferric chloride (FeCl₃) and ferric sulphate (Fe₂(SO₄)₃). Alum has historically dominated the global water treatment market due to its low unit cost, well-established supply chains, and extensive documentation of its chemistry (Daud et al., 2009). When dissolved in water, aluminium sulphate hydrolyses to form a series of cationic aluminium hydroxide species and the neutral precipitate Al(OH)₃, which acts as the sweep-floc matrix:

Al₂(SO₄)₃ + 6H₂O → 2Al(OH)₃↓ + 3H₂SO₄

Alum is effective over a narrow pH window of 6.0–7.5; outside this range, aluminium solubility increases markedly, and residual Al3+ concentrations in treated water exceed regulatory limits (Oregon Health Authority, 2010). The acid released during hydrolysis depresses the alkalinity of the treated water, requiring pH correction with lime. Alum also produces large volumes of light, gelatinous, high-water-content sludge that is difficult to dewater, significantly increasing sludge management costs. At the extreme turbidity levels now regularly recorded in Ghana’s Galamsey-impacted river basins the required alum doses become impractically large, the pH depression severe, and the resulting sludge volumes unmanageable for treatment plants designed for conventional alum operation (Nti et al., 2021).

Ferric salts offer a broader effective pH range (5.0–9.0) than alum and produce denser, faster-settling flocs that dewater more efficiently (Alliance Chemical, 2025). Ferric chloride and ferric sulphate are particularly effective for removing natural organic matter (NOM), phosphorus, and dissolved heavy metals, and they maintain coagulation activity in cold, low-alkalinity waters where aluminium-based coagulants often fail (Qin et al., 2006). However, ferric salts are highly corrosive, can impart a reddish-brown colour to treated water at elevated doses, and release iron into the treated water if overdosed or if pH is not carefully controlled. In waters with high natural colour or high NOM content, the colour-contributing effect of iron can be problematic.

(b) Natural and Synthetic Organic Polymer Coagulants

Natural coagulants, derived principally from plant materials such as Moringa oleifera seed extract, cactus mucilage (Opuntia ficus-indica), and tannin-based polymers, have attracted growing research interest as low-cost, biodegradable alternatives for use in rural and low-income settings (Kumar et al., 2022). Their active coagulating components are typically proteins and polysaccharides that function as bridging agents, promoting floc formation by adsorbing polymer chains onto particle surfaces. They produce minimal sludge, are non-toxic, and are readily available in tropical countries.

However, natural coagulants suffer from significant limitations that preclude their use in large-scale municipal treatment plants: their coagulation performance is variable and heavily dependent on source material quality, season, storage conditions, and water matrix chemistry; they introduce biodegradable organic matter (dissolved organic carbon) into treated water, which can promote microbial regrowth in the distribution network; and their efficacy at the extreme turbidity levels found in Galamsey-impacted Ghanaian river basins is unproven and likely insufficient (Kumar et al., 2022; Sillanpää et al., 2018). Synthetic organic polymer flocculants such as polyacrylamide (PAM) are effective coagulant aids but carry residual monomer toxicity concerns and are therefore subject to strict dose limits for drinking water applications.

(c) Inorganic Polymeric Coagulants — Poly-aluminium Chloride

Inorganic polymeric coagulants (IPCs) represent the most significant technological advance in water treatment chemistry. They are pre-polymerised, high-charge-density polynuclear metal hydroxo complexes that are stable in concentrated solution and exert immediate and powerful charge neutralisation upon dosing, without the need for in-situ hydrolysis. PAC is the pre-eminent IPC, and its superior performance across a wide range of water quality conditions is the central justification for its selection as the target product of this design project (Wang et al., 2004; Zouboulis and Tzoupanos, 2010).

Table 2.1 below presents a systematic technical comparison of the principal coagulant classes against the performance criteria most relevant to the Ghanaian water treatment context.

| Performance Criterion | Alum (Al₂(SO₄)₃) | Ferric Salts (FeCl₃/Fe₂(SO₄)₃) | Natural Coagulants (e.g. Moringa) | PAC (pre-polymerised) |
| --- | --- | --- | --- | --- |
| Effective pH Range | 6.0–7.5 (narrow) | 5.0–9.0 (broad) | 6.0–8.0 (variable) | 5.0–8.5 (broad) |
| Required Dosage | High | Moderate–High | High; variable | 30–50% lower than alum |
| Sludge Volume | Large; difficult to dewater | Moderate; dewaters well | Low; compact | 30–50% less than alum |
| pH Depression | Significant | Moderate–High | Minimal | Minimal (pre-hydrolysed) |
| Performance at High Turbidity (>1,000 NTU) | Poor; fails at extreme loads | Moderate | Poor; inconsistent | Excellent; robust |
| Residual in Treated Water | Elevated Al³⁺ risk | Iron colour risk if overdosed | Organic DOC addition | Low residual Al³⁺ |
| Cold Water Performance | Degrades at <5°C | Good | Variable | Good; stable |
| Unit Cost | Lowest | Moderate | Low (local sources) | Moderate; justified by dosage savings |
| Suitability for Ghanaian Galamsey-Impacted Water | Inadequate | Partial | Inadequate for large plants | Optimal |

Figure 2-1 Technical Comparison of Principal Coagulant Classes for Water Treatment

Sources: Wang et al. (2004); Zouboulis and Tzoupanos (2010); Kumar et al. (2022); Nti et al. (2021); Alliance Chemical (2025)

### Advantages of PAC over Conventional Coagulants

The superiority of is well-established in the water treatment literature and is rooted in fundamental differences in speciation, charge density, and reaction kinetics (Wang et al., 2004; Zouboulis and Tzoupanos, 2010). The following specific advantages are most relevant to the operational context of water treatment in Ghana.

First, PAC is a pre-polymerised coagulant. When alum is dosed into raw water, it must first undergo a time-dependent hydrolysis sequence before the active polynuclear hydroxo species capable of charge neutralisation are formed. This hydrolysis is sensitive to temperature and pH, and the process is slow in cold or low-alkalinity water. PAC, by contrast, contains pre-formed polynuclear species that are already active and immediately exert charge neutralisation upon addition to raw water, without any lag period (Wang et al., 2004). This confers a kinetic advantage that is particularly significant during high-turbidity flood events when rapid clarification is critical.

Second, the charge density of PAC is substantially higher than that of alum. The Al₁₃ polycation carries a charge of +7, compared to the +3 charge of the monomeric Al³⁺ in alum. This 2.3-fold higher charge-to-aluminium ratio means that a given mass of aluminium in PAC has approximately 2.3 times the charge-neutralisation capacity of the same mass in alum (Zouboulis and Tzoupanos, 2010). This directly translates into the lower dosage requirement observed in field studies, including the Barekese Water Treatment Plant study by Nti et al. (2021).

Third, because PAC is pre-hydrolysed, its addition to water generates less free acid than alum and consequently causes significantly less depression of treated water pH. This means that either no pH-correction agent (lime or soda ash) or a reduced dose is required, reducing chemical consumption costs. The lower alkalinity consumption of PAC compared to alum is quantified as approximately 0.17 mg/L as CaCO₃ per mg/L PAC dose (at 10% Al₂O₃ and 40% basicity), compared to 0.24 mg/L as CaCO₃ per mg/L for an equivalent alum dose (Simmonds Bristow, 2020).

Fourth, PAC generates less sludge than alum. The pre-polymerised structure of PAC means that less precipitate forms per unit of aluminium added, and the resulting flocs are denser and more amenable to sedimentation and dewatering than the light, gelatinous flocs. Al(OH)₃ floc produced by alum hydrolysis. Field data consistently show a 30–50% reduction in sludge volume with PAC relative to alum at equivalent treatment performance (Nti et al., 2021). For a large utility such as the Ghana Water Company Limited (GWCL), which manages sludge disposal as a major operational cost, this reduction represents a significant financial saving.

Fifth, PAC maintains effective coagulation performance over a wider pH range (5.0–8.5) than alum (6.0–7.5) and is more tolerant of natural organic matter (NOM) and high-colour waters. This is particularly important in the Ghanaian context: rivers contaminated by Galamsey activities often carry elevated NOM, fulvic acids, and humic substances derived from disturbed riparian vegetation and organic soils, in addition to mineral turbidity. PAC’s broad pH tolerance and strong NOM removal capacity make it robust against these variable and extreme water quality conditions.

Sixth, the higher charge density and pre-polymerised state of PAC result in lower residual aluminium concentrations in treated water compared to alum, even at the same or higher aluminium dose rates. Residual aluminium in drinking water is a human health concern and elevated residual aluminium from alum overdosing has been associated with concerns regarding neurotoxicity (Alzheimer’s disease risk), though the epidemiological evidence remains debated. PAC’s lower residual improves compliance with drinking water safety standards (Kumar et al., 2022).

## CHEMISTRY AND STRUCTURE OF POLYALUMINIUM CHLORIDE (PAC)

Poly-aluminium Chloride is an inorganic polymeric coagulant of the general formula Alₙ(OH)ₘCl₃ₙ₋ₘ, where n and m are integers defining the degree of polymerisation and hydroxylation, respectively. The key distinction between PAC and simple aluminium salts, such as alum, lies in the fact that PAC contains pre-formed polynuclear hydroxo-aluminium species that are stable in concentrated solution prior to dosing. Upon addition to raw water, these species immediately exert strong charge neutralisation without the need for the hydrolysis step that alum must first undergo in situ. (Wang et al., 2004).

The most important and most thoroughly characterised polynuclear species in PAC is the Al₁₃ Keggin polycation, [Al₁₃O₄(OH)₂₄(H₂O)₁₂]⁺⁷. This species consists of a central tetrahedral AlO₄ unit surrounded by twelve octahedrally coordinated aluminium atoms arranged in four groups of three (trimers), with the whole assembly stabilised by µ₂-hydroxide and µ₃-oxide bridging ligands. The charge of +7 confers exceptional capacity for the charge neutralisation of negatively charged colloidal surfaces, far exceeding the +3 charge of the simple Al³⁺ monomer present in alum solutions. (Zouboulis & Tzoupanos, 2010).

Figure 2-2 Ball and Stick model of the Keggin polycation

The basicity of PAC, defined as the molar ratio of hydroxide to aluminium, is the principal quality parameter governing the speciation and performance of the product. At low basicity (below 40%), the solution contains monomeric and low-oligomeric species, like those formed from alum; coagulation performance is not significantly superior to that of alum.

B =OH-Al3+

At the optimal industrial basicity of 65–75% (OH/Al = 2.0–2.4), the Al₁₃ Keggin polycation dominates the speciation, conferring superior coagulation efficiency. At excessively high basicity (above 85%), uncontrolled oxolation drives the Keggin ions to aggregate into insoluble, amorphous. Al(OH)₃ gel, permanently destroying the product’s coagulant activity (Zouboulis & Tzoupanos, 2010).

The chloride counter-ion plays a specific and key role in PAC chemistry, distinguishing it from poly-aluminium sulphate (PAS) and poly-aluminium phosphate variants. According to Hard-Soft Acid-Base (HSAB) theory, Al³⁺ is a hard Lewis acid with an electronic configuration of [Ne] (i.e., 1s² 2s² 2p⁶), devoid of d-electrons and thus with zero Crystal Field Stabilisation Energy. As a hard base, chloride occupies the outer coordination sphere through weak electrostatic interactions, providing charge balance without forming the strong inner-sphere covalent bonds that would structurally rigidify the polymer and prevent the desired polymeric network architecture from forming. Sulphate, being a harder and more strongly coordinating anion, can bridge between aluminium centres, promoting over-condensation and precipitation at lower basicity values. This is why HCl is the preferred lixiviant for PAC production rather than H2SO4 (Wang et al., 2004).

Compared with PAS (poly-aluminium sulphate), PAC displays superior performance in treating waters with high natural organic matter (NOM) content, a characteristic feature of West African tropical surface waters. PAS is used in some applications but is more likely to cause residual sulphate in treated water, which can adversely affect taste and may pose regulatory issues. For the Ghanaian context, PAC is clearly the preferred product specification.

## RAW MATERIAL CHARACTERISATION: AWASO BAUXITE

Producing advanced coagulants such as PAC depends entirely on the mineralogical and geochemical suitability of the feedstock. The Awaso bauxite deposit, located in the Bibiani-Anhwiaso-Bekwai District of the Western-North Region, is Ghana’s premier commercially exploited bauxite resource. The deposit is operated by the Ghana Bauxite Company (GBC) and holds an estimated 60 million metric tonnes of high-quality trihydrate bauxite with a high alumina (Al₂O₃) content of approximately 52 wt% and a low silica (SiO₂) content of approximately 1.5 wt% (Ghana Bauxite Company Ltd., 2026). GBC has been in continuous operation since the 1940s and currently exports approximately two million tonnes per annum through the Takoradi Port (Penresa, 2025). The shareholders are Ofori-Poku Company Limited (80%) and the Ghanaian government (20%), following the acquisition of the Chinese Bosai Minerals Group’s stake in 2021 (Ghana Bauxite Company Ltd., 2026).

Geologically, the Awaso deposit is a classic lateritic bauxite formation developed on metasediments of the Birimian supergroup. Intense tropical weathering over geological timescales has driven the extensive leaching of soluble alkaline-earth and silicate minerals, leaving a residuum highly enriched in stable aluminium and iron oxides. Investigations by (Momade & Gawu, 1998) reveal that the deposit is predominantly gibbsitic (α-Al(OH)₃), with subordinate boehmite (γ-AlO(OH)) and diaspore (α-AlO(OH)) in the deeper zones. This mineralogical character has critical implications for process design.

Figure 2-3 The Geological and Mineral Resource map of Ghana, highlighting the regional distribution of bauxite deposits.

Table 2-1 Profile Stratigraphy and mineralogical distribution of minerals at the Awaso bauxite deposit

| Profile Zone | Primary Minerals | Al2O3​ (wt %) | SiO2​ (wt %) | Fe2O3​ (wt %) |
| --- | --- | --- | --- | --- |
| Upper Bauxite | Gibbsite, Hematite | 45.0 - 60.0 | 2.0 - 5.0 | 10.0 - 25.0 |
| Middle Bauxite | Gibbsite, Goethite | 40.0 - 50.0 | 5.0 - 10.0 | 15.0 - 30.0 |
| Lower Lithomargic Clay | Kaolinite, Gibbsite | 30.0 - 40.0 | 15.0 - 35.0 | 5.0 - 15.0 |

From a chemical engineering standpoint, the dominance of gibbsite provides a decisive thermodynamic advantage. Gibbsite dissolves readily in hydrochloric acid at atmospheric pressure and temperatures of 90–110°C. This is because its hydroxyl-rich lattice (Al(OH)₃ structure) is highly susceptible to proton attack; the Al-O bonds are relatively labile compared to the structurally denser boehmitic and diasporic phases (Al-O in a double-layer octahedral framework), which requires high-pressure autoclaving at 140–280°C to dissolve. The ability to operate at atmospheric pressure in conventional glass-lined or rubber-lined steel reactors represents a substantial capital cost saving over pressure-vessel-based alternatives.

However, the ore also contains significant gangue minerals, primarily kaolinite (Al₂Si₂O₅(OH)₄), iron oxides (haematite, α-Fe₂O₃, and goethite, α-FeO(OH)), and titanium dioxide (rutile/anatase, TiO₂). The presence of reactive silica is particularly problematic: dissolved silicic acid can polymerise to form gelatinous silica gel, which severely impairs solid-liquid separation by blinding filter media and hydro-cyclone nozzles, and which can co-precipitate with aluminium, compromising coagulant purity. The iron content presents an equally significant technical challenge: haematite and goethite are partially soluble in hydrochloric acid, releasing Fe³⁺ ions that form FeCl₃ in solution. While GB 15892-2009 (General Administration of Quality Supervision & Standardisation Administration of China (SAC), 2009) does not set an explicit iron content limit, excessive iron dissolution lowers the effective Al₂O₃ content, discolours the product, and introduces potentially harmful iron chloride residuals into treated water. Industry practice targets Fe below approximately 0.5 wt% in the product (normalised to 10% Al₂O₃ basis), while the raw ore contains 10–25 wt% Fe₂O₃, making upstream iron removal the single most critical beneficiation objective.

Two primary beneficiation strategies are under consideration for iron removal:

- Magnetic separation, which preferentially captures the paramagnetic haematite and goethite phases and reduces iron content from ~20 wt% to below 5 wt% while simultaneously upgrading the alumina grade (Ofori-Sarpong, 2014); and

- Reductive roasting at 870–950°C, which converts haematite to magnetite, enhancing the efficiency of subsequent magnetic separation.

While reductive roasting achieves higher iron removal efficiency, it is an energy-intensive pretreatment step that significantly increases operating (OPEX) and capital (CAPEX) costs. For the current design basis, single-stage magnetic separation followed by hydrocyclone classification is adopted as the preferred beneficiation strategy.

## COMMINUTION OF BAUXITE ORE

Comminution, the unit operation of size reduction, is the first and one of the most energy-intensive processing steps in the bauxite-to-PAC production route. Its purpose is to reduce the run-of-mine (ROM) ore from coarse lump size (typically 50–300 mm) to a fine powder (target top size. ≤3 mm) that presents sufficient surface area for rapid and complete dissolution during acid leaching.

The relationship between particle size and leaching efficiency is direct and fundamental: because the gibbsite-HCl dissolution reaction is surface-reaction controlled (shrinking core model; (Reddy et al., 1998), reducing the Sauter mean diameter of the feed particles proportionally increases the specific surface area available for proton attack. This directly reduces the required residence time and acid consumption in the downstream leaching reactor. The required product fineness is dictated by the kinetics of the acid dissolution reaction and is strictly maintained by the closed-circuit comminution design.

### Comminution Theory: Energy–Size Relationships

Three classical laws relate the specific energy input (kWh/tonne) to the degree of size reduction achieved in comminution equipment. These laws are empirical in origin but are widely validated and remain the industry standard for sizing crushers and mills.

Kick’s Law (1885) proposes that the energy required for comminution is proportional to the ratio of the feed size to the product size and is therefore independent of the absolute particle size. It is expressed as:

W=CK⋅lnf80p80

where W is the specific energy input (kWh/t), CK is Kick’s constant (material-dependent), f80 is the feed 80% passing size, and p80 is the product 80% passing size. Kick’s Law is most accurate for coarse primary crushing (particles greater than approximately 50 mm) and for size ratios exceeding 8:1, where the energy is dominated by crack propagation through large mineral grains.

Bond’s Law (1952), the most widely used relationship in mineral processing, defines a specific energy requirement based on the Bond Work Index (Wi), a material-specific parameter measured empirically. Bond’s Third Theory states that the energy required to reduce a material is proportional to the square root of the surface area generated:

W=10Wi1p80-1f80

Bond’s Law is most accurate in the intermediate size range (0.05–50 mm). Published Bond Work Index values for tropical lateritic bauxite typically range from 8 to 14 kWh/short ton, depending on the degree of lateritisation, moisture content, and clay mineral content (Napier-Munn et al., 1999). Based on the relatively low silica and moderate clay content of the Awaso gibbsitic bauxite, a Bond Work Index (WiA value of 11.0 kWh/short ton was established as the primary design assumption to anchor the power requirements of the modelled milling circuit.

Rittinger’s Law (1867) proposes that energy is proportional to the new surface area created, making it most applicable to fine grinding (below approximately. 75 μm). It is not the dominant descriptor for the current design circuit, as the target product fineness is primarily optimised within the Bond operating regime.

### Primary Crushing: Jaw Crushers

Jaw crushers are the universally applied primary crushers for hard and moderately abrasive ores and are well suited to Awaso bauxite. While gibbsite is relatively soft (Mohs hardness ~2.5–3.5), the ore exhibits a competent lump structure that requires primary compressive breakage before wet processing.

Within the computational process model, the primary jaw crusher is governed by the closed-side setting (CSS) and modelled using standard empirical performance functions to predict the breakage of ROM bauxite. The ROM ore is fed from the top and progressively reduced by compressive fracture as material works its way downward towards the discharge opening.

Key design considerations for the primary jaw crusher include:

- Selection of jaw plate material resistant to abrasion from the iron-oxide gangue in the bauxite (e.g., manganese steel or chromium-molybdenum alloy).

- Dust suppression at the feed hopper and discharge chute, as dry bauxite crushing generates fine particulates presenting both health and explosion hazards.

- Provision of a vibrating grizzly screen ahead of the crusher to scalp out fine material that has already reached the CSS, thereby protecting the crusher chamber from packing with wet clay-rich material and improving overall mechanical efficiency.

### Secondary Comminution: Wet Ball Milling

To achieve the critical sub-3 mm target particle size distribution required for optimal digester performance, the primary crushed ore is routed to a continuous wet ball mill operating in closed circuit with a hydrocyclone classification system.

Ball milling relies on the cascading and cataracting action of heavy grinding media to reduce the intermediate bauxite particles via attrition and heavy impact forces. To satisfy the stringent purity requirements of a food-grade water treatment coagulant and prevent the introduction of parasitic, reactive metallic iron into the PAC matrix, the ball mill is specified to utilise high-alumina ceramic grinding media rather than forged steel balls, operating within a rubber-lined cylindrical shell.

For the process simulation, the continuous breakage kinetics within the ball mill were modelled to generate a mathematically predictable Particle Size Distribution (PSD). The product distribution exiting the mill is anchored by a Rosin-Rammler distribution model, a standard mathematical function utilised for modelling skewed fine-milling distributions:

Rd=exp-dd63.2n

where R(d)  is the mass fraction retained on a sieve of size? d, d63.2 is the absolute size parameter (representing the size at which 63.2% of the particles pass), and n is the distribution spread parameter.

By comparing the simulated mill power draw against the bauxite's Bond Work Index (11.0 kWh/short ton) and specifying the Rosin-Rammler parameters to restrict the top size, the Sauter mean diameter of the feed is mathematically minimised. This directly satisfies the quadratic sensitivity of the shrinking core model. (Reddy et al., 1998), yielding a disproportionately significant reduction in theoretical dissolution time and substantially lowering the required size of the downstream leaching reactor.

### Wet Classification: Hydro-cyclones

Following ball-mill comminution, the crushed ore slurry is continuously classified in a hydrocyclone operating in closed circuit with the mill. The hydro-cyclone performs a critical dual function: it separates and recycles oversize particles back to the ball mill via the underflow, while simultaneously desliming the finest clay-rich fraction, which is disproportionately high in reactive silica and iron oxides, via the overflow. This pre-leach classification aggressively reduces the parasitic iron and silica load entering the reactor, substantially improving leach selectivity and downstream filtration performance.

The hydro-cyclone geometry utilised in the process design is governed by the Rietema optimal geometry, which defines the set of dimensionless ratios that minimise the cut size (d50) for a given pressure drop (Rietema, 1961). The computational model relies on the following structural and operational parameters:

- Cyclone Body Diameter (Dc): 0.50 m

- Cylinder Length: ≈ 1.48 m.

- Optimal Aspect Ratio (LDc): 2.96

- Cone Half-Angle: 20°

The pressure-drop across the unit is maintained between 0.5 and 2.0 bar, generating the high centrifugal forces required for efficient phase separation. An inherent characteristic of hydro-cyclone operation is the carry-over of a fraction of the feed liquid to the underflow along with the oversize solids; this liquid split was rigorously accounted for in the overall steady-state water mass balance around the classification circuit.

Several operational advantages justify the integration of a hydro-cyclone rather than a vibrating screen:

- Hydro-cyclones contain no moving parts, ensuring exceptionally minimal maintenance requirements for continuous 24-hour operation.

- They are highly compact relative to their volumetric throughput capacity.

- They are insensitive to the highly variable moisture and sticky clay content of the Awaso ore, which would rapidly blind a static or vibrating fine screen.

- The underflow can be directly and continuously recycled to the ball mill, creating a highly stable and efficient closed-circuit grinding loop.

## KINETIC CONSIDERATIONS FOR ACID LEACHING

The extraction of aluminium from gibbsite bauxite with hydrochloric acid is a heterogeneous solid-liquid reaction governed by well-established kinetic principles. Because the process simulation utilises the Electrolyte Non-Random Two-Liquid with Redlich-Kwong (ENRTL-RK) thermodynamic property method, the complete dissociation of strong mineral acids is explicitly accounted for. Therefore, the hydronium ion (H3O+) is the active lixiviant attacking the solid mineral lattice, while chloride ions act as spectators in the bulk aqueous phase.

The overall molecular stoichiometry for the primary extraction is:

AlOH3 s+3HClaq→AlCl3aq+3H2O(l)

Represented in the rigorous electrolytic form utilised for the simulation framework:

AlOH3s+3H3Oaq+→Al3++6H2Ol

(Reddy et al., 1998) performed systematic kinetic studies on gibbsitic bauxite dissolution in hydrochloric acid and demonstrated that the process conforms to the shrinking core model (SCM) in which the rate-controlling step is the chemical reaction at the particle surface, rather than diffusion through a product layer or the bulk solution. This finding implies that rate enhancement is most effectively achieved by increasing temperature (which increases the rate constant) and decreasing particle size (which increases the specific surface area).

### Primary Reaction Kinetic Modelling (Power Law)

To accurately simulate the digestion process in a Continuous Stirred-Tank Reactor (CSTR) block, a macroscopic Power Law kinetic model was established. Because the stoichiometric requirement for HCl (at an 33 wt% concentration) is maintained at a high liquid-to-solid (L/S) ratio of 4.6:1 to 6:1 (L/kg), the proton activity remains in vast excess. Therefore, the reaction is modelled as pseudo-first order with respect to the solid mineral and zero-order with respect to the acid concentration, thereby preventing numerical instability while accurately reflecting the SCM surface-control regime.

The empirical rate equation for gibbsite dissolution is defined as:

rAl=k⋅AlOH31⋅H3O+0

Where the rate constant k follows the Arrhenius temperature dependence:

k=A⋅exp-EaRT

For the simulation basis, the activation energy (Ea) is set to 81,000 J/mol (81.0 kJ/mol) (Reddy et al., 1998) and the pre-exponential kinetic factor (A) is mathematically fixed at 3.0×107s-1 to accurately predict the target >90% extraction efficiency over a standard 2 to 4-hour reactor residence time at 110°C.

### Competing Iron Dissolution Kinetics

A critical competing reaction that must be accounted for is the partial dissolution of iron-bearing impurity minerals. Haematite (Fe2O3) reacts with hydrochloric acid according to the molecular equation:

Fe2O3s+6HClaq→2FeCl3aq+3H2Ol

Represented in the electrolytic form utilised for the simulation:

Fe2O3s+6H3O+→2Fe3++9H2Ol

These reactions consume acid and introduce parasitic. Fe3+ into the leach liquor. The Fe3+ must be removed downstream by neutralisation with CaCO3, which precipitates solid FeOH3 while generating CO2 gas as a by-product).

The kinetic parameters for this parasitic side-reaction are based on the foundational hydrometallurgical models established by (Majima et al., 1985) for the dissolution of iron oxides in aqueous acidic solutions. Hematite dissolution is notoriously stubborn, characterised by a lower activation energy but a drastically smaller pre-exponential factor compared to gibbsite, which dictates its slower leaching rate.

The empirical rate equation for hematite dissolution in the digester is modelled similarly as a pseudo-first-order surface reaction:

rFe=k⋅Fe2O31⋅H3O+0

For the simulation framework, the activation energy (Ea) for chemically controlled hematite dissolution is specified as 77,700 J/mol (77.7 kJ/mol), with a pre-exponential baseline factor (A) of 1.0×107 s-1 (Majima et al., 1985). This parameterisation successfully allows the Aspen Plus model to simultaneously calculate the rapid, high-yield extraction of aluminium alongside the slow, parasitic fractional leaching of iron.

## REVIEW OF PRODUCTION TECHNOLOGIES

### The Alkaline (Bayer) Route

The Bayer process is the principal industrial method for refining bauxite into high-purity alumina (Al₂O₃). Developed by Carl Josef Bayer in 1887–1892, the process involves the dissolution of bauxite in concentrated sodium hydroxide (NaOH) solution at temperatures of 140–280°C and elevated pressures of 3–5 bar, forming soluble sodium aluminate (NaAlO₂). After clarification, to remove the insoluble iron-rich residue (red mud), the sodium aluminate solution is seeded with Al(OH)₃ crystals and cooled, driving precipitation of pure aluminium trihydrate (gibbsite). The precipitate is then calcined at 950–1,200°C to produce smelter-grade alumina (Al₂O₃).

The Bayer process is indispensable for aluminium smelting, where the highest-purity alumina is required. It achieves almost complete rejection of silica, iron, and titanium impurities. However, for PAC production, the Bayer route introduces a critically wasteful intermediate step: the high-purity alumina must be re-dissolved in hydrochloric acid to generate the aluminium chloride solution, which is then subjected to controlled polymerisation. This “dissolve–calcine–re-dissolve” sequence is both energy-intensive and capital-intensive, requiring autoclave pressure vessels, NaOH recovery systems and high-temperature calciners that are economically redundant for a product (PAC) that does not require the extreme purity levels achieved by the Bayer process (Zouboulis & Tzoupanos, 2010).

The Bayer process also generates copious quantities of red mud (typically 1.0–1.5 tonnes per tonne of alumina produced), a strongly alkaline (pH 10–13) slurry containing iron oxides, titanium oxide, residual silica, and trace heavy metals. Red mud disposal represents one of the most challenging environmental liabilities in the aluminium industry globally, requiring engineered impoundment structures and active monitoring. The 2010 Ajkai Timföldgyár alumina plant disaster in Hungary, in which approximately one million cubic metres of red mud escaped from a storage reservoir, resulting in fatalities and widespread environmental contamination, illustrates the severity of this hazard. For a developing economy such as Ghana, the environmental and financial regulatory and management burden of managing Bayer red mud is a significant additional argument against adopting this route for PAC production.

The importance of the Bayer process should nonetheless be acknowledged within the broader GIADEC framework. If Ghana were to develop a fully integrated aluminium industry, including a smelter, the Bayer process would be indispensable for producing smelter-grade alumina. In that future scenario, the Bayer alumina plant could supply alumina as the feedstock for a PAC production unit using the re-dissolution route. However, for the current project, in the absence of an integrated aluminium smelter, the direct acid leaching route is unambiguously the more appropriate technology.

### Direct Acid Leaching Route

Direct acid leaching offers a streamlined synthesis route well suited to the production of liquid or solid coagulants. Raw or beneficiated bauxite is reacted directly with a mineral acid, in this case hydrochloric acid, at temperatures of 90–110°C and atmospheric pressure, dissolving the aluminium-bearing minerals and generating aluminium chloride (AlCl₃) solution. Following solid-liquid separation to remove insoluble iron oxide and silica residue (red mud), the filtrate is subjected to controlled basicity adjustment by careful addition of a base (calcium carbonate, sodium carbonate, or sodium aluminate), thereby inducing hydrolytic polymerisation to form the Al₁₃ Keggin polycation. The resulting PAC solution is then processed into the desired product form.

The superiority of hydrochloric acid over sulphuric acid as the lixiviant for PAC production is well established in the literature and is rooted in the coordination chemistry of the Al³⁺ ion. As discussed in Section 2.2, the chloride counter-ion preferentially occupies the outer coordination sphere, providing charge balance without forming the bridging bonds that would cause premature condensation of the polymeric network. Sulphuric acid, by contrast, can form inner-sphere sulphate bridges between aluminium centres, disrupting the controlled polymerisation and leading to precipitation at lower basicity values (Wang et al., 2004). The use of HCl also prevents sulphate carry-over into the product, a quality concern for drinking water coagulants.

Table 2-2  Comparative Evaluation of the Alkaline (Bayer) Route and the Direct Acid Leaching Route for PAC Production

| Criterion | Alkaline (Bayer) Route | Direct Acid Leaching Route |
| --- | --- | --- |
| Raw Material | Requires prior alumina (Al₂O₃) refining | Uses raw bauxite directly |
| Process Steps | Digestion → Precipitation → Calcination → Re-dissolution | Leaching → Solid-liquid separation → Polymerisation |
| Temperature | 140–280°C at high pressure | 90–110°C at atmospheric pressure |
| CAPEX/OPEX | Extremely high (pressure vessels, NaOH recovery) | Moderate (atmospheric reactors, HCl handling) |
| Impurity Rejection | Excellent (silica, iron removed as red mud) | Requires beneficiation upstream |
| Suitability for PAC | Redundant intermediate step; economically unviable | Directly suited; preferred for PAC production |
| Red Mud Generation | Significant (environmental burden) | Residue (red mud) is still generated from the Fe/Si gangue |

.

### The Polymerisation Process: Basicity Adjustment and the Role of the Al₁₃ Polycation

Following solid–liquid separation of the leach liquor, the clarified aluminium chloride (AlCl₃) solution has a basicity of approximately zero (no hydroxyl groups). The core chemical transformation that converts this solution into PAC is controlled basicity adjustment — the incremental addition of an alkaline reagent to raise the OH/Al molar ratio (basicity) to the target range, simultaneously inducing the formation of the polynuclear hydroxo-aluminium polymer species that define PAC’s superior coagulation activity.

#### Methods of Basicity Adjustment

Three alkaline reagents are principally used in industrial practice for basicity adjustment: calcium carbonate (CaCO₃), calcium aluminate (CaAl₂O₄ or Ca₃Al₂O₆), and sodium hydroxide (NaOH). Each has distinct chemical, economic, and process engineering characteristics that influence the choice of reagent for a specific plant design.

Calcium carbonate (CaCO₃) reacts with AlCl₃ solution according to the following stoichiometry:

2AlCl₃ + 3CaCO₃ + 3H₂O → 2Al(OH)₃ + 3CaCl₂ + 3CO₂↑

By controlling the stoichiometry to add only sufficient CaCO₃ to achieve the target basicity (OH/Al = 2.0–2.4, corresponding to 65–75%) rather than proceeding to complete Al(OH)₃ precipitation, the reaction is arrested at the polynuclear stage. The CO₂ by-product is released as a gas and must be managed by appropriate venting and scrubbing. CaCO₃ has a low unit cost, is widely available, and is safe and straightforward to handle. However, it introduces calcium ions (Ca²⁺) and chloride ions into the product solution as CaCl₂, and the CO₂ generation introduces gas handling complexity. The introduction of Ca²⁺ can also interfere with the Al₁₃ polymerisation equilibria at high concentrations (Wang et al., 2004).

Calcium aluminate (CaAl₂O₄) is the preferred basicity-adjusting agent for high-quality industrial PAC production. Its reaction with AlCl₃ proceeds without gas evolution, and, critically, it simultaneously contributes additional aluminium to the product, thereby improving aluminium utilisation efficiency from the bauxite feedstock and raising the product Al₂O₃ content. Zhang et al. (2009) demonstrated that adjusting basicity with calcium aluminate yielded PAC with basicity exceeding 90% and high stability, significantly outperforming the CaCO₃-adjusted product on both coagulation performance and long-term storage stability. The principal limitation of calcium aluminate is its higher unit cost relative to CaCO₃. Moreover, it must be sourced from a reliable industrial supplier.

Sodium hydroxide (NaOH) can also be used for basicity adjustment, reacting with AlCl₃ as follows:

AlCl₃ + 3NaOH → Al(OH)₃ + 3NaCl

NaOH-based basicity adjustment requires extremely precise dosage control, as the high reactivity and strong alkalinity of NaOH make it prone to generating localised zones of high pH near the injection point, causing irreversible precipitation of amorphous Al(OH)₃. The use of NaOH also introduces Na⁺ and Cl⁻ into the product without contributing any aluminium, reducing the effective aluminium content of the product per unit of reagent cost. While NaOH-adjusted PAC can achieve high basicity, the process is less forgiving than calcium aluminate-based adjustment and requires specialised membrane or microporous injection reactor designs to manage supersaturation. For this project, calcium aluminate is the selected basicity-adjusting agent, consistent with the established best practice identified in the literature and described in the Aspen Plus simulation framework.

##### The Role of the Al₁₃ Polycation in Coagulation Performance

The Al₁₃ Keggin polycation, Al13O4OH24H2O127+, is the species most directly responsible for PAC’s superior coagulation performance, and maximising its concentration in the product is the primary objective of the basicity adjustment and polymerisation process (Jiang & Graham, 1998; Wang et al., 2004). The Al₁₃ polycation has a characteristic ε-Keggin structure: a central AlO₄ tetrahedron surrounded by twelve octahedrally coordinated aluminium atoms arranged in four trimeric groups, the whole assembly bridged by μ₂-hydroxide and μ₃-oxide linkages. The resulting structure carries a charge of +7, conferring exceptional charge-neutralisation capacity that is unmatched by any monomeric or dimeric aluminium species.

Ferron time-resolved spectrophotometric analysis, a standard technique for quantifying aluminium species distribution in PAC solutions, classifies aluminium species into three fractions: Alₐ (monomeric, reacts with Ferron in <30 seconds), Alb (medium polymeric — principally Al₁₃ — reacts in 30 seconds to 2 hours), and Alb (slow-reacting colloidal/polymeric aggregates). The Alb fraction, dominated by Al₁₃, is the active coagulant species; the Alₐ fraction contributes minimally to charge neutralisation and may raise residual aluminium in treated water; and the Alc fraction, present at high basicities above ~85%, is partially insoluble and associated with reduced performance (Jiang & Graham, 1998; Wang et al., 2004).

Research consistently demonstrates that the maximum Al₁₃ concentration is achieved when basicity is maintained in the range of 65–85% (OH/Al = 1.5–2.5). Below 65%, the solution is dominated by monomeric and low-oligomeric Alₐ species; above 85%, uncontrolled oxolation drives the Keggin ions to aggregate into insoluble Alᶜ polymers (Bottero et al., 1980; Zouboulis & Tzoupanos, 2010). The coagulation performance advantage of high Al₁₃ content PAC is well-documented: Jiang and Graham (1998) reported that PAC with high Al₁₃ content achieved significantly higher turbidity removal than conventional PAC across the full dosage range studied and exhibited notably better UV₂₅₄ absorbance (NOM indicator) removal at higher doses.

The kinetics of Al₁₃ formation following basicity adjustment are temperature-dependent, with activation energies for the oxolation-polymerisation reaction estimated at approximately 45.5 kJ/mol (Wang et al., 2004). This lower activation energy compared to the primary acid leaching reaction confirms that, while polymerisation proceeds at moderate temperatures (60–80°C), the reaction requires substantial residence time (3–5 hours in a well-mixed reactor) to complete. This kinetic constraint informs the design of the maturation/polymerisation tank detailed in the process design section.

### Calcination: Effect of Temperature on Alumina Solubility

Calcination, or thermal activation, is a pre-treatment step applied to bauxite before acid leaching to improve the rate and efficiency of aluminium extraction. The underlying principle is that thermal dihydroxylation of crystalline aluminium hydroxide minerals generates reactive, disordered (amorphous or nanocrystalline) transition alumina phases with high specific surface area and structural disorder, which are substantially more susceptible to acid attack than the original crystalline minerals.

The thermal transformation sequence of gibbsite (Al(OH)₃) upon heating follows a well-characterised pathway. At 200–300°C, constitutional water is expelled as OH⁻ groups in a first de-hydroxylation stage, and gibbsite begins to transform to the poorly crystalline χ-Al₂O₃ phase ((Fysh & Clark, 1983) as cited in Vereshchagin et al., 2020). At 560–700°C, a second, more complete dehydroxylation stage occurs in which the residual hydroxyl groups are removed to form γ-Al₂O₃ (reactive transition alumina), metakaolinite from kaolinite impurities, and, for boehmite content, its characteristic dehydroxylation to γ-Al₂O₃. At temperatures above 950°C, γ-Al₂O₃ progressively transforms to the thermodynamically stable α-Al₂O₃ (corundum), which is highly refractory and extremely resistant to acid dissolution.

The range of 700–800°C has been specifically identified in the literature as the optimal calcination temperature window for maximising acid leaching efficiency from bauxite. Valeev et al. (2016) systematically investigated the calcination of high-silica boehmite-kaolinite bauxite. They demonstrated that pre-calcination at 700°C for 30 minutes resulted in a dramatic increase in HCl leaching efficiency — from 23% for uncalcined ore to 89% for calcined ore — under identical leaching conditions (20% HCl, 110°C). The authors attributed this improvement to two simultaneous effects: the dehydroxylation of gibbsite and boehmite to the reactive γ-Al₂O₃ phase, and the thermal conversion of kaolinite (Al₂Si₂O₅(OH)₄) to metakaolinite (Al₂Si₂O₇), both of which are far more readily attacked by HCl than their parent crystalline phases. Beyond 700°C, and particularly above 800°C, the onset of γ → δ → θ alumina phase transitions begin to reduce surface area and reactivity, with no further benefit to extraction efficiency (Smith, 1996 as cited in Vereshchagina et al., 2020).

The evolution of specific surface area during calcination is directly correlated with leaching performance. Flash calcination of gibbsite at 700°C has been shown to increase specific surface area from less than 1 m²/g in the raw crystalline material to approximately 278 m²/g in the calcined product. This ~280-fold increase in specific surface area directly translates into a proportional increase in the rate of acid dissolution, as the gibbsite–HCl reaction is surface-reaction controlled (Reddy et al., 1998).

For the Awaso gibbsite bauxite, with its high gibbsite content and low boehmite fraction, the calcination step is incorporated into the design at 700°C as an upstream activation stage within the rotary kiln, consistent with the design basis adopted in the process simulation. Crucially, the reduction in required residence time and reactor volume that calcination permits in the downstream leaching reactor more than offsets the energy cost of the calcination step at commercial scale, as demonstrated by the simulation results in the material and energy balance section. The reducing atmosphere (CO injection) simultaneously achieves partial reduction of Fe₂O₃ to Fe₃O₄ (magnetite), which is then removed by magnetic separation — an upstream iron beneficiation strategy that reduces parasitic iron dissolution in the leaching reactor.

# PROCESS SELECTION

## Process Route Selection

Figure 3-1 Process Flow Diagram

The Alkaline (Bayer) route, being the industrial method for refining bauxite into highly purified alumina (Al2O3), operates at high-pressure and highly caustic environments making it rather capital-intensive and utilises redundant processes as the alumina produced would have to be redissolved in acid again to produce our poly-aluminium chloride. On the other hand, the direct acid leaching approach operates at atmospheric pressure and at ~110oC making it the preferred process route.

For the direct acid leaching as chosen for this plant, the lixiviant preferred was hydrochloric acid, with the reason being that the chloride ion (Cl-) acts purely as a spectator in the outer coordination sphere of the aluminium ion. Other plants use sulphuric acid or a blend of hydrochloric and sulphuric acids. Our choice of hydrochloric acid is that the sulphate ions (SO42-) form stronger inner-sphere bridging bonds, causing premature precipitation, but chloride allows for the controlled and stable polymerisation of the aluminium chains.

### Feed Preparation and Beneficiation

The assay data from (Momade & Gawu, 1998) provided that the Awaso bauxite deposit has a substantial amount of haematite and if left untreated, this iron component consumes expensive acid and further degrades the final poly-aluminium chloride quality (violating the GB 15892-2009 standard).

The plant incorporates a reductive roasting (Magnetising reduction) feed pre-treatment stage to convert the paramagnetic haematite, α-Fe2O3 at 870-950oC to magnetite, Fe3O4 (Ofori-Sarpong, 2014). The reducing agent used by the plant is carbon monoxide.

Due to the high head and prolonged exposure, the Aluminium hydroxide content of the bauxite was converted to aluminium oxide, which is the more reactive form.

3Fe2O3+CO→2Fe3O4+CO2

2AlOH3→Al2O3+3H2O

The newly formed magnetite allows for efficient magnetic separation of the iron content, thereby reducing the parasitic acid consumption downstream and the reduction in the quality of the final product.

Heat from the hot roaster product was harvested using a heat exchanger to heat up high-pressure air to be used downstream, thereby cutting cost for heating other utility fluids.

Figure 3-2 Schematic representation of the magnetic separation of magnetite from the roasted ore.

### Milling and Slurry Formation

Process water is added to the high-alumina-content feed to form a slurry with the right viscosity, corresponding to about 60% water, for effective milling in a ball mill.

The wet slurry is fed into a ball mill, achieving particle sizes less than 245 μm (Reddy et al., 1998). The milled product is fed into a hydrocyclone to ensure that the slurry that proceeds to the next stage has the required particle size.

A recirculation loop was set up that passes oversize from the hydrocyclone back to the mixing tank for re-milling.

### Digester Kinetics and Thermodynamics

To gain full control of the polymer formation stage, aluminium is to be separated from the alumina present in the slurry by leaching using hydrochloric acid as the lixiviant. Commercial grade hydrochloric acid (32 wt% HCl) is dosed into the leaching tank to maintain a highly acidic pH (< 1.0) with 18 – 24 % HCl to drive the proton-promoted dissolution of the alumina phase and prevent premature hydrolysis (Reddy et al., 1998).

For effective leaching, the leaching tank is to be maintained at a temperature around 120oC and 3 atm pressure and a residence time of around 2 to 4 hours. The acid and the slurry were heated to 80oC using heat recovered from the products from the Roaster.

Residual haematite from the roasting stage also consumes some of the acid to leach out iron (III) ions into the slurry, which contributes to a reddish-brown colour formation.

The surface-reaction-controlled dissolution reaction was modelled using the Shrinking Core Model with a power-law architecture.

### Solid-Liquid Separation

With the active component of the poly-aluminium chloride dissolved in the slurry, the undissolved components (mostly quartz, un-separated magnetite, and unreacted alumina) are removed from the slurry by passing through a filter press.

### Speciation Control via Basicity (OH/Al Ratio)

Following solid-liquid separation, the clarified aluminium chloride liquor proceeds to the basicity adjustment and polymerisation stage. The operating temperature for this stage was chosen as 70oC: sufficiently high to maintain kinetic viability of the condensation reactions, but below the threshold (~85°C) at which irreversible over-condensation of the Al₁₃ Keggin polycation begins. A residence time of 3–5 hours in a well-mixed CSTR is required to achieve complete basicity stabilisation (Wang et al., 2004).

The basicity adjustment step in PAC synthesis involves the controlled addition of a base (calcium carbonate, sodium aluminate and sodium carbonate are the principal industrial options) to the partially neutralised aluminium chloride (AlCl₃) solution to promote the formation of hydrolysed aluminium species while avoiding complete precipitation. For this process, calcium aluminate (CaAl2O4) was used as the basifying agent, offering both a hydroxyl generation and supplementary aluminium species, which enhances polymer formation efficiency.

The initial dissolution step in the acidic medium proceeds as follows:

8H3O+(aq)+CaAl2O4s→12H2O(aq)+Ca2+aq+2Al3+aq

Following dissolution, the released aluminium ions, and residual solid CaAl₂O₄ participate in controlled hydrolysis reactions that generate aluminium hydroxide monomers.

For aluminium-only systems:

3CaAl2O4s+2Al3+aq+12H2Oaq→3Ca2+aq+8AlOH3aq

In systems containing ferric impurities, which are common in bauxite-derived feeds, there is a controlled generation of amorphous AlOH3, which serves as the fundamental precursor to subsequent polymerisation. Importantly, under carefully regulated conditions (pH, temperature, and residence time), these hydroxide species remain in a reactive, non-crystalline, and partially soluble form, enabling further condensation rather than precipitation. For such feeds, the reaction occurs as follows:

CaAl2O4+2Fe3++12H2O→3Ca2++6AlOH3+2FeOH3

The hydrolysed aluminium monomers subsequently undergo olation and oxolation reactions, leading to the formation of polynuclear aluminium species. The dominant pathway yields the well-known Keggin-type Al₁₃ polycation, which is primarily responsible for PAC coagulation efficiency in the following reaction:

7Al3++32AlOH3+21Cl-→3Al13O4OH24Cl7

This step represents the core polymerisation mechanism, where monomeric and oligomeric aluminium species condense into highly charged, stable polycations. The presence of chloride ions ensures charge balance and stabilisation of the polymer structure in solution.

The reaction must be arrested at partial neutralisation, targeting a molar basicity ratio:

Basicity=OH-Al3+≈2.0-2.4

corresponding to approximately 65–75% basicity, which is optimal for PAC performance. Excess basification drives the system toward:

Al3++3OH-→AlOH3(s)

This results in an irreversible precipitation and loss of coagulant activity.

### Ageing (Maturation) of PAC

Following the basicity adjustment and initial polymerisation, the Poly-Aluminium Chloride (PAC) liquor undergoes a critical maturation (ageing) phase. This stage is not merely a holding period but a kinetically driven unit operation where metastable, low-molecular-weight aluminium monomers and dimers are transformed into high-performance polynuclear species, primarily the ε-Al13 Keggin polycation (Al13O4OH24H2O127+).

#### Stoichiometry and Structural Evolution

During maturation, structural reorganisation occurs through ligand exchange and oxidation. In the process simulation, this is modelled as a transformation involving the interaction of residual monomeric aluminium, chloride ions, and the freshly formed hydroxylated intermediates. The overall stoichiometric representation established for the maturation block is:

7Al3++21Cl-+32AlOH3s+24H2Ol→3[Al13O4OH24Cl7⋅nH2O]

This reaction represents the "fixing" of the aluminium into the stable polymeric framework. The transition from simple monomeric hydrolysis products to the Al13 polycation is governed by the basicity ratio (B = [OH]/Al). Research indicates that the highest concentration of Al13 is achieved when B is maintained between 1.5 and 2.2 (Wang et al., 2004). At these ratios, the "Al13 species becomes the dominant coagulant agent, providing the high positive charge density (7+) required for efficient destabilisation of negatively charged colloidal particles in raw water.

1.1.5.2 Maturation Kinetics and Power Law Modelling

The transformation of metastable species into the Keggin architecture is a time-dependent process. To satisfy the requirements of the Continuous Stirred-Tank Reactor (CSTR) model used for the maturation tank, a Power Law kinetic expression was utilised.

The rate of PAC formation is defined as:

rPAC=k⋅Al3+a⋅AlOH3b

For the simulation basis, the reaction is modelled as second order overall. The activation energy (Ea) for the oxolation and polymerisation of aluminium hydroxides is significantly lower than that of the primary acid leaching, estimated at 45.5 kJ/mol based on the temperature sensitivity studies of PAC stability (Wang et al., 2004). This lower activation energy confirms that while the reaction proceeds at moderate temperatures (60-80∘C), it requires significant residence time to reach equilibrium.

#### Process Parameter Justification

The maturation tank is designed to operate at 75∘C with a residence time of 3 to 4.5 hours. These parameters are justified by two competing thermodynamic factors:

- Rate of Oxolation: At temperatures below 60∘C, the rate of formation for the Al13 species is prohibitively slow, leading to a product dominated by less effective monomers (Ala) and small oligomers (Alb).

- Thermal Degradation: Operating above 90∘C for extended periods triggers the degradation of the Keggin ion into larger, less soluble Al-aggregates and eventually amorphous AlOH3 precipitate, which significantly reduces the active "Alumina" content of the final liquid PAC (Bottero et al., 1980).

1.1.5.4 Equipment Configuration

The maturation tank serves as a unit distinct from the polymerisation reactor to ensure a plug-flow-like age distribution. The tank is equipped with:

- A low-shear agitator: Designed to maintain suspension of any micro-precipitates without providing enough shear energy to break the developing polymeric chains.

- Thermal Control: An external heating jacket maintains the 75∘C setpoint. The simulation results for stream 18 confirm that the enthalpy required to maintain this temperature is minimal, as the polymerisation reaction is slightly exothermic (Bottero et al., 1980)

- Corrosion Resistance: Given the high chloride activity and moderate temperature, the tank internals are specified as G-30 nickel-based alloy or PTFE-lined carbon steel to prevent metal leaching, which would compromise the purity of the PAC.

## Quality Standards for PAC

The production of PAC for use in municipal drinking water treatment must comply with recognised quality standards. In the absence of a current Ghanaian national standard for PAC (a standard development opportunity that GIADEC and the Ghana Standards Authority could address), the applicable international reference standards are the Chinese National Standard GB 15892-2009 (the most widely applied standard globally given China’s dominance of the PAC market), the British Standard BS EN 883:2021 (for water treatment chemicals) and the AWWA Standard B408 (American Water Works Association).

The GB 15892-2009 standard is adopted as the primary quality benchmark for this design. Issued jointly by China’s General Administration of Quality Supervision, Inspection and Quarantine (AQSIQ) and the Standardisation Administration (SAC) on 8 April 2009 (implemented 1 September 2009), this standard superseded GB 15892-2003 and is the most widely referenced PAC quality standard globally, given China’s dominant position in PAC production and trade. Critically, GB 15892-2009 does not differentiate between a “Grade I” (drinking water) and a “Grade II” (industrial) classification. It specifies a single set of compulsory indices applicable to all PAC for drinking water treatment, with separate limit columns for liquid PAC and solid PAC products. This distinction is important: the standard covers both product forms within a unified quality framework rather than through a tiered grading system. Table 2-iii below reproduces the compulsory technical indices from Table 1 of GB 15892-2009.

Several important observations arise from examining GB 15892-2009 in detail. First, the standard does not set an explicit limit on iron (Fe) content. Iron control is instead implicit in the product appearance specification (colourless to yellowish-brown liquid; white to yellowish-brown solid) and in the Al₂O₃ purity requirement: excessive iron dissolution from the ore dilutes the effective aluminium content and discolours the product, making iron removal an indirect but critical quality constraint. Industry practice and analogous standards (such as ANSI/AWWA B408) generally target Fe below 0.5 wt% in the liquid product, equivalent to 10% Al₂O₃; for the Awaso bauxite process, this remains the most technically demanding constraint and is the primary driver for the upstream iron beneficiation and the downstream CaAl2O4 neutralisation steps. Second, the liquid PAC specification requires a minimum of 10.0 wt% Al₂O₃ for liquid PAC. Third, the heavy metal limits for arsenic (≤0.0002 wt%), lead (≤0.001 wt%), cadmium (≤0.0002 wt%), mercury (≤0.00001 wt%), and hexavalent chromium (≤0.0005 wt%) are stringent and apply to both liquid and solid grades. In the Ghanaian context, given that Awaso bauxite contains trace heavy metals, particularly arsenic from hydrothermal mineralisation and lead from weathering profiles, these limits require analytical characterisation of leachate composition during the detailed design phase. The standard further specifies that all heavy metal limits for the liquid product are normalised to a 10% Al₂O₃ basis, meaning that if the actual Al₂O₃ content exceeds 10%, the impurity concentrations must be proportionally corrected. This normalisation provision is important for product characterisation and for the development of quality control protocols.

# MATERIAL AND ENERGY BALANCE

## Material Balance

The Poly-Aluminium Chloride (PAC) production plant is designed to process a feed equivalent to 6,600 tonnes per annum (TPA) of PAC product, at a plant attainment of 90%. The material balance was simulated using Aspen Plus version 14.2, which provided the property data for all components and the thermodynamic framework required to resolve reaction and phase equilibria across the flowsheet.

The base property method adopted was the Electrolyte Non-Random Two-Liquid (ELECNRTL) activity-coefficient model, coupled with the Redlich–Kwong equation of state for the vapour phase. This combination is well suited to the present plant design, which involves the concurrent handling of solids, vapours, and electrolytic aqueous slurries across the roasting, leaching, and precipitation/polymerisation circuits.

Owing to the limitation of the standard Aspen Plus databank in representing polymeric species, the Poly-Aluminium Chloride product was manually specified as a conventional component. The formula adopted for PAC in this project is the hydrated form of the Al₁₃ Keggin ion, Al13O4OH24H2O127+. The polymer's electrolytic properties were not separately accounted for, owing to the scarcity of published data for this species.

### Assumptions

#### Thermodynamic and Property Method

It was assumed that the ELECNRTL property method, together with the Redlich–Kwong equation of state, accurately predicts the highly non-ideal behaviour of the system, including the ionic speciation and activity coefficients of the complex aqueous electrolyte systems, across the temperature range encountered in the process.

The dissolution of trace gases into the aqueous phase was assumed to strictly obey Henry's Law, with the corresponding binary interaction parameters adequately extrapolated from the native Aspen Plus databanks.

Because Poly-Aluminium Chloride is a highly variable inorganic polymer rather than a single defined molecule, it was modelled using a surrogate stoichiometric empirical formula, and its thermophysical properties were assumed to approximate those of the base aluminium chloride hydrates for the purposes of the material and energy balance.

#### Reaction Engineering and Kinetics

In the roasting/calcination stage (RK-201), the thermal conversion of bauxite was simulated using a stoichiometric reactor model, assuming negligible heat- and mass-transfer resistance within the solid bauxite matrix, with the specified conversions of Al(OH)₃ and Fe₂O₃ achieved without recourse to a thermodynamic-equilibrium deadband.

Perfect macro- and micro-mixing was assumed in the acid leaching reactor (R-601). The leaching reaction was assumed to be governed by Power Law rate expressions, with diffusion limitations arising from shrinking-core effects at the particle scale neglected.

The precipitation reactor (R-701) was modelled assuming instantaneous, complete neutralisation of residual acidity by the dosed CaAl2O4 reagent, with AlOH3 precipitation proceeding to the extents dictated by the specified reaction stoichiometry and solution electroneutrality. The PAC polymerisation reactor (R-702) was modelled as a stoichiometric reactor in which AlOH3 and residual dissolved aluminium species condense to the surrogate Al13 Keggin-ion formula at a specified conversion, with polymerisation kinetics not explicitly resolved.

### Unit Operation Material Balance Descriptions

#### FD-101 — Feed Bin / Storage Hopper

Unit Type: Bin / Storage Vessel	Inlet streams: 101    Outlet streams: 102

Run-of-mine (ROM) bauxite ore reporting from the mine boundary is received and surge-stored in the feed bin FD-101 ahead of size reduction. The unit performs no chemical transformation and provides residence time to decouple upstream ore delivery from downstream crushing.

| Stream 101 |  |  |
| --- | --- | --- |
| Component | Mass Flow (kg/hr) | Mass Fraction |
| Al(OH)₃ | 451.350 | 0.5970 |
| Fe₂O₃ | 11.997 | 0.0159 |
| SiO₂ | 265.500 | 0.3512 |
| TiO₂ | 21.633 | 0.0286 |
| K₂O | 4.032 | 0.0053 |
| Na₂O | 1.180 | 0.0016 |
| MgO | 0.295 | 0.0004 |
| Total | 755.987 | 1.0000 |
| Temperature (°C) = 32.00 Pressure (atm) = 1.0000 |  |  |

| Stream 102 |  |  |
| --- | --- | --- |
|  | Mass Flow (kg/hr) | Mass Fraction |
| Al(OH)₃ | 451.350 | 0.5970 |
| Fe₂O₃ | 11.997 | 0.0159 |
| SiO₂ | 265.500 | 0.3512 |
| TiO₂ | 21.633 | 0.0286 |
| K₂O | 4.032 | 0.0053 |
| Na₂O | 1.180 | 0.0016 |
| MgO | 0.295 | 0.0004 |
| Total | 755.987 | 1.0000 |
| Temperature (°C) = 32.00 Pressure (atm) = 1.0000 |  |  |

#### CRK-201 — Jaw Crusher

Unit Type: Size-Reduction Unit	Inlet streams: 102	Outlet streams: 103

The ROM ore (102) is reduced in particle size by the jaw crusher CRK-201 to a top size suitable for the crushed-ore bin. Size reduction is a purely mechanical/physical operation; no mass is created, destroyed, or chemically converted across the unit.

| Stream 102 |  |  |
| --- | --- | --- |
|  | Mass Flow (kg/hr) | Mass Fraction |
| Al(OH)₃ | 451.350 | 0.5970 |
| Fe₂O₃ | 11.997 | 0.0159 |
| SiO₂ | 265.500 | 0.3512 |
| TiO₂ | 21.633 | 0.0286 |
| K₂O | 4.032 | 0.0053 |
| Na₂O | 1.180 | 0.0016 |
| MgO | 0.295 | 0.0004 |
| Total | 755.987 | 1.0000 |
| Temperature (°C) = 32.00 Pressure (atm) = 1.0000 |  |  |

| Stream 103 |  |  |
| --- | --- | --- |
|  | Mass Flow (kg/hr) | Mass Fraction |
| Al(OH)₃ | 451.350 | 0.5970 |
| Fe₂O₃ | 11.997 | 0.0159 |
| SiO₂ | 265.500 | 0.3512 |
| TiO₂ | 21.633 | 0.0286 |
| K₂O | 4.032 | 0.0053 |
| Na₂O | 1.180 | 0.0016 |
| MgO | 0.295 | 0.0004 |
| Total | 755.987 | 1.0000 |
| Temperature (°C) = 32.20 Pressure (atm) = 1.0000 |  |  |

#### BN-101 — Crushed Ore Bin

Unit Type: Bin / Storage Vessel	Inlet streams: 103    Outlet streams: 104

The crushed ore (103) discharged from CRK-201 is held in the intermediate storage bin BN-101, which buffers the crushing circuit against the batch-wise thermal demand of the rotary kiln RK-201. Composition and temperature are effectively unchanged across the bin.

| Stream 103 |  |  |
| --- | --- | --- |
|  | Mass Flow (kg/hr) | Mass Fraction |
| Al(OH)₃ | 451.350 | 0.5970 |
| Fe₂O₃ | 11.997 | 0.0159 |
| SiO₂ | 265.500 | 0.3512 |
| TiO₂ | 21.633 | 0.0286 |
| K₂O | 4.032 | 0.0053 |
| Na₂O | 1.180 | 0.0016 |
| MgO | 0.295 | 0.0004 |
| Total | 755.987 | 1.0000 |
| Temperature (°C) = 32.20 Pressure (atm) = 1.0000 |  |  |

| Stream 104 |  |  |
| --- | --- | --- |
|  | Mass Flow (kg/hr) | Mass Fraction |
| Al(OH)₃ | 451.350 | 0.5970 |
| Fe₂O₃ | 11.997 | 0.0159 |
| SiO₂ | 265.500 | 0.3512 |
| TiO₂ | 21.633 | 0.0286 |
| K₂O | 4.032 | 0.0053 |
| Na₂O | 1.180 | 0.0016 |
| MgO | 0.295 | 0.0004 |
| Total | 755.987 | 1.0000 |
| Temperature (°C) = 32.20 Pressure (atm) = 1.0000 |  |  |

#### E,BR-201 — Air Pre-heater / Burner

Unit Type: Fired Heater	Inlet streams: 201    Outlet streams: 202

Ambient combustion air (201) drawn from the plant boundary is raised in temperature across the air pre-heater/burner E,BR-201 prior to injection into the rotary kiln. The duty is supplied by combustion of the reducing gas (CO) admitted to the kiln; the air stream itself undergoes only a physical (sensible-heat) change.

Inlet stream condition and composition:

| Stream 201 |  |  |
| --- | --- | --- |
|  | Mass Flow (kg/hr) | Mass Fraction |
| CO₂ | 174.267 | 0.1781 |
| CO | 297.447 | 0.3040 |
| O₂ | 133.620 | 0.1366 |
| N₂ | 373.066 | 0.3813 |
| Total | 978.400 | 1.0000 |
| Temperature (°C) = 32.00 Pressure (atm) = 1.0000 |  |  |

| Stream 202 |  |  |
| --- | --- | --- |
|  | Mass Flow (kg/hr) | Mass Fraction |
| CO₂ | 174.267 | 0.1781 |
| CO | 297.447 | 0.3040 |
| O₂ | 133.620 | 0.1366 |
| N₂ | 373.066 | 0.3813 |
| Total | 978.400 | 1.0000 |
| Temperature (°C) = 700.00 Pressure (atm) = 1.1000 |  |  |

#### RK-201 — Rotary Kiln (Roaster/Calciner)

Unit Type: Reactor	Inlet streams: 104, 202    Outlet streams: 203, 301

| Stream: 104. Component; Mass, kg/hr; Mass, %. Al(OH)₃; 451.35; 59.70. Fe₂O₃; 12.00; 1.59. SiO₂; 265.50; 35.12. TiO₂; 21.63; 2.86. K₂O; 4.03; 0.53. Na₂O; 1.18; 0.16. MgO; 0.30; 0.04. Total; 755.99; 100.00. Temperature: 32.2 °C Pressure: 1.0132 bar. |  | Stream: 202. Component; Mass, kg/hr; Mass, %. CO₂; 174.267; 0.1781. CO; 297.447; 0.3040. O₂; 133.620; 0.1366. N₂; 373.066; 0.3813. Total; 978.400; 1.0000. Temperature: 32.0 °C Pressure: 1.0132 bar. |
| --- | --- | --- |

| Stream: 203. Component; Mass, kg/hr; Mass, %. Al(OH)₃; 8.332; 0.0150. Fe₂O₃; 0.554; 0.0010. SiO₂; 245.066; 0.4408. TiO₂; 19.968; 0.0359. K₂O; 3.721; 0.0067. Na₂O; 1.089; 0.0020. MgO; 0.272; 0.0005. Fe₃O₄; 10.168; 0.0183. Al₂O₃; 266.839; 0.4799. Total; 556.010; 1.0000. Temperature: 870.0 °C Pressure: 1.0132 bar. |  | Stream: 301. Component; Mass, kg/hr; Mass, %. H₂O; 153.235; 0.1300. CO₂; 542.865; 0.4607. Al(OH)₃; 0.695; 0.0006. Fe₂O₃; 0.046; 0.0000. SiO₂; 20.434; 0.0173. TiO₂; 1.665; 0.0014. K₂O; 0.310; 0.0003. Na₂O; 0.091; 0.0001. MgO; 0.023; 0.0000. CO; 62.849; 0.0533. Fe₃O₄; 0.848; 0.0007. Al₂O₃; 22.250; 0.0189. N₂; 373.066; 0.3166. Total; 1,178.377; 1.0000. Temperature: 700.0 °C Pressure: 1.0132 bar. |
| --- | --- | --- |

#### CL-201 — Product Cooler

Unit Type: Heat Exchanger		Inlet streams: 203    Outlet streams: 204

The hot calcined solid (203) leaving the kiln is cooled in the product cooler CL-201 prior to magnetic separation. Composition is unchanged across the unit; only sensible heat is removed to a cooling utility.

Inlet stream condition and composition:

| Stream 203 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| Al(OH)₃ | 8.332 | 0.0150 |
| Fe₂O₃ | 0.554 | 0.0010 |
| SiO₂ | 245.066 | 0.4408 |
| TiO₂ | 19.968 | 0.0359 |
| K₂O | 3.721 | 0.0067 |
| Na₂O | 1.089 | 0.0020 |
| MgO | 0.272 | 0.0005 |
| Fe₃O₄ | 10.168 | 0.0183 |
| Al₂O₃ | 266.839 | 0.4799 |
| Total | 556.010 | 1.0000 |
| Temperature (°C) | 870.00 |  |
| Pressure (atm) | 1.0000 |  |

Outlet stream condition and composition:

| Stream 204 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| Al(OH)₃ | 8.332 | 0.0150 |
| Fe₂O₃ | 0.554 | 0.0010 |
| SiO₂ | 245.066 | 0.4408 |
| TiO₂ | 19.968 | 0.0359 |
| K₂O | 3.721 | 0.0067 |
| Na₂O | 1.089 | 0.0020 |
| MgO | 0.272 | 0.0005 |
| Fe₃O₄ | 10.168 | 0.0183 |
| Al₂O₃ | 266.839 | 0.4799 |
| Total | 556.010 | 1.0000 |
| Temperature (°C) | 32.00 |  |
| Pressure (atm) | 1.0000 |  |

#### E,CL-301 — Off-gas Cooler

Unit Type: Heat Exchanger	Inlet streams: 301    Outlet streams: 303B, CONDENSA

The kiln off-gas (301) is cooled in the off-gas cooler E,CL-301, condensing part of its water content. The condensate is withdrawn as stream CONDENSA, while the remaining, cooled gas phase proceeds as stream 303B to the cyclone separator. Composition changes as H₂O partitions between the vapour (303B) and condensate (CONDENSA) streams; the non-condensable gas species (CO₂, CO) remain in the gas phase.

Composition changes across the unit; components gained across the unit: H₃O⁺, OH⁻.

Inlet stream condition and composition:

| Stream 301 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 153.235 | 0.1300 |
| CO₂ | 542.865 | 0.4607 |
| Al(OH)₃ | 0.695 | 0.0006 |
| Fe₂O₃ | 0.046 | 0.0000 |
| SiO₂ | 20.434 | 0.0173 |
| TiO₂ | 1.665 | 0.0014 |
| K₂O | 0.310 | 0.0003 |
| Na₂O | 0.091 | 0.0001 |
| MgO | 0.023 | 0.0000 |
| CO | 62.849 | 0.0533 |
| Fe₃O₄ | 0.848 | 0.0007 |
| Al₂O₃ | 22.250 | 0.0189 |
| N₂ | 373.066 | 0.3166 |
| Total | 1,178.377 | 1.0000 |
| Temperature (°C) = 870.00 Pressure (atm) = 1.0000 |  |  |

Outlet stream condition and composition:

| Stream 303B | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 0.000 | 0.0000 |
| CO₂ | 542.865 | 0.5296 |
| Al(OH)₃ | 0.695 | 0.0007 |
| Fe₂O₃ | 0.046 | 0.0000 |
| SiO₂ | 20.434 | 0.0199 |
| TiO₂ | 1.665 | 0.0016 |
| K₂O | 0.310 | 0.0003 |
| Na₂O | 0.091 | 0.0001 |
| MgO | 0.023 | 0.0000 |
| CO | 62.849 | 0.0613 |
| Fe₃O₄ | 0.848 | 0.0008 |
| Al₂O₃ | 22.250 | 0.0217 |
| N₂ | 373.066 | 0.3639 |
| Total | 1,025.142 | 1.0000 |
| Temperature (°C) | 30.00 |  |
| Pressure (atm) | 1.0000 |  |

| Stream CONDENSA | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 153.234 | 1.0000 |
| H₃O⁺ | 0.000 | 0.0000 |
| OH⁻ | 0.000 | 0.0000 |
| Total | 153.234 | 1.0000 |
| Temperature (°C) | 30.00 |  |
| Pressure (atm) | 1.0000 |  |

#### CY-301 — Cyclone Separator

Unit Type: Gas-Solid Separator      Inlet streams: 303B    Outlet streams: 304, CY-DUST

The cooled off-gas (303B) enters the cyclone separator CY-301, where entrained particulate matter is removed by inertial separation and withdrawn as the cyclone dust product (CY-DUST). The clarified gas (304) proceeds to the gas compressor/fan. The operation is a physical solids-gas separation; no component is chemically converted.

Inlet stream condition and composition:

| Stream 303B | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 0.000 | 0.0000 |
| CO₂ | 542.865 | 0.5296 |
| Al(OH)₃ | 0.695 | 0.0007 |
| Fe₂O₃ | 0.046 | 0.0000 |
| SiO₂ | 20.434 | 0.0199 |
| TiO₂ | 1.665 | 0.0016 |
| K₂O | 0.310 | 0.0003 |
| Na₂O | 0.091 | 0.0001 |
| MgO | 0.023 | 0.0000 |
| CO | 62.849 | 0.0613 |
| Fe₃O₄ | 0.848 | 0.0008 |
| Al₂O₃ | 22.250 | 0.0217 |
| N₂ | 373.066 | 0.3639 |
| Total | 1,025.142 | 1.0000 |
| Temperature (°C) | 30.00 |  |
| Pressure (atm) | 1.0000 |  |

Outlet stream condition and composition:

| Stream 304 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 0.000 | 0.0000 |
| CO₂ | 2.571 | 0.2773 |
| Al(OH)₃ | 0.069 | 0.0075 |
| Fe₂O₃ | 0.005 | 0.0005 |
| SiO₂ | 2.043 | 0.2204 |
| TiO₂ | 0.166 | 0.0180 |
| K₂O | 0.031 | 0.0033 |
| Na₂O | 0.009 | 0.0010 |
| MgO | 0.002 | 0.0002 |
| CO | 0.298 | 0.0321 |
| Fe₃O₄ | 0.085 | 0.0091 |
| Al₂O₃ | 2.225 | 0.2400 |
| N₂ | 1.767 | 0.1906 |
| Total | 9.272 | 1.0000 |
| Temperature (°C) | 30.00 |  |
| Pressure (atm) | 1.2000 |  |

| Stream CY-DUST | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 0.000 | 0.0000 |
| CO₂ | 540.294 | 0.5319 |
| Al(OH)₃ | 0.625 | 0.0006 |
| Fe₂O₃ | 0.042 | 0.0000 |
| SiO₂ | 18.391 | 0.0181 |
| TiO₂ | 1.499 | 0.0015 |
| K₂O | 0.279 | 0.0003 |
| Na₂O | 0.082 | 0.0001 |
| MgO | 0.020 | 0.0000 |
| CO | 62.551 | 0.0616 |
| Fe₃O₄ | 0.763 | 0.0008 |
| Al₂O₃ | 20.025 | 0.0197 |
| N₂ | 371.299 | 0.3655 |
| Total | 1,015.870 | 1.0000 |
| Temperature (°C) | 30.00 |  |
| Pressure (atm) | 1.2000 |  |

#### CP-301 — Gas Compressor / Fan

Unit Type: Rotating Equipment (Compressor)

Inlet streams: 304    Outlet streams: 304B

The clarified kiln off-gas (304) is conveyed by the gas compressor/fan CP-301 to overcome downstream pressure drop before entering the baghouse filter. Composition is unchanged across the unit; a small amount of mechanical work is imparted to the gas stream.

Inlet stream condition and composition:

| Stream 304 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 0.000 | 0.0000 |
| CO₂ | 2.571 | 0.2773 |
| Al(OH)₃ | 0.069 | 0.0075 |
| Fe₂O₃ | 0.005 | 0.0005 |
| SiO₂ | 2.043 | 0.2204 |
| TiO₂ | 0.166 | 0.0180 |
| K₂O | 0.031 | 0.0033 |
| Na₂O | 0.009 | 0.0010 |
| MgO | 0.002 | 0.0002 |
| CO | 0.298 | 0.0321 |
| Fe₃O₄ | 0.085 | 0.0091 |
| Al₂O₃ | 2.225 | 0.2400 |
| N₂ | 1.767 | 0.1906 |
| Total | 9.272 | 1.0000 |
| Temperature (°C) | 30.00 |  |
| Pressure (atm) | 1.2000 |  |

Outlet stream condition and composition:

| Stream 304B | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 0.000 | 0.0000 |
| CO₂ | 2.571 | 0.2773 |
| Al(OH)₃ | 0.069 | 0.0075 |
| Fe₂O₃ | 0.005 | 0.0005 |
| SiO₂ | 2.043 | 0.2204 |
| TiO₂ | 0.166 | 0.0180 |
| K₂O | 0.031 | 0.0033 |
| Na₂O | 0.009 | 0.0010 |
| MgO | 0.002 | 0.0002 |
| CO | 0.298 | 0.0321 |
| Fe₃O₄ | 0.085 | 0.0091 |
| Al₂O₃ | 2.225 | 0.2400 |
| N₂ | 1.767 | 0.1906 |
| Total | 9.272 | 1.0000 |
| Temperature (°C) | 117.52 |  |
| Pressure (atm) | 5.0000 |  |

#### BH-101 — Baghouse Filter

Unit Type: G-S Separator	Inlet streams: 304B    Outlet streams: BH-AIR, BH-DUST

The compressed off-gas (304B) is polished in the baghouse filter BH-101, which removes residual fine particulate via fabric filtration. Clean air is discharged to atmosphere as BH-AIR, while the collected fines report as the baghouse dust product, BH-DUST. The operation is a physical gas-solid separation with no chemical conversion.

The component identity of the process stream(s) is unchanged across the unit; only physical conditions (temperature, pressure, and/or phase distribution) or stream routing are altered.

Inlet stream condition and composition:

| Stream 304B | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 0.000 | 0.0000 |
| CO₂ | 2.571 | 0.2773 |
| Al(OH)₃ | 0.069 | 0.0075 |
| Fe₂O₃ | 0.005 | 0.0005 |
| SiO₂ | 2.043 | 0.2204 |
| TiO₂ | 0.166 | 0.0180 |
| K₂O | 0.031 | 0.0033 |
| Na₂O | 0.009 | 0.0010 |
| MgO | 0.002 | 0.0002 |
| CO | 0.298 | 0.0321 |
| Fe₃O₄ | 0.085 | 0.0091 |
| Al₂O₃ | 2.225 | 0.2400 |
| N₂ | 1.767 | 0.1906 |
| Total | 9.272 | 1.0000 |
| Temperature (°C) | 117.52 |  |
| Pressure (atm) | 5.0000 |  |

Outlet stream condition and composition:

| Stream BH-AIR | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 0.000 | 0.0000 |
| CO₂ | 2.571 | 0.5546 |
| CO | 0.298 | 0.0642 |
| N₂ | 1.767 | 0.3812 |
| Total | 4.636 | 1.0000 |
| Temperature (°C) | 117.52 |  |
| Pressure (atm) | 4.9975 |  |

| Stream BH-DUST | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| Al(OH)₃ | 0.069 | 0.0150 |
| Fe₂O₃ | 0.005 | 0.0010 |
| SiO₂ | 2.043 | 0.4408 |
| TiO₂ | 0.166 | 0.0359 |
| K₂O | 0.031 | 0.0067 |
| Na₂O | 0.009 | 0.0020 |
| MgO | 0.002 | 0.0005 |
| Fe₃O₄ | 0.085 | 0.0183 |
| Al₂O₃ | 2.225 | 0.4799 |
| Total | 4.636 | 1.0000 |
| Temperature (°C) | 117.52 |  |
| Pressure (atm) | 4.9975 |  |

#### MS-401 — Magnetic Separator

Unit Type: Magnetic Separator

Inlet streams: 204    Outlet streams: 401, FE3O4

The cooled, calcined ore (204) is processed through the magnetic separator MS-401. The magnetically susceptible fraction, rich in the newly formed Fe₃O₄, is diverted as the magnetic reject stream FE3O4 and removed at the plant boundary, while the non-magnetic, alumina-silica-rich fraction (401) proceeds to the milling circuit. Composition therefore changes markedly, with Fe₃O₄ and part of the iron oxides concentrated in the reject.

Inlet stream condition and composition:

| Stream 204 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| Al(OH)₃ | 8.332 | 0.0150 |
| Fe₂O₃ | 0.554 | 0.0010 |
| SiO₂ | 245.066 | 0.4408 |
| TiO₂ | 19.968 | 0.0359 |
| K₂O | 3.721 | 0.0067 |
| Na₂O | 1.089 | 0.0020 |
| MgO | 0.272 | 0.0005 |
| Fe₃O₄ | 10.168 | 0.0183 |
| Al₂O₃ | 266.839 | 0.4799 |
| Total | 556.010 | 1.0000 |
| Temperature (°C) | 32.00 |  |
| Pressure (atm) | 1.0000 |  |

Outlet stream condition and composition:

| Stream 401 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| Fe₂O₃ | 0.526 | 0.0010 |
| SiO₂ | 240.165 | 0.4777 |
| Fe₃O₄ | 0.508 | 0.0010 |
| Al₂O₃ | 261.502 | 0.5202 |
| Total | 502.701 | 1.0000 |
| Temperature (°C) | 32.00 |  |
| Pressure (atm) | 1.0000 |  |

| Stream FE3O4 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| Al(OH)₃ | 8.332 | 0.1563 |
| Fe₂O₃ | 0.028 | 0.0005 |
| SiO₂ | 4.901 | 0.0919 |
| TiO₂ | 19.968 | 0.3746 |
| K₂O | 3.721 | 0.0698 |
| Na₂O | 1.089 | 0.0204 |
| MgO | 0.272 | 0.0051 |
| Fe₃O₄ | 9.660 | 0.1812 |
| Al₂O₃ | 5.337 | 0.1001 |
| Total | 53.309 | 1.0000 |
| Temperature (°C) | 32.00 |  |
| Pressure (atm) | 1.0000 |  |

#### P-501 — Process Water Feed Pump

Unit Type: Pump	Inlet streams: PW-1    Outlet streams: PW-2

Fresh process water (PW-1), admitted at the plant boundary, is pressurised by the process water feed pump P-501 for delivery to the slurry conditioning tank TK-501. Composition and temperature are essentially unchanged across the pumping step; only the stream pressure is increased.

Inlet stream condition and composition:

| Stream PW-1 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 833.333 | 1.0000 |
| H₃O⁺ | 0.000 | 0.0000 |
| OH⁻ | 0.000 | 0.0000 |
| Total | 833.333 | 1.0000 |
| Temperature (°C) | 32.00 |  |
| Pressure (atm) | 1.0000 |  |

Outlet stream condition and composition:

| Stream PW-2 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 833.333 | 1.0000 |
| H₃O⁺ | 0.000 | 0.0000 |
| OH⁻ | 0.000 | 0.0000 |
| Total | 833.333 | 1.0000 |
| Temperature (°C) | 32.01 |  |
| Pressure (atm) | 1.5000 |  |

#### TK-501 — Slurry Conditioning Tank

Unit Type: Tank / Mixer	Inlet streams: 501, PW-2, 506B    Outlet streams: 502

The mineral solids (501), fresh process water (PW-2), and the hydrocyclone underflow recycle (506B) are combined in the slurry conditioning tank TK-501 to form the conditioned mill-feed slurry, stream 502. The operation is a mixing step; the total mass and component inventory of stream 502 is the algebraic sum of the three inlet streams, and the resulting slurry temperature reflects the weighted thermal contribution of each feed.

The component identity of the process stream(s) is unchanged across the unit; only physical conditions (temperature, pressure, and/or phase distribution) or stream routing are altered.

Inlet stream condition and composition:

| Stream 501 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| Fe₂O₃ | 0.526 | 0.0010 |
| SiO₂ | 240.165 | 0.4777 |
| Fe₃O₄ | 0.508 | 0.0010 |
| Al₂O₃ | 261.502 | 0.5202 |
| Total | 502.701 | 1.0000 |
| Temperature (°C) | 32.00 |  |
| Pressure (atm) | 1.0000 |  |
| Stream PW-2 | Mass Flow (kg/hr) | Mass Fraction |
| H₂O | 833.333 | 1.0000 |
| H₃O⁺ | 0.000 | 0.0000 |
| OH⁻ | 0.000 | 0.0000 |
| Total | 833.333 | 1.0000 |
| Temperature (°C) | 32.01 |  |
| Pressure (atm) | 1.5000 |  |
| Stream 506B | Mass Flow (kg/hr) | Mass Fraction |
| H₂O | 427.298 | 0.6235 |
| Fe₂O₃ | 0.270 | 0.0004 |
| SiO₂ | 123.284 | 0.1799 |
| H₃O⁺ | 0.000 | 0.0000 |
| OH⁻ | 0.000 | 0.0000 |
| Fe₃O₄ | 0.261 | 0.0004 |
| Al₂O₃ | 134.237 | 0.1959 |
| Total | 685.351 | 1.0000 |
| Temperature (°C) | 32.30 |  |
| Pressure (atm) | 1.9738 |  |

Outlet stream condition and composition:

| Stream 502 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 1,260.632 | 0.6236 |
| Fe₂O₃ | 0.796 | 0.0004 |
| SiO₂ | 363.449 | 0.1798 |
| H₃O⁺ | 0.000 | 0.0000 |
| OH⁻ | 0.000 | 0.0000 |
| Fe₃O₄ | 0.769 | 0.0004 |
| Al₂O₃ | 395.739 | 0.1958 |
| Total | 2,021.385 | 1.0000 |
| Temperature (°C) | 32.12 |  |
| Pressure (atm) | 1.0000 |  |

#### P-502 — Slurry Feed Pump

Unit Type: Pump	Inlet streams: 502    Outlet streams: 503

The conditioned slurry (502) is pumped from TK-501 to the ball mill by the slurry feed pump P-502. Composition and temperature are essentially unchanged across this pumping step; only the stream pressure is increased to overcome the mill circuit head.

The component identity of the process stream(s) is unchanged across the unit; only physical conditions (temperature, pressure, and/or phase distribution) or stream routing are altered.

Inlet stream condition and composition:

| Stream 502 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 1,260.632 | 0.6236 |
| Fe₂O₃ | 0.796 | 0.0004 |
| SiO₂ | 363.449 | 0.1798 |
| H₃O⁺ | 0.000 | 0.0000 |
| OH⁻ | 0.000 | 0.0000 |
| Fe₃O₄ | 0.769 | 0.0004 |
| Al₂O₃ | 395.739 | 0.1958 |
| Total | 2,021.385 | 1.0000 |
| Temperature (°C) | 32.12 |  |
| Pressure (atm) | 1.0000 |  |

Outlet stream condition and composition:

| Stream 503 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 1,260.632 | 0.6236 |
| Fe₂O₃ | 0.796 | 0.0004 |
| SiO₂ | 363.449 | 0.1798 |
| H₃O⁺ | 0.000 | 0.0000 |
| OH⁻ | 0.000 | 0.0000 |
| Fe₃O₄ | 0.769 | 0.0004 |
| Al₂O₃ | 395.739 | 0.1958 |
| Total | 2,021.385 | 1.0000 |
| Temperature (°C) | 32.12 |  |
| Pressure (atm) | 1.1000 |  |

#### ML--501 — Ball Mill

Unit Type: Mechanical Size-Reduction Unit	    Inlet streams: 503    Outlet streams: 504

The slurry (503) undergoes wet grinding in the ball mill ML--501 to achieve the particle-size distribution required for efficient downstream leaching. Grinding is a physical size-reduction operation; the mass composition of the mill discharge (504) is identical to that of the mill feed, with only a marginal temperature rise reflecting mechanical energy dissipation.

Inlet stream condition and composition:

| Stream 503 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 1,260.632 | 0.6236 |
| Fe₂O₃ | 0.796 | 0.0004 |
| SiO₂ | 363.449 | 0.1798 |
| H₃O⁺ | 0.000 | 0.0000 |
| OH⁻ | 0.000 | 0.0000 |
| Fe₃O₄ | 0.769 | 0.0004 |
| Al₂O₃ | 395.739 | 0.1958 |
| Total | 2,021.385 | 1.0000 |
| Temperature (°C) | 32.12 |  |
| Pressure (atm) | 1.1000 |  |

Outlet stream condition and composition:

| Stream 504 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 1,260.632 | 0.6236 |
| Fe₂O₃ | 0.796 | 0.0004 |
| SiO₂ | 363.449 | 0.1798 |
| H₃O⁺ | 0.000 | 0.0000 |
| OH⁻ | 0.000 | 0.0000 |
| Fe₃O₄ | 0.769 | 0.0004 |
| Al₂O₃ | 395.739 | 0.1958 |
| Total | 2,021.385 | 1.0000 |
| Temperature (°C) | 32.28 |  |
| Pressure (atm) | 1.1000 |  |

#### P-503 — Mill Discharge Pump

Unit Type: Pump	Inlet streams: 504    Outlet streams: 505

The ground slurry (504) is transferred from the ball mill to the hydrocyclone classifier by the mill discharge pump P-503. Composition and temperature are essentially unchanged across this pumping step.

Inlet stream condition and composition:

| Stream 504 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 1,260.632 | 0.6236 |
| Fe₂O₃ | 0.796 | 0.0004 |
| SiO₂ | 363.449 | 0.1798 |
| H₃O⁺ | 0.000 | 0.0000 |
| OH⁻ | 0.000 | 0.0000 |
| Fe₃O₄ | 0.769 | 0.0004 |
| Al₂O₃ | 395.739 | 0.1958 |
| Total | 2,021.385 | 1.0000 |
| Temperature (°C) | 32.28 |  |
| Pressure (atm) | 1.1000 |  |

Outlet stream condition and composition:

| Stream 505 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 1,260.632 | 0.6236 |
| Fe₂O₃ | 0.796 | 0.0004 |
| SiO₂ | 363.449 | 0.1798 |
| H₃O⁺ | 0.000 | 0.0000 |
| OH⁻ | 0.000 | 0.0000 |
| Fe₃O₄ | 0.769 | 0.0004 |
| Al₂O₃ | 395.739 | 0.1958 |
| Total | 2,021.385 | 1.0000 |
| Temperature (°C) | 32.30 |  |
| Pressure (atm) | 1.9738 |  |

1.1.2.18 HC-101 — Hydrocyclone Classifier

Unit Type: Classifier	Inlet streams: 505    Outlet streams: 506, 506B

The milled slurry (505) is classified in the hydrocyclone HC-101. The fine-particle overflow (506), meeting the target size specification, proceeds to the leach feed tank TK-601, while the coarse underflow (506B) is recycled back to the slurry conditioning tank TK-501 for further grinding. Composition differs between the two product streams as a function of particle size, though both retain the parent slurry's component identities.

Inlet stream condition and composition:

| Stream 505 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 1,260.632 | 0.6236 |
| Fe₂O₃ | 0.796 | 0.0004 |
| SiO₂ | 363.449 | 0.1798 |
| H₃O⁺ | 0.000 | 0.0000 |
| OH⁻ | 0.000 | 0.0000 |
| Fe₃O₄ | 0.769 | 0.0004 |
| Al₂O₃ | 395.739 | 0.1958 |
| Total | 2,021.385 | 1.0000 |
| Temperature (°C) | 32.30 |  |
| Pressure (atm) | 1.9738 |  |

Outlet stream condition and composition:

| Stream 506 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 833.324 | 0.6237 |
| Fe₂O₃ | 0.526 | 0.0004 |
| SiO₂ | 240.161 | 0.1798 |
| H₃O⁺ | 0.000 | 0.0000 |
| OH⁻ | 0.000 | 0.0000 |
| Fe₃O₄ | 0.508 | 0.0004 |
| Al₂O₃ | 261.498 | 0.1957 |
| Total | 1,336.017 | 1.0000 |
| Temperature (°C) | 32.30 |  |
| Pressure (atm) | 1.9738 |  |
| Stream 506B | Mass Flow (kg/hr) | Mass Fraction |
| H₂O | 427.298 | 0.6235 |
| Fe₂O₃ | 0.270 | 0.0004 |
| SiO₂ | 123.284 | 0.1799 |
| H₃O⁺ | 0.000 | 0.0000 |
| OH⁻ | 0.000 | 0.0000 |
| Fe₃O₄ | 0.261 | 0.0004 |
| Al₂O₃ | 134.237 | 0.1959 |
| Total | 685.351 | 1.0000 |
| Temperature (°C) | 32.30 |  |
| Pressure (atm) | 1.9738 |  |

#### TK-601 — Leach Feed Tank

Unit Type: Tank (Surge Vessel)	Inlet streams: 506    Outlet streams: 601

The hydrocyclone overflow (506) is held in the leach feed tank TK-601, which provides surge capacity between the milling/classification circuit and the leaching section. Composition and temperature are unchanged across the tank.

Inlet stream condition and composition:

| Stream 506 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 833.324 | 0.6237 |
| Fe₂O₃ | 0.526 | 0.0004 |
| SiO₂ | 240.161 | 0.1798 |
| H₃O⁺ | 0.000 | 0.0000 |
| OH⁻ | 0.000 | 0.0000 |
| Fe₃O₄ | 0.508 | 0.0004 |
| Al₂O₃ | 261.498 | 0.1957 |
| Total | 1,336.017 | 1.0000 |
| Temperature (°C) | 32.30 |  |
| Pressure (atm) | 1.9738 |  |

Outlet stream condition and composition:

| Stream 601 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 833.324 | 0.6237 |
| Fe₂O₃ | 0.526 | 0.0004 |
| SiO₂ | 240.161 | 0.1798 |
| H₃O⁺ | 0.000 | 0.0000 |
| OH⁻ | 0.000 | 0.0000 |
| Fe₃O₄ | 0.508 | 0.0004 |
| Al₂O₃ | 261.498 | 0.1957 |
| Total | 1,336.017 | 1.0000 |
| Temperature (°C) | 32.30 |  |
| Pressure (atm) | 1.9738 |  |

#### P-601 — HCl Acid Feed Pump

Unit Type: Pump	Inlet streams: HCL-1    Outlet streams: HCL-2

Concentrated hydrochloric acid feed (HCL-1), admitted at the plant boundary, is pressurised by the HCl acid feed pump P-601 for delivery to the acid leach reactor R-601. Composition and temperature are essentially unchanged across the pumping step.

The component identity of the process stream(s) is unchanged across the unit; only physical conditions (temperature, pressure, and/or phase distribution) or stream routing are altered.

Inlet stream condition and composition:

| Stream HCL-1 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 939.397 | 0.5219 |
| HCl | 0.000 | 0.0000 |
| H₃O⁺ | 300.517 | 0.1670 |
| Cl⁻ | 560.085 | 0.3112 |
| Total | 1,800.000 | 1.0000 |
| Temperature (°C) | 32.00 |  |
| Pressure (atm) | 2.9608 |  |

Outlet stream condition and composition:

| Stream HCL-2 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 939.397 | 0.5219 |
| HCl | 0.000 | 0.0000 |
| H₃O⁺ | 300.517 | 0.1670 |
| Cl⁻ | 560.085 | 0.3112 |
| Total | 1,800.000 | 1.0000 |
| Temperature (°C) | 32.01 |  |
| Pressure (atm) | 3.4542 |  |

#### R-601 — Acid Leach Reactor

Unit Type: Reactor (Kinetic)	Inlet streams: 601, HCL-2    Outlet streams: 602

The leach feed slurry (601) and the concentrated HCl acid feed (HCL-2) are combined in the acid leach reactor R-601, where alumina, Al₂O₃, dissolves in hydrochloric acid to yield dissolved aluminium species (Al³⁺ and associated hydrolysis products) in solution.

Inlet stream condition and composition:

| Stream 601 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 833.324 | 0.6237 |
| Fe₂O₃ | 0.526 | 0.0004 |
| SiO₂ | 240.161 | 0.1798 |
| H₃O⁺ | 0.000 | 0.0000 |
| OH⁻ | 0.000 | 0.0000 |
| Fe₃O₄ | 0.508 | 0.0004 |
| Al₂O₃ | 261.498 | 0.1957 |
| Total | 1,336.017 | 1.0000 |
| Temperature (°C) | 32.30 |  |
| Pressure (atm) | 1.9738 |  |
| Stream HCL-2 | Mass Flow (kg/hr) | Mass Fraction |
| H₂O | 939.397 | 0.5219 |
| HCl | 0.000 | 0.0000 |
| H₃O⁺ | 300.517 | 0.1670 |
| Cl⁻ | 560.085 | 0.3112 |
| Total | 1,800.00 | 1.0000 |
| Temperature (°C) | 32.01 |  |
| Pressure (atm) | 3.4542 |  |

Outlet stream condition and composition:

| Stream 602 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 2,185.458 | 0.6969 |
| Fe₂O₃ | 0.039 | 0.0000 |
| SiO₂ | 240.161 | 0.0766 |
| H₃O⁺ | 9.973 | 0.0032 |
| Cl⁻ | 560.085 | 0.1786 |
| Al³⁺ | 137.195 | 0.0437 |
| Fe₂(OH)₂ | 0.000 | 0.0000 |
| Fe³⁺ | 0.341 | 0.0001 |
| FeOH²⁺ | 0.000 | 0.0000 |
| Fe₃O₄ | 0.508 | 0.0002 |
| Al₂O₃ | 2.257 | 0.0007 |
| Total | 3,136.017 | 1.0000 |
| Temperature (°C) | 120.00 |  |
| Pressure (atm) | 2.9608 |  |

#### FP-601 — Filter Press / Thickener

Unit Type: Solid-Liquid Separator	Inlet streams: 602    Outlet streams: 603, SIO2

The leach liquor (602) from R-601 is dewatered in the filter press/thickener FP-601, which separates the chemically inert, undissolved silica residue (withdrawn as the waste stream SIO2, at the plant boundary) from the aluminium- and iron-bearing leachate (603), which proceeds to the precipitation stage.

Inlet stream condition and composition:

| Stream 602 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 2,185.458 | 0.6969 |
| Fe₂O₃ | 0.039 | 0.0000 |
| SiO₂ | 240.161 | 0.0766 |
| H₃O⁺ | 9.973 | 0.0032 |
| Cl⁻ | 560.085 | 0.1786 |
| Al³⁺ | 137.195 | 0.0437 |
| Fe₂(OH)₂ | 0.000 | 0.0000 |
| Fe³⁺ | 0.341 | 0.0001 |
| FeOH²⁺ | 0.000 | 0.0000 |
| Fe₃O₄ | 0.508 | 0.0002 |
| Al₂O₃ | 2.257 | 0.0007 |
| Total | 3,136.017 | 1.0000 |
| Temperature (°C) | 120.00 |  |
| Pressure (atm) | 2.9608 |  |

Outlet stream condition and composition:

| Stream 603 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 2,185.458 | 0.7554 |
| H₃O⁺ | 9.973 | 0.0034 |
| Cl⁻ | 560.085 | 0.1936 |
| Al³⁺ | 137.195 | 0.0474 |
| Fe₂(OH)₂ | 0.000 | 0.0000 |
| Fe³⁺ | 0.341 | 0.0001 |
| FeOH²⁺ | 0.000 | 0.0000 |
| Total | 2,893.053 | 1.0000 |
| Temperature (°C) | 120.00 |  |
| Pressure (atm) | 2.9608 |  |
| Stream SIO2 | Mass Flow (kg/hr) | Mass Fraction |
| Fe₂O₃ | 0.039 | 0.0002 |
| SiO₂ | 240.161 | 0.9885 |
| Fe₃O₄ | 0.508 | 0.0021 |
| Al₂O₃ | 2.257 | 0.0093 |
| Total | 242.964 | 1.0000 |
| Temperature (°C) | 120.00 |  |
| Pressure (atm) | 2.9608 |  |

#### H-701 — CaAl₂O₄ Reagent Hopper

Unit Type: Dosing Hopper	Inlet streams: CAAL2O4    Outlet streams: CAAL2O4B

Solid calcium aluminate, CaAl₂O₄, admitted at the plant boundary as the precipitating reagent, is metered from the reagent hopper H-701 to the precipitation reactor R-701 at a controlled dosing rate. Composition and temperature are unchanged across the hopper.

Inlet stream condition and composition:

| Stream CAAL2O4 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| CaAl₂O₄ | 687.215 | 1.0000 |
| Total | 687.215 | 1.0000 |
| Temperature (°C) | 32.00 |  |
| Pressure (atm) | 1.0000 |  |

Outlet stream condition and composition:

| Stream CAAL2O4B | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| CaAl₂O₄ | 687.215 | 1.0000 |
| Total | 687.215 | 1.0000 |
| Temperature (°C) | 32.00 |  |
| Pressure (atm) | 1.0000 |  |

#### R-701 — Precipitation Reactor

Unit Type: Reactor	Inlet streams: 603, CAAL2O4B    Outlet streams: 701

The clarified leachate (603) and the metered calcium aluminate reagent (CAAL2O4B) are combined in the precipitation reactor R-701.

Inlet stream condition and composition:

| Stream 603 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 2,185.458 | 0.7554 |
| H₃O⁺ | 9.973 | 0.0034 |
| Cl⁻ | 560.085 | 0.1936 |
| Al³⁺ | 137.195 | 0.0474 |
| Fe₂(OH)₂ | 0.000 | 0.0000 |
| Fe³⁺ | 0.341 | 0.0001 |
| FeOH²⁺ | 0.000 | 0.0000 |
| Total | 2,893.053 | 1.0000 |
| Temperature (°C) | 120.00 |  |
| Pressure (atm) | 2.9608 |  |
| Stream CAAL2O4B | Mass Flow (kg/hr) | Mass Fraction |
| CaAl₂O₄ | 687.215 | 1.0000 |
| Total | 687.215 | 1.0000 |
| Temperature (°C) | 32.00 |  |
| Pressure (atm) | 1.0000 |  |

Outlet stream condition and composition:

| Stream 701 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 1,924.138 | 0.5374 |
| Al(OH)₃ | 794.738 | 0.2220 |
| H₃O⁺ | 0.000 | 0.0000 |
| OH⁻ | 0.000 | 0.0000 |
| Cl⁻ | 560.085 | 0.1564 |
| Al³⁺ | 72.134 | 0.0201 |
| Fe₂(OH)₂ | 0.000 | 0.0000 |
| Fe³⁺ | 0.000 | 0.0000 |
| FeOH²⁺ | 0.000 | 0.0000 |
| Fe(OH)₂⁺ | 0.000 | 0.0000 |
| Ca²⁺ | 155.839 | 0.0435 |
| CaAl₂O₄ | 72.681 | 0.0203 |
| CaOH⁺ | 0.000 | 0.0000 |
| Iron-(s) | 0.000 | 0.0000 |
| Fe(OH)₃ | 0.652 | 0.0002 |
| Total | 3,580.268 | 1.0000 |
| Temperature (°C) | 70.00 |  |
| Pressure (atm) | 1.0000 |  |

#### R-702 — PAC Polymerisation Reactor

Unit Type: Reactor	Inlet streams: 701    Outlet streams: PRODUCT

The precipitation-reactor effluent (701) is fed to the PAC polymerisation reactor R-702, where Al(OH)₃ and residual dissolved aluminium species condense and polymerise under controlled conditions to form the target poly-aluminium chloride product, modelled as the hydrated Al₁₃ Keggin-ion species. Chloride is incorporated into the polymer structure, so the free Cl⁻ concentration decreases across the unit while the PAC component appears in the product stream, PRODUCT, which reports at the plant boundary together with residual water and minor ionic species.

Composition changes across the unit; components gained across the unit: PAC.

Inlet stream condition and composition:

| Stream 701 | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 1,924.138 | 0.5374 |
| Al(OH)₃ | 794.738 | 0.2220 |
| H₃O⁺ | 0.000 | 0.0000 |
| OH⁻ | 0.000 | 0.0000 |
| Cl⁻ | 560.085 | 0.1564 |
| Al³⁺ | 72.134 | 0.0201 |
| Fe₂(OH)₂ | 0.000 | 0.0000 |
| Fe³⁺ | 0.000 | 0.0000 |
| FeOH²⁺ | 0.000 | 0.0000 |
| Fe(OH)₂⁺ | 0.000 | 0.0000 |
| Ca²⁺ | 155.839 | 0.0435 |
| CaAl₂O₄ | 72.681 | 0.0203 |
| CaOH⁺ | 0.000 | 0.0000 |
| Iron-(s) | 0.000 | 0.0000 |
| Fe(OH)₃ | 0.652 | 0.0002 |
| Total | 3,580.268 | 1.0000 |
| Temperature (°C) | 70.00 |  |
| Pressure (atm) | 1.0000 |  |

Outlet stream condition and composition:

| Stream PRODUCT | Mass Flow (kg/hr) | Mass Fraction |
| --- | --- | --- |
| H₂O | 1,786.477 | 0.4990 |
| Al(OH)₃ | 0.000 | 0.0000 |
| PAC | 1,229.578 | 0.3434 |
| H₃O⁺ | 0.000 | 0.0000 |
| OH⁻ | 0.000 | 0.0000 |
| Cl⁻ | 323.038 | 0.0902 |
| Al³⁺ | 12.003 | 0.0034 |
| Fe₂(OH)₂ | 0.000 | 0.0000 |
| Fe³⁺ | 0.000 | 0.0000 |
| FeOH²⁺ | 0.000 | 0.0000 |
| Fe(OH)₂⁺ | 0.000 | 0.0000 |
| Ca²⁺ | 155.839 | 0.0435 |
| CaAl₂O₄ | 72.681 | 0.0203 |
| CaOH⁺ | 0.000 | 0.0000 |
| Iron-(s) | 0.000 | 0.0000 |
| Fe(OH)₃ | 0.652 | 0.0002 |
| Total | 3,580.268 | 1.0000 |
| Temperature (°C) | 75.00 |  |
| Pressure (atm) | 1.0000 |  |

### Overall Material Balance Table

| Parameter | 101 | 102 | 103 | 104 | 201 | 202 | 203 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Stream Name | 101 | 102 | 103 | 104 | 201 | 202 | 203 |
| From (unit) | — (plant boundary) | FD-101 | CRK-201 | BN-101 | — (plant boundary) | E,BR-201 | RK-201 |
| To (unit) | FD-101 | CRK-201 | BN-101 | RK-201 | E,BR-201 | RK-201 | CL-201 |
| Temperature (°C) | 32.00 | 32.00 | 32.20 | 32.20 | 32.00 | 700.00 | 870.00 |
| Pressure (atm) | 1.0000 | 1.0000 | 1.0000 | 1.0000 | 1.0000 | 1.1000 | 1.0000 |
| Mass Vapour Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 1.0000 | 1.0000 | 0.0000 |
| Mass Liquid Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Mass Solid Fraction | 1.0000 | 1.0000 | 1.0000 | 1.0000 | 0.0000 | 0.0000 | 1.0000 |
| Total Mass Flow (kg/hr) | 755.987 | 755.987 | 755.987 | 755.987 | 978.400 | 978.400 | 556.010 |
| Mass Enthalpy (J/kg) | -15,580,575.7003 | -15,580,575.7003 | -15,580,375.7001 | -15,580,375.7001 | -2,787,626.5933 | -2,065,274.9466 | -14,436,077.0685 |
| Enthalpy Flow (W) | -3,271,865.8587 | -3,271,865.8587 | -3,271,820.8997 | -3,271,820.8997 | -757,614.9608 | -561,295.8355 | -2,229,611.9525 |
| Mass Density (kg/m³) | 2,564.1066 | 2,564.1066 | 2,564.1066 | 2,564.1066 | 1.2193 | 0.4201 | 3,236.8808 |
| Average MW | 71.1840 | 71.1840 | 71.1840 | 71.1840 | 30.5063 | 30.5063 | 77.6140 |
| H₂O — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| HCl — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| CO₂ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 174.267 | 174.267 | 0.000 |
| Al(OH)₃ — Mass Flow (kg/hr) | 451.350 | 451.350 | 451.350 | 451.350 | 0.000 | 0.000 | 8.332 |
| Fe₂O₃ — Mass Flow (kg/hr) | 11.997 | 11.997 | 11.997 | 11.997 | 0.000 | 0.000 | 0.554 |
| SiO₂ — Mass Flow (kg/hr) | 265.500 | 265.500 | 265.500 | 265.500 | 0.000 | 0.000 | 245.066 |
| TiO₂ — Mass Flow (kg/hr) | 21.633 | 21.633 | 21.633 | 21.633 | 0.000 | 0.000 | 19.968 |
| PAC — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| K₂O — Mass Flow (kg/hr) | 4.032 | 4.032 | 4.032 | 4.032 | 0.000 | 0.000 | 3.721 |
| Na₂O — Mass Flow (kg/hr) | 1.180 | 1.180 | 1.180 | 1.180 | 0.000 | 0.000 | 1.089 |
| MgO — Mass Flow (kg/hr) | 0.295 | 0.295 | 0.295 | 0.295 | 0.000 | 0.000 | 0.272 |
| H₃O⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| OH⁻ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Cl⁻ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Al³⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Fe₂(OH)₂ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Fe³⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| FeOH²⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Fe(OH)₂⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Ca²⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| CO — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 297.447 | 297.447 | 0.000 |
| Fe₃O₄ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 10.168 |
| Al₂O₃ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 266.839 |
| O₂ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 133.620 | 133.620 | 0.000 |
| CaAl₂O₄ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| CaOH⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Iron-(s) — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| HCO₃⁻ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| CO₃²⁻ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| N₂ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 373.066 | 373.066 | 0.000 |
| Fe(OH)₃ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| H₂O — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| HCl — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| CO₂ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.1781 | 0.1781 | 0.0000 |
| Al(OH)₃ — Mass Fraction | 0.5970 | 0.5970 | 0.5970 | 0.5970 | 0.0000 | 0.0000 | 0.0150 |
| Fe₂O₃ — Mass Fraction | 0.0159 | 0.0159 | 0.0159 | 0.0159 | 0.0000 | 0.0000 | 0.0010 |
| SiO₂ — Mass Fraction | 0.3512 | 0.3512 | 0.3512 | 0.3512 | 0.0000 | 0.0000 | 0.4408 |
| TiO₂ — Mass Fraction | 0.0286 | 0.0286 | 0.0286 | 0.0286 | 0.0000 | 0.0000 | 0.0359 |
| PAC — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| K₂O — Mass Fraction | 0.0053 | 0.0053 | 0.0053 | 0.0053 | 0.0000 | 0.0000 | 0.0067 |
| Na₂O — Mass Fraction | 0.0016 | 0.0016 | 0.0016 | 0.0016 | 0.0000 | 0.0000 | 0.0020 |
| MgO — Mass Fraction | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0000 | 0.0000 | 0.0005 |
| H₃O⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| OH⁻ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Cl⁻ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Al³⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Fe₂(OH)₂ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Fe³⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| FeOH²⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Fe(OH)₂⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Ca²⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| CO — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.3040 | 0.3040 | 0.0000 |
| Fe₃O₄ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0183 |
| Al₂O₃ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.4799 |
| O₂ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.1366 | 0.1366 | 0.0000 |
| CaAl₂O₄ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| CaOH⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Iron-(s) — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| HCO₃⁻ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| CO₃²⁻ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| N₂ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.3813 | 0.3813 | 0.0000 |
| Fe(OH)₃ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |

Figure 4-1 Streams 101, 102, 103, 104, 201, 202, 203

| Parameter | 204 | 301 | 303B | 304 | 304B | 401 | 501 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Stream Name | 204 | 301 | 303B | 304 | 304B | 401 | 501 |
| From (unit) | CL-201 | RK-201 | E,CL-301 | CY-301 | CP-301 | MS-401 | BN-501 |
| To (unit) | MS-401 | E,CL-301 | CY-301 | CP-301 | BH-101 | BN-501 | TK-501 |
| Temperature (°C) | 32.00 | 870.00 | 30.00 | 30.00 | 117.52 | 32.00 | 32.00 |
| Pressure (atm) | 1.0000 | 1.0000 | 1.0000 | 1.2000 | 5.0000 | 1.0000 | 1.0000 |
| Mass Vapour Fraction | 0.0000 | 0.9607 | 0.9548 | 0.5000 | 0.5000 | 0.0000 | 0.0000 |
| Mass Liquid Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Mass Solid Fraction | 1.0000 | 0.0393 | 0.0452 | 0.5000 | 0.5000 | 1.0000 | 1.0000 |
| Total Mass Flow (kg/hr) | 556.010 | 1,178.377 | 1,025.142 | 9.272 | 9.272 | 502.701 | 502.701 |
| Mass Enthalpy (J/kg) | -15,373,116.6625 | -5,636,805.0979 | -5,671,460.3458 | -10,293,303.2556 | -10,215,016.1714 | -15,795,399.9710 | -15,795,399.9710 |
| Enthalpy Flow (W) | -2,374,335.1116 | -1,845,077.5812 | -1,615,014.8221 | -26,511.3879 | -26,309.7520 | -2,205,655.8277 | -2,205,655.8277 |
| Mass Density (kg/m³) | 3,236.8808 | 0.3450 | 1.4801 | 3.3919 | 10.9648 | 3,212.6193 | 3,212.6193 |
| Average MW | 77.6140 | 31.8485 | 35.9780 | 48.3261 | 48.3261 | 76.5457 | 76.5457 |
| H₂O — Mass Flow (kg/hr) | 0.000 | 153.235 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| HCl — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| CO₂ — Mass Flow (kg/hr) | 0.000 | 542.865 | 542.865 | 2.571 | 2.571 | 0.000 | 0.000 |
| Al(OH)₃ — Mass Flow (kg/hr) | 8.332 | 0.695 | 0.695 | 0.069 | 0.069 | 0.000 | 0.000 |
| Fe₂O₃ — Mass Flow (kg/hr) | 0.554 | 0.046 | 0.046 | 0.005 | 0.005 | 0.526 | 0.526 |
| SiO₂ — Mass Flow (kg/hr) | 245.066 | 20.434 | 20.434 | 2.043 | 2.043 | 240.165 | 240.165 |
| TiO₂ — Mass Flow (kg/hr) | 19.968 | 1.665 | 1.665 | 0.166 | 0.166 | 0.000 | 0.000 |
| PAC — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| K₂O — Mass Flow (kg/hr) | 3.721 | 0.310 | 0.310 | 0.031 | 0.031 | 0.000 | 0.000 |
| Na₂O — Mass Flow (kg/hr) | 1.089 | 0.091 | 0.091 | 0.009 | 0.009 | 0.000 | 0.000 |
| MgO — Mass Flow (kg/hr) | 0.272 | 0.023 | 0.023 | 0.002 | 0.002 | 0.000 | 0.000 |
| H₃O⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| OH⁻ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Cl⁻ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Al³⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Fe₂(OH)₂ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Fe³⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| FeOH²⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Fe(OH)₂⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Ca²⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| CO — Mass Flow (kg/hr) | 0.000 | 62.849 | 62.849 | 0.298 | 0.298 | 0.000 | 0.000 |
| Fe₃O₄ — Mass Flow (kg/hr) | 10.168 | 0.848 | 0.848 | 0.085 | 0.085 | 0.508 | 0.508 |
| Al₂O₃ — Mass Flow (kg/hr) | 266.839 | 22.250 | 22.250 | 2.225 | 2.225 | 261.502 | 261.502 |
| O₂ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| CaAl₂O₄ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| CaOH⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Iron-(s) — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| HCO₃⁻ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| CO₃²⁻ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| N₂ — Mass Flow (kg/hr) | 0.000 | 373.066 | 373.066 | 1.767 | 1.767 | 0.000 | 0.000 |
| Fe(OH)₃ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| H₂O — Mass Fraction | 0.0000 | 0.1300 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| HCl — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| CO₂ — Mass Fraction | 0.0000 | 0.4607 | 0.5296 | 0.2773 | 0.2773 | 0.0000 | 0.0000 |
| Al(OH)₃ — Mass Fraction | 0.0150 | 0.0006 | 0.0007 | 0.0075 | 0.0075 | 0.0000 | 0.0000 |
| Fe₂O₃ — Mass Fraction | 0.0010 | 0.0000 | 0.0000 | 0.0005 | 0.0005 | 0.0010 | 0.0010 |
| SiO₂ — Mass Fraction | 0.4408 | 0.0173 | 0.0199 | 0.2204 | 0.2204 | 0.4777 | 0.4777 |
| TiO₂ — Mass Fraction | 0.0359 | 0.0014 | 0.0016 | 0.0180 | 0.0180 | 0.0000 | 0.0000 |
| PAC — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| K₂O — Mass Fraction | 0.0067 | 0.0003 | 0.0003 | 0.0033 | 0.0033 | 0.0000 | 0.0000 |
| Na₂O — Mass Fraction | 0.0020 | 0.0001 | 0.0001 | 0.0010 | 0.0010 | 0.0000 | 0.0000 |
| MgO — Mass Fraction | 0.0005 | 0.0000 | 0.0000 | 0.0002 | 0.0002 | 0.0000 | 0.0000 |
| H₃O⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| OH⁻ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Cl⁻ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Al³⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Fe₂(OH)₂ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Fe³⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| FeOH²⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Fe(OH)₂⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Ca²⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| CO — Mass Fraction | 0.0000 | 0.0533 | 0.0613 | 0.0321 | 0.0321 | 0.0000 | 0.0000 |
| Fe₃O₄ — Mass Fraction | 0.0183 | 0.0007 | 0.0008 | 0.0091 | 0.0091 | 0.0010 | 0.0010 |
| Al₂O₃ — Mass Fraction | 0.4799 | 0.0189 | 0.0217 | 0.2400 | 0.2400 | 0.5202 | 0.5202 |
| O₂ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| CaAl₂O₄ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| CaOH⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Iron-(s) — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| HCO₃⁻ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| CO₃²⁻ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| N₂ — Mass Fraction | 0.0000 | 0.3166 | 0.3639 | 0.1906 | 0.1906 | 0.0000 | 0.0000 |
| Fe(OH)₃ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |

Figure 4-2 Streams 204, 301, 303B, 304, 304B, 401, 501

| Parameter | 502 | 503 | 504 | 505 | 506 | 506B | 601 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Stream Name | 502 | 503 | 504 | 505 | 506 | 506B | 601 |
| From (unit) | TK-501 | P-502 | ML--501 | P-503 | HC-101 | HC-101 | TK-601 |
| To (unit) | P-502 | ML--501 | P-503 | HC-101 | TK-601 | TK-501 | R-601 |
| Temperature (°C) | 32.12 | 32.12 | 32.28 | 32.30 | 32.30 | 32.30 | 32.30 |
| Pressure (atm) | 1.0000 | 1.1000 | 1.1000 | 1.9738 | 1.9738 | 1.9738 | 1.9738 |
| Mass Vapour Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Mass Liquid Fraction | 0.6236 | 0.6236 | 0.6236 | 0.6236 | 0.6237 | 0.6235 | 0.6237 |
| Mass Solid Fraction | 0.3764 | 0.3764 | 0.3764 | 0.3764 | 0.3763 | 0.3765 | 0.3763 |
| Total Mass Flow (kg/hr) | 2,021.385 | 2,021.385 | 2,021.385 | 2,021.385 | 1,336.017 | 685.351 | 1,336.017 |
| Mass Enthalpy (J/kg) | -15,826,905.1187 | -15,826,893.5734 | -15,826,445.5260 | -15,826,344.6283 | -15,826,349.1421 | -15,826,285.6137 | -15,826,349.1461 |
| Enthalpy Flow (W) | -8,886,742.2014 | -8,886,735.7187 | -8,886,484.1419 | -8,886,427.4883 | -5,873,409.7802 | -3,012,933.6400 | -5,873,409.7817 |
| Mass Density (kg/m³) | 1,344.3213 | 1,344.3256 | 1,344.2687 | 1,344.3068 | 1,344.1936 | 1,344.5246 | 1,344.1936 |
| Average MW | 25.2944 | 25.2944 | 25.2944 | 25.2944 | 25.2920 | 25.2991 | 25.2920 |
| H₂O — Mass Flow (kg/hr) | 1,260.632 | 1,260.632 | 1,260.632 | 1,260.632 | 833.324 | 427.298 | 833.324 |
| HCl — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| CO₂ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Al(OH)₃ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Fe₂O₃ — Mass Flow (kg/hr) | 0.796 | 0.796 | 0.796 | 0.796 | 0.526 | 0.270 | 0.526 |
| SiO₂ — Mass Flow (kg/hr) | 363.449 | 363.449 | 363.449 | 363.449 | 240.161 | 123.284 | 240.161 |
| TiO₂ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| PAC — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| K₂O — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Na₂O — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| MgO — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| H₃O⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| OH⁻ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Cl⁻ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Al³⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Fe₂(OH)₂ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Fe³⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| FeOH²⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Fe(OH)₂⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Ca²⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| CO — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Fe₃O₄ — Mass Flow (kg/hr) | 0.769 | 0.769 | 0.769 | 0.769 | 0.508 | 0.261 | 0.508 |
| Al₂O₃ — Mass Flow (kg/hr) | 395.739 | 395.739 | 395.739 | 395.739 | 261.498 | 134.237 | 261.498 |
| O₂ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| CaAl₂O₄ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| CaOH⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Iron-(s) — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| HCO₃⁻ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| CO₃²⁻ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| N₂ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Fe(OH)₃ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| H₂O — Mass Fraction | 0.6236 | 0.6236 | 0.6236 | 0.6236 | 0.6237 | 0.6235 | 0.6237 |
| HCl — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| CO₂ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Al(OH)₃ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Fe₂O₃ — Mass Fraction | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 |
| SiO₂ — Mass Fraction | 0.1798 | 0.1798 | 0.1798 | 0.1798 | 0.1798 | 0.1799 | 0.1798 |
| TiO₂ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| PAC — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| K₂O — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Na₂O — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| MgO — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| H₃O⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| OH⁻ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Cl⁻ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Al³⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Fe₂(OH)₂ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Fe³⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| FeOH²⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Fe(OH)₂⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Ca²⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| CO — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Fe₃O₄ — Mass Fraction | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 | 0.0004 |
| Al₂O₃ — Mass Fraction | 0.1958 | 0.1958 | 0.1958 | 0.1958 | 0.1957 | 0.1959 | 0.1957 |
| O₂ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| CaAl₂O₄ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| CaOH⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Iron-(s) — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| HCO₃⁻ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| CO₃²⁻ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| N₂ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Fe(OH)₃ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |

Figure 4-3 Streams 502, 503, 504, 505, 506, 506B, 601

| Parameter | 602 | 603 | 701 | BH-AIR | BH-DUST | CAAL2O4 | CAAL2O4B |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Stream Name | 602 | 603 | 701 | BH-AIR | BH-DUST | CAAL2O4 | CAAL2O4B |
| From (unit) | R-601 | FP-601 | R-701 | BH-101 | BH-101 | — (plant boundary) | H-701 |
| To (unit) | FP-601 | R-701 | R-702 | — (plant boundary) | — (plant boundary) | H-701 | R-701 |
| Temperature (°C) | 120.00 | 120.00 | 70.00 | 117.52 | 117.52 | 32.00 | 32.00 |
| Pressure (atm) | 2.9608 | 2.9608 | 1.0000 | 4.9975 | 4.9975 | 1.0000 | 1.0000 |
| Mass Vapour Fraction | 0.0000 | 0.0000 | 0.0000 | 1.0000 | 0.0000 | 0.0000 | 0.0000 |
| Mass Liquid Fraction | 0.9225 | 1.0000 | 0.7577 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Mass Solid Fraction | 0.0775 | 0.0000 | 0.2423 | 0.0000 | 1.0000 | 1.0000 | 1.0000 |
| Total Mass Flow (kg/hr) | 3,136.017 | 2,893.053 | 3,580.268 | 4.636 | 4.636 | 687.215 | 687.215 |
| Mass Enthalpy (J/kg) | -13,694,510.4367 | -13,579,158.4185 | -14,083,724.3612 | -5,129,221.8292 | -15,300,920.4607 | -14,714,464.0160 | -14,714,464.0160 |
| Enthalpy Flow (W) | -11,929,505.7622 | -10,912,561.7231 | -14,006,529.7784 | -6,605.4723 | -19,704.2797 | -2,808,889.7085 | -2,808,889.7085 |
| Mass Density (kg/m³) | 1,250.6111 | 1,197.3410 | 1,470.4460 | 5.4891 | 3,236.8808 | 5,267.9560 | 5,267.9560 |
| Average MW | 21.3703 | 20.2702 | 25.6062 | 35.0864 | 77.6140 | 158.0387 | 158.0387 |
| H₂O — Mass Flow (kg/hr) | 2,185.458 | 2,185.458 | 1,924.138 | 0.000 | 0.000 | 0.000 | 0.000 |
| HCl — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| CO₂ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 2.571 | 0.000 | 0.000 | 0.000 |
| Al(OH)₃ — Mass Flow (kg/hr) | 0.000 | 0.000 | 794.738 | 0.000 | 0.069 | 0.000 | 0.000 |
| Fe₂O₃ — Mass Flow (kg/hr) | 0.039 | 0.000 | 0.000 | 0.000 | 0.005 | 0.000 | 0.000 |
| SiO₂ — Mass Flow (kg/hr) | 240.161 | 0.000 | 0.000 | 0.000 | 2.043 | 0.000 | 0.000 |
| TiO₂ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.166 | 0.000 | 0.000 |
| PAC — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| K₂O — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.031 | 0.000 | 0.000 |
| Na₂O — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.009 | 0.000 | 0.000 |
| MgO — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.002 | 0.000 | 0.000 |
| H₃O⁺ — Mass Flow (kg/hr) | 9.973 | 9.973 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| OH⁻ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Cl⁻ — Mass Flow (kg/hr) | 560.085 | 560.085 | 560.085 | 0.000 | 0.000 | 0.000 | 0.000 |
| Al³⁺ — Mass Flow (kg/hr) | 137.195 | 137.195 | 72.134 | 0.000 | 0.000 | 0.000 | 0.000 |
| Fe₂(OH)₂ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Fe³⁺ — Mass Flow (kg/hr) | 0.341 | 0.341 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| FeOH²⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Fe(OH)₂⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Ca²⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 155.839 | 0.000 | 0.000 | 0.000 | 0.000 |
| CO — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.298 | 0.000 | 0.000 | 0.000 |
| Fe₃O₄ — Mass Flow (kg/hr) | 0.508 | 0.000 | 0.000 | 0.000 | 0.085 | 0.000 | 0.000 |
| Al₂O₃ — Mass Flow (kg/hr) | 2.257 | 0.000 | 0.000 | 0.000 | 2.225 | 0.000 | 0.000 |
| O₂ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| CaAl₂O₄ — Mass Flow (kg/hr) | 0.000 | 0.000 | 72.681 | 0.000 | 0.000 | 687.215 | 687.215 |
| CaOH⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Iron-(s) — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| HCO₃⁻ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| CO₃²⁻ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| N₂ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 1.767 | 0.000 | 0.000 | 0.000 |
| Fe(OH)₃ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.652 | 0.000 | 0.000 | 0.000 | 0.000 |
| H₂O — Mass Fraction | 0.6969 | 0.7554 | 0.5374 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| HCl — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| CO₂ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.5546 | 0.0000 | 0.0000 | 0.0000 |
| Al(OH)₃ — Mass Fraction | 0.0000 | 0.0000 | 0.2220 | 0.0000 | 0.0150 | 0.0000 | 0.0000 |
| Fe₂O₃ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0010 | 0.0000 | 0.0000 |
| SiO₂ — Mass Fraction | 0.0766 | 0.0000 | 0.0000 | 0.0000 | 0.4408 | 0.0000 | 0.0000 |
| TiO₂ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0359 | 0.0000 | 0.0000 |
| PAC — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| K₂O — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0067 | 0.0000 | 0.0000 |
| Na₂O — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0020 | 0.0000 | 0.0000 |
| MgO — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0005 | 0.0000 | 0.0000 |
| H₃O⁺ — Mass Fraction | 0.0032 | 0.0034 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| OH⁻ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Cl⁻ — Mass Fraction | 0.1786 | 0.1936 | 0.1564 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Al³⁺ — Mass Fraction | 0.0437 | 0.0474 | 0.0201 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Fe₂(OH)₂ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Fe³⁺ — Mass Fraction | 0.0001 | 0.0001 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| FeOH²⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Fe(OH)₂⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Ca²⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0435 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| CO — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0642 | 0.0000 | 0.0000 | 0.0000 |
| Fe₃O₄ — Mass Fraction | 0.0002 | 0.0000 | 0.0000 | 0.0000 | 0.0183 | 0.0000 | 0.0000 |
| Al₂O₃ — Mass Fraction | 0.0007 | 0.0000 | 0.0000 | 0.0000 | 0.4799 | 0.0000 | 0.0000 |
| O₂ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| CaAl₂O₄ — Mass Fraction | 0.0000 | 0.0000 | 0.0203 | 0.0000 | 0.0000 | 1.0000 | 1.0000 |
| CaOH⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Iron-(s) — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| HCO₃⁻ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| CO₃²⁻ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| N₂ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.3812 | 0.0000 | 0.0000 | 0.0000 |
| Fe(OH)₃ — Mass Fraction | 0.0000 | 0.0000 | 0.0002 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |

Figure 4-4 Streams 602, 603, 701, BH-AIR, BH-DUST, CAAL2O4, CAAL2O4B

Sub-table 1.5: Streams CONDENSA, CY-DUST, FE3O4, HCL-1, HCL-2, PRODUCT, PW-1

| Parameter | CONDENSA | CY-DUST | FE3O4 | HCL-1 | HCL-2 | PRODUCT | PW-1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Stream Name | CONDENSA | CY-DUST | FE3O4 | HCL-1 | HCL-2 | PRODUCT | PW-1 |
| From (unit) | E,CL-301 | CY-301 | MS-401 | — (plant boundary) | P-601 | R-702 | — (plant boundary) |
| To (unit) | — (plant boundary) | — (plant boundary) | — (plant boundary) | P-601 | R-601 | — (plant boundary) | P-501 |
| Temperature (°C) | 30.00 | 30.00 | 32.00 | 32.00 | 32.01 | 75.00 | 32.00 |
| Pressure (atm) | 1.0000 | 1.2000 | 1.0000 | 2.9608 | 3.4542 | 1.0000 | 1.0000 |
| Mass Vapour Fraction | 0.0000 | 0.9589 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Mass Liquid Fraction | 1.0000 | 0.0000 | 0.0000 | 1.0000 | 1.0000 | 0.9797 | 1.0000 |
| Mass Solid Fraction | 0.0000 | 0.0411 | 1.0000 | 0.0000 | 0.0000 | 0.0203 | 0.0000 |
| Total Mass Flow (kg/hr) | 153.234 | 1,015.870 | 53.309 | 1,800.000 | 1,800.000 | 3,580.268 | 833.333 |
| Mass Enthalpy (J/kg) | -15,854,842.7945 | -5,629,466.1470 | -11,391,019.3117 | -12,160,504.4369 | -12,160,446.6689 | -9,244,657.3957 | -15,846,487.6903 |
| Enthalpy Flow (W) | -674,863.1787 | -1,588,557.2527 | -168,679.2839 | -6,080,252.2184 | -6,080,223.3344 | -9,193,986.3194 | -3,668,168.4467 |
| Mass Density (kg/m³) | 995.7605 | 1.7695 | 3,485.0676 | 1,154.0754 | 1,154.0856 | 1,302.1999 | 995.1344 |
| Average MW | 18.0153 | 35.8943 | 89.3778 | 21.4951 | 21.4951 | 31.3974 | 18.0153 |
| H₂O — Mass Flow (kg/hr) | 153.234 | 0.000 | 0.000 | 939.397 | 939.397 | 1,786.477 | 833.333 |
| HCl — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| CO₂ — Mass Flow (kg/hr) | 0.000 | 540.294 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Al(OH)₃ — Mass Flow (kg/hr) | 0.000 | 0.625 | 8.332 | 0.000 | 0.000 | 0.000 | 0.000 |
| Fe₂O₃ — Mass Flow (kg/hr) | 0.000 | 0.042 | 0.028 | 0.000 | 0.000 | 0.000 | 0.000 |
| SiO₂ — Mass Flow (kg/hr) | 0.000 | 18.391 | 4.901 | 0.000 | 0.000 | 0.000 | 0.000 |
| TiO₂ — Mass Flow (kg/hr) | 0.000 | 1.499 | 19.968 | 0.000 | 0.000 | 0.000 | 0.000 |
| PAC — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 1,229.578 | 0.000 |
| K₂O — Mass Flow (kg/hr) | 0.000 | 0.279 | 3.721 | 0.000 | 0.000 | 0.000 | 0.000 |
| Na₂O — Mass Flow (kg/hr) | 0.000 | 0.082 | 1.089 | 0.000 | 0.000 | 0.000 | 0.000 |
| MgO — Mass Flow (kg/hr) | 0.000 | 0.020 | 0.272 | 0.000 | 0.000 | 0.000 | 0.000 |
| H₃O⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 300.517 | 300.517 | 0.000 | 0.000 |
| OH⁻ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Cl⁻ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 560.085 | 560.085 | 323.038 | 0.000 |
| Al³⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 12.003 | 0.000 |
| Fe₂(OH)₂ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Fe³⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| FeOH²⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Fe(OH)₂⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Ca²⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 155.839 | 0.000 |
| CO — Mass Flow (kg/hr) | 0.000 | 62.551 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Fe₃O₄ — Mass Flow (kg/hr) | 0.000 | 0.763 | 9.660 | 0.000 | 0.000 | 0.000 | 0.000 |
| Al₂O₃ — Mass Flow (kg/hr) | 0.000 | 20.025 | 5.337 | 0.000 | 0.000 | 0.000 | 0.000 |
| O₂ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| CaAl₂O₄ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 72.681 | 0.000 |
| CaOH⁺ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Iron-(s) — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| HCO₃⁻ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| CO₃²⁻ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| N₂ — Mass Flow (kg/hr) | 0.000 | 371.299 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Fe(OH)₃ — Mass Flow (kg/hr) | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.652 | 0.000 |
| H₂O — Mass Fraction | 1.0000 | 0.0000 | 0.0000 | 0.5219 | 0.5219 | 0.4990 | 1.0000 |
| HCl — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| CO₂ — Mass Fraction | 0.0000 | 0.5319 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Al(OH)₃ — Mass Fraction | 0.0000 | 0.0006 | 0.1563 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Fe₂O₃ — Mass Fraction | 0.0000 | 0.0000 | 0.0005 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| SiO₂ — Mass Fraction | 0.0000 | 0.0181 | 0.0919 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| TiO₂ — Mass Fraction | 0.0000 | 0.0015 | 0.3746 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| PAC — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.3434 | 0.0000 |
| K₂O — Mass Fraction | 0.0000 | 0.0003 | 0.0698 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Na₂O — Mass Fraction | 0.0000 | 0.0001 | 0.0204 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| MgO — Mass Fraction | 0.0000 | 0.0000 | 0.0051 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| H₃O⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.1670 | 0.1670 | 0.0000 | 0.0000 |
| OH⁻ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Cl⁻ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.3112 | 0.3112 | 0.0902 | 0.0000 |
| Al³⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0034 | 0.0000 |
| Fe₂(OH)₂ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Fe³⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| FeOH²⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Fe(OH)₂⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Ca²⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0435 | 0.0000 |
| CO — Mass Fraction | 0.0000 | 0.0616 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Fe₃O₄ — Mass Fraction | 0.0000 | 0.0008 | 0.1812 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Al₂O₃ — Mass Fraction | 0.0000 | 0.0197 | 0.1001 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| O₂ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| CaAl₂O₄ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0203 | 0.0000 |
| CaOH⁺ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Iron-(s) — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| HCO₃⁻ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| CO₃²⁻ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| N₂ — Mass Fraction | 0.0000 | 0.3655 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| Fe(OH)₃ — Mass Fraction | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0002 | 0.0000 |

| Parameter | PW-2 | SIO2 |
| --- | --- | --- |
| Stream Name | PW-2 | SIO2 |
| From (unit) | P-501 | FP-601 |
| To (unit) | TK-501 | — (plant boundary) |
| Temperature (°C) | 32.01 | 120.00 |
| Pressure (atm) | 1.5000 | 2.9608 |
| Mass Vapour Fraction | 0.0000 | 0.0000 |
| Mass Liquid Fraction | 1.0000 | 0.0000 |
| Mass Solid Fraction | 0.0000 | 1.0000 |
| Total Mass Flow (kg/hr) | 833.333 | 242.964 |
| Mass Enthalpy (J/kg) | -15,846,419.8099 | -15,068,042.6220 |
| Enthalpy Flow (W) | -3,668,152.7336 | -1,016,944.0391 |
| Mass Density (kg/m³) | 995.1551 | 2,659.5202 |
| Average MW | 18.0153 | 60.4144 |
| H₂O — Mass Flow (kg/hr) | 833.333 | 0.000 |
| Fe₂O₃ — Mass Flow (kg/hr) | 0.000 | 0.039 |
| SiO₂ — Mass Flow (kg/hr) | 0.000 | 240.161 |
| Fe₃O₄ — Mass Flow (kg/hr) | 0.000 | 0.508 |
| Al₂O₃ — Mass Flow (kg/hr) | 0.000 | 2.257 |
| Fe₂O₃ — Mass Fraction | 0.0000 | 0.0002 |
| SiO₂ — Mass Fraction | 0.0000 | 0.9885 |
| Fe₃O₄ — Mass Fraction | 0.0000 | 0.0021 |
| Al₂O₃ — Mass Fraction | 0.0000 | 0.0093 |

Figure 4-5 Streams PW-2, SIO2

## Energy Balance

### Energy Balance Framework and Sign Convention

For each unit operation, the following energy balance framework is applied:

Σ Ḥin+ Qadded = Σ Ḥout+ Qremoved

where Ḥ denotes the total stream enthalpy flow (Gcal/hr) as reported by Aspen Plus for each material stream. Stream enthalpies are referenced to the Aspen Plus standard state (25 °C, 1 atm, ideal-gas reference) and include sensible heat, latent heat, and heat-of-mixing contributions.

For each stream, the partial enthalpy contribution of each component is computed as

Ḥᵢ = ṁᵢ hᵢ

where ṁᵢ is the component mass flow (kg/hr) and hᵢ is the component-specific mass enthalpy (kcal/kg); the sum over all components equals the total stream enthalpy flow reported by Aspen Plus. Where Aspen Plus does not export a pure-component specific enthalpy — typically for ionic species in aqueous solution — the entry is marked "solid/ionic"; the contribution of these species is nonetheless captured within the total Aspen stream enthalpy via the electrolyte thermodynamics framework.

Aspen Plus reports all stream enthalpies in this plant as negative absolute values on the chosen reference state, i.e. at a lower energy state than the reference condition. The quantity ΔḤ = Ḥₒᵤₜ − Ḥᵢₙ indicates the net energy change across a unit: a negative ΔḤ indicates that the unit is a net energy sink (endothermic, requiring heat addition), while a positive ΔḤ indicates that the unit is a net energy source (exothermic, or products leaving at a higher enthalpy than the feeds, consistent with heat removal from the process).

### Per-Unit Energy Balance

#### FD-101 — Feed Bin / Storage Hopper

Unit FD-101 (Feed Bin / Storage Hopper, Bin / Storage Vessel) is classified as thermally neutral / adiabatic on the basis of the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -2.8152 Gcal/hr and the total outlet enthalpy flow is -2.8152 Gcal/hr, giving a net ΔḤ of 0.0000 Gcal/hr.

Table A — Stream Enthalpy Flow Summary:

| Stream / Term | Direction | Enthalpy Flow (Gcal/hr) |
| --- | --- | --- |
| 101 | Inlet | -2.8152 |
| Total Inlet | — | -2.8152 |
| 102 | Outlet | -2.8152 |
| Total Outlet | — | -2.8152 |
| ΔḤ (Net) | Net | 0.0000 |

Table B — Component-Specific Enthalpy (kcal/kg) per stream:

Stream 101 (Inlet)

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| Al(OH)₃ | -3,924.1067 |
| Fe₂O₃ | -1,232.5287 |
| SiO₂ | -3,621.3665 |
| TiO₂ | -2,825.6322 |
| K₂O | -919.9908 |
| Na₂O | -1,597.7727 |
| MgO | -3,565.2905 |

Stream 102 (Outlet) — T = 32.00 °C, P = 1.0000 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| Al(OH)₃ | -3,924.1067 |
| Fe₂O₃ | -1,232.5287 |
| SiO₂ | -3,621.3665 |
| TiO₂ | -2,825.6322 |
| K₂O | -919.9908 |
| Na₂O | -1,597.7726 |
| MgO | -3,565.2905 |

#### CRK-201 — Jaw Crusher

Unit CRK-201 (Jaw Crusher, Size-Reduction Unit) is classified as thermally neutral / adiabatic based on the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -2.8152 Gcal/hr and the total outlet enthalpy flow is -2.8151 Gcal/hr, giving a net ΔḤ of 0.0000 Gcal/hr.

Table A — Stream Enthalpy Flow Summary:

| Stream / Term | Direction | Enthalpy Flow (Gcal/hr) |
| --- | --- | --- |
| 102 | Inlet | -2.8152 |
| Total Inlet | — | -2.8152 |
| 103 | Outlet | -2.8151 |
| Total Outlet | — | -2.8151 |
| ΔḤ (Net) | Net | 0.0000 |

Table B — Component-Specific Enthalpy (kcal/kg) per stream:

Stream 102 (Inlet) — T = 32.00 °C, P = 1.0000 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| Al(OH)₃ | -3,924.1067 |
| Fe₂O₃ | -1,232.5287 |
| SiO₂ | -3,621.3665 |
| TiO₂ | -2,825.6322 |
| K₂O | -919.9908 |
| Na₂O | -1,597.7726 |
| MgO | -3,565.2905 |

Stream 103 (Outlet) — T = 32.20 °C, P = 1.0000 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| Al(OH)₃ | -3,924.0505 |
| Fe₂O₃ | -1,232.4977 |
| SiO₂ | -3,621.3309 |
| TiO₂ | -2,825.5994 |
| K₂O | -919.9487 |
| Na₂O | -1,597.7197 |
| MgO | -3,565.2465 |

#### BN-101 — Crushed Ore Bin

Unit BN-101 (Crushed Ore Bin, Bin / Storage Vessel) is classified as thermally neutral / adiabatic based on the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -2.8151 Gcal/hr and the total outlet enthalpy flow is -2.8151 Gcal/hr, giving a net ΔḤ of 0.0000 Gcal/hr.

Table A — Stream Enthalpy Flow Summary:

| Stream / Term | Direction | Enthalpy Flow (Gcal/hr) |
| --- | --- | --- |
| 103 | Inlet | -2.8151 |
| Total Inlet | — | -2.8151 |
| 104 | Outlet | -2.8151 |
| Total Outlet | — | -2.8151 |
| ΔḤ (Net) | Net | 0.0000 |

Table B — Component-Specific Enthalpy (kcal/kg) per stream:

Stream 103 (Inlet) — T = 32.20 °C, P = 1.0000 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| Al(OH)₃ | -3,924.0505 |
| Fe₂O₃ | -1,232.4977 |
| SiO₂ | -3,621.3309 |
| TiO₂ | -2,825.5994 |
| K₂O | -919.9487 |
| Na₂O | -1,597.7197 |
| MgO | -3,565.2465 |

Stream 104 (Outlet) — T = 32.20 °C, P = 1.0000 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| Al(OH)₃ | -3,924.0505 |
| Fe₂O₃ | -1,232.4977 |
| SiO₂ | -3,621.3309 |
| TiO₂ | -2,825.5994 |
| K₂O | -919.9487 |
| Na₂O | -1,597.7197 |
| MgO | -3,565.2465 |

#### E,BR-201 — Air Pre-heater / Burner

Unit E,BR-201 (Air Pre-heater / Burner, Fired Heater) is classified as exothermic on the basis of the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -0.6519 Gcal/hr and the total outlet enthalpy flow is -0.4830 Gcal/hr, giving a net ΔḤ of 0.1689 Gcal/hr.

Unit is a net energy source: 0.1689 Gcal/hr of enthalpy is released by the process (exothermic, or heat is rejected to a cooling utility).

Table A — Stream Enthalpy Flow Summary:

| Stream / Term | Direction | Enthalpy Flow (Gcal/hr) |
| --- | --- | --- |
| 201 | Inlet | -0.6519 |
| Total Inlet | — | -0.6519 |
| 202 | Outlet | -0.4830 |
| Total Outlet | — | -0.4830 |
| ΔḤ (Net) | Net | 0.1689 |

Table B — Component-Specific Enthalpy (kcal/kg) per stream:

Stream 201 (Inlet) — T = 32.00 °C, P = 1.0000 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| CO₂ | -2,137.2547 |
| CO | -942.1779 |
| O₂ | 1.4688 |
| N₂ | 1.6797 |

Stream 202 (Outlet) — T = 700.00 °C, P = 1.1000 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| CO₂ | -1,964.8523 |
| CO | -766.1972 |
| O₂ | 162.9239 |
| N₂ | 175.8111 |

RK-201 — Rotary Kiln (Roaster/Calciner)

Unit RK-201 (Rotary Kiln (Roaster/Calciner), Reactor (Stoichiometric)) is classified as endothermic based on the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -3.2981 Gcal/hr and the total outlet enthalpy flow is -3.5059 Gcal/hr, giving a net ΔḤ of -0.2079 Gcal/hr.

Unit is a net energy sink: 0.2079 Gcal/hr of enthalpy is absorbed by the process (endothermic; heat must be supplied by a utility, e.g. steam or fired duty).

Table A — Stream Enthalpy Flow Summary:

| Stream / Term | Direction | Enthalpy Flow (Gcal/hr) |
| --- | --- | --- |
| 104 | Inlet | -2.8151 |
| 202 | Inlet | -0.4830 |
| Total Inlet | — | -3.2981 |
| 203 | Outlet | -1.9184 |
| 301 | Outlet | -1.5875 |
| Total Outlet | — | -3.5059 |
| ΔḤ (Net) | Net | -0.2079 |

Table B — Component-Specific Enthalpy (kcal/kg) per stream:

Stream 104 (Inlet) — T = 32.20 °C, P = 1.0000 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| Al(OH)₃ | -3,924.0505 |
| Fe₂O₃ | -1,232.4977 |
| SiO₂ | -3,621.3309 |
| TiO₂ | -2,825.5994 |
| K₂O | -919.9487 |
| Na₂O | -1,597.7197 |
| MgO | -3,565.2465 |

Stream 202 (Inlet) — T = 700.00 °C, P = 1.1000 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| CO₂ | -1,964.8523 |
| CO | -766.1972 |
| O₂ | 162.9239 |
| N₂ | 175.8111 |

Stream 203 (Outlet) — T = 870.00 °C, P = 1.0000 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| Al(OH)₃ | -3,685.1343 |
| Fe₂O₃ | -1,052.7885 |
| SiO₂ | -3,390.9689 |
| TiO₂ | -2,654.5769 |
| K₂O | -701.4388 |
| Na₂O | -1,314.4366 |
| MgO | -3,327.4366 |
| Fe₃O₄ | -971.4037 |
| Al₂O₃ | -3,703.6312 |

Stream 301 (Outlet) — T = 870.00 °C, P = 1.0000 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| H₂O | -2,785.6846 |
| CO₂ | -1,913.9743 |
| Al(OH)₃ | -3,685.1344 |
| Fe₂O₃ | -1,052.7885 |
| SiO₂ | -3,390.9690 |
| TiO₂ | -2,654.5770 |
| K₂O | -701.4388 |
| Na₂O | -1,314.4367 |
| MgO | -3,327.4366 |
| CO | -717.7410 |
| Fe₃O₄ | -971.4038 |
| Al₂O₃ | -3,703.6313 |
| N₂ | 223.5801 |

#### CL-201 — Product Cooler

Unit CL-201 (Product Cooler, Heat Exchanger) is classified as endothermic based on the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -1.9184 Gcal/hr and the total outlet enthalpy flow is -2.0429 Gcal/hr, giving a net ΔḤ of -0.1245 Gcal/hr.

Unit is a net energy sink: 0.1245 Gcal/hr of enthalpy is absorbed by the process (endothermic; heat must be supplied by a utility, e.g. steam or fired duty).

Table A — Stream Enthalpy Flow Summary:

| Stream / Term | Direction | Enthalpy Flow (Gcal/hr) |
| --- | --- | --- |
| 203 | Inlet | -1.9184 |
| Total Inlet | — | -1.9184 |
| 204 | Outlet | -2.0429 |
| Total Outlet | — | -2.0429 |
| ΔḤ (Net) | Net | -0.1245 |

Table B — Component-Specific Enthalpy (kcal/kg) per stream:

Stream 203 (Inlet) — T = 870.00 °C, P = 1.0000 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| Al(OH)₃ | -3,685.1343 |
| Fe₂O₃ | -1,052.7885 |
| SiO₂ | -3,390.9689 |
| TiO₂ | -2,654.5769 |
| K₂O | -701.4388 |
| Na₂O | -1,314.4366 |
| MgO | -3,327.4366 |
| Fe₃O₄ | -971.4037 |
| Al₂O₃ | -3,703.6312 |

Stream 204 (Outlet) — T = 32.00 °C, P = 1.0000 atm, From: CL-201 → To: MS-401

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| Al(OH)₃ | -3,924.1067 |
| Fe₂O₃ | -1,232.5287 |
| SiO₂ | -3,621.3665 |
| TiO₂ | -2,825.6322 |
| K₂O | -919.9908 |
| Na₂O | -1,597.7727 |
| MgO | -3,565.2905 |
| Fe₃O₄ | -1,153.3510 |
| Al₂O₃ | -3,926.6763 |

#### E,CL-301 — Off-gas Cooler

Unit E,CL-301 (Off-gas Cooler, Heat Exchanger) is classified as endothermic on the basis of the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -1.5875 Gcal/hr and the total outlet enthalpy flow is -1.9703 Gcal/hr, giving a net ΔḤ of -0.3827 Gcal/hr.

Unit is a net energy sink: 0.3827 Gcal/hr of enthalpy is absorbed by the process (endothermic; heat must be supplied by a utility, e.g. steam or fired duty).

Table A — Stream Enthalpy Flow Summary:

| Stream / Term | Direction | Enthalpy Flow (Gcal/hr) |
| --- | --- | --- |
| 301 | Inlet | -1.5875 |
| Total Inlet | — | -1.5875 |
| 303B | Outlet | -1.3896 |
| CONDENSA | Outlet | -0.5807 |
| Total Outlet | — | -1.9703 |
| ΔḤ (Net) | Net | -0.3827 |

Table B — Component-Specific Enthalpy (kcal/kg) per stream:

Stream 301 (Inlet) — T = 870.00 °C, P = 1.0000 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| H₂O | -2,785.6846 |
| CO₂ | -1,913.9743 |
| Al(OH)₃ | -3,685.1344 |
| Fe₂O₃ | -1,052.7885 |
| SiO₂ | -3,390.9690 |
| TiO₂ | -2,654.5770 |
| K₂O | -701.4388 |
| Na₂O | -1,314.4367 |
| MgO | -3,327.4366 |
| CO | -717.7410 |
| Fe₃O₄ | -971.4038 |
| Al₂O₃ | -3,703.6313 |
| N₂ | 223.5801 |

Stream 303B (Outlet) — T = 30.00 °C, P = 1.0000 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| H₂O | -3,209.3791 |
| CO₂ | -2,137.6290 |
| Al(OH)₃ | -3,924.6770 |
| Fe₂O₃ | -1,232.8427 |
| SiO₂ | -3,621.7265 |
| TiO₂ | -2,825.9645 |
| K₂O | -920.4180 |
| Na₂O | -1,598.3095 |
| MgO | -3,565.7368 |
| CO | -942.6684 |
| Fe₃O₄ | -1,153.6674 |
| Al₂O₃ | -3,927.0516 |
| N₂ | 1.1911 |

Stream CONDENSA (Outlet) — T = 30.00 °C, P = 1.0000 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| H₂O | -3,787.6051 |
| H₃O⁺ | solid/ionic |
| OH⁻ | solid/ionic |

#### CY-301 — Cyclone Separator

Unit CY-301 (Cyclone Separator, Gas-Solid Separator) is classified as thermally neutral / adiabatic based on the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -1.3896 Gcal/hr and the total outlet enthalpy flow is -1.3896 Gcal/hr, giving a net ΔḤ of -0.0000 Gcal/hr.

Table A — Stream Enthalpy Flow Summary:

| Stream / Term | Direction | Enthalpy Flow (Gcal/hr) |
| --- | --- | --- |
| 303B | Inlet | -1.3896 |
| Total Inlet | — | -1.3896 |
| 304 | Outlet | -0.0228 |
| CY-DUST | Outlet | -1.3668 |
| Total Outlet | — | -1.3896 |
| ΔḤ (Net) | Net | -0.0000 |

Table B — Component-Specific Enthalpy (kcal/kg) per stream:

Stream 303B (Inlet) — T = 30.00 °C, P = 1.0000 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| H₂O | -3,209.3791 |
| CO₂ | -2,137.6290 |
| Al(OH)₃ | -3,924.6770 |
| Fe₂O₃ | -1,232.8427 |
| SiO₂ | -3,621.7265 |
| TiO₂ | -2,825.9645 |
| K₂O | -920.4180 |
| Na₂O | -1,598.3095 |
| MgO | -3,565.7368 |
| CO | -942.6684 |
| Fe₃O₄ | -1,153.6674 |
| Al₂O₃ | -3,927.0516 |
| N₂ | 1.1911 |

Stream 304 (Outlet) — T = 30.00 °C, P = 1.2000 atm, From: CY-301 → To: CP-301

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| H₂O | -3,209.8410 |
| CO₂ | -2,137.7081 |
| Al(OH)₃ | -3,924.6770 |
| Fe₂O₃ | -1,232.8427 |
| SiO₂ | -3,621.7265 |
| TiO₂ | -2,825.9645 |
| K₂O | -920.4180 |
| Na₂O | -1,598.3095 |
| MgO | -3,565.7368 |
| CO | -942.6908 |
| Fe₃O₄ | -1,153.6674 |
| Al₂O₃ | -3,927.0516 |
| N₂ | 1.1683 |

Stream CY-DUST (Outlet) — T = 30.00 °C, P = 1.2000 atm, From: CY-301 → To: Plant Boundary

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| H₂O | -3,209.8410 |
| CO₂ | -2,137.7081 |
| Al(OH)₃ | -3,924.6770 |
| Fe₂O₃ | -1,232.8427 |
| SiO₂ | -3,621.7265 |
| TiO₂ | -2,825.9645 |
| K₂O | -920.4180 |
| Na₂O | -1,598.3095 |
| MgO | -3,565.7368 |
| CO | -942.6908 |
| Fe₃O₄ | -1,153.6674 |
| Al₂O₃ | -3,927.0516 |
| N₂ | 1.1683 |

#### CP-301 — Gas Compressor / Fan

Unit CP-301 (Gas Compressor / Fan, Rotating Equipment (Compressor)) is classified as thermally neutral / adiabatic based on the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -0.0228 Gcal/hr and the total outlet enthalpy flow is -0.0226 Gcal/hr, giving a net ΔḤ of 0.0002 Gcal/hr.

Thermally neutral — negligible net enthalpy change across the unit (adiabatic / pass-through).

Table A — Stream Enthalpy Flow Summary:

| Stream / Term | Direction | Enthalpy Flow (Gcal/hr) |
| --- | --- | --- |
| 304 | Inlet | -0.0228 |
| Total Inlet | — | -0.0228 |
| 304B | Outlet | -0.0226 |
| Total Outlet | — | -0.0226 |
| ΔḤ (Net) | Net | 0.0002 |

Table B — Component-Specific Enthalpy (kcal/kg) per stream:

Stream 304 (Inlet) — T = 30.00 °C, P = 1.2000 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| H₂O | -3,209.8410 |
| CO₂ | -2,137.7081 |
| Al(OH)₃ | -3,924.6770 |
| Fe₂O₃ | -1,232.8427 |
| SiO₂ | -3,621.7265 |
| TiO₂ | -2,825.9645 |
| K₂O | -920.4180 |
| Na₂O | -1,598.3095 |
| MgO | -3,565.7368 |
| CO | -942.6908 |
| Fe₃O₄ | -1,153.6674 |
| Al₂O₃ | -3,927.0516 |
| N₂ | 1.1683 |

Stream 304B (Outlet) — T = 117.52 °C, P = 5.0000 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| H₂O | -3,173.4600 |
| CO₂ | -2,119.5271 |
| Al(OH)₃ | -3,899.7199 |
| Fe₂O₃ | -1,218.1698 |
| SiO₂ | -3,604.5843 |
| TiO₂ | -2,810.6794 |
| K₂O | -901.0739 |
| Na₂O | -1,573.7370 |
| MgO | -3,544.9580 |
| CO | -920.9481 |
| Fe₃O₄ | -1,138.7762 |
| Al₂O₃ | -3,908.9827 |
| N₂ | 22.8706 |

#### BH-101 — Baghouse Filter

Unit BH-101 (Baghouse Filter, Gas-Solid Separator) is classified as thermally neutral / adiabatic based on the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -0.0226 Gcal/hr and the total outlet enthalpy flow is -0.0226 Gcal/hr, giving a net ΔḤ of 0.0000 Gcal/hr.

Table A — Stream Enthalpy Flow Summary:

| Stream / Term | Direction | Enthalpy Flow (Gcal/hr) |
| --- | --- | --- |
| 304B | Inlet | -0.0226 |
| Total Inlet | — | -0.0226 |
| BH-AIR | Outlet | -0.0057 |
| BH-DUST | Outlet | -0.0170 |
| Total Outlet | — | -0.0226 |
| ΔḤ (Net) | Net | 0.0000 |

Table B — Component-Specific Enthalpy (kcal/kg) per stream:

Stream 304B (Inlet) — T = 117.52 °C, P = 5.0000 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| H₂O | -3,173.4600 |
| CO₂ | -2,119.5271 |
| Al(OH)₃ | -3,899.7199 |
| Fe₂O₃ | -1,218.1698 |
| SiO₂ | -3,604.5843 |
| TiO₂ | -2,810.6794 |
| K₂O | -901.0739 |
| Na₂O | -1,573.7370 |
| MgO | -3,544.9580 |
| CO | -920.9481 |
| Fe₃O₄ | -1,138.7762 |
| Al₂O₃ | -3,908.9827 |
| N₂ | 22.8706 |

Stream BH-AIR (Outlet) — T = 117.52 °C, P = 4.9975 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| H₂O | -3,173.4580 |
| CO₂ | -2,119.5269 |
| CO | -920.9482 |
| N₂ | 22.8705 |

Stream BH-DUST (Outlet) — T = 117.52 °C, P = 4.9975 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| Al(OH)₃ | -3,899.7199 |
| Fe₂O₃ | -1,218.1698 |
| SiO₂ | -3,604.5843 |
| TiO₂ | -2,810.6794 |
| K₂O | -901.0739 |
| Na₂O | -1,573.7370 |
| MgO | -3,544.9580 |
| Fe₃O₄ | -1,138.7762 |
| Al₂O₃ | -3,908.9827 |

#### MS-401 — Magnetic Separator

Unit MS-401 (Magnetic Separator, Magnetic Separator) is classified as thermally neutral / adiabatic based on the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -2.0429 Gcal/hr and the total outlet enthalpy flow is -2.0429 Gcal/hr, giving a net ΔḤ of 0.0000 Gcal/hr.

Table A — Stream Enthalpy Flow Summary:

| Stream / Term | Direction | Enthalpy Flow (Gcal/hr) |
| --- | --- | --- |
| 204 | Inlet | -2.0429 |
| Total Inlet | — | -2.0429 |
| 401 | Outlet | -1.8978 |
| FE3O4 | Outlet | -0.1451 |
| Total Outlet | — | -2.0429 |
| ΔḤ (Net) | Net | 0.0000 |

Table B — Component-Specific Enthalpy (kcal/kg) per stream:

Stream 204 (Inlet) — T = 32.00 °C, P = 1.0000 atm, From: CL-201 → To: MS-401

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| Al(OH)₃ | -3,924.1067 |
| Fe₂O₃ | -1,232.5287 |
| SiO₂ | -3,621.3665 |
| TiO₂ | -2,825.6322 |
| K₂O | -919.9908 |
| Na₂O | -1,597.7727 |
| MgO | -3,565.2905 |
| Fe₃O₄ | -1,153.3510 |
| Al₂O₃ | -3,926.6763 |

Stream 401 (Outlet) — T = 32.00 °C, P = 1.0000 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| Fe₂O₃ | -1,232.5287 |
| SiO₂ | -3,621.3665 |
| Fe₃O₄ | -1,153.3510 |
| Al₂O₃ | -3,926.6763 |

Stream FE3O4 (Outlet) — T = 32.00 °C, P = 1.0000 atm

| Component | Specific Enthalpy (kcal/kg) |
| --- | --- |
| Al(OH)₃ | -3,924.1067 |
| Fe₂O₃ | -1,232.5287 |
| SiO₂ | -3,621.3665 |
| TiO₂ | -2,825.6322 |
| K₂O | -919.9908 |
| Na₂O | -1,597.7727 |
| MgO | -3,565.2905 |
| Fe₃O₄ | -1,153.3510 |
| Al₂O₃ | -3,926.6763 |

#### BN-501 — Mill Feed Bin

Unit BN-501 (Mill Feed Bin, Bin / Storage Vessel) is classified as thermally neutral / adiabatic based on the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -1.8978 Gcal/hr and the total outlet enthalpy flow is -1.8978 Gcal/hr, giving a net ΔḤ of 0.0000 Gcal/hr.

#### P-501 — Process Water Feed Pump

Unit P-501 (Process Water Feed Pump, Pump) is classified as thermally neutral / adiabatic on the basis of the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -3.1562 Gcal/hr and the total outlet enthalpy flow is -3.1562 Gcal/hr, giving a net ΔḤ of 0.0000 Gcal/hr.

Thermally neutral — negligible net enthalpy change across the unit (adiabatic / pass-through).

#### TK-501 — Slurry Conditioning Tank

Unit TK-501 (Slurry Conditioning Tank, Tank / Mixer) is classified as thermally neutral / adiabatic based on the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -7.6463 Gcal/hr and the total outlet enthalpy flow is -7.6463 Gcal/hr, giving a net ΔḤ of -0.0000 Gcal/hr.

Thermally neutral — negligible net enthalpy change across the unit (adiabatic / pass-through).

#### P-502 — Slurry Feed Pump

Unit P-502 (Slurry Feed Pump, Pump) is classified as thermally neutral / adiabatic based on the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -7.6463 Gcal/hr and the total outlet enthalpy flow is -7.6463 Gcal/hr, giving a net ΔḤ of 0.0000 Gcal/hr.

Thermally neutral — negligible net enthalpy change across the unit (adiabatic / pass-through).

#### ML--501 — Ball Mill

Unit ML--501 (Ball Mill, Mechanical Size-Reduction Unit) is classified as thermally neutral / adiabatic based on the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -7.6463 Gcal/hr and the total outlet enthalpy flow is -7.6461 Gcal/hr, giving a net ΔḤ of 0.0002 Gcal/hr.

Thermally neutral — negligible net enthalpy change across the unit (adiabatic / pass-through).

#### P-503 — Mill Discharge Pump

Unit P-503 (Mill Discharge Pump, Pump) is classified as thermally neutral / adiabatic based on the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -7.6461 Gcal/hr and the total outlet enthalpy flow is -7.6461 Gcal/hr, giving a net ΔḤ of 0.0000 Gcal/hr.

Thermally neutral — negligible net enthalpy change across the unit (adiabatic / pass-through).

#### HC-101 — Hydrocyclone Classifier

Unit HC-101 (Hydrocyclone Classifier, Classifier) is classified as thermally neutral / adiabatic on the basis of the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -7.6461 Gcal/hr and the total outlet enthalpy flow is -7.6460 Gcal/hr, giving a net ΔḤ of 0.0001 Gcal/hr.

Thermally neutral — negligible net enthalpy change across the unit (adiabatic / pass-through).

#### TK-601 — Leach Feed Tank

Unit TK-601 (Leach Feed Tank, Tank (Surge Vessel)) is classified as thermally neutral / adiabatic on the basis of the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -5.0536 Gcal/hr and the total outlet enthalpy flow is -5.0536 Gcal/hr, giving a net ΔḤ of -0.0000 Gcal/hr.

Thermally neutral — negligible net enthalpy change across the unit (adiabatic / pass-through).

#### P-601 — HCl Acid Feed Pump

Unit P-601 (HCl Acid Feed Pump, Pump) is classified as thermally neutral / adiabatic based on the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -5.2316 Gcal/hr and the total outlet enthalpy flow is -5.2316 Gcal/hr, giving a net ΔḤ of 0.0000 Gcal/hr.

Thermally neutral — negligible net enthalpy change across the unit (adiabatic / pass-through).

#### R-601 — Acid Leach Reactor

Unit R-601 (Acid Leach Reactor, Reactor (Kinetic)) is classified as exothermic based on the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -10.2852 Gcal/hr and the total outlet enthalpy flow is -10.2644 Gcal/hr, giving a net ΔḤ of 0.0208 Gcal/hr.

Unit is a net energy source: 0.0208 Gcal/hr of enthalpy is released by the process (exothermic, or heat is rejected to a cooling utility).

#### FP-601 — Filter Press / Thickener

Unit FP-601 (Filter Press / Thickener, Solid-Liquid Separator) is classified as thermally neutral / adiabatic based on the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -10.2644 Gcal/hr and the total outlet enthalpy flow is -10.2644 Gcal/hr, giving a net ΔḤ of -0.0000 Gcal/hr.

Thermally neutral — negligible net enthalpy change across the unit (adiabatic / pass-through).

#### H-701 — CaAl₂O₄ Reagent Hopper

Unit H-701 (CaAl₂O₄ Reagent Hopper, Bin / Dosing Hopper) is classified as thermally neutral / adiabatic based on the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -2.4168 Gcal/hr and the total outlet enthalpy flow is -2.4168 Gcal/hr, giving a net ΔḤ of 0.0000 Gcal/hr.

Thermally neutral — negligible net enthalpy change across the unit (adiabatic / pass-through).

#### R-701 — Precipitation Reactor

Unit R-701 (Precipitation Reactor, Reactor) is classified as endothermic based on the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -11.8062 Gcal/hr and the total outlet enthalpy flow is -12.0515 Gcal/hr, giving a net ΔḤ of -0.2453 Gcal/hr.

Unit is a net energy sink: 0.2453 Gcal/hr of enthalpy is absorbed by the process (endothermic; heat must be supplied by a utility, e.g. steam or fired duty).

#### R-702 — PAC Polymerisation Reactor

Unit R-702 (PAC Polymerisation Reactor, Reactor) is classified as exothermic based on the Aspen Plus stream enthalpy data. The total inlet enthalpy flow is -12.0515 Gcal/hr and the total outlet enthalpy flow is -7.9107 Gcal/hr, giving a net ΔḤ of 4.1408 Gcal/hr.

Unit is a net energy source: 4.1408 Gcal/hr of enthalpy is released by the process (exothermic, or heat is rejected to a cooling utility).

### Plant-Wide Energy Balance Summary

The table below consolidates the overall energy balance across all 25 unit operations of the PAC production plant. All values are reported on the Aspen Plus reference state. A positive ΔḤ denotes a net energy release from the unit; a negative ΔḤ denotes a net energy absorption, requiring the addition of utility heat.

| Unit Operation | Total Inlet Ḥ (Gcal/hr) | Total Outlet Ḥ (Gcal/hr) | ΔḤ (Gcal/hr) | Interpretation |
| --- | --- | --- | --- | --- |
| FD-101 — Feed Bin / Storage Hopper | -2.8152 | -2.8152 | 0.0000 | Thermally neutral |
| CRK-201 — Jaw Crusher | -2.8152 | -2.8151 | 0.0000 | Thermally neutral |
| BN-101 — Crushed Ore Bin | -2.8151 | -2.8151 | 0.0000 | Thermally neutral |
| E,BR-201 — Air Pre-heater / Burner | -0.6519 | -0.4830 | 0.1689 | Net energy source (exothermic) |
| RK-201 — Rotary Kiln (Roaster/Calciner) | -3.2981 | -3.5059 | -0.2079 | Net energy sink (endothermic) |
| CL-201 — Product Cooler | -1.9184 | -2.0429 | -0.1245 | Net energy sink (endothermic) |
| E,CL-301 — Off-gas Cooler | -1.5875 | -1.9703 | -0.3827 | Net energy sink (endothermic) |
| CY-301 — Cyclone Separator | -1.3896 | -1.3896 | -0.0000 | Thermally neutral |
| CP-301 — Gas Compressor / Fan | -0.0228 | -0.0226 | 0.0002 | Thermally neutral |
| BH-101 — Baghouse Filter | -0.0226 | -0.0226 | 0.0000 | Thermally neutral |
| MS-401 — Magnetic Separator | -2.0429 | -2.0429 | 0.0000 | Thermally neutral |
| BN-501 — Mill Feed Bin | -1.8978 | -1.8978 | 0.0000 | Thermally neutral |
| P-501 — Process Water Feed Pump | -3.1562 | -3.1562 | 0.0000 | Thermally neutral |
| TK-501 — Slurry Conditioning Tank | -7.6463 | -7.6463 | -0.0000 | Thermally neutral |
| P-502 — Slurry Feed Pump | -7.6463 | -7.6463 | 0.0000 | Thermally neutral |
| ML--501 — Ball Mill | -7.6463 | -7.6461 | 0.0002 | Thermally neutral |
| P-503 — Mill Discharge Pump | -7.6461 | -7.6461 | 0.0000 | Thermally neutral |
| HC-101 — Hydrocyclone Classifier | -7.6461 | -7.6460 | 0.0001 | Thermally neutral |
| TK-601 — Leach Feed Tank | -5.0536 | -5.0536 | -0.0000 | Thermally neutral |
| P-601 — HCl Acid Feed Pump | -5.2316 | -5.2316 | 0.0000 | Thermally neutral |
| R-601 — Acid Leach Reactor | -10.2852 | -10.2644 | 0.0208 | Net energy source (exothermic) |
| FP-601 — Filter Press / Thickener | -10.2644 | -10.2644 | -0.0000 | Thermally neutral |
| H-701 — CaAl₂O₄ Reagent Hopper | -2.4168 | -2.4168 | 0.0000 | Thermally neutral |
| R-701 — Precipitation Reactor | -11.8062 | -12.0515 | -0.2453 | Net energy sink (endothermic) |
| R-702 — PAC Polymerisation Reactor | -12.0515 | -7.9107 | 4.1408 | Net energy source (exothermic) |

# Equipment specification

## Solid Handling and Comminution Infrastructure

The physical beneficiation and sequential size reduction of the raw bauxite ore are critical preliminary operations. The specific surface area of the mineral matrix dictates the thermodynamic efficiency and reaction kinetics of the subsequent pyrometallurgical roasting and hydrometallurgical leaching phases. The comminution circuit is designed to progressively fracture and grind the run-of-mine ore into a microscopic powder, maximizing the interfacial area available for chemical attack.

### Primary Jaw Crusher (Block: CRK-201)

The initial size reduction of the incoming raw bauxite ore is executed by a heavy-duty, continuous jaw crusher designated as Block CRK-201. This unit operates on the fundamental principles of Kick's Law of comminution, which postulates that the energy required to reduce the size of a given quantity of material is proportional to the logarithmic ratio of the initial and final particle sizes. The crusher serves to fracture the bulk ore into a manageable particle size distribution suitable for downstream secondary milling.

The crusher is specified to process a continuous inlet stream (Stream 102) with a total mass flow rate of 755.987 kg/hr. Operating with a specified mass-specific power consumption of 4,000.00 J/kg, the unit utilizes a Bond Work Index of 43,051.5 J/kg for the solid substream, reflecting the considerable hardness and high crushing resistance characteristic of unweathered bauxite. A single unit is sufficient to handle this capacity.

The inlet material is characterized by a D80 (the particle diameter at which 80% of the mass is smaller) of 0.98 mm, which is systematically reduced to an outlet D80 of 88 μm. The Sauter mean diameter undergoes a shift from 0.9358 μm at the inlet to 0.9122 μm at the outlet.

Thermodynamically, the process of mechanically fracturing the mineral lattice converts kinetic energy into thermal energy which is reflected in the slight enthalpy variation. The relative difference in the total balance is an infinitesimal -12.84 μW, validating the energy conservation of the comminution model. Due to the highly abrasive nature of high-silica bauxite, the materials of construction for the crushing jaws and internal wear plates must utilize high-manganese austenitic steel (Hadfield steel) or hard-faced chromium-molybdenum alloys to mitigate rapid mechanical degradation and ensure a continuous operational lifecycle.

| Parameter | Value |
| --- | --- |
| Capacity | 755.987 kg/hr |
| Enthalpy (In) | -3.27 MW |
| Maximum no. Of flash iterations | 30 |
| Larger than 80% of inlet mass | 0.98038 mm |
| Larger than 80% of outlet mass | 8.84771 μm |
| D80 reduction ratio | 11.0806 |
| D50 reduction ratio | 12.0452 |
| Sauter diameter at inlet meter | 0.935837μm |
| Sauter diameter at outlet meter | 0.912178 μm |
| Material of construction | Austenitic steel |

Figure 5-1 Design summary for primary jaw crusher, CRK-201

Figure 5-2 Particle size distribution of the input and output of the Jaw Crusher, CRK-201

### Solid Handling Hopper and Mixer (Block: H-301)

This unit is designed to continuously process an inlet stream of 687.215 kg/hr. The thermodynamic calculation parameters specify a two-phase flash operation with a maximum of 30 iterations and a convergence tolerance of 0.0001. The unit operates isobarically at the minimum of the inlet stream pressures. The mass and energy balances are completely neutral, with both inlet and outlet streams registering a mass of 687.215 kg/hr and an enthalpy of -2.81 MW. A single unit is sufficient. The vessel is constructed of standard carbon steel and equipped with internal pneumatic fluidisers or mechanical paddle agitators to prevent rat-holing, bridging, and the agglomeration of the hygroscopic calcium aluminate powder.

## High-Temperature Pyrometallurgical Pre-Treatment

### High-Temperature Fabric Filter Baghouse (Block: BH-101)

The particulate-laden flue gases exiting the roasting circuit at elevated temperatures must be rigorously filtered. This serves a dual purpose: recovering valuable, entrained bauxite fines and ensuring strict compliance with atmospheric emission limits. The Baghouse (Block: BH-301) is engineered to process the complex vapor-solid stream at a total mass flow rate of 179.600 kg/hr, separating it into a clean flue gas stream and a recovered particulate matter stream. One total unit is specified for this operation.

#### Filtration Geometry and Active Area Calculations

The physical geometry of the filtration unit is calculated based on strict resistance models and spatial requirements. The baghouse comprises 2 distinct filtration cells, operated under a staggered cleaning regime where 1 cell undergoes continuous online cleaning (pulsing) at any given moment, leaving 1 cell actively filtering.

Each cell contains 26 cylindrical filter bags. With a specified bag diameter (D) of 0.15400 m and an individual surface area (Abag) of 1.48000 m², the required length (L) of each bag is implicitly designed at approximately 3.06 m, derived from the standard cylinder surface area equation:

Abag=π⋅D⋅L

Because only one cell is actively filtering the process gas at any given time, the total active filtration area (Aactive) is governed entirely by the online bags:

Aactive=Nactive_bags ⋅ Abag

Aactive=26⋅1.48= 38.4800 m2

To accommodate these two cells, their respective hoppers, and maintenance clearances, spatial allocation models require a total floor footprint of 7.02928 m².

#### Gas Velocity and Volumetric Flow

While the initial design velocity was specified at 0.025 m/s, the actual calculated operational gas velocity (v) is reduced to a 0.0057824 m/s. This extremely slow face velocity guarantees maximum particle interception and minimizes interstitial penetration, preventing the premature blinding of the filter media.

Using the active filtration area and the actual gas velocity, the operating volumetric flow rate (Q) handled by the active cell is calculated as:

Q = Aactive⋅v

Q = 38.4800 m2⋅0.0057824 m/s ≈ 0.2225 m3/sec

#### Pressure Drop (ΔP) and Cleaning Cycle Modelling

The filtration cycle involves a filtering time (t) of 30 seconds followed by an equivalent cleaning time of 30 seconds. To determine the maximum allowable pressure-drop across the baghouse just before the cleaning pulse is triggered, the system relies on the classical empirical resistance model for fabric filters:

ΔPtotal= ΔPcloth+ ΔPcake

The pressure-drop inherent to the clean filter cloth (ΔPcloth) is modelled at 0.0024673 atm (approximately 250.0 Pa). The pressure-drop across the accumulating filter cake (ΔPcake) increases dynamically as dust builds up, calculated using the specific dust resistance parameter (K2), the filtration velocity (v), and the areal dust loading (W):

ΔPcake=K2⋅v⋅W

The areal dust loading (W, in kg/m²) after 30 seconds of filtration is a function of the inlet solids concentration (Ci = 0.031191 kg/m3):

W=Ci⋅v⋅t

W = 0.031191⋅0.0057824⋅30 ≈ 0.00541 kg/m2

Applying the modeled dust resistance parameter (K2 = 60, 000.0 s−1):

ΔPcake=60000.0⋅ 0.0057824 ⋅ 0.00541 ≈ 1.877 Pa

Converting this transient cake resistance to atmospheres (1 atm = 101, 325 Pa), the cake contributes an additional 0.0000185 atm of resistance. Summing these factors validates the system's maximum allowable pressure drop constraint:

ΔPtotal=0.0024673 atm+0.0000185 atm = 0.0024858 atm

#### Collection Efficiency and Mass Balance

The unit operates with an assumed overall collection efficiency (η) of 1.00 (100%). Therefore, the inlet gas, featuring a solids concentration of 0.031191 kg/m³, is entirely stripped of particulate matter, effectively reducing the outlet solids concentration to 0.0 kg/m³ at the vapor exhaust. Based on the calculated volumetric flow rate (0.2225 m3/sec), the baghouse successfully recovers approximately 25.0 kg/hr of bauxite fines for recirculation or product capture.

Material of Construction Warning: Because the inlet gases originate from an 870 °C roasting process, even with upstream heat recovery, the temperature will exceeded the thermal degradation limits of standard polyester, acrylic, or aramid fabric filter bags. To prevent catastrophic bag failure and subsequent combustion, the filtration media utilises advanced low-density ceramic filter candles (Lang et al., 2021). Ceramic candle filters provide a rigid, fragile-yet-stable structural barrier capable of operating continuously at temperatures exceeding 800 °C up to 1000 °C while maintaining >99.0% collection efficiency. The structural housing of the baghouse is fabricated from thermally insulated carbon steel.

## Hydrometallurgical Reaction and Leaching Systems

The core chemical transformations required to synthesize poly-aluminium chloride occur within the wet hydrometallurgical circuit. These continuous reactors manage the aggressive dissolution of roasted bauxite in concentrated hydrochloric acid, followed by precise stoichiometric basification and carefully controlled polymerization. The convergence of high operating temperatures, elevated pressures, and highly concentrated halide solutions creates a localized environment requiring the most stringent material specifications in the entire facility.

### Polymerisation and Maturation Tank (Block: R-702)

The final chemical synthesis occurs in the Polymerisation Tank (Block: R-702), where the freshly basified, purified liquor undergoes controlled thermal aging to properly form the final Al2OHnCl6-nm poly-aluminium chloride complexes. This vessel is modelled as a continuous CSTR. One unit handles the final product stream.

The maturation process relies on a prolonged residence time of 3 hours to allow the slow cross-linking of the aluminium hydroxide-chloride polymers to reach equilibrium. To accommodate the 3,580.33 kg/hr continuous flow over 3 hours, the reactor demands a massive physical volume of 8.2535 cubic meters. The operating temperature is tightly controlled at 75.000 °C at 1.0000 atm. The reaction is completely liquid-phase, with a vapor volume fraction of 0.0000 and a liquid volume fraction of 0.99492, taking up 8.2116 cubic meters of the vessel, alongside a minor precipitated salt volume of 5.8914×10-4 m3.

The complex power-law maturation kinetics exhibit a pre-exponential factor of 1.93×10-10 and an activation energy of 5.00×108 J/kmol based on the AlOH3 solid substream. This continuous cross-linking reaction demands a massive influx of thermal energy, registering a required reactor heat duty of (4.81 MW). The mass balance demonstrates a molar reduction from 139.819 kmol/hr to 114.023 kmol/hr.

Because this vessel contains the finished liquid PAC long-term structural integrity against acid attack is paramount. The massive 8.25 cubic meter vessel must be constructed of premium Fiberglass Reinforced Plastic (FRP). The tank's interior must feature a thick, resin-rich corrosion barrier utilizing high-grade vinyl ester to prevent acidic chemical wicking into the structural fiberglass layers. Furthermore, the tank geometry must feature a conical bottom. This ensures complete drainage and eliminates the accumulation of standing, concentrated liquid at the base, which accelerates localized stress-corrosion cracking at the knuckle radius commonly seen in flat-bottom designs.

## Separation and Clarification Equipment

To achieve a commercially viable, pure liquid PAC product, the unreacted gangue materials must be continuously separated from the dense aqueous phase. This relies on an optimized sequence of gravimetric, mechanical, and magnetic separation technologies.

### Liquid-Solid Hydrocyclone Array (Block: HC-501)

The hydrocyclone array is the major and most important liquid-solid separation unit for the hot, abrasive, highly acidic post-leach slurry. The feed of the system is a heavy slurry flow of 2083.54 kg/hr (80.7189 kmol/hr). The mathematical model provides a round geometry for the Rietema classification, which is optimized geometrically for the ultra-fine classification necessary to separate the abrasive kaolinite and reactive silica slimes from the valuable bauxite liquor.

The system is based on 4 parallel cyclones with a diameter of 0.30 m for cylinder. The hydrocyclone flow properties are given by a Bradley efficiency correlation with an inlet liquid velocity of 0.015890 m/sec and a fractional pressure drop of 1.334×10-5 atm. The liquid volumetric flow per cyclone can be determined to be 0.31701 m3/h.

The physical separation is made possible using strong centrifugal forces resulting in a calculated overall separation efficiency of 0.33832. The particle cut diameter that gives 50% collection efficiency (d50c) is very accurate, with a value of 0.00082848 m for the solid substream.

Abrasive wear from the high velocity silica particles in the fluid is always occurring on the inside of the hydrocyclone, and the chemical attack from any residual hydrochloric acid in the fluid will take the rest of the story. As a result, the internal wetted surfaces, apex and vortex finders must be injection moulded from high density, corrosion resistant polyurethane. In high-abrasion applications, such as in a cyclone, polyurethane offers far greater elastomeric wear resistance, which will maintain the geometric integrity of the cyclone.

### Mechanical Filter Press (Block: FP-601)

To extract the absolute maximum volumetric yield of valuable PAC liquor from the densified solid waste matrix rejected by the hydrocyclones, a robust mechanical Filter Press (Block: FP-601) is deployed. Modelled as a single ideal generic separator (SEP), the unit receives a massive, combined feed of 3,177.94 kg/hr (147.296 kmol/hr).

The press executes an absolute, mathematically ideal split. It forces the valuable aqueous phase components (H2O, H3O+, Cl-, Al3+, Fe3+ ) through the filtration media with a split fraction of 1.00000 into Stream 16, while completely retaining the solid gangue substream (AlOH3, Fe2O3, SiO2, Kaolinite, TiO2) into the reject stream designated SIO2. The unit features a negligible internal heat duty (-0.64413 W), validating its function as a purely mechanical separator devoid of thermal phase changes.

The filter plates must be moulded from solid, high-density polypropylene (HDPP), which offers excellent structural rigidity and total resistance to acidic chlorides. The structural skeleton and hydraulic rams of the press should be constructed of heavy-duty, epoxy-coated carbon steel. The filter cloths themselves must utilize tightly woven, multi-filament polypropylene or PTFE fibres to withstand the acidic environment without degrading, ensuring a crystal-clear PAC filtrate.

| Category | Parameter | Value / Description |
| --- | --- | --- |
| General Details | Equipment Type | Mechanical Filter Press |
|  | Block Designation | FP-601 |
|  | Process Model | Single ideal generic separator (SEP) |
|  | Purpose | Extract maximum volumetric yield of PAC liquor from densified solid waste. |
| Feed & Flow Data | Total Feed Mass Flow | 3,177.94 kg/hr |
|  | Total Feed Molar Flow | 147.296 kmol/hr |
| Separation Dynamics | Aqueous Split Fraction | 1.00 (100% routed to Stream 16) |
|  | Solid Retention Fraction | 1.00 (100% retained in SIO2 stream) |
|  | Filtrate Stream (Stream 16) | Valuable aqueous phase components [Specifics not specified] |
|  | Reject Stream (SIO2) | Solid gangue substream |
|  | Internal Heat Duty | Negligible |
| Materials of Construction | Filter Plates | Solid, high-density polypropylene (HDPP) |
|  | Skeleton & Hydraulic Rams | Heavy-duty, epoxy-coated carbon steel |
|  | Filter Cloths | Tightly woven, multi-filament polypropylene or PTFE fibres |

Figure 5-3 Design summary for Mechanical Filter Press, FP-601

### High-Intensity Magnetic Separator (Block: MS-401)

To selectively eliminate the reduced magnetic gangue—primarily the magnetite (Fe3O4) deliberately generated during the pyrometallurgical roasting phase—a wet, high-intensity Magnetic Separator (Block: MS-401) is integrated into the intermediate slurry line. Processing a feed of 602.372 kg/hr (7.76112 kmol/hr), the unit is modelled as an ideal separator (SEP) operating under two-phase TP flash conditions at an isobaric pressure drop of 0.0 atm. One unit is sufficient.

The magnetic separation mechanics achieve a highly selective 0.95 (95%) recovery rate of the highly susceptible Fe3O4 solid phase into the reject stream, alongside minor entrained fractions of Fe2O3 (0.05) and SiO2 (0.02). The remaining 95% of the non-magnetic Fe2O3 and 98% of the SiO2 and Al2O3 safely pass through the magnetic field into the product stream (5A). The unit operates physically and thermally neutral. The wetted rotating drum, stationary tank, and internal magnetic matrix elements must be fabricated from highly passivated 316L stainless steel or titanium to resist the acidic slurry during the brief contact time.

## HEAT EXCHANGER NETWORK (HEN)

The overarching thermal efficiency and economic viability of the entire plant are maximized through an extensive, highly integrated network of six critical heat exchangers. These units are tasked with recovering high-grade waste heat from the pyrometallurgical section to intensely pre-heat combustion air and utility water, while precisely regulating the temperatures of the violently corrosive hydrometallurgical acidic streams.

### Air and Flue Gas Recovery Exchangers (Blocks: CL-201, HX-102, HX-103)

This primary cluster of heat exchangers is dedicated entirely to clean air and hot flue gas heat recovery, operating free from the constraints of acidic chloride corrosion.

#### High-Temperature Gas Exchanger Specifications

| Parameter | CL-201 | HX-102 | HX-103 |
| --- | --- | --- | --- |
| Model Type | Multiple Tube Pass Shell & Tube | Counter-current | Counter-current |
| Number Required | 1 | 1 | 1 |
| Heat Duty (Watts) | 153,040.44 | 27,077.68 | 117,263.40 |
| Hot Stream Inlet Temp (°C) | 870.00 | 594.04 | 650.00 |
| Hot Stream Outlet Temp (°C) | 60.00 | 563.58 | 263.47 |
| Cold Stream Inlet Temp (°C) | 43.65 | 563.58 | 32.00 |
| Cold Stream Outlet Temp (°C) | 563.58 | 650.00 | 95.00 |
| Required Area (sqm) | 3.3441 | 0.3323 | 0.3729 |
| Average U (W/sqm-K) | 850 | 850 | 850 |
| LMTD (Corrected) (K) | 53.8409 | 95.8619 | 369.9551 |
| Number of Shells | 5 | 1 | 1 |

Figure 5-4 Design summary for high temperature Gas Exchangers

- CL-201: This unit is used to remove sensible heat from the 870 °C solid/gas stream to energise compressed air with its pre-heat to 563.58 °C with a violent pre-heat. This large temperature difference means that the am LMTD correction factor is 0.5440, which is accomplished by the physical arrangement of 5 independent shells, giving a required 3.3441 sqm of area. The inlet gas is forced to hit 870 °C, which means that normal metals will melt or structurally yield. The hot side tubes and sheets should be made of special refractory lined carbon steel or super alloy such as Incoloy 800H.

- HX-102 & HX-103: These are small, very efficient single shell counter current units. HX-102 utilizes a tiny 0.3323 sqm area to extract 27 kW of heat, while HX-103 extracts 117.2 kW over 0.3729 sqm, dropping the 650 °C air down to 263.47 °C to heat process water. Such gas-to-water applications can be quite well managed by a high grade austenitic stainless steel, such as 310S or 316L.

### Acidic and Corrosive Heat Exchangers (Blocks: HX-201, HX-601, HX-203)

This secondary cluster handles the highly aggressive, chloride-rich liquid streams entering and exiting the main leaching and polymerization reactors. The metallurgical constraints here are absolute; a single misapplication of material will result in immediate, catastrophic failure.

| Parameter | HX-201 | HX-601 | HX-203 |
| --- | --- | --- | --- |
| Model Type | Counter-current | Counter-current | Counter-current |
| Number Required | 1 | 1 | 1 |
| Heat Duty (Watts) | 62,839.46 | 48,680.05 | 155,696.10 |
| Hot Stream Inlet Temp (°C) | 95.00 | 95.00 | 120.00 |
| Hot Stream Outlet Temp (°C) | 33.62 | 36.89 | 60.00 |
| Cold Stream Inlet Temp (°C) | 32.01 | 35.43 | 34.57 |
| Cold Stream Outlet Temp (°C) | 80.00 | 80.00 | 99.86 |
| Required Area (sqm) | 12.3279 | 9.8510 | 25.6241 |
| LMTD (Corrected) (K) | 5.9969 | 5.8137 | 7.1484 |
| Zone Analysis Notes | 1 Zone (Liquid/Liquid) | 1 Zone (Liquid/Liquid) | 2 Zones (Vapor fraction 0.0598 generated on cold side) |

Figure 5-5 Design summary for Acidic and corrosive heat exchangers

- The unit is required to transfer 62.8 kW of thermal energy, to pre-heat the highly concentrated hydrochloric acid feed stream (HCL-2) from 32.01 °C to 80.00 °C, with a required area of 12.3279 sqm, and an extremely tight LMTD of 5.9969 K, making metallic materials totally unsuitable against hot HCL. This exchanger should be defined as Impervious Graphite Block or Graphite Plate heat exchanger (e.g., SGL Carbon Diabon). Graphite is uniquely suitable for heat transfer in boiling hydrochloric acid because it has extremely high thermal conductivity and absolute chemical inertness, allowing for trouble-free heat transfer without leak.

- This is an operating duty of 48.6 kW and requires an area of 9.8510 sqm and LMTD of 5.8137 K depending on the type of chlorides in Stream 11, either a secondary graphite block unit or a heavy-duty glass-lined double-pipe heat exchanger is needed. The glass lined, tubular exchangers are ideal for maximum heat transfer and a complete separation between the aggressive media and the carbon steel construction.

- ●	HX-203: The most complicated and largest exchanger in acid circuit. It delivers a very large energy transfer of 155.6 kW, cooling the very corrosive, abrasive high pressure post-leach slurry (Stream 13) from 120.00 °C to 60.00 °C and simultaneously heating the process water coming into the plant to 99.85 °C. The total area is enormous at 25.6241 sqm with an LMTD of 7.1484 K. Thermodynamic zone analysis indicates that the cold side partially boils under the high heat, creating a vapor fraction of 0.0598 and thus two thermal zones. The sensible liquid heating in Zone 1 is 51.5 kW and the boiling heat transfer in Zone 2 is 104.1 kW. This unit is used to process a very abrasive bauxite slurry at temperatures of 120 °C, which leads to rapid mechanical erosion-corrosion on the standard graphite. The specification therefore calls for a Silicon Carbide (SiC) block heat exchanger or a very special Tantalum lined tube and shell exchanger to resist both the chemical turbulence and the physical abrasion.

## Fluid Transport: Pumping Infrastructure

The motive hydraulic force required to drive the heavy slurries, aggressive acids, and pure process water through the extensive heat exchanger networks and high-pressure reactors is provided by a network of heavy-duty centrifugal pumps. The proper selection of these pumps requires a delicate balance between resisting the intense chemical corrosion of the chlorides and surviving the severe mechanical abrasion of the dense bauxite slurries.

| Parameter | P-101 | P-501 | P-502 | P-201 |
| --- | --- | --- | --- | --- |
| Fluid Handled | Process Water | Intermediate Slurry | High-Pressure Slurry | Concentrated HCl |
| Number Required | 1 | 1 | 1 | 1 |
| Mass Flow (kg/hr) | 833.333 | 2,083.54 | 2,083.54 | 1,800.00 |
| Volumetric Flow (m³/hr) | 0.83741 | 1.26801 | 1.26808 | 1.55969 |
| Inlet/Outlet Mole Flow (kmol/hr) | 46.2570 | 80.7189 | 80.7189 | 83.7402 |
| Pressure Change (atm) | 0.50000 | 0.10249 | 0.87385 | 0.49346 |
| Head Developed (meters) | 5.19140 | 1.06526 | 9.08340 | 4.41789 |
| NPSH Available (meters) | 9.89356 | 9.78119 | 10.8417 | 25.7791 |
| Fluid Power (Watts) | 11.7848 | 3.65763 | 31.1885 | 21.6624 |
| Brake Power / Net Work (Watts) | 15.7130 | 6.65024 | 56.7064 | 28.8832 |
| Pump Efficiency Used | 0.75000 | 0.55000 | 0.55000 | 0.75000 |
| Driver Efficiency | 1.00000 | 1.00000 | 1.00000 | 1.00000 |

Figure 5-6 Comprehensive Pump Performance Specifications

- P-101: Process water of 0.837 m3/hr is treated for clean process water. The thermodynamic risk of cavitation at the impeller eye is almost unthinkably low with a high NPSHa of 9.89 m. The 316 stainless steel centrifugal pumps with single mechanical seal are just fine at standard operating efficiency of 0.75000.

- P-501 and P-502: These pumps can pump high abrasiveness, dense bauxite and intermediate gangue slurries. The most challenging task is P-502, which must pump the dense slurry pressure up by almost 0.9 atm (9.08 meters of head) to propel the slurry into the high-pressure leaching reactor. The simulation deliberately reduces the efficiency parameter to 0.55000 to take into consideration the severe viscous drag and rheological issues associated with pumping heavy heterogeneous solid-liquid mixtures. Thin-vane metallic impellers would be destroyed by erosion within days. Hence, these pumps shall be precisely described as heavy duty vertical or horizontal centrifugal slurry pump. The internal casing and open impellers should be coated with thick, replaceable natural rubber elastomeric liner or hardened 27% Chrome Alloy.

- P-201 is a dedicated line exclusively for pumping concentrated and highly toxic hydrochloric acid to the leaching reactor - main reactor. Requires 28.8W of brake power to produce 4.41m of head with a flow rate of 1.55m3/hr. Absolute uncompromising corrosion resistance is required for pumping HCl. EFTE, PFA, or solid PTFE lined, seal-less centrifugal pumps with a magnetic drive are the industry standard for this application. There are no susceptible mechanical seals required and there is no toxic acid vapours lost to the environment by Mag-drive pumps. The very large NPSHa (25.77 meters) resulting from the simulation guarantees that operation of the valve is very smooth and free from cavitation even with the use of acids having relatively high vapor pressures.

## Fluid Transport: Piping Network and Rheological Guidelines

The physical integration of the various unit operations requires a sophisticated piping network. The primary operational challenges are dichotomous: preventing the settling of heavy solid particles in the slurry lines (which causes plugging and catastrophic process halting) while simultaneously avoiding excessively high fluid velocities (which accelerates kinetic erosive wear on the pipe walls, particularly at elbows, bends, and tees).

| Fluid Service | Pipe Material Specification | Design Velocity (m/s) | Fitting Specifications | Application Notes |
| --- | --- | --- | --- | --- |
| Bauxite Slurry (Heterogeneous) | Carbon Steel w/ Natural Rubber Lining or High-Density Polyurethane | 1.50 – 2.00 | Schedule 160, Long Radius Sweeps, Capped Tees | Velocity must be strictly maintained 0.3 to 0.5 m/s above the critical settling velocity calculated via Durand or Wasp equations. Avoid dead legs. |
| Concentrated HCl (up to 38%) | Dual-Laminate CPVC with structural FRP Wrap, or PTFE-Lined Steel | 1.00 – 1.50 | Flanged, PTFE Envelope Gaskets | CPVC Schedule 80 is acceptable for ambient up to 82 °C. Above 82 °C, PTFE lined steel is mandatory. |
| Liquid PAC Product (Aqueous) | Fiberglass Reinforced Plastic (FRP) with premium Vinyl Ester inner liner | 1.50 – 2.50 | Standard FRP Fittings, bonded | FRP provides exceptional resistance to chloride ion attack, low pH, and high specific gravity. |
| Hot Gases / Roaster Off-Gas | Refractory-Lined Carbon Steel / Inconel Alloys | 15.00 – 25.00 | High-Temp Expansion Joints Required | Must safely accommodate massive thermal expansion stresses and cyclic fatigue from 870 °C continuous streams. |
| Process Water / Utility | Schedule 40 Carbon Steel, 304 SS, or industrial PVC | 1.50 – 3.00 | Standard Threaded or Welded | General velocity rule of thumb applies: 1.5 + d/10. |

Figure 5-7 Piping Specification and Fluid Dynamics

Hydraulic and Rheological Considerations: Hydraulic design of slurry transport lines from the ball mill, hydrocyclones and main leaching reactors must be extremely precise and based on empirical models of fluid flow. When the bulk fluid velocity is less than the critical settling velocity (which is usually around 1.0 to 1.5 m/s for fine heavy bauxite particles) a sliding bed will rapidly develop along the bottom of the pipe, causing a significant increase in the frictional pressure drop and eventually the line will be completely blocked. On the other hand, if the wind speed is more than 2.0-3.0 m/s, the kinetic erosion-corrosion process will accelerate rapidly, and the pressure limit of the pipe wall will be destroyed exponentially. Thus, it is imperative that all slurry piping is carefully sized so that the velocity of the slurry within the pipe is exactly 1.5–2.0 m/s. Also, direction changes should be carefully designed to avoid regular short radius 90-degree elbows and should use long radius turns or special "capped tees" (which will provide a stationary pocket of slurry to protect and absorb the kinetic energy of the turning fluid).

Chemical corrosion is the only dominant failure mode for the acid and liquid PAC transport lines. Normal metallic piping quickly penetrates and rapidly and catastrophically breaks down concentrated hydrochloric acid. The highly engineered Dual-Laminate pipe is chemically inert due to the impermeable chemical barrier (CPVC) completely enclosed in a load bearing structural barrier of Fiberglass Reinforced Polymer (FRP) material that offers the required mechanical rigidity. For the unavoidable vibration of a pump and the thermal expansion that must occur in all systems, highly engineered PTFE expansion joints must be used alongside all major rotating equipment and large volumetric storage tanks.

## Auxiliary Fluid Storage, Mixing, and Flow Control

The plant relies heavily on multiple auxiliary continuous mixing units, fluid splitters, and volumetric holding tanks to hydraulically balance flows, buffer operational surges, and perfectly homogenize liquid reagents prior to primary reaction.

- Fluid Flow Splitter (Block: SS-201): This unit acts as a precision hydraulic divider, receiving 1,600.00 kg/hr (88.8135 kmol/hr) of heated process water (W-2) and dividing it into two streams: WA-3 (taking a 0.45000 mass fraction split) and WB-3 (taking a 0.55000 mass fraction split). One unit (Number Required: 1) is required. The simulation applies a programmed, realistic pressure drop of 0.0050000 atm across the dividing manifold. Standard PTFE-lined carbon steel construction is appropriate to prevent any long-term internal scaling.

- Inline Continuous Mixers (Blocks: MX-101, MX-201): These units provide continuous, isobaric inline homogenization. MX-101 continuously blends the recovered particulate matter into a unified solid stream (602.372 kg/hr, 7.76112 kmol/hr) with an enthalpy of -0.241552E+07 Watts. MX-201 unifies two distinct liquid water streams totalling 1,370.00 kg/hr (76.0466 kmol/hr) with an enthalpy of -0.602638E+07 Watts. One unit of each (Number Required: 1 per block) is specified. Both mixers operate mathematically at the minimum pressure of their respective inlet streams, ensuring forward flow.

- Volumetric Holding Tanks (Blocks: TK-501, TK-103): Serving as critical hydraulic volumetric buffers, the Mixing Tank (TK-501) safely combines 2,083.54 kg/hr (80.7189 kmol/hr) of aggressive intermediate slurry streams prior to pumping. The Holding Tank (TK-103) buffers 1,377.94 kg/hr (53.3714 kmol/hr) of acidic fluid. One unit of each (Number Required: 1 per block) is utilized. For liquid PAC and acidic precursor storage, these massive tanks must utilize high-density cross-linked polyethylene (XLPE) or filament-wound FRP equipped with a pure vinyl ester corrosion barrier. Standard flat-bottom tanks must be explicitly avoided for slurry and dense PAC service; instead, conical bottoms or Integrally Moulded Flanged Outlets (IMFO) must be utilized to prevent the hazardous accumulation of abrasive sludges and facilitate safe, zero-entry cleaning protocols.

- Pressure Reduction Valve (Block: V-201): This critical control valve handles a massive continuous flow of 3,177.94 kg/hr (147.296 kmol/hr) of liquid fluid. One unit (Number Required: 1) is needed. The unit violently drops the system pressure by a massive 1.96077 atm to safely reach atmospheric downstream conditions (1.00 atm outlet). The enthalpy remains constant at -0.122683E+08 Watts. To completely resist the high-velocity acidic erosion and potential localized cavitation induced by such a massive pressure drop, the internal valve body, plug, and trim must be constructed of solid moulded PTFE, or highly specialized ceramic-lined steel.

# DESIGN OF SPECIFIC EQUIPMENT

## ONE-DIMENSIONAL STEADY-STATE SIMULATION OF A COUNTER-CURRENT ROTARY KILN FOR BAUXITE REDUCTIVE ROASTING – Gyasi-Denteh, Prince Sugar

### Abstract

The present Report contains the full chemical and mechanical engineering analysis on a counter-current rotary kiln designed for the reductive roasting of Awaso deposit bauxite ore. Using a reducing gas (CO-rich), the function of the kiln is to dehydrate gibbsite (AlOH3) to alumina (Al2O3) and reduce Haematite (Fe2O3) to magnetite (Fe3O4). A one-dimensional, steady-state heterogeneous two-temperature model was developed and implemented in the MATLAB environment using the bvp4c solver, which consists of nine coupled ordinary differential equations (ODEs) as a boundary value problem (BVP). The governing equations are for solid-phase mass balances, gas-phase mass balances, coupled energy balances for the gas and solid phases that feature the inclusion of convective and radiative heat transfer, Arrhenius reaction kinetics with harmonic mean transport-kinetic rate limiters, and a dynamic calculation of the fill-degree using Sullivan-Friedman-Maier correlation. The kiln is to be 10 m long and 0.70 m in inner diameter, for which the solid feed rate is about 755.9 kg/h and the reducing gas flow rate is about 607.6 kg/h. The mechanical design includes shell thickness calculation, refractory lining thermal resistance calculation, tyre and riding-ring sizing, and load estimation of the structure. The results show that simultaneous dehydration and selective reductive roasting was found to be feasible in the operating window mentioned.

### INTRODUCTION

#### Background and Motivation

The Awaso bauxite deposit in the Western Region is one of the largest lateritic bauxite deposits in the African continent with proven reserves exceeding 700 million tonnes and the anchor of Ghana's aluminium industry. It is characterized by the presence of high amounts of gibbsite (AlOH3) in addition to Haematite (Fe2O3), silica (SiO2) and titania (TiO2) gangue; the bauxite is the main source feedstock to produce alumina and downstream aluminium. Traditional processing methods involve a lot of energy consumption and chemical usage. Hence thermochemical pre-treatment steps that enhance the processing efficiency in the downstream process are of great interest to both industry and academia.

- Reductive roasting in a rotary kiln is a thermochemically preferred process for achieving two purposes: drying of gibbsite to transition alumina, which decreases the energy demand of the later calcination steps

- Wet separation using magnetic beams to remove iron from the product stream after selective reduction of Haematite to magnetite. Selective reduction of Haematite to magnetite followed by wet magnetic separation to reduce the amount of iron in the product stream. The reducing gas used contains CO and is supplied from a burner or gasifier with a fuel-rich mixture.

This report contains a complete chemical and mechanical engineering design basis for the counter-current rotary kiln of the poly-aluminium chloride (PAC) production plant. The PAC production context is important as the presence of iron impurities in the alumina feedstock can have a direct and adverse impact on the colour and performance of the PAC, which underlies the selective reduction step that converts the paramagnetic Haematite to the ferrimagnetic magnetite for magnetic separation downstream.

#### Scope of This Report

The scope of this document encompasses the following:

- A one-dimensional, steady-state mathematical model of the counter-current rotary kiln, formulated as a two-temperature heterogeneous boundary value problem.

- Derivation and justification of all heat transfer, reaction kinetics, and thermophysical property correlations adopted.

- The full set of governing ordinary differential equations, boundary conditions, and numerical solution strategy.

- Mechanical design verification of the kiln shell, refractory lining, and rotating support structures.

- A comprehensive reference list of primary literature underpinning each modelling decision.

The numerical implementation was developed in MATLAB R2024b and employs the built-in bvp4c solver.

#### Objectives

The specific engineering objectives addressed in this report are:

- To establish a mathematically consistent 1D heterogeneous model that captures the coupled mass and energy transport in a counter-current solid-gas rotary kiln.

- To quantify the axial temperature profiles of the freeboard gas and the solid bed and predict the extent of gibbsite dehydration and Haematite reduction at the kiln discharge.

- To derive all heat transfer coefficients from peer-reviewed correlations applicable to industrial rotary kilns.

- To verify the mechanical integrity of the kiln shell and support structure under process thermal and mechanical loads.

- To provide a complete specification document suitable for procurement and detailed engineering.

### LITERATURE REVIEW

#### Rotary Kiln Technology in Mineral Processing

The rotary kiln has been used for over a hundred years and has been used in the process industries for a variety of applications, including waste incineration, non-ferrous metallurgy, production of titanium dioxide, lime calcination and cement manufacture. As the bauxite processing rotary kiln, the rotary kiln is used as a combined reactor and heat exchanger, and it takes advantage of the counter-current arrangement to recover the heat between the hot gas and the hot solid, realize the efficient recovery of the heat energy.

The engineering description of the rotary kiln transport phenomena was first given by (Sass, 1967), who developed an analytical model for heat transfer in the axial direction of rotary kilns, assuming given profiles of wall and gas temperature. Sass was able to show that the overall gas-to-bed heat flux was controlled by the heat transfer coefficient from the exposed wall, and this result has been repeated in later modelling. The present model uses the correlation approach that is consistent with Sass's parametric definitions but also uses updated heat transfer coefficient expressions from later experimental data.

#### Volumetric Kiln Feeding

Crushed bauxite is extracted from the base of the day bin via a precision Screw Feeder (FD-201). Screw feeders differ profoundly from standard screw conveyors; they operate flood-loaded, meaning the inlet trough is 100% full of material (capacity constrained by the flight volume rather than the feed rate). To accurately meter the solid flow into the hostile environment of the kiln and prevent material bridging in the hopper, the feeder utilizes variable-pitch or twin-tapered flighting.

Furthermore, the screw feeder is equipped with a mechanical rotary airlock valve at its discharge point. This mechanism strictly prevents ambient atmospheric oxygen from leaking into the kiln. Any ingress of extraneous oxygen would rapidly disturb the delicate reducing atmosphere required for the hematite-to-magnetite transformation, resulting in poor magnetic recovery yields later in the process. The rotational speed (N, in RPM) required for the specified mass flow rate (Q, in kg/h) is theoretically calculated using the volumetric displacement equation for a screw flight:

N=Q60⋅π4Ds2-Dp2⋅P⋅ρb⋅α⋅CE

Where Ds represents the outer screw diameter, Dp is the inner pipe shaft diameter, P is the pitch of the screw, ρb is the bulk density of the bauxite, α is the loading ratio (assumed to be 1.0 for a flood-loaded feeder), and CE is the volumetric efficiency factor accounting for material slip.

#### Counter-Current Rotary Kiln Design and Sizing

The principal pyrometallurgical unit operation is the Counter-Current Rotary Kiln (R-201). It comprises a massive, refractory-lined cylindrical steel shell that is slightly inclined to the horizontal and rotated slowly upon trunnion rollers to induce a tumbling, cascading bed flow. The counter-current configuration provides vastly superior thermal efficiency and heat transfer profiles compared to co-current alternatives. The kiln features distinct longitudinal thermal zones: a drying zone to drive off unbound moisture, a high-temperature calcination zone (approximately 700°C) to induce gibbsite phase transitions, and a reduction zone where the carbon monoxide sweeps the bed.

The physical sizing of the rotary kiln relies heavily on determining the requisite Mean Residence Time (MRT) necessary for the solid particles to undergo complete endothermic transformation (Liu & Specht, 2006). For a rotary kiln devoid of internal constriction dams, the standard empirical model originally established by Sullivan and the U.S. Bureau of Mines defines the MRT as an inverse function of the kiln's geometric and dynamic parameters:

MRT=1.77⋅L⋅∅D⋅N⋅S

Where:

- MRT = Mean residence time (minutes)

- L = Internal length of the kiln (m)

- D = Internal diameter of the kiln inside the refractory lining (m)

- ∅ = Dynamic angle of repose of the bauxite bed (degrees)

- N = Rotational speed of the shell (RPM)

- S = Kiln slope or longitudinal inclination (m/m)

To achieve the necessary material hold-up and ensure optimal gas-solid contact time, the kiln slope is strictly designed between 3.0% and 3.5% (approximately 1.7o to 2.0o). The operational speed is maintained exceedingly low, typically 1.0 to 3.0 RPM, to prevent the material from entering a centrifuging regime. At these speeds, the bauxite remains in a gentle cascading regime characterized by a volumetric fill fraction (or bed depth) of 10% to 15%. The ratio of length to diameter (L/D) is optimized to approximately 10:1 to 15:1 to balance the thermal profile against the mechanical stresses of the long span.

#### Burner Stoichiometry and Thermal Integration

Thermal energy is introduced via a specialised dual-feed Kiln Burner (BR-201) located at the lower, solids-discharge end of the kiln. The burner combusts a hydrocarbon fuel (such as natural gas or atomized fuel oil) under strictly controlled sub-stoichiometric conditions. By deliberately starving the primary flame of oxygen, the combustion yields a Carbon Monoxide (CO) enriched, highly reducing atmosphere. This reducing gas sweeps over the cascading bauxite bed, chemically stripping oxygen atoms from the hematite lattice to form magnetite

Because incomplete combustion presents severe deflagration and asphyxiation hazards, the burner is fully integrated with a programmable flame safeguard system and continuous infrared CO analysers to monitor the exhaust gas composition. To maximize the thermal efficiency of the plant, the primary Combustion Air Blower (BL-201) forces atmospheric air through a high-temperature Gas-Gas Air Preheater (E-201), where it absorbs sensible heat from the exiting flue gases before injection into the burner.

#### Product Discharge and Cooling

The calcined and reduced bauxite exits the lower lip of the kiln at temperatures approaching 900°C and falls directly into the Rotary Kiln Cooler (200-CL-001). This auxiliary vessel is a tube-type cooler mounted concentrically to the kiln discharge. It actively cools the hot solids to below 100°C by drafting ambient, counter-current air over the cascading material. In a hallmark of aggressive energy integration, the air heated by the cooling process is subsequently routed to serve as the preheated primary combustion air for the main burner, recovering massive amounts of sensible heat. The cooled solids are finally discharged onto a water-jacketed drag conveyor which provides the final temperature reduction while safely transporting the highly abrasive clinker to the magnetic separation circuit.

#### Two-Temperature Heterogeneous Models

One of the most important modelling issues for any rotary kiln simulation involves handling the solid-bed and gas temperatures. Single-temperature (homogeneous) models assume that there is no distinction between the thermal properties of the gas and solid, which is valid only when the heat transfer resistance between gas and solid is very low compared to the axial heat conduction resistance. The solid bed and freeboard gas regions within a rotary kiln are in different spatial regions with the exposed bed surface and can have differences of several tens to hundreds of Kelvin at any lengthwise position within the bed.

(Boateng & Barr, 1996) proposed a detailed two-temperature model of the rotary kiln which was based on dividing the kiln cross section into two parts, one for the solid bed region (which would occupy a chord of a circular cross section) and another for the freeboard gas region (which would occupy the remaining arc). Their model included for the first time the concept of the covered-wall heat transfer coefficient for heat exchange between the wall arc that is in contact with the bed and the bed itself, a process that is very important for kilns heated at moderate temperatures, where radiative contribution is not significant. The present model adopts the formulation of Boateng-Barr and its rotating-bed thermal conductivity enhancement.

Figure 6-1 Labelled longitudinal section of a charged rotary kiln; Source: (Tscheng & Watkznson, 1979)

One of the most frequently quoted experimental data set and semi-empirical correlations for radiative and convective heat transfer within rotary kilns was given by (Gorog et al., 1983). They showed that the convective heat transfer is dominant over the radiation heat transfer across the gas-to-exposed-wall interface and the gas-to-exposed-bed interface at the temperatures below about 900°C. Due to the low inlet temperatures of gas and solid in the present kiln, 600oC and 500oC, the major heat transfer mode is convective, as indicated by the model formulation, and the (Gorog et al., 1983) correlations are used.

#### Heat Transfer Correlations in Rotary Kilns

(Tscheng & Watkznson, 1979) conducted systematic experimental measurements of convective heat transfer between the covered wall and the bed in a pilot-scale rotary kiln. They correlated the covered-wall Nusselt number as a function of a Peclet number defined in terms of the bed thermal diffusivity and kiln rotational speed:

Nucw=11.6×Pe0.3,     Pe=π²×θ×ωαb

where θ is the subtended bed angle, ω is the rotational speed [rev/s], and αb is the bed thermal diffusivity. This correlation has been validated over a wide range of kiln diameters (0.19 m to 0.41 m) and rotational speeds and is considered the standard reference for covered-wall heat transfer in the open literature. The Tscheng-Watkinson correlation is employed without modification in the present model.

The effective thermal conductivity of the packed bed has a significant influence on the covered-wall heat transfer coefficient through its role in the bed thermal diffusivity. (Schotte, 1960) derived an expression for the effective conductivity of a packed bed at elevated temperatures that accounts for both solid conduction and interparticle radiation:

kb=1-ε1ks+14σεpdpT3+ε×4σεpdpT3

For the bauxite particles (dp=2000 μm) and operating temperatures of interest, the radiation contribution to effective conductivity is small and the expression is dominated by the solid conduction term.

#### Gibbsite Dehydration Kinetics

The thermal dehydration of gibbsite (AlOH3) to produce alumina proceeds through a complex sequence of intermediate phases. At temperatures above approximately 150°C, gibbsite dehydrates to boehmite (γ-AlOOH) and bayerite, which further dehydrate to chi- and kappa-alumina (χ-Al2O3, κ-Al2O3) before ultimately converting to the thermodynamically stable α-Al2O3 (corundum) above 1100°C.

Figure 6-2 Phase diagram for Al(OH)3; Source: (Ioelovich, 2017)

For engineering design purposes, the dehydration is represented as a single lumped reaction:

2 AlOH3s→Al2O3s+3H2Og    ΔH = +75,300 J/mol AlOH3

The endothermic enthalpy value is consistent with experimental measurements reported in the thermal analysis literature. The Arrhenius pre-exponential factor and activation energy adopted in the present model (A = 7.58 × 1015 s-1, Ea = 157,000 J/mol) (Jovanovic et al., 1992) are within the range of values reported for particle sizes in the 50-100 μm range, where intraparticle diffusion begins to compete with intrinsic surface reaction.

The rate expression used in this design was adopted from (Amiri et al., 2015):

rdeh=kdeh⋅CAlOH3,0⋅CH2O,0

In the design, the initial water concentration was assumed to be negligible, making the rate expression;

rdeh=kdeh⋅CAlOH3,0

The harmonic mean rate limiter with a mass-transfer-limited constant kdeh,mt= 7.4 × 10⁻⁴ s⁻¹ ensures that the model does not predict physically unrealisable conversion rates when the kinetics are fast relative to the available contact area.

#### Haematite Reduction Kinetics

The reduction of Haematite to magnetite by carbon monoxide is a well-studied gas-solid reaction of industrial importance in direct reduction ironmaking:

3 Fe2O3(s)+CO(g)→2Fe3O4(s)+CO2(g)     ΔH = -47,000 J/mol CO

The reaction is exothermic and proceeds at appreciable rates above approximately 400°C. The rate expression adopted follows the classical grain model framework, where the intrinsic surface reaction rate is expressed in terms of the CO partial pressure and the specific surface area of Haematite particles:

rred,kin=kCO⋅PCO⋅as⋅ρm,Fe2O3⋅FFe2O3FFe2O3,0

The Arrhenius parameters (A = 2.32 × 10⁻⁷ mol/m²/s/atm, Ea = 76,000 J/mol) are representative of values reported for Haematite reduction in the 500-900°C temperature range (Chen et al., 2019).

#### CO Combustion Kinetics

Carbon monoxide combustion in the freeboard gas is described by an Arrhenius kinetic expression combined with a turbulent eddy-dissipation mixing limit, following the approach of the Eddy Dissipation Model (EDM) developed for turbulent combustion:

CO(g)+0.5 O2(g)→CO2(g)     ΔHcomb= -283,000 J/mol CO

The kinetic rate coefficient A = 2.24 × 10¹² m³/mol/s with Ea = 170,000 J/mol is consistent with the global CO combustion rate parameters reported by (Dryer & Glassman, n.d.) and widely employed in combustion engineering models. The turbulent mixing limit is expressed as the inverse of the macromixing timescale (Dk/vgas), ensuring that the effective combustion rate cannot exceed the rate at which fresh reactants are brought into contact.

#### Sullivan-Friedman-Maier Residence Time Correlation

The mean solid residence time in a rotary kiln has been the subject of extensive experimental investigation since the early twentieth century. The most widely applied correlation for indirect-fired kilns without lifters is the Sullivan-Friedman-Maier equation, which expresses residence time as a function of kiln geometry and operating variables:

τ =0.23LsN0.9D     [min]

where L is the kiln length [m], s is the axial slope [m/m], N is the rotational speed [rpm], and D is the internal diameter [m]. The exponent 0.9 on rotational speed reflects the weak but measurable influence of kiln rotation on solid transport velocity at low-to-moderate fill degrees.(Ferron & Singh, 1991) reviewed the available experimental data and concluded that the Sullivan-Friedman-Maier correlation provides reliable residence time predictions within ±15% for fill degrees below 15% and L/D ratios between 10 and 50. For the present design (L/D = 10/0.70 = 14.3, fill degree 18.8%), the correlation is well within its validated range.

Figure 6-3 Cross-section of a charged rotary kiln; Source: (Tscheng & Watkznson, 1979)

#### Boundary Value Problem Formulation for Counter-current Systems

For a counter-current system conditions are set at one end of the reactor for the solid phase (entering at z = 0) and the other end of the reactor is set for the gas phase (entering at z = L). This is a split-boundary structure, which leads to a classical two-point BVP.

MATLAB's bvp4c solver adaptively refines the mesh to meet the specified relative and absolute tolerances, and various numerical methods textbooks and MATLAB documentation have been used to establish the performance of the solver (Shampine et al., 2009).

### CHEMICAL ENGINEERING CALCULATIONS

#### Process Description and Flow Configuration

Unit RK-201 is modelled as a one-dimensional plug-flow reactor in the axial direction z, with z = 0 at the solid feed end (upstream) and z = Lkiln= 10 m at the solid discharge end (downstream).

Table 6-1 Counter-current flow arrangement in Unit RK-201

| Phase | Flow Direction | Inlet Location | Outlet Location |
| --- | --- | --- | --- |
| Solid bed | +z (feed → discharge) | z = 0 | z = L |
| Freeboard gas | −z (discharge → feed end) | z = L | z = 0 |

Because some state variables are specified at z = 0 (solid feed conditions) and others at z = L (gas feed conditions), the system constitutes a two-point boundary value problem that cannot be integrated in a single direction from one boundary.

#### Feed Compositions and Mass Balances

#### Solid Feed Composition (z = 0)

The solid feed is Awaso deposit bauxite, crushed and screened to a mean particle size of 2000 μm. All molar flow rates are in mol/s; mass flows are derived from F×Mw×3600.

Table 6-2 Solid feed composition and mass flow rates, Unit RK-201.

| Species | Molar Flow (mol/s) | Mw (kg/mol) | Mass Flow (kg/h) |
| --- | --- | --- | --- |
| Fe2O3 (Haematite) | 0.02087 | 0.15969 | 12.00 |
| AlOH3 (Gibbsite) | 1.6072 | 0.07800 | 451.35 |
| SiO2 (Silica) | 1.2275 | 0.06008 | 265.44 |
| TiO2 (Titania) | 0.0752 | 0.07986 | 21.62 |
| K2O | 0.0119 | 0.09420 | 4.04 |
| Na2O | 0.0053 | 0.06198 | 1.18 |
| MgO | 0.0021 | 0.04030 | 0.30 |
| Total | — | — | 755.9 |

#### Gas Feed Composition (Z=L)

The reducing gas, derived from a fuel-rich burner, enters the kiln at the solid discharge end (z = L) at 600°C (873.15 K).

Table 6-3 Gas feed composition and mass flow rates, Unit RK-201

| Species | Molar Flow (mol/s) | Mw (kg/mol) | Mass Flow (kg/h) |
| --- | --- | --- | --- |
| CO | 2.95 | 0.02801 | 297.5 |
| CO2 | 1.10 | 0.04401 | 174.3 |
| N2 (carrier) | 3.70 | 0.02801 | 373.0 |
| O2 | 1.16 | 0.03200 | 133.6 |
| H2O | 0 | 0.01802 | 0 |
| Total | 8.91 | — | 978.4 |

Note: The CO/O₂ molar ratio of 2.95/1.16 = 2.54 is significantly above the stoichiometric ratio of 2.0, ensuring a net reducing atmosphere throughout the kiln and preventing re-oxidation of magnetite product.

#### Chemical Reactions and Thermochemistry

Three chemical reactions are modelled. The enthalpy values are standard state values referenced to 298 K and adjusted at operating temperatures through heat capacity integration.

Table 6-4 Chemical reactions and standard enthalpies.

| # | Reaction | Phase | Type | ΔH (J/mol) |
| --- | --- | --- | --- | --- |
| R1 | Gibbsite dehydration | Solid-Gas | Endothermic | +75,300 per mol AlOH3 |
| R2 | Reduction of Haematite | Solid-Gas | Exothermic | −47,000 per mol CO |
| R3 | Combustion of Carbon monoxide | Gas-phase | Exothermic | −283,000 per mol CO |

#### Kiln Geometry and Kinematic Parameters

Table 6-5 Kiln geometric and kinematic parameters

| Parameter | Value | Unit |
| --- | --- | --- |
| Kiln length | 10 | m |
| Outer diameter | 1.0 | m |
| Steel shell thickness | 0.01 | m |
| Refractory lining thickness | 0.14 | m |
| Outer shell radius | 0.500 | m |
| Inner shell radius | 0.490 | m |
| Inner refractory radius (effective) | 0.350 | m |
| Effective internal diameter | 0.700 | m |
| Axial slope | 0.02 | m/m |
| Rotational speed | 2.00 | rpm |
| Rotational speed (rev/s) | 0.0333 | rev/s |
| Mean particle diameter | 75 × 10⁻⁶ | m |
| Bed void fraction | 0.30 | — |
| Particle emissivity | 0.89 | — |

#### Dynamic Fill Degree and Interface Geometry

Solid Residence Time (Sullivan-Friedman-Maier)

The mean solid residence time is calculated from the Sullivan-Friedman-Maier empirical correlation:

τmin=0.23 × Lkilns×N0.9×Din

τmin=0.23 × 100.02 × 2.000.9× 0.70

τmin=2.300.02 × 1.866 × 0.70=2.300.02612≈88.1 min

Dynamic Bulk Density

The bulk density is computed dynamically from the instantaneous solid composition:

ṁsolid,in=ΣFi,0×Mwi  [kg/s]

Vsolid,true=ΣFi,0×Mwiρtrue,i  [m³/s]

ρtrue,bed=ṁsolid,inVsolid,true  [kg/m³]

ρbulk,in=(1-εbed)×ρtrue,bed  [kg/m³]

Figure 6-4 Variation of bauxite bulk density along kiln length

Volumetric Flow and Fill Degree

Qvol,in=ṁsolid,inρbulk,in  [m³/s]

vsolid=Lkilnτsec  [m/s]

Abed=Qvol,invsolid  [m²]

f =AbedπR2=fractional fill degree  [-]

Interface Geometry

The subtended central angle θ [rad] of the bed cross-section is found from the implicit equation:

θ-sinθ=2πf

Solved iteratively using Newton-Raphson. The per-unit-length interface areas are:

Table 6-6 Per-unit-length interface areas

| Interface | Formula | Physical Meaning |
| --- | --- | --- |
| Exposed wall (gas-side) | 2π-θ×R | Wall arc facing freeboard gas |
| Exposed bed surface | 2R sinθ2 | Chord at gas-solid interface |
| Covered wall (bedside) | θ×R | Wall arc in contact with solid bed |

The freeboard cross-sectional area is:

Afb=πR2-Abed  [m²]

Heat Capacity Correlations

  - Gas Phase [J/mol/K]

All polynomial correlations use the reduced temperature t = T/1000 (T in Kelvin), consistent with NIST/JANAF data. The temperature range of validity is 298-1500 K.

Table 6-7 Gas-phase heat capacity polynomials (NIST WebBook)

| Species | Cp Polynomial [J/mol/K] |
| --- | --- |
| CO | 25.568 + 6.096t + 4.055t2- 2.671t3+0.131t2 |
| CO₂ | 24.997 + 55.187t - 33.691t2+ 7.948t3-0.137t2 |
| H₂O(g) | 30.092 + 6.833t + 6.793t2- 2.534t3+0.082t2 |
| N₂ | 30.0 (constant approximation) |

The total gas-phase heat capacity flow rate [W/K] is:

GCp,g= FCO⋅cp,CO+ FCO2⋅cp,CO2+ FH2O⋅cp,H2O+ FN2⋅cp,N2

  - Solid Phase [J/kg/K]

All solid-phase Cp polynomials are derived from NIST data in J/mol/K and converted to J/kg/K by dividing by the molecular weight.

Table 6-8 Solid-phase heat capacity expressions

| Species | Cp Expression [J/mol/K] (t = Tb/1000) |
| --- | --- |
| AlOH3 | 58.62 + 0.126Tb |
| Al2O3 | 104.157 + 16.481t - 8.125t2+ 1.638t3-3.208t2 |
| Fe2O3 | 98.806 + 111.725t - 52.687t2+ 9.871t3-1.246t2 |
| Fe3O4 | 200.832 + 1.586t -4.113t2 |
| SiO2 | 46.94 + 34.31t -1.13t2 |
| TiO2 | 73.35 + 5.02t -1.715t2 |
| MgO | 42.59 + 7.28t -0.619t2 |
| K2O | 1000 J/kg/K (constant estimate) |
| Na2O | 1000 J/kg/K (constant estimate) |

The total bed heat capacity flow rate [W/K] is:

GCp,b=ΣjFj×Mwj×cp,j  (summed over all solid species)

#### Heat Transfer Coefficient Derivations

#### Gas-to-Exposed Wall (hew)

The convective heat transfer coefficient from the freeboard gas to the exposed wall arc is calculated using the (Gorog et al., 1983) modified Dittus-Boelter correlation adapted for kiln geometry:

hew= 0.036×kgDk×Re0.8×Pr0.33×DkLkiln0.055

where:

Table 6-9 Parameters for gas-to-exposed-wall heat transfer coefficient.

| Symbol | Definition | Value |
| --- | --- | --- |
| Dk | Hydraulic diam. of freeboard: 4AfbAew+Aeb | Computed at each z |
| Re | Gas Reynolds no.: GgDkμg | Computed at each z |
| Gg | Gas mass flux: ṁgAfb [kg/m²/s] | Computed at each z |
| kg | Gas thermal conductivity | 0.07 W/m/K |
| μg | Gas dynamic viscosity | 4 × 10-5 kg/m/s |
| Pr | Gas Prandtl number | 0.70 |

#### Gas-to-Exposed Bed (heb)

The (Gorog et al., 1983) correlation for gas-to-bed convective heat transfer:

heb=0.4 × Gg0.62    [W/m²/K]

where Gg is expressed in kg/m²/h. This empirical correlation captures the enhanced heat transfer due to the impingement of the recirculating gas flow on the exposed bed surface.

#### Covered Wall-to-Bed (hcw)

The (Tscheng & Watkznson, 1979) correlation for the covered-wall-to-bed coefficient:

Nucw=11.6×Pe0.3

Pe=π2×θ×ωαb,    αb=kbρb×cp,b

hcw=Nucw×kbR1 × θ     [W/m²/K]

The covered-wall coefficient reflects the periodic heating of the wall by the gas and subsequent heat release to the bed as the kiln rotates. It is sensitive to rotational speed and bed thermal diffusivity.

#### Effective Bed Thermal Conductivity (Schotte, 1960)

kb=1-ε1ks+14σεpdpTb3+ 4εσεpdpTb3

with

σ = 5.67 × 10⁻⁸ W/m²/K⁴ (Stefan-Boltzmann constant)

ks= 1.3 W/m/K (solid conductivity)

εbed= 0.30

εp= 0.89 (particle emissivity), dp = 2000 μm.

#### Wall Temperature and Shell Heat Loss

The wall inner-surface temperature Tw is algebraically determined at each axial position from a steady-state energy balance on the composite cylindrical wall (the wall has negligible thermal mass in steady state):

Tw=hewAewTg+ hcwAcwTb+TambRwallhewAew+ hcwAcw+1Rwall

The composite wall thermal resistance per unit axial length [m·K/W] comprises the refractory lining, the steel shell, and the external ambient convection resistance:

Rwall=lnR2R12π kref+lnR3R22πkshell+12πR3hamb

Table 6-10 Wall thermal property values.

| Layer | Value |
| --- | --- |
| Refractory thermal conductivity | 1.5 W/m/K (Boateng, 2008) |
| Steel shell thermal conductivity | 45 W/m/K (Bergman et al., 2011) |
| Ambient convection coefficient | 15 W/m2/K (Boateng, 2008) |
| Ambient temperature | 305.15 |

The shell heat loss per unit length is:

Qshell=Tw- TambRwall     [W/m]

#### Reaction Kinetics and Rate-Limiting Analysis

All effective rates are computed as the harmonic mean of the kinetically controlled rate and the transport (mass-transfer or mixing) limited rate. This formulation naturally interpolates between the two limiting regimes:

reff=rkin× rlimitrkin+ rlimit+ 10-12

The 10⁻¹² term is a numerical safeguard against division by zero during solver Jacobian evaluations.

##### Gibbsite Dehydration (R1)

Arrhenius kinetic rate constant:

kdeh= 7.58 × 1015× exp-157,0008.314 × Tb    [s⁻¹]

Mass-transfer limited constant:

kdeh,mt= 7.4 × 10-4      [s⁻¹]

Rate expressions [kg AlOH3/m³/s]:

rdeh,kin=CAlOH3,0×kdeh×FAlOH3FAlOH3,0

rdeh,mt = CAlOH3,0× kdeh,mt× FAlOH3FAlOH3,0

rdeh=rdeh,kin×rdeh,mtrdeh,kin+rdeh,mt+10-12

where CAlOH3,0 [kg/m³] is the initial gibbsite bulk concentration computed from the feed bulk density and the gibbsite mass fraction.

##### Haematite Reduction (R2)

Arrhenius rate constant for CO-Haematite surface reaction:

kCO= 2.32 × 10-7× exp-76,0008.314 × Tb     [mol/m²/s/atm]

Specific surface area of Haematite particles:

as=61 - εdp     [m²/m³]

CO partial pressure:

PCO= Ptotal×FCOFCO+ FCO2+ FH2O+ FN2     [atm]

Rate expressions [mol/m³/s]:

rred,kin = kCO× PCO× as× ρm,Fe2O3× FFe2O3FFe2O3,0

rred,mt = kred,mt× PCO× as× ρm,Fe2O3 × FFe2O3FFe2O3,0   [kred,mt = 0.9]

rred    = rred,kin×rred,mtrred,kin+ rred,mt+ 10-12

##### CO Combustion (R3)

Arrhenius kinetic rate constant:

kcomb= 2.24 × 1012× exp-170,0008.314 × Tg     [m³/mol/s]

Local volumetric flow of gas:

Qlocal=FCO+ FCO2+ FH2O+ FN2Rgas TgPtotal× 101,325    [m³/s]

Local concentrations:

CCO=FCOQlocal,    CO2=FO2Qlocal     [mol/m³]

Turbulent mixing limit (Eddy Dissipation Model):

vgas=QlocalAfb,    τmix=Dkvgas,    kcomb,mix=1τmix     [s⁻¹]

Rate expressions [mol/m³/s]:

rcomb,kin=kcomb⋅CCO⋅CO2       (Pure kinetic combustion rate)

rcomb,mix= kcomb,mix⋅CCO     Mixing limited combustion rate based on fuel

rcomb    = rcomb,kin×rcomb,mixrcomb,kin+ rcomb,mix+ 10-12

#### Governing ODEs and Boundary Conditions

#### State Vector

The model resolves 9 coupled state variables along z ∈ [0, 10 m]:

y =  FCO, FCO2, FH2O, FAlOH3, FFe3O4, FFe2O3, Tg, Tb, FO2ᵀ

#### Solid-Phase Mass Balances

Table 6-11 Solid-phase mass balance ODEs.

| Species | ODE | Physical Meaning |
| --- | --- | --- |
| AlOH3 | dFAlOH3dz=-rdeh×AbedMwAlOH3 | Gibbsite consumed by dehydration |
| Fe2O3 | dFFe2O3dz= -3 × rred× Abed | Haematite consumed by reduction |
| Fe3O4 | dFFe3O4dz= +2 × rred× Abed | Magnetite produced by reduction |
| Al2O3 | FAl2O3=FAlOH3,0- FAlOH32 | Stoichiometric (not a state ODE) |

#### Gas-Phase Mass Balances

Gas flows in the −z direction; to express all ODEs in the +z coordinate, gas-phase flow rates increase with z for consumed species:

Table 6-12 Gas-phase mass balance ODEs.

| Species | ODE | Physical Meaning |
| --- | --- | --- |
| CO | dFCOdz= +rred⋅Abed+ rcomb⋅Afb | CO consumed (reduction + combustion) |
| CO2 | dFCO2dz= -rred⋅Abed- rcomb⋅Afb | CO₂ generated |
| H2O | dFH2Odz= 32×dFAlOH3dz | Steam from dehydration |
| O2 | dFO2dz= -0.5rcomb⋅Afb | O₂ consumed by combustion |

#### Energy Balances

Gas temperature (note: gas temperature increases toward z = L, hence dTgdz>0):

GCp,gdTgdz=hewAewTg-Tw+hebAebTg-Tb+dFH2Odzcp,H2OTb-Tg - Qcomb,gas

where Qcomb,gas=ΔHcomb⋅rcomb⋅Afb [W/m] is the heat released to the gas by CO combustion.

Bed temperature (solid moves in +z, so co-current sign convention applies):

GCp,bdTbdz=hebAebTg-Tb+hcwAcwTw-Tb+Qrxn

where the bed reaction heat source:

Qrxn= -ΔHdeh⋅rdehMwAlOH3⋅Abed+ ΔHred⋅rred⋅Abed     [W/m]

Sign convention: ΔHdeh>0 makes the first term negative (endothermic heat sink); ΔHred<0 makes the second term negative but contributes a positive heat release to the bed.

#### Boundary Conditions

Table 6-13 Nine boundary conditions for the two-point BVP.

| BC # | Boundary | Value | Meaning |
| --- | --- | --- | --- |
| 1 | z = 0 | 1.6072 mol/s | Fresh gibbsite feed |
| 2 | z = 0 | 0 mol/s | No magnetite in feed |
| 3 | z = 0 | 0.02087 mol/s | Fresh Haematite feed |
| 4 | z = 0 | 773.15 K | Solid feed temperature |
| 5 | z = L | 2.95 mol/s | Fresh CO from burner |
| 6 | z = L | 1.10 mol/s | CO₂ from burner |
| 7 | z = L | 0 mol/s | Fresh gas, no steam |
| 8 | z = L | 873.15 K | Gas inlet temperature |
| 9 | z = L | 1.16 mol/s | O₂ from burner air |

#### Numerical Solution Method

Solver: bvp4c

The system is solved using MATLAB's bvp4c, which implements the three-stage Lobatto IIIa collocation formula, a fourth-order implicit Runge-Kutta method. The solver adaptively refines the axial mesh to meet the user-specified error tolerances. Solver settings:

Table 6-14 bvp4c solver options.

| Option | Value | Purpose |
| --- | --- | --- |
| NMax | 10,000 | Maximum number of mesh points |
| RelTol | 1 × 10⁻⁴ | Relative error tolerance per mesh interval |
| AbsTol | 1 × 10⁻⁶ | Absolute error tolerance |

Table 6-15 Key performance metrics computed in post-processing.

| Quantity | Formula |
| --- | --- |
| Al2O3 molar flow | FAl2O3=FAlOH3,0- FAlOH32 |
| Gibbsite conversion | XAlOH3= 1 -FAlOH3LFAlOH3,0 |
| Haematite conversion | XFe2O3= 1 -FFe2O3LFFe2O3,0 |
| CO conversion | XCO=FCO,in- FCOzFCO,in |
| Total heat duty | Q_total = 0LQdutyz dz |
| Shell heat loss | Qshell,total=0LQshellz dz |

### SIMULATION RESULTS AND DISCUSSION

#### Thermal Profiles and Heat Transfer

The steady-state one-dimensional temperature profiles of the freeboard gas, the solid bed, and the exposed refractory wall are presented below. The counter-current flow arrangement is distinctly visible, with the hot reducing gases entering at z = 10 m at 600°C and transferring sensible heat to the solid bed, which enters at z = 0 m at 500°C. The wall temperature consistently tracks between the gas and bed temperatures, underscoring its dual role as a primary heat sink from the gas phase and a vital heat source to the solid bed via the covered-wall interface.

Figure 6-5 Temperature profiles of Freeboard, solid bed and wall

To further understand the thermal demands of the reactor, the figure below illustrates the axial heat duty profiles. A massive endothermic heat sink is observed in the upstream section of the kiln (near z=0), corresponding to the high energy demand of gibbsite dehydration. As the solid progresses longitudinally, this endothermic demand subsides, and the exothermic contributions from carbon monoxide combustion and haematite reduction become the dominant thermal driving forces, sustaining the bed temperature required for optimal kinetics.

Figure 6-6 Heat duty profiles

The figure below demonstrates the variation in the bed's effective thermal conductivity and the corresponding heat transfer coefficients along the kiln axis. Because the dynamic bulk density and bed composition change continuously as volatile moisture is driven off, the effective thermal conductivity exhibits a distinct gradient. This property shift subsequently influences the covered-wall heat transfer coefficient, proving that the thermal resistance of the bed is highly dependent on local composition and temperature rather than acting as a static parameter.

Figure 6-7 Thermal conductivity profiles

#### Mass Transport and Reaction Extents

The conversion of the Awaso bauxite feed is governed by the simultaneous depletion of solid reactants and the generation of gas-phase products. Figure 0-1 highlights the molar flow rate of gibbsite undergoing thermal dehydration. A sharp initial decline in AlOH3 is observed as the bed temperature rapidly overcomes the dehydration activation energy, evolving a significant volume of steam into the freeboard gas.

Figure 6-8 Molar flow rate profiles for gibbsite dehydration

Simultaneously, the selective reductive roasting of the ore is captured in the figure above, which overlays the depletion of paramagnetic haematite (Fe2O3) with the stoichiometric generation of ferrimagnetic magnetite (Fe3O4). The reduction rate accelerates in the downstream sections of the kiln (z > 5 m) where the bed temperature is elevated, and the concentration of the reducing gas is highest.

Figure 6-9 Solid molar flow rates of Fe2O3 and Fe3O4

The mass transport in the freeboard is detailed above, showing the gas and vapour molar flow rates flowing in the -z direction. The sweeping effect of the counter-current gas is evident: steam generated from gibbsite dehydration accumulates toward the gas exhaust at z = 0. Concurrently, CO is continuously consumed by both the reduction of haematite and partial combustion with O2, resulting in a rising profile of CO2.

Figure 6-10 Gas/Vapour molar flow rates

Maintaining a highly reducing atmosphere is critical to preventing the re-oxidation of magnetite. The figure above confirms process viability by plotting the gas-phase partial pressures. Despite the generation of steam and CO2, the partial pressure of CO (PCO) remains sufficiently dominant throughout the reaction zone to thermodynamically drive the haematite reduction forward.

Figure 6-11 Gas phase partial pressures

Figure 0-5 validates the rigorous kinetic limits applied to the numerical model. By plotting the intrinsic kinetic rate against the mass-transfer limit, it is evident that in regions of extreme temperature, the effective reaction rate successfully clamps to the transport limit. This ensures the solver predicts physically realizable conversions rather than theoretically infinite reaction rates.

Figure 6-12 Reaction Rate Profiles

#### Kiln Kinematics and Bed Properties

The physical transformation of the bauxite bed as it travels down the kiln is summarized in Figure 0-6. As the solid bed loses nearly 34 % of its mass due to the liberation of structural water from gibbsite, the true density of the solid phase alters significantly. Because the volumetric flow rate and axial solid velocity are interlinked, this densification causes a dynamic shift in both the bulk density and the fractional fill degree (f) along the kiln axis. The simulation confirms that the fill degree safely remains between 11 % and 12 % which is well within the optimal operating window for indirect-fired kilns and validates the Sullivan-Friedman-Maier residence time correlation used in the mechanical design basis.

Figure 6-13 Dynamic fill degree (f) and bulk density profiles

### MECHANICAL ENGINEERING CALCULATIONS

#### Shell Thickness and Structural Loading

##### Design Basis

The kiln shell is a carbon steel cylinder of outer diameter Dout=1.0 m. The shell must withstand its own weight over a span between tyre stations, the weight of the refractory lining, and the weight of the solid bed, all under steady-state thermal conditions. The design follows the ASME BPV Code Section VIII Div. 1 framework for cylindrical shells under external circumferential loads.

Table 6-16 Wall Thickness Specification

| Layer | Material | Thickness [m] | Inner Radius [m] | Outer Radius [m] |
| --- | --- | --- | --- | --- |
| Refractory lining | High-alumina castable | 0.140 | 0.350 | 0.490 |
| Steel shell | Carbon steel A36 | 0.010 | 0.490 | 0.500 |

##### Shell Hoop Stress Estimate

The primary structural load on the shell is the circumferential bending stress arising from the eccentric weight of the refractory lining and bed. A conservative estimate uses the dead weight of the refractory per unit length:

Wref= ρref×π×R22- R12×g     [N/m]

Using ρref≈ 2564.12 kg/m3 (high-alumina castable), R2 = 0.490 m, R1 = 0.350 m, g = 9.81 m/s2:

Wref= 2800 × π × 0.4902- 0.3502× 9.81≈9,293.1843 N/m

The shell wall cross-sectional area per unit length:

Ashell= 2π R2 × δwall= 2π × 0.490 × 0.010 = 0.0308 m²/m

Nominal circumferential stress from dead weight:

σhoop=WrefAshell=10,1340.0308≈ 302 kPa

This is well below the allowable stress for A36 steel (250 MPa minimum yield) (Ikumapayi et al., 2021), confirming that the 10 mm shell thickness is adequate from a structural standpoint. The governing mechanical design criterion is typically the shell ovality under the tyre loads, addressed subsequently.

#### Refractory Lining Thermal Resistance

##### Steady-State Radial Temperature Distribution

Under steady-state operation, the temperature distribution across the composite cylindrical wall follows the solution to the cylindrical Laplace equation. The thermal resistance formulation gives the total wall resistance:

Rwall=lnR2R12πkref+lnR3R22πkshell+12πR3 hamb

Substituting values:

lnR2R1= ln0.4900.350= ln1.400= 0.3365

Rref=0.33652π × 15=0.336594.25= 3.57 × 10⁻³ m·K/W

lnR3R2= ln0.5000.490= ln1.0204= 0.0202

Rsteel   =0.02022π × 45=0.0202282.7=  7.14 × 10⁻⁵ m·K/W

Rconv    =12π × 0.500 × 15=147.12= 0.02122 m·K/W

Rwall,total = 3.57×10-3+ 7.14×10-5+ 0.02122 =  0.02500 m·K/W

##### Shell Outer Surface Temperature

Assuming a typical inner wall temperature of Tw≈700 K (427°C):

Qshelllength=Tw- TambRwall=700 - 305.150.02500=394.850.0250≈ 15,794 W/m

Touter= Tamb+ Qshelllength× Rconv= 305.15 + 15,794 × 0.02122  ≈ 640 K (367°C)

The outer shell surface temperature of approximately 367°C is consistent with industrial practice for kilns of this type and within the safe operating range for carbon steel (maximum continuous operating temperature approximately 400°C for A36).

#### Tyre and Riding-Ring Design

##### Tyre Load Calculation

Tyres (also called riding rings) support the entire rotating mass of the kiln on trunnion rollers. The supported mass per unit length includes the shell, refractory, and solid bed:

mshell= ρsteel× 2πR2 × δwall= 7850 × 2π0.4900.010 ≈ 241 kg/m

mref  = ρref× πR22-R12= 2800 × π0.2401-0.1225 ≈ 1035 kg/m

mbed  = ρbulk× Abed  (variable along z, use peak value of ≈1000 kg/m³ × 0.05 m² ≈ 50 kg/m)

mtotal≈ 241 + 1035 + 50 = 1326 kg/m

For a 10 m kiln with two tyre stations at 2.5 m from each end (5 m span), the load on each tyre:

Ftyre= mtotal× g ×Lspan2= 1326 × 9.81 ×5.02 ≈ 32,500 N = 32.5 kN

##### Tyre Width Selection

The contact pressure between the tyre and the trunnion roller must not exceed the allowable Hertzian contact stress for the materials (typically 200-400 MPa for steel-on-steel). For an initial tyre width estimate:

Tyre face width b ≥FtyrePallow× Dtyre=32,500250×103× 1.0 = 0.13 m

A standard tyre face width of 200 mm (0.20 m) is selected, providing an adequate safety margin. The tyre is fabricated from cast carbon steel (Grade E or equivalent) with a minimum Brinell hardness of 220 HB to resist rolling contact fatigue (Ikumapayi et al., 2021).

#### Drive Power Estimation

Gross Torque

The drive power for the rotary kiln is estimated using the Kramer formula, which relates the torque to the kiln dimensions, fill fraction, and bed unit weight:

Tgross=f×mtotal×g×R  [N·m]

where f is the friction factor (typically 0.12-0.20 for steel-on-steel trunnions). Taking f = 0.15 and R = R3 = 0.5 m:

Tgross= 0.15 × 1326 × 10× 9.81 × 0.5≈ 9,756 N·m

Drive Motor Power

The gross mechanical power at the shell:

Pmech= Tgross× ωrad= 9,756 × 2π ×260= 9,756 × 0.2094 ≈ 2,042 W

Accounting for drive train efficiency (η ≈ 0.80 for girth-gear and pinion drive):

Pmotor=Pmechη=2,0420.80≈ 2,553 W   ≈  2.6 kW

A standard 5.5 kW (7.5 hp) variable-frequency drive motor is specified, providing a generous 2.1× service factor for startup and process upsets.

#### Summary of Mechanical Specifications

Table 6-17 Summary of mechanical design parameters, Unit RK-201.

| Parameter | Value | Unit |
| --- | --- | --- |
| Shell outer diameter | 1.000 | m |
| Shell inner diameter (to refractory) | 0.980 | m |
| Internal process diameter | 0.700 | m |
| Shell wall thickness | 10 | mm |
| Refractory thickness | 140 | mm |
| Kiln length | 10.0 | m |
| Tyre count | 2 | — |
| Tyre face width | 200 | mm |
| Tyre material | Cast steel, ≥220 HB | — |
| Rotational speed | 2.0 | rpm |
| Drive motor power (installed) | 5.5 | kW |
| Total supported mass | ~13,260 | kg |
| Shell outer surface temperature (peak) | ~367 | °C |
| Shell hoop stress (nominal) | ~329 | kPa |

## ALUMINA ACID DIGESTION REACTOR DESIGN – OSABUTEY, SYLVESTER NII ABBEY

### Problem Statement

This design addresses a continuous acid digestion system processing 1,336.02 kg/h of calcined bauxite slurry with 1,800 kg/h of hydrochloric acid at 120 °C and 3 bar(a), with the reaction maintained at pH < 1.5, to produce aluminium chloride solution, which serves as a precursor in the production of poly-aluminium chloride. Pressurised operation at 3 bar(a) is essential because, at atmospheric pressure, concentrated hydrochloric acid solution has a boiling point of approximately 108 °C, which would prevent operation at 120 °C and promote HCl volatilisation, acid loss, and increased off-gas generation. Maintaining the reactor at 3 bar(a) elevates the boiling temperature of the reacting HCl–H₂O system, thereby enabling operation at 120 °C while limiting vapour formation and HCl loss from the liquid phase. The reaction is exothermic, releasing 232.2 kW and 35.1 kW in the first and second stages, respectively. The design objective is therefore to develop a two-stage jacketed CSTR system capable of achieving 90% alumina conversion while maintaining stable temperature control and safe operation at the specified conditions.

### Introduction

Poly-aluminium chloride (PAC) is a widely used inorganic coagulant in water and wastewater treatment because of its high coagulation efficiency, relatively low sludge production, and broad effective pH range (Kassa et al., 2023). Commercial PAC production requires a soluble aluminium precursor, commonly aluminium chloride, which can be obtained by acid digestion of aluminium-bearing minerals such as bauxite.

Hydrochloric acid digestion is a hydrometallurgical operation in which the mineral feed reacts with hydrochloric acid to solubilise alumina, while a substantial fraction of the insoluble gangue remains in the solid phase. For calcined bauxite, the principal digestion reaction is represented by:

Al2O3+6HCl→2AlCl3+3H2O     (7.2.0)

The reaction is heterogeneous, and its rate is influenced by temperature, hydrochloric acid concentration, particle size, residence time, and solid–liquid mass transfer. Adequate agitation is therefore required to maintain a uniform slurry, suspend the solid particles, and minimise external mass-transfer limitations (Levenspiel, 1999). Continuous stirred-tank reactors (CSTRs) are well suited to slurry-phase reactions because they provide effective mixing, relatively uniform temperature conditions, and continuous solids suspension (Towler and Sinnott, 2022). For the present design, CSTRs arranged in series provide progressive alumina conversion while maintaining continuous operation and enabling staged control of the exothermic digestion process. A two-stage configuration is therefore adopted to achieve the target overall alumina conversion of 90%.

### Equipment Selection and Justification

Reactor selection was based on reaction kinetics, solids-handling requirements, exothermic heat generation, target conversion, controllability, and capital economics.

The digestion process follows the Shrinking Core Model (SCM), in which reaction progresses from the particle surface toward an unreacted core as a porous silica-rich residue accumulates, introducing progressive mass-transfer resistance (Levenspiel, 1999). Effective liquid–solid contact, continuous solids suspension, and uniform reactant distribution are therefore essential.

Batch reactors are unsuitable for continuous production, requiring parallel charge–heat–discharge cycles that increase capital cost. PFRs provide higher conversion per unit volume but are impractical in slurry service owing to particle settling, channelling, and the risk of axial temperature gradients.

CSTRs provide excellent mixing, uniform temperature, and reliable solids suspension in strongly acidic service. Two equal-volume CSTRs in series require approximately 52% less total residence time than a single CSTR for 90% conversion, reducing total vessel volume, capital cost, and distributing the exothermic heat load (Levenspiel, 1999). External cooling jackets were selected over internal coils to avoid solids fouling on heat-transfer surfaces. A two-stage jacketed CSTR system in series was therefore selected, consistent with industrial practice for continuous HCl leaching of mineral slurries (Wills and Finch, 2016).

### Process and Equipment Description

#### Process Description

Calcined bauxite slurry is fed continuously to the first digestion reactor at 1,336.02 kg h⁻¹, together with hydrochloric acid at 1,800 kg h⁻¹. Acid is supplied in 20% stoichiometric excess to maintain a strongly acidic environment (pH < 1.5) and prevent reactant limitation. The excess serves two engineering purposes. First, iron oxides present in the bauxite (Fe₂O₃ and Fe₃O₄ combined at 0.081 wt.% of slurry) consume acid via

Fe2O3+6HCl→2FeCl₃+3H₂O and Fe3O4+8HCl→FeCl2+2FeCl3+4H₂O, reducing available acid for alumina dissolution. At the feed rates specified, this impurity consumption is minor (approximately 1–2% of total acid demand); however, maintaining excess acid compensates for this and for any variability in ore composition between batches. Second, a low pH environment (pH < 1.5) suppresses re-precipitation of aluminium hydroxide, which would otherwise reduce product yield and foul downstream equipment. The exact impurity acid consumption is not incorporated into the stoichiometric calculation at this preliminary stage — the 20% excess is conservatively sufficient to cover it. Digestion proceeds at 120 °C, 3 bar(a), according to Equation (10.0). The reaction releases 232.2 kW and 35.1 kW in the first and second reactors respectively.

Slurry from the first reactor overflows continuously to the second, where additional residence time achieves the target overall alumina conversion of 90%. The effluent is transferred to downstream solid–liquid separation for removal of insoluble residues.

#### Equipment Description

The digestion system comprises two identical jacketed CSTRs in series with equal working volumes and residence times. Each reactor is a vertical cylindrical vessel with 2:1 ellipsoidal closures, rated for 5 bar(a) design pressure and 150°C design temperature. A centrally mounted six-bladed 45° pitched-blade turbine impeller, driven via a motor–gearbox assembly, maintains complete solids suspension, uniform reactant distribution, and enhanced liquid–solid mass transfer. Four vertical wall baffles suppress vortex formation and improve axial circulation. An external annular cooling jacket with spiral-flow guides circulates pressurised cooling water to maintain 120 °C. Each reactor is fitted with nozzles for slurry feed, acid feed, product discharge, vent, pressure relief, level measurement, temperature measurement, and a 500 mm elliptical manway. Key process safety provisions are integrated into the design:

- a pressure relief valve (PRV) set at 5 bar(a) with a rupture disc upstream protects against overpressure during loss of cooling or process runaway

- cooling water supply is monitored by flow and temperature transmitters with high-temperature alarms — loss of cooling water flow triggers automatic isolation of acid and slurry feeds

- agitator failure is alarmed on loss of drive current or speed signal, and failure shuts off all feeds within 30 s to prevent acid attack of unsuspended solids

- the product discharge overflow arrangement ensures vessel level is self-regulating, eliminating blocked-outlet overpressure risk

- startup requires cooling water flow established before acid addition, with controlled heating at ≤30°C/min to protect the glass lining from thermal shock.

### Chemical Engineering Design

#### Design Basis and Design Criteria

The design basis was derived from process requirements, literature on hydrochloric acid leaching of alumina-bearing materials, and Aspen Plus® V14 steady-state simulation using the ELECNRTL property package. Table 10.1 summarises the principal design parameters.

Table 6-18 Design basis and criteria for the acid digestion reactor

| Parameter | Design Value | Basis |
| --- | --- | --- |
| Reactor configuration | 2 × jacketed CSTRs in series | Reduced total volume vs. single CSTR; improved solids suspension (Levenspiel, 1999) |
| Operating mode | Continuous | Consistent with continuous PAC production |
| Feed temperature | 120°C | Preheated to reduce reactor heating demand |
| Operating temperature | 120 °C | Literature-supported enhanced dissolution; Aspen Plus® optimisation |
| Operating pressure | 3 bar(a) | Maintains liquid phase; minimises HCl volatilisation |
| Target alumina conversion | 90% | Economic optimum; higher conversion requires disproportionate residence time |
| HCl concentration (feed) | 32 wt.% | Commercially available concentration providing adequate acidity |
| Acid stoichiometry | 20% molar excess | Prevents acid limitation; compensates for Fe-oxide acid consumption |
| Slurry feed rate | 1,336.02 kg h⁻¹ | Overall process mass balance |
| HCl feed rate | 1,800 kg h⁻¹ | Stoichiometric requirement plus 20% excess |
| Al₂O₃ content of slurry | 19.57 wt.% | Upstream beneficiation/calcination mass balance |
| SiO₂ content of slurry | 17.98 wt.% | Principal gangue component |
| H₂O content of slurry | 60.37 wt.% | Slurry preparation requirement |
| Fe₂O₃ / Fe₃O₄ (slurry) | 0.041 / 0.040 wt.% | Residual upstream impurities |
| Heat removal method | External spiral-flow cooling jacket | Continuous heat removal without disturbing agitation |

#### Reaction Kinetics

The reactor residence time and volume are governed by the rate at which alumina is transferred from the solid bauxite particles into the liquid phase. Because the digestion involves reaction between solid particles and aqueous hydrochloric acid, the overall rate is influenced not only by the intrinsic chemical reaction but also by mass transfer through the liquid boundary layer and the porous residue formed around the reacting particle. The kinetic analysis therefore establishes the controlling mechanism and provides the conversion–time relationship required for determining the residence time of the two-stage CSTR system.

##### Heterogeneous Leaching Mechanism

Alumina digestion in calcined bauxite is treated as a heterogeneous solid–liquid reaction because the reacting alumina is initially contained within solid mineral particles, while hydrochloric acid is present in the surrounding liquid phase. For dissolution to occur, hydrochloric acid must first reach the particle surface and subsequently penetrate the porous reaction-product layer to reach the unreacted alumina-containing core. As digestion proceeds, the reacting interface moves progressively inward, while the remaining unreacted core decreases in size and a silica-rich porous residue accumulates around it. The increasing thickness of this product layer introduces additional resistance to transport of hydrochloric acid to the reaction interface.

This physical progression provides the basis for applying the Shrinking Core Model (SCM). The figure below illustrates the idealised particle structure adopted in the present design, showing the original particle radius, the shrinking unreacted core, the reaction interface, and the accumulating product layer. The model therefore represents the conversion of a bauxite particle as a moving reaction front rather than as a spatially uniform reaction throughout the particle.

Figure 6-14 Schematic diagram illustrating the shrinking core model - product layer diffusion

##### Product-Layer Diffusion Shrinking Core Model

The SCM is applied under the following assumptions:

- bauxite particles are spherical and of approximately uniform size

- reaction occurs at the core–product-layer interface

- temperature and acid concentration are spatially uniform within the reactor

- external liquid-film resistance is negligible under vigorous agitation

- the silica-rich product layer provides the dominant mass-transfer resistance; and

- particle breakage and attrition are neglected.

Under product-layer diffusion control, fractional particle conversion X at reaction time t satisfies (Levenspiel, 1999; Fogler, 2020):

1-32X23- 131 - X=tτₑ=kt

where X is fractional particle conversion (dimensionless), t is the particle residence time (h), τₑ the characteristic time required for complete conversion of a particle under the assumed rate-controlling mechanism, and k = 1/τₑ (h⁻¹) is the apparent rate constant. The left-hand side represents the growing diffusion resistance of the accumulating product layer, distinguishing diffusion-controlled leaching from surface-reaction control.

##### Kinetic Parameters

The governing rate mechanism is adopted from Valeev et al. (2020), who identified this as the controlling step for hydrochloric acid leaching of calcined aluminosilicate materials across conditions broadly consistent with those employed here. Deng and Li (2013) similarly confirmed diffusion-through-product-layer control for bauxite HCl leaching at elevated temperature, providing independent corroboration of the mechanism.

The characteristic reaction time τₑ is an effective lumped parameter that integrates intrinsic reaction kinetics, effective diffusivity through the silica-rich product layer, particle geometry, and reactor hydrodynamics. Its value is inherently process-specific: it depends on bauxite mineralogy and gibbsite/boehmite phase ratio, calcination degree (which determines product-layer porosity), particle size distribution, HCl concentration and activity, liquid-to-solid ratio, and operating temperature (Levenspiel, 1999). Direct extraction of τₑ from published studies is therefore not appropriate because no published dataset replicates the specific Ghanaian bauxite ore, calcination conditions, and slurry composition of this design.

In the absence of project-specific leaching testwork at this preliminary design stage, τₑ was selected as a calibrated engineering parameter guided by four convergent lines of evidence. First, the Aspen Plus® ELECNRTL simulation predicts 90% alumina dissolution at 120 °C and 3 bar(a) with the feed composition specified, providing a thermodynamically and stoichiometrically consistent conversion target against which the kinetic model must be matched. Second, published leaching rates for similar calcined bauxite ores at 80–100°C with concentrated HCl indicate characteristic reaction times in the range 1–4 h (Valeev et al., 2020; Deng and Li, 2013); operation at 120 °C accelerates diffusion and reaction, placing the expected τₑ toward the lower end of this range. Third, the 20% stoichiometric acid excess and vigorous agitation (Rei=5.89×10⁵) maintain a strongly acidic interfacial environment and high mass-transfer coefficient, suppressing external film resistance and sustaining product-layer diffusion as the sole rate-limiting step throughout the design conversion range. Fourth, a two-stage CSTR configuration with τₑ = 2.5 h yields τ = 1.15 h per reactor, corresponding to a total hydraulic residence time of 2.31 h — a commercially reasonable value for industrial acid leaching of mineral slurries (Wills and Finch, 2016). A value of τₑ = 2.5 h is therefore adopted as a defensible preliminary design parameter. Pilot-scale leaching test work would refine this value during front-end engineering design (FEED); a ±30% sensitivity analysis on τₑ would constitute appropriate design robustness verification at that stage.

Table 6-19 Adopted kinetic parameters

| Parameter | Symbol | Value | Basis |
| --- | --- | --- | --- |
| Controlling mechanism | — | Product-layer diffusion SCM | Valeev et al. (2020) |
| Characteristic reaction time | τₑ | 2.5 h | Calibrated design parameter |
| Target aluminium extraction | X | 0.90 | Process design specification |

##### Residence Time Distribution Analysis

The SCM describes conversion of an individual particle as a function of its residence time. In a CSTR, particles experience a broad exponential distribution of residence times owing to perfect mixing — some leave shortly after entry, others after much longer periods. Mean reactor exit conversion must therefore be obtained by weighting particle conversion X(t) by the CSTR residence time distribution (RTD) E(t).

For a single ideal CSTR with mean residence time τ:

E(t)=1τexp-tτ

The mean conversion leaving Reactor 1 is:

X̄₁=0∞ XtE1(t)dt

Since X(t) = 1 for t ≥ τₑ (complete conversion), the integral splits:

X1=0τeXt1τexp-tτdt+τe∞Xt1τexp-tτdt

For two equal-volume CSTRs in series, the combined RTD follows a gamma distribution:

E₂(t)=tτ2exp-tτ

The mean conversion leaving Reactor 2 is:

X2=0τeXt1τ2exp-tτdt+τe∞Xt1τ2exp-tτdt

The two-CSTR RTD is narrower with a distinct peak near the mean residence time, reducing the fraction of particles with very short residence times and improving conversion efficiency compared with a single CSTR of equal total volume.

The ideal CSTR RTD assumes perfect macro-mixing instantaneous on the timescale of reaction. Deviations can arise in concentrated mineral slurries through dead zones near baffles or the vessel base, bypassing of short-circuit flow between inlet and overflow, and macro-mixing limitations if the mixing time exceeds the reaction timescale. At Rei=5.89× 105 and P/V = 0.136 kW m⁻³, the mixing time estimated from the correlation of Nienow (1997) is approximately 15–25 s, negligible compared with the mean residence time of 1.15 h. Dead-zone and bypassing effects are likewise minimised by the four-baffle geometry, fully turbulent flow, and bottom-clearance impeller positioning. The ideal CSTR RTD is therefore a sound and conservative design assumption for this application; any real RTD narrowing due to imperfect mixing would reduce the fraction of short-residence-time particles and increase actual conversion above the design prediction, providing a conservative safety margin. Detailed residence time validation by tracer testing would be appropriate at the commissioning stage.

##### Determination of Reactor Residence Time

Figure 6-15 Residence distribution of single CSTR vs. Two equal CSTRs in series

The required mean residence time per reactor was determined by combining the SCM kinetics with the RTD analysis. MATLAB evaluated mean exit conversion iteratively, adjusting τ until the two-stage system achieved the design criterion:

X̄₂=0.90

The optimisation yielded:

τ=1.1533 h per reactor

τtotal=2τ=2.307 h

Substituting into the RTD–SCM model: mean alumina conversion after Reactor 1 is 71.35%, rising to 90.00% after Reactor 2. Results are summarised in Table 6-20.

Table 6-20 Residence time optimisation results

| Parameter | Value |
| --- | --- |
| Characteristic conversion time, τₑ | 2.50 h |
| Residence time per reactor, τ | 1.1533 h |
| Total system residence time, τtotal | 2.3066 h |
| Mean conversion after Reactor 1, X̄₁ | 71.35% |
| Mean conversion after Reactor 2, X̄₂ | 90.00% |

Figure 6-16 Mean alumina conversion vs. residence time per reactor (Two-Stage CSTR system)

The figure above shows conversion increasing monotonically with residence time but approaching an asymptote at high τ, reflecting the progressive increase in diffusional resistance from the thickening silica-rich product layer. The diminishing slope beyond X̅ = 0.80 reflects the progressive increase in product-layer diffusion resistance, indicating that each additional percentage point of conversion beyond 90% would require a disproportionate increase in reactor volume and capital cost. The 90% design point therefore represents a sound engineering optimum. Substantially larger reactor volumes are required for incremental conversion gains beyond 90%, justifying the selected design point. The residence-time advantage of the two-stage configuration relative to a single CSTR of equal total volume is illustrated in the figure below;

Figure 6-17 Conversion vs Total Residence Time — Single CSTR and Two Equal CSTRs in series

##### Reactor Volume Determination

Mixture Density

Mixture density was estimated using the volumetric reciprocal mixing rule, assuming additive component volumes:

1ρmix=wᵢρᵢ

Total feed mass flowrate:

ṁtotal=ṁslurry+ṁacid=1,336.02+1,800.00=3,136.02 kg h⁻¹

Table 6-21 Component mass flowrates

| Component | Mass Flowrate (kg h⁻¹) |
| --- | --- |
| Al₂O₃ | 261.498 |
| SiO₂ | 240.161 |
| Fe₂O₃ | 0.526 |
| Fe₃O₄ | 0.508 |
| HCl (32 wt.% aq.) | 860.603 |
| H₂O | 1,772.721 |
| Total | 3,136.02 |

Note: the density applied to HCl is that of the 32 wt.% aqueous solution, since the feed is a concentrated aqueous acid rather than anhydrous hydrogen chloride.

Q=Vtotal=ṁᵢρᵢ=2.834 m³ h⁻¹   (from Aspen Plus simulation)

ρmix=ṁtotalQ=3,136.022.834=1,106.6 kg m⁻³

Working Reactor Volume

Vw=Q·τ=3.311×1.1533=3.818 m³

Total Vessel Volume (Freeboard Allowance)

A 20% freeboard allowance — standard for mechanically agitated slurry reactors (Towler and Sinnott, 2022; Paul et al., 2004) — ensures the working liquid volume occupies 80% of the total cylindrical shell volume, accommodating agitation-induced surface fluctuations, slurry turbulence, thermal expansion, and transient upsets:

Vw=0.80VT⟹VT=Vw0.80=3.8180.80=4.773 m3

The design total cylindrical shell volume is 4.773 m³. The additional internal volume provided by the two 2:1 ellipsoidal head contributes operational freeboard above and below the cylindrical section and is not included in the liquid residence-time calculation.

#### Thermal Design and Heat Transfer

Both reactors operate at identical conditions — 120 °C, 3 bar(a), glass-lined carbon steel construction, six-bladed 45° pitched-blade turbine agitators, and spiral-flow cooling jackets — so the thermal design methodology applies equally to both. Thermal duties were obtained from Aspen Plus® V14 steady-state simulation using the ELECNRTL property package, which is specifically formulated for strong electrolyte systems and accounts rigorously for ionic speciation, activity coefficients, and heat of mixing in concentrated HCl–AlCl₃–H₂O solutions. The simulated heat duties are grounded in reaction stoichiometry (Equation 10.0) and the standard enthalpies of formation of all species. They are therefore consistent with the overall energy balance and appropriate for preliminary equipment sizing. The values are conservative in that they do not credit sensible heat recovery or heat integration. Independent validation would be performed during FEED through laboratory reaction calorimetry and comparison with industrial plant data from comparable bauxite HCl leaching operations:

Table 6-22 Reactor heat duties from Aspen Plus® simulation

| Parameter | Reactor 1 | Reactor 2 |
| --- | --- | --- |
| Operating temperature (°C) | 120 | 120 |
| Operating pressure (bar(a)) | 3 | 3 |
| Aspen Plus® heat duty (kW) | −232.2 | −35.1 |
| Required cooling duty (kW) | 232.2 | 35.1 |

Negative heat duties confirm both reactors are exothermic, requiring continuous heat removal for isothermal operation. Since Reactor 1 accounts for approximately 87% of total heat released, the jacket was designed on this duty as the controlling condition. The following analysis is presented for Reactor 1; identical methodology applies to Reactor 2.

#### Cooling Medium Selection

Pressurised cooling water was selected owing to its high specific heat capacity, favourable thermal conductivity, low viscosity, non-toxicity, and wide industrial availability (Towler and Sinnott, 2022). Thermal oils or refrigerants are unnecessary, as the required duty is achievable with cooling water. To enhance the jacket-side convective coefficient, a spiral-flow guide directs coolant in a helical path through the annular jacket, increasing velocity and turbulence, and minimising stagnant zones (Paul et al., 2004; Oldshue, 1983).

Cooling Water Requirement

Cooling water is assumed to enter at 30°C and leave at 50°C (ΔTw=20°C). The required mass flowrate follows from the steady-state energy balance with Cp,w=4.179 kJ kg⁻¹ K⁻¹:

Q=ṁwCp,wΔTw⟹ṁw=232.24.179×20=2.78 kg s⁻¹

A design flow rate 50% above the minimum was adopted to enhance jacket-side turbulence and reduce thermal resistance without modifying jacket geometry:

ṁw,design=1.5×2.78=4.17 kg s⁻¹

#### Thermophysical Properties of the Reaction Slurry

Dynamic Viscosity

Slurry viscosity was estimated using the Einstein–Roscoe correlation for dilute suspensions of rigid particles:

μ=μl1-φφm-2.5φm

where μl=liquid-phase viscosity (Pa·s), φ=solids volume fraction, φm = maximum packing fraction. For irregular mineral particles, φm=0.63 was adopted. The continuous phase is 32 wt.% aqueous HCl at 120 °C with μl=0.91×10⁻³ Pa·s; solids volume fraction from the material balance is φ = 0.063. Substituting:

μ=0.91×10-3×1 - 0.0630.63-2.5×0.63=1.07×10⁻³ Pa·s

The Einstein–Roscoe correlation tends to underpredict viscosity of reacting mineral slurries owing to particle irregularity and dissolved species effects. A conservative design viscosity of μ=1.18×10⁻³ Pa·s was adopted.

Thermal Conductivity

Effective slurry thermal conductivity was estimated using the Maxwell model for dispersed spherical particles:

ks=klkp+2kl+2φkp-klkp+2kl-φkp-kl

With kl=0.67 W m⁻¹ K⁻¹, kp=30 W m⁻¹ K⁻¹, φ = 0.063:

ks=0.796 W m⁻¹ K⁻¹

Prandtl Number

Pr=μCpks=1.18×10-3× 3,424.80.796=5.08

#### Reactor-Side Heat Transfer Coefficient

The impeller Reynolds number:

Rei=ρNDi2μ=1,106.6×2.07×0.55121.18×10-3=5.89 × 10⁵

Since Rei>10⁴, the reactor is fully turbulent. The Nagata correlation was used to estimate the Nusselt number for agitated vessels:

Nu=0.54Re0.67Pr0.33DiT0.10

Nu=0.54×5.89×1050.67×5.080.33×0.5511.670.10=6,100

hi,theoretical=Nu·ksDi=6,100×0.7960.551=8,810 W m⁻² K⁻¹

The theoretical value of 8,810 W m⁻² K⁻¹ represents idealised turbulent convection at a smooth, clean wall, and is not realisable in industrial glass-lined slurry service for several compounding reasons. The glass lining (thickness 2 mm, kg=1.15 W m⁻¹ K⁻¹) introduces a conduction resistance of 1.74 × 10⁻³ m² K W⁻¹ that alone reduces the effective wall-side coefficient to approximately 4,200 W m⁻² K⁻¹. Mineral slurry fouling and scale deposition on the lining progressively increase wall resistance; a combined fouling factor Rf=2.0×10⁻⁴ m² K W⁻¹ is applied in the overall coefficient calculation to account for this long-term effect. Particle–wall interactions in concentrated slurries create a hindered boundary layer that impairs near-wall turbulence relative to a clean single-phase fluid, reducing the effective convective coefficient below the Nagata prediction. Taken together, these effects typically reduce reactor-side coefficients in glass-lined agitated vessels to 2,000–5,000 W m⁻² K⁻¹ in industrial practice (Paul et al., 2004; Green and Southard, 2019). A design value of hi=3,500 W m⁻² K⁻¹ is accordingly adopted, representing the mid-point of this industrial range and providing reasonable conservatism without excessive over-design of the cooling jacket.

#### Jacket-Side Heat Transfer Coefficient

Cooling water flows through the annular jacket with hydraulic diameter Dh=Do-Di = 1.82-1.67=0.15 m. The Reynolds number at design flowrate:

Rej=ρwuwDhμw=4.79×10⁴

Flow is turbulent; the Dittus–Boelter equation (heating of fluid) applies:

Nu=0.023Re0.8Pr0.4=0.023×4.79×1040.8×4.350.4 = 230

ho=Nu·kwDh=230×0.6280.15=963 W m⁻² K⁻¹

#### Overall Heat Transfer Coefficient

The overall heat transfer coefficient accounts for reactor-side convection, glass-lining conduction (thickness tg=2 mm, kg=1.15 W m⁻¹ K⁻¹), shell conduction (ts=10 mm, ks=45 W m⁻¹ K⁻¹), jacket-side convection, and a combined fouling resistance Rf=2.0×10⁻⁴ m² K W⁻¹:

1U=1hi+tgkg+tsks+1ho+Rf

1U=13500+0.0021.15+0.01045+1963+2.0×10⁻⁴

U=287 W m⁻² K⁻¹

#### Required Heat Transfer Area

Applying the log-mean temperature difference method (isothermal reactor at 120 °C, cooling water 30→50°C):

ΔT₁=120-30=90°C;   ΔT₂=120-50=70°C

ΔTLM=90 - 70ln9070=79.6°C

Areq=QU·ΔTLM=232,213287×79.6=10.16 m²

Available heat-transfer area: cylindrical shell Ac=πDH=π×1.67×2.17 = 11.39 m²; lower ellipsoidal head (partial jacket coverage) Ah≈2.20 m². Total available area:

Aavail=11.39+2.20=13.59 m²

Design margin=13.59-10.1610.16×100=34%

The available jacket area exceeds the requirement by 34%, providing adequate margin for long-term fouling resistance and transient process disturbances such as feed rate excursions or temporary reduction in cooling water temperature.

### Mechanical Design

Both CSTRs operate under identical conditions, so the mechanical design applies equally to each reactor.

#### Reactor Geometry Optimisation

For a cylindrical vessel with internal diameter D and liquid height H:

VT=π4D²H=πα4D3⟹D=4VTπα13

where α = H/D is the aspect ratio. With VT=4.773 m³, several aspect ratios were evaluated:

Table 6-23 Effect of aspect ratio on reactor dimensions

| H/D Ratio | Diameter D (m) | Cylindrical Height H (m) |
| --- | --- | --- |
| 1.0 | 1.825 | 1.825 |
| 1.2 | 1.719 | 2.063 |
| 1.3 | 1.674 | 2.176 |
| 1.5 | 1.596 | 2.394 |
| 2.0 | 1.447 | 2.894 |

An aspect ratio H/D = 1.3 was selected as a compromise between mixing performance, solids suspension, vessel height, and fabrication requirements, consistent with commercially available industrial digestion reactors of comparable capacity. Substituting α = 1.3:

Di=4×4.773π×1.313=1.67 m

H=1.3×1.67=2.17 m

Total internal vessel volume including two 2:1 ellipsoidal heads (Vh=πD324≈ 0.61 m³ each):

Vvessel=4.773+2×0.61=5.99 m³ ≈ 6.0 m³

#### Material Selection

Glass-lined carbon steel was selected. Carbon steel provides excellent structural strength at low cost but requires internal protection against hydrochloric acid attack. Austenitic stainless steels (304, 316L) corrode rapidly by localised chloride-induced attack at the operating temperature. FRP loses mechanical strength at elevated temperature and is generally unsuitable for pressurised service at 120 °C. Titanium alloys resist many aggressive media but are rapidly attacked by reducing acids such as HCl unless specialised alloys are used, and their high cost is economically unjustified here. The glass lining provides an inert, non-porous vitreous barrier with exceptional resistance to hydrochloric acid and aluminium chloride solutions, preventing product contamination and fouling. Glass-lined carbon steel is the established industry choice for hydrochloric acid digestion and inorganic chemical processing.

Maintenance of glass-lining integrity is critical to process reliability and vessel longevity. Following installation and after any repair, the lining should be holiday-tested by high-voltage spark testing (typically 15–20 kV for 2 mm lining thickness) to detect pinholes, holidays, and insufficient coverage before commissioning. The lining is susceptible to thermal shock: temperature differentials exceeding approximately 120 °C applied suddenly can initiate cracking. Startup and shutdown procedures must therefore impose controlled heating and cooling rates (typically ≤30°C min⁻¹) and avoid steam injection or cold-water quench into a hot vessel. Routine inspection intervals of 12–24 months are recommended for inspection of lining condition via visual examination and spark re-testing, particularly at nozzle welds, baffle attachment points, and areas subject to slurry impingement. Any lining damage exposing bare carbon steel should be repaired by acid-resistant plug or patch before further operation to prevent accelerated corrosion and metallic contamination of the aluminium chloride product.

#### Pressure Vessel Design Criteria

The reactor shell was designed to ASME Boiler and Pressure Vessel Code (BPVC), Section VIII, Division 1 (ASME, 2025).

Table 6-24 Pressure vessel design criteria

| Parameter | Value | Basis |
| --- | --- | --- |
| Design code | ASME BPVC Sec. VIII, Div. 1 | International pressure vessel standard |
| Design pressure, Pd | 5 bar(a) (0.4 MPa gauge) | Operating pressure plus design margin |
| Design temperature | 150°C | Operating temperature plus design margin |
| Material | Glass-lined carbon steel (SA-516 Gr. 70) | See Section 10.7.2 |
| Weld joint efficiency, E | 0.85 | Spot radiographed welded joint (ASME) |
| Corrosion allowance | 0 mm | Corrosion protection provided by enamlining |

#### Shell Thickness Determination

Minimum shell thickness under internal pressure per ASME BPVC Section VIII, Division 1, UG-27(c)(1):

t=PRSE-0.6P

where P = design gauge pressure = 5.0 − 1.0 = 4.0 bar = 0.4 MPa, R=Di2=1.672=0.835 m, S = allowable design stress at 150°C = 138 MPa (ASME BPVC Sec. II, Part D, Table 1A), E = 0.85:

tASME=0.4×0.835138×0.85-0.6×0.4=0.00285 m=2.85 mm

The ASME formula is a lower bound for pressure containment only; the governing criterion for a glass-lined agitated vessel of this type is structural rigidity and fabrication practicality rather than pressure load. Several additional requirements dictate a substantially greater thickness. Glass-lining (enamelling) subjects the carbon steel shell to temperatures of 820–880°C during firing; thin shells distort during this thermal cycle, causing glass delamination and void formation. Nozzle loads, agitator dynamic forces (torque reaction and bending through the mechanical seal housing), jacket attachment weld stresses, and the deadweight of the agitator assembly all impose structural demands absent from the ASME pressure formula. Minimum shell thickness for transport without permanent deformation and for reliable glass-lining adhesion in vessels of 1.5–2 m diameter is empirically 8–12 mm; this is confirmed by De Dietrich Process Systems, who publish standard wall thicknesses of 10–12 mm for enamel-lined vessels in the 4–8 m³ range. The adopted shell thickness is:

tselected=10 mm

This exceeds the ASME minimum by a factor of 3.5, providing adequate mechanical stiffness for continuous agitation, nozzle reinforcement, and glass-lining manufacture.

#### Head Design (2:1 Ellipsoidal Heads)

2:1 ellipsoidal heads were selected for both closures, offering a favourable balance of mechanical strength, fabrication cost, and hydraulic performance for mechanically agitated vessels at moderate pressure (Towler and Sinnott, 2022). Their smooth contour promotes uniform stress distribution and reduces solids accumulation at vessel extremities.

For a standard 2:1 ellipsoidal head: head diameter Dh=Dv=1.67 m; head depth h=Dv4=0.42 m. Required head thickness per ASME BPVC Section VIII, Division 1, UG-32:

th=0.5PDSE-0.1P

Substituting P = 0.4 MPa, D = 1,670 mm, S = 138 MPa, E = 0.85 yields th,min ≈1.8 mm. As with the shell, the calculated minimum is governed in practice by fabrication and structural considerations. A nominal thickness:

th=10 mm

was adopted for both heads, consistent with the shell thickness. Each ellipsoidal head contributes Vh=πD324=0.61 m3 of additional internal volume, serving as operational freeboard for vapour disengagement, agitation-induced surface fluctuations, slurry expansion, and process upsets.

#### Nozzle Design

Nozzle sizes were determined from process flowrates and recommended fluid velocities for corrosive slurry service, then rounded to standard nominal bore (DN) sizes. For each nozzle: cross-sectional area A=Qvdesign,  then d=4Aπ0.5.

Hydrochloric acid feed — feed rate 2,285 kg h⁻¹, density 1,160 kg m⁻³, design velocity 2.0 m s⁻¹ (corrosive liquid service):

Qa=2,2851,160=1.97 m3h-1=5.47×10-4m3s-1; d=18.7 mm  →  DN25 selected

Slurry feed — 1,378 kg h⁻¹, density 1,106.6 kg m⁻³, design velocity 1.5 m s⁻¹ (to minimise erosion):

Qs=1,3781,106.6=1.245 m3h-1=3.46×10-4m3s-1; d=17.1 mm  →  DN25 selected

Product discharge — total flowrate 3,663 kg h⁻¹, Q = 3.311 m³ h⁻¹ = 9.20×10⁻⁴ m³ s⁻¹, design velocity 1.5 m s⁻¹:

d=27.9 mm  →  DN40 selected

Thermowell insertion depth: Lt=0.30Dv=0.30×1.67=0.50 m (within the well-mixed zone, clear of impeller). Nozzle reinforcement calculations per ASME UG-37 are required during detailed mechanical design to verify that shell metal removed by each opening is compensated by the nozzle neck, pad, or excess shell thickness. Given that the selected shell thickness of 10 mm substantially exceeds the minimum calculated value of 2.85 mm, nozzle reinforcement is expected to be satisfied by excess shell thickness alone for the smaller nozzles (DN25–DN50); this will be confirmed at the detailed engineering stage. The present design adopts standard industrial nozzle configurations consistent with glass-lined reactor practice.

Table 6-25 Summary of nozzle sizes

| Connection | Selected Size |
| --- | --- |
| Hydrochloric acid inlet | DN25 (1 in.) |
| Slurry feed inlet | DN25 (1 in.) |
| Product outlet | DN40 (1.5 in.) |
| Vent nozzle | DN50 (2 in.) |
| Thermowell | DN25 (1 in.) |
| Pressure transmitter | DN15 (½ in.) |
| Level transmitter | DN50 (2 in.) |
| Pressure relief valve | DN50 (2 in.) |
| Rupture disc | DN50 (2 in.) |
| Manway | 500 mm elliptical |

#### Agitation System Design

Reactor hydrodynamics govern solids suspension, liquid–solid mass transfer, temperature uniformity, and particle residence time distribution. The SCM assumes negligible external liquid-film resistance relative to product-layer diffusion; this requires vigorous agitation to continuously renew the liquid boundary layer around every particle. Agitation also redistributes the exothermic heat released, minimises concentration and temperature gradients, and ensures uniform acid contact.

The primary hydrodynamic objective is complete particle suspension, defined by Zwietering (1958) as the condition where no particle remains stationary on the vessel floor for more than 1–2 seconds. A six-bladed 45° pitched-blade turbine (PBT) was selected for its superior axial flow, efficient particle lifting, and lower power consumption compared with radial-flow impellers for solid–liquid suspension (Paul et al., 2004).

#### Minimum Suspension Speed (Zwietering Correlation)

The Zwietering correlation predicts the just-suspended impeller speed Njs for geometrically standard baffled vessels (Zwietering, 1958):

Njs=S·g0.45·ρs-ρl0.45·ρl-0.55·dp0.13·X0.13·Di-0.85·μ0.10

where Njs=just-suspended speed (s⁻¹), S = impeller geometry constant, g = 9.81 m s⁻², ρs=particle density (kgm⁻³), ρl=slurry density (kg m⁻³), dp=particle diameter (m), X = solids mass fraction, Di=impeller diameter (m), μ = slurry viscosity (Pa·s).

Table 6-26 Physical properties for agitation design

| Parameter | Symbol | Value |
| --- | --- | --- |
| Slurry density | ρl | 1,106.6 kg m⁻³ |
| Particle density (calcined bauxite) | ρs | 3,290 kg m⁻³ |
| Mean particle diameter | dp | 75 μm |
| Solids mass fraction | X | 0.2094 |
| Design slurry viscosity | μ | 2.0 × 10⁻³ Pa·s |
| Gravitational acceleration | g | 9.81 m s⁻² |

A viscosity of 2.0 × 10⁻³ Pa·s was adopted for the suspension calculation (conservatively higher than the heat-transfer design value), reflecting the reduced liquid viscosity at 120 °C partially offset by the increased apparent viscosity from suspended solids. The particle density of 3,290 kg m⁻³ represents high-alumina calcined bauxite, which contains silica and other mineral constituents rather than pure alumina (3,950 kg m⁻³).

For the 45° PBT with impeller-to-tank diameter ratio DiT=0.33:

Di=0.33×1.67=0.551 m

Using S = 4.7 for a 45° pitched-blade turbine (Zwietering, 1958):

Njs=1.72 s-1=103 rpm

#### Operating Speed and Flow Regime

Industrial slurry reactors operate at 1.1-1.3×Njs to ensure reliable suspension under variations in particle size distribution, solids loading, and rheology (Nienow, 1997). An operating factor of 1.2 was selected:

N=1.2×Njs=1.2×103=124 rpm=2.07 s⁻¹

Impeller Reynolds number:

Rei=ρNDi2μ =1,106.6×2.07×0.55121.18×10-3 = 5.89 × 10⁵

Since Rei≫10⁴, the agitation system operates in the fully turbulent regime, where the power number is independent of Reynolds number (Oldshue, 1983). Fully turbulent flow promotes strong bulk circulation, efficient solids suspension, and rapid liquid boundary-layer renewal, preserving the SCM assumptions.

#### Agitator Shaft Power

In the fully turbulent regime, shaft power is determined from the dimensionless power number Np:

P=Np·ρ·N³·Di5

For a six-bladed 45° PBT in turbulent flow, literature values of Np range from 1.2 to 1.5 (Nienow, 1997; Paul et al., 2004); a design value of Np=1.30 was adopted:

P=1.30×1,106.6×2.073×0.5515=648 W=0.648 kW

Specific power input:

PV=0.6484.773=0.136 kW m⁻³

A power density of 0.136 kW m⁻³ may appear modest relative to typical rules of thumb for slurry mixing, which often quote 0.5–2.0 kW m⁻³ for solids suspension duties. However, this relatively low value is consistent with several characteristics of this application: (i) the bauxite particles are fine (dp=75 μm), requiring less energy for suspension than coarse solids; (ii) the density difference between particles (3,290 kg m⁻³) and slurry (1,106.6 kg m⁻³) is moderate; (iii) the chosen PBT geometry is optimally efficient for axial-flow solid suspension; and (iv) the relatively low aspect ratio (H/D = 1.3) reduces the pumping head required for full vessel circulation. The Zwietering Njs approach explicitly accounts for all these factors, providing a more reliable power estimate than generic volume-specific rules. At the installed motor rating of 2.0 kW, the installed-to-shaft-power ratio of 3.1 provides ample margin for start-up on a settled bed and for viscosity increases during process upsets or startup from cold. The calculated shaft power is therefore accepted as physically realistic for this duty (Towler and Sinnott, 2022; Nienow, 1997).

#### Motor and Gearbox Sizing

Installed motor power must accommodate transmission losses, start-up loads, and process fluctuations:

Pmotor=Pshaft×SFηg

Applying a service factor SF = 1.50 (appropriate for slurry-handling equipment; Towler and Sinnott, 2022) and gearbox efficiency ηg=0.95:

Pmotor=0.648×1.500.95=1.02 kW→standard rating 2.0 kW selected

Required gearbox reduction ratio (motor speed 1,450 rpm, design agitator speed 124 rpm):

i=NmotorNagitator=1,450124=11.7→12:1 standard gearbox selected

Noutput=1,45012=121 rpm

(deviation: -2.4%; acceptable for slurry duty)

#### Baffle Configuration

Four vertical wall-mounted baffles are required to suppress vortex formation, convert tangential fluid momentum into axial and radial circulation, and improve solids suspension (Paul et al., 2004). The standard baffle configuration used in the Zwietering correlation and power number correlations — four baffles at 90° intervals — was adopted. Baffle width per Towler and Sinnott (2022):

B=T10=1.6710=0.167 m

Four baffles of width 0.167 m, extending the full straight-side height with small wall clearance, provide a fully baffled mixing environment consistent with the suspension and power calculations.

#### Mechanical Seal

The agitator shaft penetrates the pressurised reactor shell, requiring an effective seal against slurry leakage. Packed gland seals are unsuitable: they require controlled leakage for lubrication, wear rapidly in abrasive slurry service, and allow HCl leakage with associated corrosion and safety hazards. A dual mechanical seal was selected, comprising two sealing interfaces separated by a pressurised barrier fluid, providing secondary containment for hazardous or corrosive service (Bloch and Geitner, 2019). Seal face material: reaction-bonded silicon carbide (SiC/SiC) for abrasive-wear resistance. Secondary sealing elements: PTFE (excellent mineral acid resistance). Metallic wetted components: Hastelloy C-276 (chloride corrosion resistance). Barrier fluid pressure is maintained 1–2 bar above reactor operating pressure, ensuring leakage direction is from the barrier system into the reactor rather than outward.

#### Agitator Shaft Design

Torque Calculation

ω=2πN60=2π×12460 = 12.99 rad s⁻¹

T=Pω=64812.99=49.9 N·m≈50 N·m

Shaft Diameter

Minimum shaft diameter from ASME torsion equation for solid circular shafts (Budynas and Nisbett, 2020), allowable shear stress τallow=40 MPa:

ds=16Tπτallow13=16×50π×40×10613=18.5 mm

Applying a design factor of 2 for fluctuating loads, start-up torque, and erosion effects typical of slurry service:

ddesign=2×18.5=37 mm →dselected=40 mm (standard)

Combined Loading

The equivalent twisting moment for combined bending and torsion (Budynas and Nisbett, 2020), with bending shock factor Kb=1.5 and torsion shock factor Kt=1.0 for moderate-duty agitator service, and assuming a conservative bending moment M=T=50 N·m:

Te=[KbM2+KtT2]=1.5×502+1.0×502=90.1 N·m

This remains well within the capacity of the 40 mm shaft, confirming adequate torsional strength and bending stiffness for continuous agitation.

Table 6-27 Agitator shaft design summary

| Parameter | Value |
| --- | --- |
| Shaft power | 0.648 kW |
| Operating speed | 124 rpm |
| Transmitted torque | 50.0 N·m |
| Equivalent combined torque | 90.1 N·m |
| Minimum shaft diameter (torsion) | 18.5 mm |
| Selected shaft diameter | 40 mm |

#### Support Structure Design

#### Support Type Selection

Four equally spaced welded lug supports were selected for both reactors. Lug supports are the standard choice for vertical process vessels with aspect ratios H/D < 2.0, moderate operating weights, and indoor or sheltered installation (Brownell and Young, 1959; Towler and Sinnott, 2022). For this vessel (H/D = 1.3, operating weight ~6,500 kg), lug supports offer several advantages over a cylindrical skirt:

- They allow unobstructed access to the bottom head and product discharge nozzle, facilitating slurry piping installation and maintenance.

- The vessel rests on structural steel columns attached to the lug base plates, enabling the vessel to be elevated to a process-appropriate height without a continuous skirt ring.

- Thermal expansion of the glass-lined carbon steel shell is less constrained than with a welded skirt, reducing differential thermal stresses at the vessel–support junction during heat-up and cooldown.

- Four lugs at 90° intervals are consistent with the four-baffle geometry and provide balanced load distribution around the vessel circumference.

Lug supports are not recommended for tall vessels (H/D > 2.5), high wind or seismic zones, or vessels above approximately 50,000 kg, none of which apply here.

#### Operating Weight

The support was designed for maximum operating weight:

(a) Reactor contents: Wcontents=ρs×Vw=1,106.6×3.818=4,225 kg

(b) Shell: Vshell=πDHt=π×1.67×2.17×0.01=0.114 m3;Wshell=0.114×7,850=895 kg

(c) Two 2:1 ellipsoidal heads: surface area≈1.084D2=3.02 m2 each; Wh=3.02×0.01×7,850=237 kg; Wheads=2×237=474 kg

(d) Agitator assembly (motor, gearbox, shaft, impeller): Wagitator=250 kg

(e) Jacket: Wjacket=650 kg

Wtotal=4,225+895+474+250+650=6,494 kg

F=Wtotal×g=6,494×9.81=63.7 kN

#### Design Load

A structural design factor of 1.5 was applied to account for dynamic agitation loads, maintenance loads, and minor seismic effects:

Fd=1.5×63.7=95.6 kN (≈ 96 kN)

#### Load Per Lug

With four equally-spaced lugs, the vertical load carried per lug under design conditions:

Flug=Fd4=95.64=23.9 kN≈24 kN per lug

#### Lug Geometry

Lug dimensions were selected following Brownell and Young (1959) for a vessel of this diameter and loading. Each lug comprises a welded steel plate assembly consisting of a bearing plate (resting on the supporting column cap plate), a gusset plate transferring load to the vessel shell, and a top plate providing attachment continuity with the shell.

Table 6-28 Lug support geometry

| 11 | Value | Basis |
| --- | --- | --- |
| Number of lugs | 4 | Symmetric spacing; consistent with baffle geometry |
| Lug spacing | 90° | Equal circumferential distribution |
| Bearing plate width, B | 150 mm | Brownell & Young (1959), D = 1.67 m |
| Bearing plate length, C | 200 mm | Contact length over supporting column |
| Gusset plate height, HL | 200 mm | Load transfer to shell |
| Plate thickness | 12 mm | Exceeds shell thickness; provides adequate weld area |
| Material | Carbon steel (SA-516 Gr. 70) | Consistent with vessel shell |
| Weld to shell | 8 mm fillet weld, full perimeter | Per AWS D1.1 minimum for structural attachment |

#### Shell Bending Stress at Lug Attachment

Lug supports introduce a local concentrated load at the shell weld, generating bending and membrane stresses in the shell wall. The local shell bending stress at the lug attachment per Zick (1951) and Brownell and Young (1959):

The bearing area of each lug on the shell:

Abearing=B×HL=0.150×0.200=0.030 m2

Local shell bearing stress:

σbearing=FlugAbearing=23,9000.030=797 kPa=0.80 MPa

This is well below the allowable shell membrane stress of 138 MPa (ASME BPVC Sec. II, Part D) and confirms that shell bearing stress is not a limiting condition.

The local bending moment at the lug-to-shell junction, using a conservative moment arm equal to half the gusset height:

Mlocal=Flug×HL2=23,900×0.100=2,390 N·m

Shell section modulus at the lug location (per unit circumferential length, using the shell thickness t = 10 mm):

Z=t26=0.01026=1.67×10-5 m3/m

Local bending stress:

σlocal=MlocalZ×lug width=2,3901.67×10-5×0.150=953 kPa≈1.0 MPa

The combined local stress (bearing + bending) of approximately 1.8 MPa is negligible relative to the allowable shell stress of 138 MPa, confirming that no stiffening rings or pad reinforcement are required for this lug geometry and shell thickness. This conclusion is consistent with published lug design charts for vessels of this diameter and shell thickness (Brownell and Young, 1959; Moss, 2004).

#### Supporting Column Design

Each lug bears on a structural steel column (leg). A preliminary column section is selected for the compressive load per leg of 24 kN. Using a standard hollow square section (100 × 100 × 5 mm SHS, carbon steel Grade S275):

Column cross-sectional area: Acol=4×(100×5)-4×(5×5)=1,900 mm²

Compressive stress in column:

σcol=FlugAcol=23,9001,900×10-6=12.6 MPa

This is well below the allowable compressive stress for S275 steel (~165 MPa at the column height of approximately 1.0 m, where buckling is not critical). Standard 100 × 100 × 5 SHS columns are adequate for this duty. Final column section selection, base plate sizing, and foundation bolt design will be confirmed at the detailed engineering stage.

#### Anchor Bolt Arrangement

Each lug base plate is anchored to the supporting column cap plate or concrete pad by two M20 bolts per lug (eight bolts total). The tensile load per bolt arises primarily from overturning moments due to agitation forces and piping loads rather than from vertical dead weight alone. For the operating weight and agitator torque:

Overturning moment from agitator (conservative upper bound, using torque arm = vessel radius):

MOT=Tagitator×D2=50×0.835=41.75 N·m

This is negligible relative to the weight-induced compressive load, confirming that anchor bolts are loaded primarily in compression-relief (rocking) rather than tension. Two M20 bolts per lug with standard embedment depth are more than adequate. Detailed anchor bolt design for wind and seismic loads (where applicable) will be performed during detailed structural engineering per local building codes and ASCE 7.

## DESIGN OF A BALL MILL – AMOAH, Kelvin

### INTRODUCTION

Size reduction stands as one of the most energy-intensive and operationally critical unit operations in chemical and mineral process engineering. It is estimated that approximately 3 to 4 percent of the total electrical energy consumed globally is expended on grinding operations, with mineral processing circuits alone accounting for most of this demand (Wills & Finch, 2016). Despite its apparent simplicity as a mechanical operation, the science governing grinding encompasses fracture mechanics, surface chemistry, fluid dynamics, and thermodynamics in a deeply interrelated manner. Achieving the correct product particle size is not merely an engineering convenience; in hydrometallurgical extraction processes, it constitutes the primary determinant of downstream reactor performance, reagent efficiency, and ultimately, process economics.

In the context of this design project, the ball mill occupies a uniquely pivotal position within the process flowsheet. The raw bauxite ore, as delivered from the primary jaw crushing circuit at a feed size (F80) of 9,000 microns (9.0 mm), possesses an extremely low reactive surface area per unit mass. In this coarse state, the dissolution reaction between the aluminium hydroxide minerals of the ore and the leaching acid is severely kinetically limited. The ball mill designated ML-501 is therefore designed as a critical kinetic enabler: its sole mandate is to transform the coarse crusher product into a fine, reactive slurry with an 80% passing size (P80) of 74 microns, which is the standard 200-mesh specification identified in the hydrometallurgical literature as optimal for achieving maximum aluminium extraction under atmospheric HCl leaching conditions. s

#### PROBLEM STATEMENT

Ghanaian gibbsitic bauxite, sourced principally from the Nyinahin-Mpasaaso deposits in the Ashanti and Western Regions and from the Atewa Range of the Eastern Region, is mineralogically characterised by a tri-hydrate aluminium hydroxide mineral, gibbsite (AlOH3), as the dominant aluminium-bearing phase. Subsidiary aluminium phases include boehmite (AlOOH) and minor kaolinite (Al2Si2O5OH4). Iron-bearing gangue minerals, including goethite (FeO(OH)) and haematite (Fe2O3), are present at varying concentrations, along with residual quartz (SiO2) in certain deposit profiles (Ntoni et al., 2022).

The primary acid leaching reaction responsible for aluminium extraction into solution is the heterogeneous fluid-solid dissolution:

Al OH3s+ 3 HCl aq→ AlCl3(aq) + 3 H2O (l)

This reaction is thermodynamically highly favoured under mild conditions. However, as a heterogeneous reaction, its overall rate is governed not by intrinsic chemical kinetics alone but rather by the rate at which the reacting acid species can diffuse through the boundary layer surrounding each solid particle and access the reactive mineral surface. For coarse particles emerging from the crusher at F80 = 9,000 microns, this boundary layer diffusion resistance is severe.

Without adequate grinding, the following harmful consequences are observed in the downstream leaching reactor:

- severely reduced aluminium extraction yields below the design target of 90%;

- substantially increased HCl acid consumption per unit of PAC product, since unreacted ore particles consume acid stoichiometrically only at their surfaces without full dissolution;

- increased leaching residence time requirements, potentially doubling or trebling the reactor volume needed; and

- compromised product quality, as incomplete leaching introduces iron and silica impurities into the aluminium chloride leachate that are difficult to remove in subsequent purification stages. The ball mill ML-501 therefore serves as an indispensable prerequisite for any economically and technically viable HCl leaching operation on this ore.

#### AIM

To perform the process and mechanical design of a wet ball mill (ML-501) operating in a closed circuit to grind Ghanaian gibbsitic bauxite from F80 = 9,000 microns to a product size of P80 = 74 microns for downstream acid leaching.

#### OBJECTIVES

- To justify the selection of a wet-grinding ball mill for bauxite size reduction prior to leaching.

- To determine the required mill dimensions (diameter and length), operating speed, grinding media charge, and motor drive power using Bond’s Work Index.

- To perform mechanical design calculations for the mill shell plate thickness, girth gear drive requirements, and trunnion support stresses.

- To make detailed engineering drawings and layout schematics of the ball mill assembly.

### LITERATURE REVIEW

#### EVOLUTION OF GRINDING MECHANICS

The mechanical reduction of solid materials to fine particle sizes has been practised in one form or another since the earliest applications of human technology. However, the attempt to relate energy input to size reduction output through quantitative laws began seriously during the nineteenth century. Rittinger (1867) proposed the first quantitative comminution law, stating that the energy required for size reduction is proportional to the new surface area created. Kick (1885) subsequently proposed an alternative law asserting proportionality to the ratio of feed to product size on a logarithmic basis, which better described the crushing of coarser materials. Neither law proved universally satisfactory across the full range of industrial particle sizes.

The most practically significant advance came with the work of Fred Chester Bond (1952, 1961) of Allis-Chalmers, who proposed his celebrated Third Theory of Comminution as an intermediate between the Rittinger and Kick approaches. Bond's empirical law, based on extensive experimental data from both laboratory and industrial grinding circuits, states that the work input is proportional to the difference between the inverse square roots of the product and feed particle sizes. The Bond Work Index (Wi), measured in kilowatt-hours per tonne, serves as the material-specific grindability constant that anchors the equation to the physical properties of each ore. Bond's Third Theory, implemented through his standardised laboratory grindability test, has remained the dominant tool for industrial mill sizing for over seven decades and is employed as the primary design basis in this work (Napier-Munn et al., 1996; Wills & Finch, 2016).

Subsequent research through the 1980s and 1990s, particularly the development of mechanistic population balance models by Austin, Klimpel & Luckie (1984) and the physics-based mill power models of Morrell (1996), has extended the theoretical framework beyond Bond's empirical approach. Modern computational tools such as JKSimMet and the JKMRC Perfect Mixing Model allow dynamic simulation of grinding circuits. Nevertheless, for the plant capacity scale of this project, the Bond method retains its dominant position in industry practice and academic design pedagogy as the most reliable and well-validated approach (Napier-Munn et al., 1996).

#### SELECTION OF GRINDING EQUIPMENT

Five principal size-reduction technologies are capable, in principle, of taking the F80 = 9,000 micron crusher product down toward a fine grind: the rod mill, the autogenous/semi-autogenous (AG/SAG) mill, the high-pressure grinding roll (HPGR), the vertical stirred mill, and the tumbling ball mill. Each was evaluated against the specific requirements of this grinding to a fine product target (P80 = 74 microns), a soft, friable, low-hardness ore, and a modest plant throughput (2.085 t/h) — before selecting the ball mill.

| Technology | Operating Principle | Reason for Rejection |
| --- | --- | --- |
| Rod mill | Steel rods perform line-contact impact and abrasion | Bottoms out at a coarser product (typically 850 µm–1 mm); cannot economically reach the 74 µm target (Wills & Finch, 2016; Soares & Almeida, 2020) |
| AG/SAG mill | Coarse ore (or ore + few large balls) is self-grinding via massive tumbling impact | Requires large, competent ore lumps and high tonnage to be economical; uneconomical and oversized for a 2.085 t/h battery-limit duty (Napier-Munn et al., 1996) |
| HPGR | Ore is compressed between two counter-rotating rolls at very high pressure | Efficiency advantage is greatest on hard, competent ores; gibbsitic bauxite's low hardness (Mohs 2.5–3.5) gives little benefit over conventional milling, while capital and maintenance complexity are both higher (Wills & Finch, 2016) |
| Vertical stirred mill | Agitated media bed grinds fine particles by low-energy attrition | Economically suited to ultra-fine regrind duty (P80 < 40 µm) downstream of primary grinding, not as a primary mill on a 9,000 µm feed (Napier-Munn et al., 1996) |
| Ball mill (selected) | Tumbling steel ball charge grinds by combined impact and attrition | Matches the full size range (9,000 µm feed to 74 µm product); well-proven on soft, friable ore; lowest capital and technical risk at this plant scale (Wills & Finch, 2016) |

The ball mill was selected on four converging grounds. First, product size range: ball mills routinely and economically achieve products in the 74 µm range required for HCl leaching, whereas rod mills are structurally limited to much coarser products and vertical stirred mills are optimised for far finer regrind duties, not primary grinding of a 9 mm feed (Wills & Finch, 2016). Second, ore hardness match: the cascading-cataracting charge motion of a ball mill, combining impact and attrition breakage, is well matched to the low Mohs hardness and porous, friable texture of gibbsitic bauxite; HPGR and SAG technologies derive their principal efficiency advantage from ore competency and coarse lump strength that this ore does not exhibit (Napier-Munn et al., 1996). Third, scale appropriateness: AG/SAG mills and HPGR circuits are capital-intensive technologies whose efficiency gains are realised at large throughput — neither is justified at the modest 2.085 t/h design capacity of this plant. Fourth, technology risk and industry precedent: the ball mill is the most extensively documented and vendor-supported technology for bauxite and lateritic ore grinding worldwide, including specifically on bauxite duty (Soares & Almeida, 2020), giving the highest confidence in the Bond-based design methodology and cost data used throughout this chapter.

The ball mill is further specified in closed circuit with a hydrocyclone classifier rather than in open circuit, for the reasons already established in Section 6.2.2: closed-circuit operation prevents over-grinding, tightly controls the P80 specification, and is the configuration universally adopted in industrial practice wherever a fine, tightly specified product is required (Wills & Finch, 2016).

#### FUNDAMENTALS OF GRINDING CIRCUITS

A grinding circuit describes the configuration of mill equipment and classifying devices through which feed material passes to achieve a defined product specification. Two fundamental configurations exist: open circuit and closed circuit.

In an open-circuit configuration, the feed material makes a single pass through the mill, and the mill discharge constitutes the final product, regardless of the particle size distribution achieved. Open circuits are simple to operate and require minimal capital investment. However, they suffer from fundamental inefficiencies when fine product sizes are required: the mill must be over-ground on average to ensure that even the largest particles achieve the target size, resulting in the generation of an excessive proportion of ultrafine particles. This over-grinding wastes energy increases grinding media and liner wear, and can generate problematic slimes in downstream liquid-solid separation operations. For applications requiring a tight product size specification such as the P80 = 74 microns required for HCl bauxite leaching, open-circuit mills are energetically and qualitatively inadequate (Wills & Finch, 2016, Chapter 7).

In a closed-circuit configuration, the mill discharge is directed to a classification device (in our plant design, a hydrocyclone) which separates the product stream into a fine overflow fraction (the circuit product) and a coarse underflow fraction (the circulating load), which is recycled to the mill inlet for further grinding.

#### BAUXITE MINERALOGY AND GRINDABILITY

Ghanaian gibbsitic bauxite belongs to the class of tropical lateritic bauxites formed through intense chemical weathering of aluminium-bearing parent rocks under humid, subtropical to tropical climatic conditions over geological timescales. The dominant aluminium mineral is gibbsite, AlOH3, present as fine crystalline aggregates within a porous, earthy to pisolitic ore texture. The Mohs hardness of gibbsite is approximately 2.5 to 3.5, placing it in the 'soft' to 'very soft' mineral category relative to industrial ores, while the overall ore hardness is moderated by the presence of goethite and haematite, which are harder (Mohs 5 to 5.5) but typically present as minor phases (Ghana Geological Survey Authority, 2020).

This mineralogical character is directly reflected in the relatively low Bond Work Index of gibbsitic bauxite (Wi = 9.45 to 11.0 kWh/tonne, metric), compared to ores such as quartz-hosted gold ore (Wi = 18 to 22 kWh/t), copper porphyry (Wi = 12 to 16 kWh/t), or iron-formation magnetite (Wi = 14 to 17 kWh/t). The low Wi has important design implications: it means that ball mills of relatively modest power can achieve the target P80 = 74 microns, and that the risk of fracturing the hard steel balls against excessively hard ore particles is low. A design Wi of 10.0 kWh/tonne (metric) has been adopted in this work as a conservative, industry-supported value for Ghanaian gibbsitic bauxite (Bond, 1961; Wills & Finch, 2016, Table 5.2).

The porous, friable (crumbly) texture of lateritic bauxite also implies that primary fracture occurs preferentially along existing micro-crack networks and grain boundaries, a mechanism more consistent with attrition and low-energy impact than with high-energy catastrophic fragmentation. This favours the cascading-cataracting charge motion of a ball mill operated at 70 to 75% of critical speed over the high-energy tumbling of rod mills or the single-particle compression of HPGR systems, as the former promotes the combination of impact and abrasion that is most efficient for soft, porous minerals.

#### GRINDING MEDIA DYNAMICS: CASCADING VS. CATARACTING

The motion of the grinding charge within a rotating ball mill drum is governed by the balance between centrifugal force, gravitational force, and inter-particle friction. At low rotational speeds, the charge slides around the bottom of the mill in a rolling, cascading motion while balls tumble over each other at the charge surface and generate grinding primarily through abrasion. As rotational speed increases, the charge is lifted to a greater height before cascading, and a component of the charge is projected into parabolic trajectories across the mill void before impacting the toe of the charge or the mill liner. This combined motion is the operational regime that maximises the combined effect of attrition grinding (from cascading) and impact grinding (from the cataracting trajectory) and therefore maximises the grinding efficiency per unit of energy input.

Figure 6-18 Motion of charge in a tumbling mill

The critical speed of a ball mill (Nc) is defined as the rotational speed at which the centrifugal force on a ball at the mill shell exactly equals the gravitational force on that ball, such that it is on the verge of losing contact with the liner on its upward travel. At or above Nc, the entire charge rotates with the shell without any relative motion, achieving zero grinding. The formula for critical speed is derived directly from the balance of centrifugal and gravitational forces:

Nc (rpm) =42.3D0.5

where D is the internal mill diameter in metres. This is the standard formula presented in both Wills & Finch (2016) and Napier-Munn et al. (1996).

Industrial ball mills are universally operated at a fraction of critical speed to maintain the cataracting-cascading charge regime. The optimal fraction depends on mill diameter, ball size distribution, and ore characteristics, but for wet overflow ball mills processing soft to medium ores in the 74 to 212 micron product size range, an operating speed of 75% of Nc is widely accepted as producing the ideal 'cascading cataract' path, sufficiently energetic for impact breakage of coarser particles, yet with enough rolling abrasion for fine-grinding the intermediate and fine fractions (Napier-Munn et al., 1996; Austin et al., 1984). This 75% of Nc operating criterion is adopted in this design.

#### JUSTIFICATION OF MATERIAL SELECTION FOR MILL SHELL AND LINERS

The structural shell of ball mill ML-501 is specified in ASTM A36 structural carbon steel, with ASTM A516 Grade 70 pressure vessel steel as an alternative acceptable specification. ASTM A36 possesses a minimum yield strength of 250 MPa and a minimum tensile strength of 400 MPa, with an allowable design stress in pressure vessel applications of 166.7 MPa (approximately two-thirds of yield strength, consistent with standard mechanical design practice as per Towler & Sinnott, 2013, and Coulson & Richardson Vol. 6). The selection of carbon steel for the structural shell is justified on the following grounds:

- it provides adequate strength-to-thickness ratios for a vessel of this size operating under modest internal pressure loads;

- it is readily available in Ghana's industrial supply chain and is fully compatible with standard plate rolling, welding, and fabrication techniques;

- its cost per unit strength is substantially lower than stainless steel or alloy steels; and

- the external shell does not contact the corrosive slurry, which is instead managed by the internal liner system.

The internal wear liner system which protects the structural shell from the severe combined abrasion and impact loading of the steel ball charge, is specified in high-chromium cast iron (Cr content 15 to 28%, Ni-Hard or similar) or high-manganese austenitic steel (Hadfield steel, Mn content 11 to 14%). Both liner materials are capable of work-hardening under impact loading, developing surface hardness values of 400 to 600 HV while retaining adequate toughness to resist fracture. For fine-grinding applications (P80 less than 100 microns) where abrasion rather than high-energy impact is the dominant wear mechanism, natural or synthetic rubber liners are also a technically viable and increasingly common alternative, offering superior abrasion resistance at lower specific gravity and generating significantly lower noise levels. The liner system is designed as a replaceable wear component on a bolted attachment system, allowing scheduled liner replacement without major structural intervention, consistent with the maintainability requirements of continuous industrial operation.

### CHEMICAL ENGINEERING DESIGN CALCULATIONS

| Stream: 503(ML-501 Inlet). Component; Mass, kg/hr; Mass, %. H₂O; 1260.97; 60.47. Fe₂O₃; 0.86; 0.04. SiO₂; 393.74; 18.88. Fe₃O₄; 0.83; 0.04. Al₂O₃; 428.72; 20.56. Total; 2085.12; 100.00. Temperature: 35.25 °C Pressure: 1.1146 bar. |  | Stream: 504(ML-501 Outlet). Component; Mass, kg/hr; Mass, %. H₂O; 1260.97; 60.47. Fe₂O₃; 0.86; 0.04. SiO₂; 393.74; 18.88. Fe₃O₄; 0.83; 0.04. Al₂O₃; 428.72; 20.56. Total; 2085.12; 100.00. Temperature: 35.42 °C Pressure: 1.1146 bar. |
| --- | --- | --- |

Table 6-29 Chemical Design Input Parameters

| Parameter | Symbol | Value |
| --- | --- | --- |
| Fresh Feed Mass Flow Rate | Ms | 2.085 t/h (2,085.12 kg/h) |
| Feed Size (80% passing) | F80 | 9,000 microns |
| Target Product Size (80% passing) | P80 | 74 microns |
| Bond Work Index (gibbsitic bauxite) | Wi | 10.0 kWh/t |
| Steel Ball Bulk (Packing) Density | ρballs | 4.65 t/m3 |
| Ore Relative Density | ρore | 2.4 t/m3 |
| Joint (Weld) Efficiency Factor | E | 0.85 |
| Corrosion Allowance | C | 0.003 m |
| Motor/Gear Drive Efficiency | η | 0.85 |

Specific Grinding Energy (W)

The specific grinding energy required to reduce the feed from F80 = 9,000 microns to the target product P80 = 74 microns is calculated using Bond's (1961) Third Theory of Comminution equation.

W = 10 × Wi ×1P80- 1F80, kWh/t

W = 10 × 10.0 ×174- 19000

W = 100×0.1162- 0.0105

W = 100×0.1057

W = 10.57

The specific grinding energy is 10.57 kWh/t

Net Pinion Power (Pnet)

The net power required at the mill pinion shaft (the shaft that directly drives the mill drum) is the product of the specific grinding energy and the total mass throughput through the mill:

Pnet = W ×Mtotal

Pnet = 10.571 ×2.085

Pnet = 22.04 kW

Gross Motor Drive Power (Pmotor)

The gross motor power required accounts for the mechanical efficiency of the motor-to-pinion transmission train, which includes gearbox friction losses, pinion and trunnion bearing losses, and coupling inefficiencies. With a combined mechanical transmission efficiency of η = 85% (0.85):

Pmotor =Pnetη

Pmotor =22.040.85

Pmotor =25.93 kW

Applying a standard 15% engineering design safety margin to account for process upsets, startup torque surges, feed variability, and potential future capacity increase:

Pinstalled=25.93×1.15

Pinstalled=29.82 kW

Selecting the nearest standard IEC motor rating above the calculated installed requirement:

Recommended Installed Motor: 30kW (40.2 HP)

A 30kW motor provides a 0.64% margin above the calculated requirement with safety factor, ensuring reliable starting torque capability while remaining within standard commercial motor frame sizes widely available in the West African industrial equipment market.

Internal Physical Sizing (D and L)

The internal diameter (D) and length (L) of the mill drum are determined from the empirical mill power-dimension relationship. For a wet overflow ball mill operating at standard conditions (35% ball charge, 75% of critical speed), the net pinion power is related to mill dimensions by the Austin-Bond empirical equation:

Pnet = 4.8×L×D2.3

Applying the industry-standard Length-to-Diameter ratio L = 1.5 * D to substitute for L:

Pnet = 4.8×1.5 ×D×D2.3

Pnet = 4.8×1.5×D(1+2.3)

Pnet = 7.2×D3.3

Solving for D:

D3.3=Pnet7.2

D3.3=22.047.2

D3.3=3.061

D=3.06113.3

D=1.404

D=1.404m

Rounding up to the nearest standard mill shell size increment, D=1.45m

Applying the L/D = 1.5 ratio:

L=1.5 ×D

L=1.5×1.45

L=2.175m

Operational Mill Volume (Vm)

Vm=(π×D24)×L

Vm=(1.65130)×2.175

Vm=3.59m3

Critical Speed (Nc) and Operating Speed (N)

The critical speed is the rotational speed at which centrifugal force exactly overcomes gravity for a ball at the mill liner surface. Using the standard Bond-derived formula for a mill of internal diameter D = 1.45 m:

Nc= 42.3D=42.31.45

Nc= 35.13 rpm

The recommended operating speed, at 75% of critical speed to achieve the optimal cascading-cataracting charge motion:

Nop=0.75×Nc

Nop=0.75×35.13≈26.35 rpm

Grinding Ball Charge Mass

Assuming a standard ball charge volume fraction of J = 0.35 (35% of internal mill volume), the volume occupied by the ball charge is:

Vballs=J×Vm

Vballs=0.35×3.59

Vballs=1.257m3

The total mass of the ball charge is calculated using the specified packing (bulk) density of the steel ball charge, ρballs=4.65t/m3 :

Mballs=Vballs ×ρballs

Mballs=1.257 ×4.65

Mballs=5.85 tonnes(total steρel ball charge mass)

Maximum Ball Size (dmax)

The maximum ball diameter required to effectively nip and fracture the coarsest particles in the mill feed (F80 = 9,000 µm) is estimated using Bond's (1958) ball-size selection formula:

dB=25.4F80k0.5×ρs*Wi100 ×φc 3.281D13

where k = 350 for wet, overflow-discharge ball mills, ρs = 2.4 t/m³ (ore relative density), Wi = 10.0 kWh/t, φC = 0.75 and D = 1.45 m (≈ 4.76 ft):

dmax = 68mm (we will select the next standard make-up ball size, which is 70 mm)

Rationalised Ball Charge Grading

| Ball Diameter (mm) | Mass Fraction (%) | Approximate Mass (tonnes) | Function |
| --- | --- | --- | --- |
| 70 (top size) | 35 | 2.05 | Top size; coarse feed nipping |
| 60 | 30 | 1.75 | Primary impact grinding |
| 50 | 20 | 1.17 | Intermediate size reduction |
| 40 | 15 | 0.88 | Fine attrition grinding |
| TOTAL | 100 | 5.85 |  |

Slurry Residence Time

The mean residence time of the slurry inside the mill is estimated from the interstitial void volume of the ball charge (the space through which slurry flows) divided by the volumetric feed rate. The slurry density is first estimated from the fresh-feed solids fraction (bauxite 544.62 kg/h : water 833.38 kg/h → 39.5% solids by mass), ε = 0.40:

ρslurry=1xsρore+1-xsρwater =1,300 kg/m³

Qv=Mtotalρslurry=2085.121300= 1.60 m³/h

Vvoid = Vm × J × ε = 3.59 × 0.35 × 0.40 = 0.503 m³

τ = Vvoid / Qv

τ = 0.503 m³ / 1.60 m³/h

τ = 0.31h ≈18.8 mins

#### SUMMARY OF CHEMICAL ENGINEERING CALCULATIONS

Table 6-30 A summary of the chemical engineering calculations.

| PARAMETER | VALUE | UNIT |
| --- | --- | --- |
| Total Mill Throughput | 2.085 | t/h |
| Feed Size (80% passing) | 9,000 | microns |
| Product Size (80% passing) | 74 | microns |
| Specific Grinding Energy | 10.571 | kWh/t |
| Net Pinion Power | 22.04 | kW |
| Gross Motor Power (85% eff.) | 25.93 | kW |
| Installed Motor (with 15% margin) | 30 | kW (standard IEC) |
| Internal Mill Diameter | 1.45 | m |
| Internal Mill Length | 2.18 | m |
| Length / Diameter Ratio | 1.50 | - |
| Operational Mill Volume | 3.59 | m3 |
| Critical Speed | 35.13 | rpm |
| Operating Speed (75% Nc) | 26.35 | rpm |
| Ball Charge Volume Fraction | 0.35 | - |
| Volume of Ball Charge | 1.257 | m3 |
| Total Ball Charge Mass | 5.85 | tonnes |
| Maximum Ball Diameter | 68 (70 std.) | mm |
| Mean Slurry Residence Time | 18.8 | m |

### MECHANICAL ENGINEERING CALCULATIONS

Table 6-31 Mechanical Design Input Parameters

| Parameter | Value | Unit |
| --- | --- | --- |
| Internal Diameter, D | 1.45 | m |
| Internal Length, L | 2.18 | m |
| Shell Radius (R = D/2) | 0.725 | m |
| Total Ball Charge Mass, Mballs | 5,845(5.85 t) | kg |
| Ball Charge Volume, Vballs | 1.257 | m3 |
| Allowable Design Stress (ASTM A36), S | 166.7 | MPa |
| Joint (Weld) Efficiency, E | 0.85 | - |
| Corrosion Allowance, C | 0.003 | m (3 mm) |
| Steel Density (A36), ρ steel | 7,850 | kg/m3 |
| Ore/Slurry Density, ρslurry | 1,690 | kg/m3 |
| Gravitational acceleration, g | 9.81 | m/s2 |
| Dynamic angle of repose, θ | 35 | degrees |

#### Hydrostatic Dynamic Pressure Load (Phydro)

The hydrostatic dynamic pressure acting on the lower hemisphere of the mill drum is generated by the combined weight of the ball charge and the slurry filling the interstices of the ball charge and the dynamic liquid holdup volume. For design purposes, the pressure is approximated as the equivalent hydrostatic head at the bottom of the drum when the charge occupies the lower 35% of the mill cross-sectional area. The effective pressure-generating column height is taken as the centroid of the ball-and-slurry charge at the drum bottom, which for a 35% charge fill is approximately 0.35 * D below the mill centreline:

Effective charge column height:

h = 0.35×D

h= 0.35 ×1.45

h= 0.5075 m

The average density of the ball-slurry mixture in the lower charge zone is estimated as a volume-weighted average of the ball charge and the slurry filling the interstices (packing void fraction of balls approximately 40%):

Average charge mixture density:

ρmix=Jball × ρtrue balls+(1-Jball) × ρslurry

where Jball = packing fraction of solid steel = 0.60 (60%, i.e. 40% voids)

ρtrue balls (solid steel) = 7,850 kg/m3

ρmix=(0.6×7850)+(0.40×1690)

ρmix=4710+676

ρmix=5386kg/m3

Phydro= ρmix × g × h   [Pa]

Phydro = 5386 kg/m 3×  9.81 m/s2  ×  0.5075 m

Phydro = 5386 × 9.81 × 0.5075

Phydro= 26,814.60 Pa

Phydro= 26.81 kPa  = 0.02681 MPa

Required Shell Plate Thickness (ts)

The minimum required wall thickness of the cylindrical mill drum shell is determined from the standard thin-walled cylindrical pressure vessel hoop stress equation, modified for weld joint efficiency and corrosion allowance, as presented in Towler & Sinnott (2013, Chapter 13) and Coulson & Richardson (2005, Volume 6):

ts = (Phydro × D)(2 × S × E - 1.2 × Phydro)  +C  [m]

ts = (0.02681 MPa * 1.45 m)(2 × 166.7 × 0.85 - 1.2 ×  0.02681 MPa)  +0.003m

ts = 0.0388745 MPa. m(283.39 MPa) -(1.2 ×  0.02681 MPa)  +0.003m

ts = 0.0388745 MPa. m(283.39 MPa) -(0.032172 MPa)  +0.003m

ts = 0.0388745 MPa. m283.358 MPa  +0.003m

ts = 0.000137 m +0.003m

ts = 0.003137m

ts = 3.14mm

The purely hydraulic calculation yields a shell thickness requirement of only 3.14 mm, a value that is structurally inadequate for a rotating mill drum of this diameter and mass. The minimum shell thickness for a ball mill of 1.45 m internal diameter, based on rigidity and vibration resistance requirements for rotating machinery, is set by industrial practice at a minimum of 10 mm (Metso Outotec, 2020; Wills & Finch, 2016). The governing design criterion is therefore structural rigidity and dynamic loading resistance, not internal pressure.

The adopted Design Shell Thickness is therefore; ts  = 0.010 m (10 mm, ASTM A36 plate)

This 10 mm minimum shell thickness standard for mills in the 1.0 to 1.5 m diameter range is consistent with standard mill manufacturer specifications (Metso, FLSmidth, Outotec) and provides adequate stiffness against the dynamic bending and torsional loads imposed by the rotating ball charge. The internal wear liner system (high-Cr cast iron or Hadfield manganese steel, 40 to 60 mm thick) is mounted independently on the shell and provides the primary wear protection; it is not counted as structural thickness in this calculation.

Torque Requirements (T)

The torque required to lift the unbalanced ball charge from its lowest point to the dynamic angle of repose and to sustain continuous rotation against the gravity-induced restoring moment, is calculated by modelling the ball charge as a concentrated mass acting at the centroid of the displaced half-annulus of the charge. For a standard charge fill fraction of J = 0.35, with the dynamic angle of repose at theta = 35 degrees from the vertical, the gross torque is:

T = Mballs × g × (D/2) × sin(θ)   [N.m]

Mballs = 5,845 kg

g = 9.81 m/s2

D/2 = 0.725 m

sin(35) = 0.5736

T = 5,845  ×  9.81  ×  0.725  ×  0.5736

T = 23,845.18 N.m  ≈  23.85 kN.m  (Operating Torque)

Tangential Force on Girth Gear (Ft)

The tangential (driving) force transmitted through the girth gear tooth faces at the outer shell radius is obtained by dividing the operating torque by the mill outer shell radius. The outer shell radius includes the shell plate thickness:

Outer shell radius: Router = D2 + ts = 0.725 + 0.010 = 0.735 m

Ft = TRouter    [N]

Ft= 23,850 N.m  /  0.735 m

Ft= 32,440 N  ≈  32.44 kN  (Tangential Gear Force)

Total Structural Dead Weight (Wtotal)

The total structural dead weight loading on the mill trunnion bearings is the sum of the weights of the steel shell, the internal liner plates, the ball charge, and the slurry charge. Each component is calculated in turn.

Component 1: Steel Shell Weight (Wshell)

Shell volume:                   Vshell =π × D × ts × L

Vshell  = 3.14159 × 1.45 × 0.010 × 2.175

Vshell  = 0.0991 m3

Wshell =ρsteel × g × Vshell

Wshell = 7850 kg/m3×  9.81  ×  0.0991

Wshell= 7,630N

Component 2: Internal Liner Weight (Wliner)

Assuming 40 mm thick high-Cr cast iron liners covering full interior surface (conservative)

Liner volume: Vliner =π × D - 2×ts× 0.040 ×L + 2 × (π × (D/2)2 × 0.040)  (approx.)

Simplified: Vliner =π × D inner× 0.040 × L (cylindrical shell only)

D inner= 1.45 - 2*0.010 = 1.43 m

Vliner= 3.14159 × 1.43 × 0.040 × 2.175 = 0.391m3

ρliner= 7,200 kg/m 3(high-Cr cast iron)

Wliner = 7200 × 9.81 × 0.391 = 27,610 N

Component 3: Ball Charge Weight (Wball)

Wball= Mballs ×g = 5,845 × 9.81 = 57,340 N

Component 4: Slurry Charge Weight (Wslurry)

Dynamic slurry volume = 10% of V = 0.10 ×3.59= 0.359 m3

Wslurry=ρslurry× g × Vslurry = 1690 × 9.81 × 0.359 = 5,950 N

TOTAL STRUCTURAL DEAD WEIGHT

Wtotal = Wshell + Wliner+ Wballs + Wslurry  [N]

Wtotal =  7,630+ 27,610 + 57,340 +  5,950

Wtotal = 98,530 N ≈98.53 kN

This load is distributed equally between the two trunnion bearings:

Load per trunnion bearing = 98,530 / 2 = 49,270 N  ≈  49.27 kN per bearing

Longitudinal Bending Stress (σL)

The mill drum, supported at its two trunnion ends, is modelled as a simply supported horizontal beam of length L = 2.175 m subjected to a uniformly distributed load from its self-weight and contents. The maximum bending moment occurs at the mid-span and is given by:

Mmax = Wtotal ×( L8)   [N.m]

Mmax = 98,530 N ×( 2.1758)

Mmax = 214,302.75 / 8

Mmax = 26,787.84 N.m

Mmax = 26,790 N.m

The section modulus Z of the cylindrical drum shell cross-section (hollow cylinder approximation for thin shell) is:

Z =π × R2 × ts   [m3]

R =D2= 1.05 m

Z = 3.14159  ×  (0.725)2  ×  0.010

Z = 3.14159  ×  0.525625  ×  0.010

Z = 3.14159  × 0.00525625

Z = 0.01651 m3

The maximum longitudinal bending stress at mid-span is:

σL = Mmax / Z   [Pa]

σL= 26,790 N.m  /  0.01651 m3

σL = 1,622,359.868 Pa

σL=1.622 MPa (Longitudinal Bending Stress)

#### SUMMARY OF MECHANICAL ENGINEERING CALCULATIONS

Table 6-32 A summary of the mechanical engineering calculations.

| PARAMETER | VALUE | UNIT |
| --- | --- | --- |
| Internal Diameter | 1.45 | m |
| Internal Length | 2.18 | m |
| Adopted Shell Plate Thickness | 10 (0.010) | mm (m) |
| Governing Criterion for ts | Rigidity (not pressure) | - |
| Hydrostatic Dynamic Pressure | 26.81 | kPa |
| Calculated Minimum Thickness | 3.14 (incl. corrosion) | mm |
| Operating Torque | 23.85 | kN.m |
| Tangential Gear Force | 32.44 | kN |
| Steel Shell Weight | 7.63 | kN |
| Internal Liner Weight | 27.61 | kN |
| Ball Charge Weight | 57.34 | kN |
| Slurry Charge Weight | 5.95 | kN |
| Total Dead Weight | 98.53 | kN |
| Load per Trunnion Bearing | 49.27 | kN |
| Section Modulus | 0.01651 | m3 |
| Mid-span Bending Moment | 26.79 | kN.m |
| Longitudinal Bending Stress | 1.622 | MPa |
| Allowable Design Stress (A36) | 166.7 | MPa |
| Stress Utilisation Ratio | 0.97 | % |

## DESIGN OF A BASICITY ADJUSTMENT REACTOR

### INTRODUCTION

Poly-aluminium chloride (PAC)  is a highly effective inorganic polymer coagulant that is widely employed in water and wastewater treatment processes. It offers several advantages over traditional coagulants such as aluminium sulphate (alum), including a higher charge density, broader effective pH range, lower dosage requirements, reduced sludge production, and a smaller impact on the pH of treated water.

Basicity is one of the most critical parameters of PAC (Jiang and Graham, 1998; Duan and Gregory, 2003).

Basicity is one of the most critical quality parameters of PAC. It is defined as the molar ratio of hydroxyl ions (OH-) to aluminium ions (Al3+), often expressed as a percentage:

Basicity (%)=[OH-]3×[Al3+]×100

Commercial PAC products commonly target 40-90% basicity with the high basicity grades, usually between 65-92%, being particularly valued for its superior performance. A higher basicity corresponds to a greater degree of pre-hydrolysis and polymerisation of aluminium species, resulting in a higher catatonic charge density. This enhances the ability of the coagulant to neutralise negatively charged colloidal particles, promote rapid floc formation, improve turbidity and organic matter removal and maintain effectiveness across varying water conditions including low temperatures and low alkalinity waters.

Furthermore, a higher basicity reduces the consumption of alkalinity in the treated water because fewer H+ ions are released during hydrolysis compared to low-basicity or non-polymerised aluminium salts like alum. This minimizes the need for additional pH adjustment chemicals, lowers operational costs and decreases residual aluminium in effluents (Zhao et al., 2009; Yan et al., 2008).

### PROBLEM STATEMENT

The production of poly-aluminium chloride (PAC) from bauxite involves acid digestion that yields a low-basicity aluminium chloride solution, which is followed by a critical basicity adjustment stage. This stage is required to increase the OH/Al molar ratio of the aluminium chloride leachate by reacting it with calcium aluminate powder. The reaction  promotes the formation of polynuclear aluminium that give high-basicity PAC its superior coagulation performance. The precise design of a reactor in which this heterogeneous, exothermic reaction occurs is therefore essential for product quality, process safety and economic operation. The design addresses a continuous basicity adjustment system processing 2893 kgh-1 of aluminium chloride leachate with 687.215 kg h-1 of calcium aluminate powder at 85℃ and 1 bar.

### AIM

To design a continuous stirred tank reactor system for the basicity adjustment stage of poly-aluminium chloride production.

### OBJECTIVES

- To evaluate and select the most appropriate reactor type for the basicity adjustment reactions.

- To apply necessary reactor design equations and models to determine the principal dimensions and operating parameters of the reactor.

- To select appropriate materials of construction and instrumentation that ensure reliable, long-term operation in a corrosive chloride slurry environment.

- To present a comprehensive mechanical engineering drawing of a basicity adjustment reactor, including vessel geometry, agitation system, heat transfer arrangements and safety considerations.

### LITERATURE REVIEW

#### HISTORICAL DEVELOPMENT

PAC technology emerged in Japan in the late 1960s as an improvement over traditional aluminium-based coagulants like alum. Early processes focused on producing active aluminium hydroxide, which was then dissolved in hydrochloric acid to create partially hydrolysed species. By the 1980s, pressurized and heated reaction processes using aluminium hydroxide and hydrochloric acid had become widespread. Industrial adoption accelerated due to PAC’s superior performance in water treatment, leading to various raw material routes including bauxite, kaolin, and aluminium hydroxide. Calcium aluminate emerged as a preferred basifying agent for its cost-effectiveness and ability to achieve high basicity grades (up to approximately 92%) while simultaneously contributing additional aluminium to the product (Gao et al., 2005; Zhao et al., 2009).

#### FUNDAMENTALS OF BASICITY ADJUSTMENT REACTIONS

Basicity adjustment is the key unit operation following acid leaching, where low-basicity aluminium chloride solution, which is essentially AlCl₃ with low OH/Al ratio, is reacted with calcium aluminate powder. The overall process increases the molar ratio of OH⁻ to Al³⁺, driving hydrolysis and polymerization into polynuclear species that possess a high cationic charge density for coagulation. The simplified representative reaction is:

CaAl2O4 + AlCl3(solution)→PAC+CaCl2

This reaction is both exothermic and heterogeneous in nature. Calcium aluminate dissolves to release alkalinity and additional aluminium ions, with the rise in pH and OH/Al ratio promoting the condensation of monomeric aluminium species into larger polymers. Target basicity for high-performance grades is typically 65–92%. Careful control of addition rate, temperature (80–100°C), and mixing intensity is essential to avoid premature precipitation of Al(OH)3 or the generation of excessive insolubles.

#### DISSOLUTION STAGE

The first stage of basicity adjustment is the dissolution stage, which involves the heterogeneous dissolution of calcium aluminate powder in the acidic aluminium chloride leachate. The acid attacks the calcium aluminate surface, releasing Ca2+, Al3+ and OH- ions. This process is mass transfer limited initially and is strongly influenced by the particle size, temperature, acid concentration, and agitation intensity. Vigorous mixing ensures complete suspension and prevents local pH spikes that could cause gelation. Dissolution is faster in the early phase when acidity is high while free acidity remains high (Levenspiel, 1999).

Literature on analogous alumina and calcium-aluminate dissolution systems indicates that the shrinking-core model (SCM) provides a suitable kinetic description. Under typical PAC conditions the early stage is often surface-reaction controlled, later transitioning to ash-layer diffusion control as a residual silica-rich or insoluble layer forms. Activation energies reported for similar systems lie in the range 40–60 kJ mol⁻¹ for chemical-reaction control and lower values (~15–25 kJ mol⁻¹) when diffusion becomes rate-limiting (Levenspiel, 1999).

#### HYDROLYSIS AND POLYMERIZATION STAGE

As the pH rises and the OH/Al ratio increases, dissolved aluminium species undergo stepwise hydrolysis and subsequent polymerization. Monomeric Al3+ ions form hydrolysed species (AlOH42+), which condense into polynuclear complexes, notably the Al13 Keggin ion [Al₁₃O₄(OH)₂₄(H₂O)₁₂]⁷⁺  and larger polymers at higher basicity. This stage is slower than dissolution, equilibrium dependent and favoured at elevated temperatures (between 80℃ and 100℃) together with the controlled addition of the basifying agent. A residence time of  2-3 hours post addition is typically required to reach a stable high-basicity product with low residual insolubles (Zhao et al., 2009; Yan et al., 2008).

The distribution of aluminium species, which is commonly classified as Ala, Alb and Alc by the Ferron timed complexation method is strongly dependent on basicity. Low basicity solutions are dominated by monomeric Ala; medium to high basicity favours the highly coagulating Alb fraction including Al13; very high basicity leads to large aggregates that may eventually precipitate, process control must therefore maintain the system within the window that maximizes Alb while avoiding precipitation.

#### REACTOR OPERATION AND SELECTION

In industrial PAC production, basicity adjustment is often conducted at 70-100℃ with residence times of 2-3 hours. The operation involves a continuous or semi-continuous feeding of leachate and powder with temperature control via jackets or coils and downstream separation of insolubles. Agitation maintains homogeneity in the slurry formed.

CSTRs are preferred over other reactor types for this application due to:

- Their excellent mixing and solids suspension. This is essential for dispersing calcium aluminate powder and ensuring uniform liquid-solid contact, making it superior to PFRs, where settling or channelling can occur in slurries.

- Uniform temperature and concentration profiles. This is critical for controlling the exothermic reaction and avoiding hot spots or local over-neutralization. Batch reactors lack continuous throughput and PFRs risk gradients.

- Ease of continuous operation and good controllability. CSTRs are suitable for steady-state production with good controllability via RTD management (multiple CSTRs in series approach plug-flow performance while retaining mixing advantages) (Levenspiel, 1999).

- Practicality in corrosive slurries. Jacketed CSTRs simplify maintenance and heat transfer compared to complex tubular designs (Towler and Sinnott, 2022).

Batch reactors are flexible but inefficient for large-scale continuous plants. PFRs offer higher conversion per unit volume for homogeneous reactions but they struggle with solids handling and temperature control in the present heterogeneous system. Industrial practice and patents consistently describe stirred tanks (CSTRs) for this stage (Gao et al., 2005).

#### BASE SELECTION

Several basifying agents can be used for basicity adjustment in PAC production, each with distinct advantages, disadvantages, and impacts on product quality, cost and process operability. Common options include sodium hydroxide (NaOH), sodium aluminate (NaAlO₂ or Na₂Al₂O₄), calcium carbonate (CaCO₃), calcium oxide (CaO), and calcium aluminate (CaAl₂O₄).

Sodium hydroxide is a strong base that allows precise pH control and rapid basicity increase however, it introduces sodium ions, which may be undesirable in certain water treatment applications like drinking water due to the taste and regulatory limits, and can lead to higher costs and potential gelation if not dosed carefully. Sodium aluminate contributes both alkalinity and additional aluminium, supporting polymerization, but it is a relatively expensive option and adds sodium, potentially increasing product conductivity or affecting stability. Calcium carbonate and calcium oxide are lower-cost alkaline earth options however, CaCO₃ releases CO₂ during reaction which requires venting and can cause foaming, while CaO is highly reactive and exothermic but may lead to gypsum formation in sulphate-containing systems or scaling issues. Both can result in higher insoluble residues if not fully reacted. Calcium aluminate was selected as the preferred basifying agent in this design for several key reasons:

- Dual contribution of aluminium and alkalinity. It supplies both Al (typically ≥50% Al₂O₃ content) and alkalinity, increasing both overall aluminium concentration in the final PAC and basicity often to 65–92%. This improves product strength and reduces the need for additional aluminium sources.

- Cost-effectiveness and wide industrial availability. It is relatively inexpensive, widely available (often produced from bauxite or recycled materials) and supports high raw material utilization (>90% dissolution in many cases).

- Favourable reaction characteristics. The solid powder form allows controlled dissolution in the acidic leachate, providing a slower, more manageable release of alkalinity compared to liquid bases. This minimizes local over-neutralization, reduces insolubles and aids in achieving stable high-basicity PAC. The reaction is exothermic but spread over time, easing heat management in a CSTR.

- By-product management. It produces soluble CaCl₂ rather than problematic sodium salts or gaseous CO₂, simplifying downstream processing and improving product purity and stability.

- Industrial precedent. It is extensively used in bauxite-based PAC processes for achieving high-basicity grades suitable for water treatment, with proven scalability and compatibility with continuous reactor operations. (Zhao et al., 2009; Gao et al., 2005).

Overall, calcium aluminate offers the best balance of performance, economics, and product quality for large-scale PAC manufacturing from aluminium chloride leachate, making it the optimal choice for this CSTR design.

### REACTION MECHANISMS OF BASICITY ADJUSTMENT

#### DISSOLUTION REACTION

The initial stage of basicity adjustment involves the heterogeneous dissolution of solid calcium aluminate powder in the acidic aluminium chloride leachate. Calcium aluminate, which is CaAl₂O₄ and is often present with other calcium aluminates such as CaAl₄O₇ or 12CaO·7Al₂O₃, reacts with the excess hydrochloric acid and Al³⁺ species according to a simplified representative dissolution reaction:

CaAl₂O₄(s) + 8HCl(aq) → CaCl₂(aq) + 2ACl₂(aq) + 4H2O.

This surface-controlled process follows shrinking-core type behaviour where acid attacks the particle surface, releasing Ca²⁺, Al³⁺, and OH⁻ ions into solution while leaving minor insoluble residues. The rate is influenced by particle size, acid concentration, temperature, and mass transfer. Vigorous agitation in the CSTR ensures rapid renewal of the liquid film around particles, enhancing dissolution efficiency. This stage is relatively fast and exothermic, contributing significantly to the initial heat release and rise in pH or basicity (Levenspiel, 1999).

#### HYDROLYSIS AND POLYMERIZATION REACTION

Following dissolution, the released aluminium species undergo hydrolysis and subsequent polymerization in the increasingly less acidic environment. Monomeric aluminium ions (Al³⁺) hydrolyse stepwise:

Al³⁺ + H₂O ⇌ Al(OH)²⁺ + H⁺

Al(OH)²⁺ + H₂O ⇌ Al(OH)₂⁺ + H⁺

leading to polynuclear complexes such as [Al₁₃O₄(OH)₂₄(H₂O)₁₂]⁷⁺ (Al₁₃ Keggin ion) and larger polymers at higher basicity.

The overall process increases the OH/Al molar ratio, driving condensation reactions that form bridged Al–OH–Al structures. This stage is slower, equilibrium-dependent, and favoured by controlled temperature (80–98°C) and gradual base addition. Proper residence time allows polymerization to reach the desired degree without excessive precipitation of Al(OH)₃. The calcium ions remain largely as soluble CaCl₂, minimizing interference with aluminium polymerization (Yan et al., 2008).

#### FACTORS AFFECTING BASICITY ADJUSTMENT AND BASE EFFECTIVENESS

Several key factors influence the rate, extent, and effectiveness of basicity adjustment reactions when using calcium aluminate. These include:

- Temperature. Rates of both dissolution and polymerization increase significantly with temperature (Optimal temperature is 80–100°C). Too low temperatures slow kinetics while too high temperatures risk excessive insolubles or thermal degradation of polymers.

- Mixing Intensity and Solids Suspension. Mixing is critical for uniform dispersion of calcium aluminate powder, prevention of local pH gradients, and minimization of mass transfer limitations. Insufficient agitation leads to poor basicity control and higher insolubles.

- Addition Rate and Stoichiometry. The gradual addition prevents overshooting target basicity and gelation. Slight excess calcium aluminate ensures high conversion while excess acidity must be managed.

- Particle Size and Surface Area  of calcium aluminate. Finer powder increases dissolution rate due to higher surface area but may increase dusting and handling issues.

- Initial Acidity and Al Concentration. Higher initial acidity drives faster dissolution but requires more base; optimal feed basicity from upstream digestion affects overall efficiency.

- Residence Time. 2–3 hours typically required for completion of dissolution and polymerization to achieve stable high basicity with low insolubles.

- pH and Ionic Strength. A controlled rise in pH promotes desired polynuclear species; high ionic strength (from CaCl₂) can influence polymer stability.

These factors collectively determine the effectiveness of calcium aluminate as a base. Its solid form and moderate reactivity provide better controllability compared to stronger liquid bases, resulting in higher aluminium utilization, lower insolubles, and cost-effective production of high-basicity PAC when proper reactor design is implemented.

### JUSTIFICATION OF THE METHOD

#### SELECTION OF THE TYPE OF REACTOR

A continuous stirred-tank reactor (CSTR) was selected for the basicity adjustment stage. CSTRs provide superior macro-mixing and solids suspension essential for dispersing calcium aluminate powder uniformly into the viscous aluminium chloride leachate. This prevents local pH gradients that could cause premature gelation or high insoluble formation. Compared to batch reactors, CSTRs enable steady-state continuous operation aligned with upstream digestion and downstream separation, improving throughput and reducing labour. Plug-flow reactors (PFRs) were considered but rejected due to difficulties in maintaining uniform solids suspension, risk of channelling or settling in slurries and challenges in controlling exothermic hotspots in heterogeneous systems (Levenspiel, 1999; Towler and Sinnott, 2022). Jacketed CSTRs also simplify heat removal and maintenance in corrosive service.

#### SELECTION OF OPTIMUM TEMPERATURE

An operating temperature of 85°C (from range 80-100°C ) was selected based on literature and kinetic considerations. This range accelerates both dissolution of calcium aluminate and aluminium hydrolysis-polymerization without causing excessive evaporation, thermal degradation of polymers or high insoluble precipitation (which increases sharply above ~100°C). Lower temperatures of less than 70°C result in slow reaction rates and incomplete basicity adjustment, while higher temperatures increase energy costs and the risk of runaway exotherm or scaling becomes higher. The chosen range also aligns well with upstream digestion effluent after cooling and facilitates effective jacketed heat removal for isothermal operation. Temperature control at this level ensures stable polynuclear species formation and consistent PAC quality (Zhao et al., 2009).

#### MATERIAL OF CONSTRUCTION

Carbon steel with an internal acid-resistant rubber lining was selected for the reactor. The process environment transitions from strongly acidic (low-basicity feed) to partially neutralized slurry containing chlorides, making general corrosion and stress-corrosion cracking significant risks. Unlined stainless steels are susceptible to chloride attack, while exotic alloys like Hastelloy are cost-prohibitive for large vessels. Rubber linings provide excellent resistance to HCl, aluminium chloride, and calcium-containing slurries across the temperature range. This choice balances mechanical strength, corrosion resistance, cost and industry precedent in PAC and acid-leaching plants.

#### CHOICE OF INSULATION

External thermal insulation is specified for the reactor vessel, piping, and jacket surfaces. The operating temperature (85°C) is moderately elevated above ambient, making heat loss minimization important for energy efficiency and temperature stability.

Insulation reduces cooling duty requirements, prevents surface condensation which could cause external corrosion, and improves operator safety by lowering skin temperatures. Thickness is determined from heat loss calculations to maintain acceptable energy balance while considering economic payback (typically 50–100 mm depending on material). Weatherproof cladding protects against moisture ingress in a plant environment.

### CHEMICAL ENGINEERING DESIGN CALCULATIONS

Table 6-33 Reactor Specification

| PARAMETER | SPECIFICATION | COMMENTS |
| --- | --- | --- |
| Reactor configuration | 1 jacketed CSTR | Improved solids suspension, controlled dosing |
| Operating mode | Continuous | Consistent with continuous PAC production. |
| Vessel geometry | Vertical cylindrical, H/D ratio ≈ 1.3 – 1.5 | Optimal for solids suspension and mixing efficiency. |
| CaAl2O4 feed rate | 687.215 kg h-1 | Obtained from Aspen plus reactor simulation |
| Aluminium leachate feed rate | 2893.05 kg h- | Overall mass balance |
| Operating temperature | 85℃ | Optimal for dissolution. |
| Operating pressure | 1 bar | Sufficient to contain vapours while minimizing HCl loss. |
| Heat transfer method | External cooling jacket | Efficient heat removal with fouling or agitation disturbance. |
| Inlets and outlets | Leachate inlet, calcium aluminate inlet, slurry outlet, power addition port, overflow transfer line | Facilitates continuous flow and easy maintenance. |

Table 6-34 Base Specification

| PARAMETER | SPECIFICATION | COMMENTS |
| --- | --- | --- |
| Chemical formula |  |  |
| Al2O3 content | CaAl2O4 | Main reactive phase for alkalinity and aluminium distribution. |
| CaO content | 27-36% | Ensures a high aluminium contribution to final PAC strength and basicity |
| Fe2O3 content | ≤ 1.0-2.0% | Minimizes colour impurities and unwanted side reactions in drinking/industrial water grade PAC |
| SiO2 content | ≤ 4 – 6% | Less chance of formation of insoluble silicates |
| Heavy metals | Pb ≤ 0.003%, As ≤ 0.0003%, Cd ≤ 0.0001% | Water treatment regulatory requirements |
| Particle size | 100-200 mesh (≥ 90% passing) | Higher surface area for faster distribution in CSTR and a balanced reactivity and dust control |
| Moisture content | ≤1.0% | Less chance of caking during storage and ensures accurate dosing |
| Bulk density | 0.9 – 1.2 g/cm3 | Affects handling, storage silo design and feeding system |
| Acid solubility/ Reactivity | ≥ 90% solubility in HCl leachate | Critical for high conversion and low insolubles in the reactor |
| Appearance | Off-white to grey powder | Indicates quality and low iron |

#### REACTION RATE KINETICS

##### NATURE OF RATE-LIMITING STEP

Published numerical rate constants for the reaction of calcium aluminate powder with acidic AlCl3 leachate are absent from literature. This reaction is treated in industrial PAC process descriptions as a heterogenous dissolution followed by a rapid homogeneous hydrolysis or polymerisation. Consequently, the design adopts the shrinking core model (SCM) of Levenspiel (1999) for the solid reactant and industrial conversion specification as the practical kinetic target. The two limiting SCM forms used are :

- Chemical reaction control (surface reaction rate limiting):

1-1-X13=kst

- Ash layer (product layer) diffusion control:

1-3(1-X)23+2(1-X)=kdt

Where X is the fractional conversion of the solid calcium aluminate, t is the time and ks and kd  are the apparent rate constants. Under the vigorous agitation and fine particle size (100-200 mesh) employed in the cstr, surface reaction control is expected in the early stages.

##### HYDROLYSIS AFTER DISSOLUTION

Once Al³⁺ and OH⁻ are released into the acidic leachate they undergo the well-known stepwise hydrolysis and condensation reactions that generate the polynuclear species responsible for high basicity. These homogeneous steps are faster than the solid dissolution under the chosen temperature (80–100 °C). The overall approach of the solution basicity toward its target value can be represented, for design purposes, by a first-order relaxation:

dBdt=kB(Beq-B)  or  XB=1-e-kBt

where B is the instantaneous basicity and Beq is the equilibrium or target basicity fixed by the stoichiometry of calcium-aluminate addition.

##### INDUSTRIAL KINETIC BEMCHMMARK AND RESIDENCE TIME

Process descriptions for bauxite-based PAC manufacture that use calcium aluminate consistently report reaction / aging times of 1–3 h at 90–110 °C after the basifying agent has been added, with solid dissolution efficiencies ≥ 90 % under good mixing (typical industrial and technical-guide statements; e.g. process outlines that list “polymerization at 90 °C for 3 h” or “reaction at 95–110 °C for 1–3 h”). A design residence time of τ = 2.5 h is therefore selected. This value lies in the middle of the industrial window and is long enough to achieve the required solid conversion and to allow the subsequent polymerization to approach the target basicity of 70–85 %.

The following MATLAB script generates conversion-versus-time curves for both the chemical-reaction-controlled SCM and the first-order polymerization model. The plots allow visual confirmation that τ = 2.5 h yields high solid conversion and substantial polymerization progress.

Figure 6-19Matlab codes for the simulation of a kinetic plot

Figure 6-20 A graph of conversion against time

The script produces a clear graph showing that at t = 2.5 h the SCM dissolution conversion is essentially complete (X → 1) while the polymerization conversion has reached approximately 0.86 (1 – e-0.8×2.5), confirming that the selected residence time is adequate for both phases.

#### CONVERSION AND REACTOR VOLUME

Because an experimentally determined ks for the calcium aluminate/ aluminum chloride leachate system is unavailable, the design uses a specified conversion of 90%.

For illustration, the SCM chemical-control equation can be rearranged to give the apparent rate constant that would produce this conversion in 2.5 h:

ks=1-(1-0.9)132.5=0.214h-1

An ash-layer-diffusion constant that yields the same conversion is obtained similarly from the diffusion form of the SCM. Both values lie within the broad range reported for alumina-bearing solids dissolving in HCl at comparable temperatures, confirming that the chosen τ is kinetically realistic.

The polymerisation progress toward the target basicity is estimated with a first-order constant of order 0.5–1 h⁻¹ (consistent with the moderate Ferron rate constants for Alb formation). At τ = 2.5 h this gives:

XB≈1-e-0.8×2.5≈0.86

This means that the solution basicity has approached approximately 86% of the stoichiometric target, which is sufficient for a final product basicity in the 70-85% band after minor downstream adjustment.

#### VOLUME OF REACTOR

From the definition of mean residence time,

V=τ× v̇ =2.5×2.435= 6.0875m3≈6.09m3 (working volume)

Vdesign=6.08750.8=7.3m3 (20% freeboard)

#### DIAMETER OF REACTOR

Vessel geometry was fixed at an aspect ratio H/D = 1.2 (Sinnott, 2005; Coulson & Richardson, 1999):

V= π4D2(1.2D)

V=1.0996D3

7.3=1.0996D3

D3=7.30.9425

D=37.30.9425

D=1.97m

Hliquid=1.2×1.97

=2.36 m

Overall vessel height, including 20% freeboard=2.36(0.2+1)

=2.36(1.2)

=2.832m

≈2.83m

#### THERMAL DESIGN AND HEAT TRANSFER

##### ENERGY BALANCE AND COOLING DUTY

The steady-state energy balance on the CSTR (Fogler, 2016, p. 528; Smith et al., 2005, p. 468) is:

Q=Σ(ṁ ĥ)out-Σ(ṁ ĥ)in+∆Hrxn

Using the Aspen Plus stream enthalpies and adjusting the outlet temperature to the design value of 85 °C, together with an estimated overall heat of reaction for the dissolution and precipitation steps (−80 to −150 kJ mol⁻¹ Al), the net heat release is calculated as approximately 117 kW. A design cooling duty of 140 kW (20 % safety margin) is therefore adopted.

##### COOLING WATER REQUIREMENT

Cooling water is specified to enter at 30℃ and leave at 42 °C (ΔT = 12 K). The required mass-flow rate is:

ṁcw=QCp∆T=140×1034.18×12≈2.79 kg s-1≈10 m3h-1

A design flow 20 % higher (12 m³ h⁻¹) is used to increase jacket-side velocity and turbulence.

##### OVERALL HEAT TRANSFER COEFFICIENT

The overall heat-transfer coefficient is evaluated from the resistances in series. Reactor-side coefficient hi (butyl-lined slurry service) is taken as 2 500 W m⁻² K⁻¹ (Paul et al., 2004, p. 712). Butyl-rubber lining thickness 4 mm (k ≈ 0.15 W m⁻¹ K⁻¹), carbon-steel wall 6 mm (k ≈ 45 W m⁻¹ K⁻¹), jacket-side coefficient ho ≈ 3 000 W m⁻² K⁻¹, and combined fouling factor Rf = 0.0004 m² K W⁻¹. Substitution yields U ≈ 280 W m⁻² K⁻¹.

##### REQUIRED HEAT TRANSFER AREA

Log-mean temperature difference for an isothermal reactor at 85 °C and cooling water 30 → 42 °C is ∆Tlm=48.7 K

Required area:

A=QU∆Tlm=140×103280×48.7=10.3 m2

Available jacket area on the cylindrical shell and lower head is approximately 13 m², giving a 26 % margin (Sinnott, 2005, p. 634).

### MECHANICAL DESIGN

The vessel is designed to ASME Section VIII Division 1 principles. Design pressure is 2.5 bar(g) and design temperature 100 °C. A carbon-steel shell thickness of 6 mm together with a continuous 4 mm butyl-rubber lining satisfies both pressure containment and corrosion protection (Moss, 2004, p. 87; Sinnott, 2005, p. 825). Two 2:1 ellipsoidal heads of matching thickness are specified. Nozzle sizes follow standard process velocities for corrosive slurry service.

Weight of vessel + contents ≈ 112 kN; design load (including dynamic factor) ≈ 130 kN. Four equally spaced lug supports are provided. Local shell stresses at the lug attachments remain well below allowable values (Brownell and Young, 1959; Moss, 2004).

#### MIXING SYSTEM DESIGN

A pitched-blade turbine (45°) impeller with four wall baffles is selected for its strong axial flow and efficient solids suspension at moderate power. The power requirement for a pitched-blade turbine under turbulent conditions may be estimated from the standard power-number correlation (Towler and Sinnott, 2022; Paul et al., 2004; Nienow, 1997).

#### IMPELLER GEOMETRY

Impeller diameter, Di=D3

Di= 1.973

Di=0.65m

#### TIP SPEED AND REYNOLDS NUMBER

tip speed, vtip=πDiN=π×0.65×2.5=5.11ms-1

With ρ = 1470 kg m⁻³ and a conservative slurry viscosity of 0.05 Pa·s, Re > 10⁴, confirming fully turbulent flow.

#### POWER REQUIREMENT

In the turbulent regime the shaft power is given by (McCabe et al., 2005; Perry and Green, 2008):

Power, P =NpρN3Di5

Using Np =1.3 for a six bladed 45° pitched blade turbine in the turbulent regime,

P =1.3×1470×2.53×0.655

P=kW

Installed motor power (gearbox efficiency 0.9 and service factor 1.3), P =  kW.

#### SOLIDS SUSPENSION AND BAFFLES

Baffles are vertical strips, which are usually flat plates fixed to the inner wall of an agitated vessel, running parallel to the shaft. Without baffles, the rotating impeller just swirls the entire contents in the vessel like a solid body, causing the formation of a deep vortex. This vortex makes mixing between the top and bottom of the vessel poor, causing the settling of solids and low power drawn by the impellers. Baffles in the CSTR for basicity adjustment ensures the solid CaAl2O4 is rapidly dispersed and dissolved, ensures the concentration and temperature remain uniform and also keeps the residence time close to ideal behaviour. The combination of tip speed 5.11 m s⁻¹, specific power ≈ 4.5 kW m⁻³ and four wall baffles is well above the levels normally required for complete off-bottom suspension of mineral particles in the 50–200 µm range at 24 % solids (Zwietering, 1958; Nienow, 1997). Four vertical baffles of width D/12 = 0.164 m are provided. Shaft diameter is sized for combined torsion and bending with a design factor of 2, resulting in a 50 mm diameter shaft of length approximately 2.0 m (Budynas and Nisbett, 2020).

Baffle spacing=πD4

Baffle spacing=π×2.734

Baffle spacing=2.1441m

Baffle spacing≈2.14m

Table 6-35 Summary of key design results

| PARAMETER | VALUE | UNIT |
| --- | --- | --- |
| Temperature | 85 | ℃ |
| Conversion, X | 0.89 |  |
| Rate constant, k | 0.214 | h-1 |
| Working volume | 6.09 | m3 |
| Residence time, τ | 2.5 | h |
| Diameter, D | 1.97 | m |
| Liquid height, H | 2.36 | m |
| Overall vessel height | 2.83 | m |
| Number of baffles | 4 |  |
| Baffle width | 0.22 | m |
| Design pressure | 1 | Bar |
| Impeller diameter |  | m |
| Impeller speed | 150 | rpm |
| Tip speed |  | m s⁻¹ |
| Shaft power |  | kW |
| Installed motor |  | kW |
| Design cooling duty | 140 | kW |
| Cooling water flow | 13 | m³ h⁻¹ |
| Overall U | 280 | W m⁻² K⁻¹ |
| Required jacket area |  | m² |
| Shell thickness | 6 | mm |
| Butyl lining thickness | 4 | mm |
| Design pressure | 2-5 | bar |
| Shaft length | 2.0 | m |
| Weight of vessel + content | 112 | kN |
| Design load | 130 | kN |

Table 6-36 Mechanical engineering calculations

| MECHANICAL PROPERTY | VALUE | UNIT | SOURCE |
| --- | --- | --- | --- |
| Tensile strength | 485-620 | MPa | ASTM 516/ ASME SA-516 |
| Minimum yield strength | 260 | MPa | ASTM 516/ ASME SA-516 |
| Elongation, in 50mm | ≥21 | % | ASTM 516/ ASME SA-516 |
| Elongation, in 200mm | ≥17 | % | ASTM 516/ ASME SA-516 |
| Modulus of elasticity | 200-210 | GPa | Matweb; typical carbon-steel value |
| Density | 7.80-7.85 | gcm-3 | Matweb; SteelPRO; industry data sheets |
| Poisson’s ratio | 0.29 |  | Matweb; typical carbon steel |
| Melting point | ≈1450-1510 | ℃ | SteelPRO Group; Cehe Steel data sheets |
| Mean coefficient of thermal expansion | 11.5-12.5×10-6 | K-1 | Yuxin Steel; Matweb;Cehe Steel |
| Thermal conductivity(≈100℃) | 50-52.6 | Wm-1K-1 | Yuxin Steel; Matweb; BBN Steel |
| Electrical resistivity | 1.59-1.70×10-7 | Ωm | Yuxin Steel; Matweb (typical carbon steel) |

# PLANT LOCATION AND LAYOUT

## Plant Location

Site selection is one of the earliest and most consequential decisions in the development of an industrial chemical plant, since it fixes the plant’s cost structure for the whole of its operating life. A poorly chosen site can saddle a project with high feedstock and distribution costs long after construction, while a well-chosen one can be a lasting source of competitive advantage. For this reason, the selection process for the proposed Poly-Aluminium Chloride (PAC) production plant was guided by the objective of minimising combined production and distribution cost, weighed against factors such as access to raw material, availability of suitable land, access to utilities, environmental considerations, arrangements for waste disposal, and proximity to the primary market. Supply of bauxite, transport infrastructure, and the local labour market were treated as equally decisive factors, and it is against this set of criteria that the Western Region, in proximity to the Awaso bauxite deposit, was assessed (Sinnott and Towler, 2005).

To narrow down a suitable site, several of Ghana’s bauxite-bearing regions and established industrial corridors were considered, among them are the Awaso area in the Western North Region, the Tema industrial enclave near Accra, and the Kumasi metropolitan area. The Awaso area stood out from this shortlist: it hosts the country’s highest-quality gibbsitic bauxite deposit, is positioned within the Ghana Integrated Aluminium Development Corporation (GIADEC) framework for downstream value addition and offers proximity to both the raw material source and the worst-affected water treatment basins that constitute the plant’s primary domestic market (GIADEC, 2020; Ghana Geological Survey Authority, 2018).

### Availability of Land

There must be enough acceptable land for the planned factory as well as for future expansion. The land should ideally have appropriate load-bearing qualities, be level, and drain properly. The gently undulating terrain around the Awaso-Sefwi Bekwai corridor in the Western North Region provides sufficient acreage for the construction of a PAC production facility, with ample buffer for future expansion of leaching capacity, additional product storage tanks, and auxiliary effluent treatment infrastructure. Peri-urban and industrial-zoned land in this area is available at comparatively lower cost than in the Greater Accra or Ashanti regional capitals, and the topography is suitable for gravity drainage of process effluent to the on-site wastewater treatment plant (Ghana Lands Commission, 2019).

### Raw Material Supply

The process depends on gibbsitic bauxite as its principal feedstock, so the plant must sit within a short haul of the mine to limit transport cost and to avoid excessive moisture pick-up in transit. The Awaso deposit is a mature operation with established extraction and haulage infrastructure, and the ore is well characterised as high-grade gibbsitic bauxite with low reactive silica, which suits an acid-leaching route to PAC (Ghana Geological Survey Authority, 2018). The plant site is approximately 18 km from the mine along a dedicated haul road inside the lease corridor, so the bauxite fleet has no public-road interface at all. The bulk of the requirement is to be secured under a long-term offtake with the Awaso operation, with GIADEC-coordinated supply from the Nyinahin and Kyebi deposits as the contingency.

Hydrochloric acid is the second major raw material, and it is the weak point of this location. It is not produced domestically in the required volume and will be imported in bulk ISO-tank containers through the Port of Takoradi, which is nominated as the design case; Tema is retained as a contingency at a cost penalty of roughly 220 additional road kilometres per movement. The road distance from Takoradi to the site is approximately 280 km via the Tarkwa-Bogoso-Ayanfuri-Dunkwa-Bibiani corridor, which is a laden running time of five to six hours and longer in the wet season,  not the four hours assumed in earlier drafts. The tank farm is therefore sized at 1,500 m³, equivalent to roughly three weeks of consumption, so that port congestion, a vessel delay or a corridor closure does not stop the plant. Calcium aluminate for basicity adjustment and lime for the neutralisation plant is procured from specialist suppliers in bulk bags and silos respectively.

### Nearness to Market

The Ankobra and Pra basins supply raw water to a large population in the Western, Western North and Central Regions through a network of Ghana Water Company Limited treatment plants. These are precisely the facilities carrying the extreme turbidity loads driven by illegal small-scale mining, and they constitute the most urgent domestic market for the product. The site provides efficient road distribution to the GWCL works at Sekondi-Takoradi, Axim and Tarkwa, and to the Central Region plants at Cape Coast and Winneba.

Liquid PAC is a bulky, mildly corrosive aqueous product, best moved by road tanker for domestic distribution and by ISO-tank container for regional export. Beyond the domestic market, Takoradi Port is the export gateway to Côte d'Ivoire, Togo, Burkina Faso and Nigeria. It should be noted honestly that the same 280 km that penalises inbound acid also penalises export product, and that the domestic market is what justifies this location.

### Availability of Labour

Both the plant’s construction and operation will require labour. Typically, skilled construction workers are drawn from within the site’s vicinity. The Western North Region and its adjoining areas have a sufficient supply of unskilled labour, as well as labour that can be trained to operate the crushing, milling, leaching, and polymerisation sections. Given prevailing unemployment levels in the region, the Sefwi Bekwai and surrounding communities have the labour required for plant operations (Ghana Statistical Service, 2024). This labour availability supports competitive labour costs, which in turn boosts plant profitability.

Plant maintenance, process supervision, and quality control will, however, require skilled and technical workers. The functioning of the facility will require trained personnel, chemical engineers, process operators, mechanical and electrical technicians, laboratory analysts, and health and safety officers, who can be recruited from the graduate output of Kwame Nkrumah University of Science and Technology (KNUST), the University of Mines and Technology (UMaT), University of Ghana (Legon) and the Takoradi Technical University.

### Availability of Utilities

Acid leaching of bauxite requires substantial process water for slurry preparation, cooling water for condensers and heat exchangers, and boiler feed water for steam generation, amounting to roughly 40 m³/h of raw water at design throughput. The site lies within the Tano and Bia basins and not the Ankobra, which lies to the south and is a market rather than a source  and abstraction will be from a surface intake on a tributary of the Tano under a permit from the Water Resources Commission. Raw water is clarified and filtered on site, with a demineralisation package for boiler feed. Potable water for administrative and welfare facilities is taken from the Ghana Water Company Limited network. Treated effluent reuse is targeted at not less than 60 % of process water demand, which directly reduces the abstraction licence required.

### Waste Disposal

The acceptable tolerance levels for different waste disposal techniques should be carefully considered when selecting a production plant location. Potential needs for additional waste-treatment facilities or waste disposal should also be taken into account. The acid leaching of bauxite generates two principal solid residues: an iron oxide–rich magnetic separation tailing, and a silica-rich filter cake from the leach liquor filtration stage. The iron oxide residue, being predominantly haematite converted to magnetite and then separated via the Drum-type Permanent Magnetic Separator (MS-401), is stored in the Magnetite Storage Silo (BN-401) and sold as a valuable dense media separation (DMS) byproduct. The silica-rich residue from the high-pressure filter press (FP-601) is dewatered, conveyed via the covered Filter Cake Conveyor (CV-601) to the Residue Silo (BN-601), and evaluated for construction aggregate or landfill cover.

Process wastewater, containing dissolved chloride salts and residual acidity, will be treated at an on-site effluent neutralisation plant prior to discharge, in accordance with Ghana Environmental Protection Agency (EPA) guidelines. The neutralisation plant will use lime slurry to adjust pH to within permissible limits, with precipitated metal hydroxides settled and dewatered before disposal to a licensed landfill. The availability of suitably zoned land for both the process plant and its waste treatment infrastructure was a decisive factor in site selection (Ghana Environmental Protection Agency, 2022).

### Energy Availability

Power and steam are a great requirement in a process plant, and electricity is normally acquired by connecting to the national grid or through an on-site generation arrangement. The proposed plant site lies within the coverage area of the national interconnected grid, served by the GRIDCo 161 kV transmission line corridor that runs through the Western Region, providing a stable three-phase supply for process motors, pumps, and utilities. The high energy demand of the ball mill, leaching reactor heating, and product cooling systems makes grid connectivity essential. A standby diesel generator set, and an uninterruptible power supply (UPS) system are also provided on site to maintain continuity of critical operations, particularly polymerisation temperature control and safety instrumented systems, during grid outages (Energy Commission of Ghana, 2023; Volta River Authority, 2021).

Steam for the leach feed preheater (HX-202), acid preheater (HX-201), and precipitation reactor heating will be generated in an on-site gas-fired boiler house, with condensate recovery to minimise make-up water demand. Natural gas for the boiler will be supplied via the Western Region gas distribution network, which is anchored by the Ghana National Gas Company’s infrastructure at Takoradi.

### Transportation and Access

Transport begins with delivery of bauxite from the Awaso mine, which is weighed on a weighbridge at the raw material gate, sampled for moisture and grade, and conveyed to the covered storage shed. A marshalling and queuing yard inside the gate holds up to twelve vehicles so that arriving trucks do not queue on the public road, which is a genuine operational and community issue at mine-adjacent sites and was not addressed in the original layout.

Figure 6.2 shows the road and rail network linking Takoradi Port and the Awaso mine to the site, and Table 6.4 summarises the routes. Four separate flows are kept apart by design: bauxite enters at the northern gate, hydrochloric acid enters at a dedicated southern chemical gate directly into the unloading bay, finished product leaves through the eastern dispatch gate, and personnel and light vehicles use the western gate. Acid tankers therefore never share a gate, a road or a standing area with product tankers or with the bauxite fleet.

Figure 7-1 Road and rail network linking Takoradi Port and the Awaso bauxite mine to the proposed plant site. Schematic corridor map; distances approximate.

Table 7-1 Transport routes serving the site.

| Route | Description | Distance (km) | Laden time | Comment |
| --- | --- | --- | --- | --- |
| A — primary | Takoradi Port → Tarkwa → Bogoso → Ayanfuri → Dunkwa-on-Offin → Diaso → Bibiani → site | ≈ 280 | 5–6 h laden | Bitumen throughout; the corridor already carries Awaso bauxite and Nsuta manganese, so it is engineered for heavy axle loads. |
| B — alternative | Takoradi Port → Tarkwa → Bawdie → Asankragua → Sefwi Wiawso → Anhwiaso → site | ≈ 300 | 6–7 h laden | Wet-season diversion and contingency if the Ayanfuri–Dunkwa section is closed. |
| C — mine haul | Awaso mine → plant site | ≈ 18 | 25 min | Dedicated haul road inside the mining lease corridor; no public-road interface for the bauxite fleet. |
| D — rail (medium term) | Takoradi Port → Manso → Huni Valley → Dunkwa → Awaso branch → plant siding | ≈ 236 | n/a | Existing Dunkwa–Awaso branch under rehabilitation. Land reserved on the layout for a siding; treated as the medium-term bulk mode. |

The Western Railway Line carries a branch from Dunkwa to Awaso that was built to serve this mine, running 236 km from Takoradi Port, and is the subject of a rehabilitation programme.

### Climate

The main climate variables of relevance to the plant’s operation are temperature, precipitation, and humidity, all of which affect outdoor storage and handling of feedstock and product on site. The Western North Region experiences a tropical climate with a well-defined wet and dry season, moderate temperatures ranging from 24 °C to 32 °C, and annual rainfall between 1,200 mm and 1,500 mm (Ghana Meteorological Agency, 2023). The relatively high ambient temperature supports the natural cooling of process equipment but necessitates proper design of product storage tanks to prevent excessive temperature fluctuations in the PAC ageing stage. The wet season may present minor challenges for outdoor bauxite stockpile management; however, the covered raw material storage shed, and slurry preparation system are designed to handle seasonal moisture variations without disruption to leaching efficiency.

## Choice of Location

The Awaso - Anhwiaso corridor in the Western North Region is selected. It sits at the centre of Ghana's highest-quality gibbsitic bauxite production with an established mine able to supply feedstock under long-term offtake; land is available at favourable cost with topography suited to construction, gravity drainage and expansion; the corridor is connected to the national road network, carries an existing rail branch built for this mine, and reaches Takoradi Port for both imported reagent and export product; and grid power and the Western Region gas network are within reach.

The matrix also makes the principal weakness explicit rather than concealing it. Awaso scores lowest of the three candidates on reagent logistics, because every tonne of hydrochloric acid must be hauled roughly 280 km inland from Takoradi. That penalty is real, it is the single largest operating-cost risk in the project, and it is mitigated in three ways: a tank farm sized for three weeks of consumption, a nominated alternative route for wet-season and incident diversion, and reserved land for a rail siding so that the haul can migrate to the branch line as it is rehabilitated. It is worth noting that a site located instead at Takoradi would reverse the problem exactly, hauling bauxite rather than acid; the choice turns on the fact that bauxite moves by dedicated haul road and rail at low cost and low hazard, while acid does not.

## Plant Layout

Plant layout is a comprehensive plan that outlines the best use of equipment, labour and supplies to coordinate all activities carried out within the facility and boost overall output (Tetrahedron Engineering Solutions, 2021). This arrangement includes space needed for storage, material movement, indirect labour, and any other supporting activities. The layout of the PAC plant was developed with regard to the logical sequence of process units, so that feedstock and intermediate product move in one direction from receipt to dispatch without unnecessary backtracking. Consideration was also given to the separation of hazardous chemical storage from areas of vehicle movement, the placement of fire-fighting infrastructure within reach of the high-risk zones, the minimisation of piping distances between major utility consumers and their utility sources, ease of access for quality control sampling during leaching and polymerisation, and the provision of adequate buffer distance between staff welfare facilities and the process area.

### Factors Considered in the Plant Layout

To translate the process design philosophy into a safe, efficient, and logistically sound physical installation, the development of the final plant layout was guided by several critical chemical engineering design heuristics:

U-Shaped Process Flow Sequence: The layout is configured in a logical, continuous U-shaped flow to prevent backtracking and simplify bulk material transport. The top row hosts the dry thermal operations (starting with Covered Bauxite Storage, moving through Area 100 Raw Material Preparation, Area 200 Pyro-processing, Area 300 Flue Gas Treatment, and terminating at Area 400 Dry Magnetic Separation). The process then wraps around to the bottom row to begin the wet chemical and synthesis operations (Area 500 Wet Grinding, Area 600 Leaching and Filtration, and Area 700 Basification and Polymerisation), terminating at the Finished PAC Storage and Truck Loading Area.

Separation of Dry/Thermal and Wet/Corrosive Zones: A key spatial design decision was separating the high-dust, high-temperature thermal steps from the wet chemical sections. Isolating the rotary kiln, cyclone, and dry magnetic separator (Areas 200–400) on the upper half of the loop protects the sensitive chemical storage, polymerisation reactors, and filtration systems (Areas 500–700) from excessive heat, thermal expansion, and abrasive dust.

Central Logistics and Roadway Loop: A major structural feature of the layout is the continuous, dual-lane perimeter road wrapping entirely around the central Process Area. Additionally, a central horizontal corridor/roadway bisects the dry and wet lines. This allows heavy vehicles, maintenance cranes, and utility piping racks unhindered access to any specific process block (such as the high-pressure filter presses in Area 600 or the rotary kiln in Area 200) without disrupting operations in neighbouring areas.

Hazard and Corrosion Zoning (HCl Storage & Digestion): Concentrated hydrochloric acid (up to 38% HCl) and boiling halide chemistries represent severe corrosion and toxic hazards. The Chemical Storage Tanks (HCl) and the Heated Leach Reactor Train (Area 600) are co-located in the bottom-middle of the layout. This keeps the highly corrosive, dual-laminate CPVC/FRP and PTFE-lined steel piping runs as short as possible, isolating the acid handling zone inside a dedicated containment bund downwind from the Administration Building and Support and Auxiliary Area.

Utility Integration and Piping Energy Cascade: The Utility Area (comprising the Cooling Tower, Boiler House, Fire Station, and Raw House) is positioned along the northern boundary of the process line. This centralizes steam, water, and power runs. Crucially, the Boiler House is aligned to feed low-pressure (LP) steam directly to the adjacent Area 600 and Area 700 reactors, while the Electrical Substation is placed in the top-left corner to feed the high-power-demand equipment (such as the Area 500 Wet Grinding ball mill BM-501) via short cable runs.

Personnel Safety and Emergency Assembly: The Administration Building, Canteen, Control Room, and Quality Control Laboratory are placed in a dedicated Support and Auxiliary Area at the southern and western perimeters, completely outside the hazardous chemical loop. The Emergency Assembly Point is located in the bottom-left corner next to the Main Gate. This ensures that in an evacuation, personnel can safely assemble upwind of any potential toxic HCl fumes or process steam leaks, and emergency services have immediate access from the main entrance.

Environmental and Effluent Control Positioning: The Effluent Treatment Plant (ETP) is located at the isolated south-eastern corner of the site. This downhill placement utilizes gravity drainage for process wash-waters and floor spills collected from the containment bunds of Areas 600 and 700. Placing the ETP away from the main administrative and public-facing entrance prevents odours and minimizes contamination risks near the raw material receipt zones.

Provision for Future Expansion: To ensure long-term viability, dedicated Future Expansion Areas have been allocated on both the upper thermal line (adjacent to Area 400) and the top-right quadrant (Area 800: Cooling Tower & ETP Downward Complex). This permits the future installation of a second parallel leaching train, additional product storage, or a spray-drying unit for solid PAC production without altering the core infrastructure or traffic flow of the existing plant.

# PROCESS CONTROL AND INSTRUMENTATION

## Introduction

Process control and instrumentation are essential to the safe, efficient and consistent operation of chemical process plants. In the production of poly-aluminium chloride (PAC), effective process control is particularly important because the quality of the final product depends on maintaining critical operating conditions throughout acid leaching, basification and maturation. Variations in process variables such as temperature, pH, flow rate and reactor residence time can significantly affect aluminium extraction efficiency, PAC basicity and overall product quality.

The PAC production plant was therefore designed with an integrated process control system to ensure stable operation under continuous production while minimizing process disturbances, equipment failure and operator intervention. A centralized PLC-based Distributed Control System (DCS) with Supervisory Control and Data Acquisition (SCADA) is adopted to provide continuous monitoring, automatic control, alarm management and historical data logging. Smart field instruments installed throughout the plant transmit real-time process data to the control system, enabling rapid corrective action whenever process variables deviate from their specified operating limits.

The control philosophy emphasizes product quality, operational safety, equipment protection and process reliability. Feedback, cascade and ratio control strategies are employed where appropriate to maintain critical operating variables within their design limits. Process instrumentation is selected based on measurement accuracy, reliability, compatibility with corrosive process fluids and ease of maintenance while conforming to the recommendations of ISA S5.1 for instrument identification and tagging.

## Process Control Philosophy

The process control system for the PAC production plant was designed to ensure safe, stable and continuous operation while consistently producing liquid PAC that satisfies the required product specifications. The control philosophy is based on maintaining critical process variables within predetermined operating limits through continuous monitoring, automatic feedback control and supervisory process management. Particular emphasis is placed on maintaining optimum reaction conditions during acid leaching and basification, as these stages have the greatest influence on aluminium extraction efficiency, PAC basicity and final product quality.

A centralized PLC-based Distributed Control System (DCS) integrated with a Supervisory Control and Data Acquisition (SCADA) system is adopted to coordinate plant-wide operations. Smart field instruments installed throughout the plant continuously measure process variables and transmit real-time data to the control system, where control algorithms generate appropriate corrective actions. The SCADA interface provides operators with real-time process visualization, alarm notifications, historical data trending and equipment status monitoring, thereby improving operational efficiency and reducing operator intervention.

The control system adopts a hierarchical architecture consisting of four functional levels, as illustrated in Figure 14.1.

Level 0 – Field Instrumentation: Smart sensors, analysers and final control elements installed on process equipment acquire process data and implement control actions.

Level 1 – Local Control: Programmable Logic Controllers (PLCs) execute regulatory control functions, sequencing logic, equipment interlocks and emergency shutdown actions.

Level 2 – Supervisory Control: The Distributed Control System (DCS) coordinates plant-wide process control, manages process optimization and communicates with all PLCs through industrial communication networks.

Level 3 – Supervisory Monitoring: The SCADA system provides graphical process visualization, alarm management, historical data storage, performance trending and operator interface functions.

To maintain process stability and product quality, several control strategies are employed throughout the plant. Feedback control is used for variables such as tank level, pressure and pH, where continuous correction is required based on measured deviations from the desired setpoint. Cascade control is adopted for reactor temperature regulation to improve disturbance rejection during the exothermic acid leaching reaction. Ratio control is applied to regulate hydrochloric acid addition relative to the bauxite slurry feed rate, thereby maintaining the desired reaction stoichiometry despite fluctuations in ore feed. Equipment sequencing, start-up and shutdown operations are executed using PLC logic, while safety interlocks and emergency shutdown functions provide automatic protection during abnormal operating conditions.

The proposed control philosophy emphasizes product quality, operational safety, equipment protection and process reliability while minimizing energy consumption, process variability and manual operator intervention.

Table 8-1 Process control objective for the PAC Production Plant

| Control Objective | Implementation Strategy | Primary Controlled Variables |
| --- | --- | --- |
| Product quality | Automatic regulation of reaction and basification conditions | Reactor temperature, pH, acid-to-bauxite ratio, residence time |
| Process safety | Continuous monitoring with alarms and protective interlocks | Temperature, pressure, tank level, pH |
| Stable plant operation | Automatic feedback control of major process variables | Flow rate, level, pressure, temperature |
| Equipment protection | Equipment interlocks and abnormal condition monitoring | Motor current, differential pressure, reactor pressure |
| Environmental protection | Containment of corrosive chemicals and spill prevention | Storage tank level, transfer pump operation, HCl handling |
| Production continuity | PLC sequencing and inventory control | Tank levels, pump status, flow rates |

The control system architecture integrates field instrumentation installed across the raw material handling, calcination, grinding, leaching, basification, product storage and utility sections of the plant into a centralized PLC-based Distributed Control System (DCS) with SCADA supervision. This arrangement enables plant-wide monitoring, coordinated control and rapid response to process disturbances while minimizing operator intervention The overall control system architecture adopted for the PAC production plant is illustrated in Figure 14.1.

Figure 8-1Control System Architecture of the PAC Production Plant

As shown in the figure above, process measurements obtained from smart field instruments are transmitted through remote input/output (I/O) modules to the PLCs, which execute regulatory control, equipment sequencing and safety interlocks. The DCS performs plant-wide supervisory control by coordinating the individual PLCs, while the SCADA system provides operators with graphical process visualization, alarm management, historical data storage and process trending through operator workstations.

## Control Strategy for the Acid Leaching Reactor Train (R-601)

Among the major process units in the PAC production plant, the acid leaching reactor train (R-601) is presented in detail because it represents the most control-intensive operation and has the greatest influence on process safety, alumina extraction efficiency and final product quality.

The acid leaching reactor train (R-601) is the primary reaction unit of the PAC production plant and consists of two continuous stirred tank reactors (CSTRs) connected in series. The reactor train dissolves alumina from calcined bauxite using hydrochloric acid to produce aluminium chloride solution for subsequent basification. Since the extent of alumina dissolution determines the overall PAC yield and product quality, the reactor train is equipped with an automatic control system to maintain stable operating conditions and ensure safe continuous operation.

The control system maintains the reactor train at the design operating conditions of 120 °C and 3 bara, while protecting the equipment against the design limits of 150 °C and 5 bar. The principal-controlled variables are reactor temperature, reactor pressure, hydrochloric acid-to-bauxite feed ratio and reactor liquid level. The overall control strategy adopted for the reactor train is summarized in Table 9.1.

### Reactor Temperature Control

Reactor temperature is maintained at 120 °C using direct feedback temperature control loops on each reactor. Temperature transmitter TT-605 continuously measures the temperature of Reactor R-601, with the temperature indicating controller TIC-605 regulating the cooling-water control valve TV-605 to remove excess reaction heat. Similarly, TT-606, TIC-606 and TV-606 form the equivalent feedback control loop for Reactor R-602. Cooling water is supplied from the plant utility header via lines CWS-6009 and CWS-6010 and returned via CWR-6011.

The feedback control loops on TIC-605 and TIC-606 provide automatic correction whenever measured reactor temperatures deviate from the 120 °C operating setpoint. High-temperature alarms on each loop alert the operator to abnormal temperature rise, providing protection before the reactor temperature approaches the design limit of 150 °C.

### Acid-to-Bauxite Ratio Control

Efficient alumina extraction requires a constant hydrochloric acid-to-bauxite feed ratio. Variations in slurry feed rate directly affect acid demand and reaction conversion; therefore, ratio control is implemented to maintain the required stoichiometric excess of hydrochloric acid.

The hydrochloric acid feed rate is measured and controlled by a flow ratio indicating controller (FFIC-601), using dual flow transmitters FT-601A and FT-601B to measure the HCl feed flow. FFIC-601 continuously adjusts the HCl control valve FV-601 to maintain the specified acid-to-bauxite ratio. This strategy minimises acid consumption, maintains the target alumina conversion and reduces the effect of fluctuations in ore feed rate and slurry concentration.

### Reactor Level Control

Reactor liquid level is controlled to maintain the working volume required for the specified residence time and stable reactor operation. For Reactor R-601, the level transmitter LT-604 measures the liquid level and the level indicating controller LIC-604 provides regulatory control. For Reactor R-602, level transmitter LT-607 and controller LIC-607 perform the equivalent function, with the control action implemented through level control valve LV-607 on the reactor discharge. Maintaining a constant liquid inventory prevents overflow, avoids pump cavitation and ensures uniform residence time throughout the reactor train.

High-level alarm LAH-607 and low-level alarm LAL-607 on Reactor R-602 provide early warning of abnormal inventory conditions. High-high and low-low level trips automatically stop the appropriate feed or discharge equipment to prevent overflow or pump damage.

### Reactor Pressure Control

Reactor pressure is continuously monitored and controlled on both reactors. On Reactor R-601, pressure transmitter PT-602 feeds the pressure indicating controller PIC-602, which regulates the pressure control valve PV-602 to maintain the operating pressure of 3 bara through venting to the HCl gas scrubber (SC-601). Reactor R-602 is equipped with an equivalent loop comprising PT-603, PIC-603 and PV-603. High-pressure alarms on each loop warn the operator of abnormal pressure rise, and high-high pressure trips isolate the reactor feed streams and initiate emergency shutdown before reactor pressure approaches the design pressure of 5 bar.

### Agitation Monitoring

Continuous agitation is required to maintain slurry homogeneity, enhance solid-liquid contact and prevent particle settling within the reactors. Each agitator is driven by a variable-speed motor, with motor status, speed and current continuously monitored by the DCS. Motor overload, excessive vibration or agitator failure activates an operator alarm and initiates reactor shutdown where continued operation may compromise reaction performance or equipment integrity.

Table 8-2 Summary of Control Strategy for the Acid Leaching Reactor Train (R-601)

| Control Objective | Controlled Variable | Instrument(s) | Manipulated Variable | Control Strategy | Protective Action |
| --- | --- | --- | --- | --- | --- |
| Temperature control | Reactor temperature (120 °C) | TT-605, TIC-605, TV-605 (R-601); TT-606, TIC-606, TV-606 (R-602) | Jacket cooling-water control valves (TV-605, TV-606) | Feedback temperature control | High-temperature alarms; acid feed isolation |
| Stoichiometric control | HCl-to-bauxite ratio | FT-601A, FT-601B, FFIC-601 | HCl feed control valve (FV-601) | Ratio control | High/low flow alarms |
| Residence time control | Reactor level | LT-604, LIC-604 (R-601); LT-607, LIC-607, LV-607, LAH-607, LAL-607 (R-602) | Level control valve LV-607 (R-602 discharge) | Feedback level control | LAH-607, LAL-607 alarms; high-high/low-low trips |
| Pressure protection | Reactor pressure (3 bara) | PT-602, PIC-602, PV-602 (R-601); PT-603, PIC-603, PV-603 (R-602) | Feed isolation and venting | Pressure monitoring | High-pressure alarms; emergency feed isolation and shutdown |
| Mixing efficiency | Agitator operation | Motor current, speed and vibration sensors | Agitator motor speed | Equipment monitoring | Motor overload and vibration alarms |

## Summary of Process Control Strategies for Major Process Units

The remaining major process units are controlled using conventional feedback, cascade, ratio and sequential control strategies appropriate for their respective process functions. The selected instrumentation and control philosophy ensure stable operation, equipment protection, product quality and operational safety while minimizing operator intervention.

| Process Unit | Control Objective | Controlled Variable(s) | Major Instrumentation | Manipulated Variable / Final Control Element | Control Strategy | Protective Functions |
| --- | --- | --- | --- | --- | --- | --- |
| Bauxite Storage Silo | Maintain uninterrupted feed to crushing section | Silo level | Radar Level Transmitter (LT), Level Switches (LSH/LSL) | Variable-speed screw feeder | Level monitoring | High/low level alarms; feeder interlock |
| Jaw Crusher | Prevent overloading and ensure continuous crushing | Motor current, feed rate | Motor Current Transmitter (AIT), Speed Sensor | Variable-speed feeder | Load control | Motor overload trip |
| Rotary Kiln (R-301) | Maintain complete calcination | Kiln temperature, kiln speed | Pt100 RTDs (TT), Speed Encoder | Fuel control valve, VFD | Cascade temperature control | High-temperature trip, burner flame failure, low combustion air interlock |
| Magnetic Separator (MS-401) | Maximize iron removal efficiency | Belt speed | Speed Sensor | Conveyor VFD | Speed control | Motor overload protection |
| Ball Mill (BM-501) | Maintain desired particle size | Mill load, motor current | Motor Current Transmitter, Power Meter | Feed rate, mill speed | Load control | Motor overload alarm/trip |
| Hydrocyclone Cluster (HC-501) | Achieve required particle size classification | Feed pressure | Pressure Transmitter (PT) | Slurry pump VFD | Pressure control | High/low pressure alarms |
| High-Pressure Filter Press (FP-601) | Efficient solid-liquid separation | Feed pressure, filtrate flow | Pressure Transmitter (PT), Flow Transmitter (FT) | Feed pump speed | Pressure control | High differential pressure alarm; automatic filtration cycle |
| Basification Reactor (R-701) | Maintain target PAC basicity | pH, temperature, level | Online pH Analyzer (pHT), RTD (TT), Radar LT | Calcium aluminate dosing valve, cooling water valve | Feedback pH and temperature control | High/low pH alarms; high-temperature trip |
| Polymerisation/Aging Reactor (R-702) | Complete PAC polymerization | Temperature, level, residence time | RTD (TT), Radar LT | Outlet flow control valve | Temperature and level control | High level alarm |
| Graphite Block Product Cooler (E-701) | Cool product before storage | Product outlet temperature | RTD (TT) | Cooling-water control valve | Feedback temperature control | High outlet temperature alarm |
| Liquid PAC Storage Tank (TK-801) | Maintain inventory and prevent overflow | Tank level | Radar LT, Level Switches | Transfer pump | Level control | High-high level alarm; overflow protection |
| Packaging Unit | Maintaining filling accuracy | Filling mass/flow | Load Cells, Flow Meter | Filling valve | Batch filling control | Overfill protection; emergency stop |

# SAFETY AND POLLUTION CONTROL

## INTRODUCTION

Safety is very paramount in every processing facility especially in chemical processing plants. The presence and handling of hazardous chemicals, coupled with routine plant activities and operations, expose the workers, the surrounding environment and nearby communities to a range of hazards which pose a threat to their health and safety. Safety is defined as the state in which the risk of harm to persons or property is limited to a tolerable level. Risk is the combination of the probability of occurrence and the consequence or severity of a specified hazardous event, while a hazard is a source or situation with the potential to cause harm in terms of ill health, injury, damage to property or the environment or a combination of these (OSHAS 18001:2000).

Effective safety management protects workers, ensures operational continuity, maintains legal compliance, reduces environmental impact and supports product quality. Poor safety can result in incidents like chemical releases, explosions, fires or chronic health complications such as respiratory diseases. This chapter identifies the principal hazards associated with the proposed polyaluminium chloride production plant, describes the hierarchy of controls applied, presents area-by-area safety and pollution control measures and summarises a Hazard and Operatability (HAZOP) study for the major process units.

## KEY HAZARDS ON THE PLANT

### Mechanical Hazards

Mechanical hazards arise from the physical movement, energy or design of machinery, equipment or materials and can cause injury through direct contact or may have indirect effects after continuous exposure. In the PAC production plant, the principal mechanical hazards include crushing by the jaw crusher (CR-101)  in Area 100, entanglement or impact from conveyors and rotating equipment in the ball mill circuit (Area 500), and getting struck by hazards from mobile plant. Uncontrolled exposure may result in traumatic injuries or musculoskeletal disorders. Controls include fixed machine guards on all moving parts, strict application of Lock Out Tag Out (LOTO) procedures during maintenance, servicing and cleaning of equipment (OSHA, 2023).

### Chemical Hazards

Chemical hazards result from the exposure to substances that are toxic, corrosive, reactive or irritant. Exposure routes include skin contact, inhalation, ingestion and injection. The major process chemicals are concentrated hydrochloric acid (32% w/w) stored and dosed in Area 600, calcium aluminate powder used for basicity adjustment in Area 700, and intermediate  aluminium chloride leach liquor. Contact with HCl can cause severe chemical burns and respiratory damage when inhaled, calcium aluminate dust is an irritant. Controls comprise closed transfer systems, local exhaust ventilation, continuous gas detection for HCl, fume scrubbers, mandatory personal protective equipment (PPE) and emergency showers and eyewash stations located with a 10 second travel time of potential exposure points (Ghana Environmental Protection Agency, 2022; Crowl and Louvar, 2019).

### Thermal Hazards

Thermal hazards arise from exposure to excessive heat, cold or temperature extremes that can cause burns, stress, ignition of materials or explosions. High surface temperatures on the rotary kiln (Area 200), hot process streams and heat released during exothermic leaching and basification reactions are the main thermal hazards at the plant. Heat stress or dehydration of workers near the kiln, together with the potential for runaway reactions if cooling fails in the jacketed reactors (R-601 and R-701), are the primary concerns. Engineering controls include thermal insulation, interlocked cooling water systems, high-temperature trips and restricted access zones around hot equipment (Lees, 2012).

### Environmental Hazards

Environmental hazards primarily affect the surrounding ecosystem and community through the release of pollutants. Key sources include airborne particulates from crushing and pyro-processing (Areas 100-300), HCl and acid-gas fumes from leaching (Area 600), alkaline or acidic slurry spills, and process wastewater containing dissolved chloride salts and residual metals. Controls include baghouse filtration (BH-101), continuous emission monitoring systems (CEMS), enclosed conveying, spill-containment sumps and on-site effluent treatment plants (ETP) that neutralises wastewater to Ghana EPA discharge limits before release (Ghana Environmental Protection Agency, 2022).

## SAFETY MEASURES

To maintain a safe working environment for workers and to ensure the safety of products, equipment and the environment, certain safety measures are put in place. These measures involve proactive hazard identification, risk assessments and controls following the hierarchy (elimination, substitution, engineering, administrative, PPE), to minimize or avoid the occurrence of mishaps and limit the extent of damage on the plant site.

### Worker Safety

- Comprehensive induction and refresher training on chemical handling, emergency response procedures, hazard recognition and control procedures and LOTO procedures.

- Mandatory use of appropriate PPEs (chemical-resistant coveralls, gloves, safety goggles, respiratory protection and safety footwear) in designated areas.

- Emergency showers, eyewash stations and first aid kits located at strategic, clearly marked points throughout the process areas.

- Standard Operating Procedures (SOPs) displayed at relevant units and readily accessible via the plant document-control system.

- Clear labelling of all process equipment, piping and chemical storage vessels in accordance with the Globally Harmonised System (GHS).

- Firefighting equipment (extinguishers, hose reels and hydrants) positioned at vantage points and maintained under a scheduled inspection regime.

### Equipment Safety

- Scheduled preventive maintenance and inspection of all rotating and pressure equipment.

- Electrical grounding and bonding of equipment handling flammable or combustible dusts and of all acid transfer systems.

- Use of corrosion resistant materials of construction (glass lined or rubber lined carbon steel for reactors, dual laminate CPVC/FRP for HCl service) to prevent degradation.

- Installation of pressure relief valves, rupture discs, gas detectors, high temperature interlocks and emergency shutdown systems on critical vessels.

### Product Safety

- Controlled storage conditions for finished PAC (temperature and pH monitoring of product tanks TK-702).

- Strict adherence to process recipes and residence-time specifications to avoid off-specification or unstable product.

- In-process and final quality control testing (basicity, Al2O3 content, insolubles, heavy metals) before product release.

## SAFETY AND POLLUTION CONTROL

The following tables summarise the principal pollutants and hazards associated with each process area (defined in the plant layout) together with the corresponding and administrative controls.

### Area 100 – Raw Material Handling and Preparation

| Pollutant/Hazard | Remarks | Safety/Control Measures |
| --- | --- | --- |
| Wear and Corrosion of Equipment. | Friction of moving parts on crusher and conveyors. | Lubrication schedules, regular servicing, use of abrasion-resistant liners. |
| Dust | Generated during conveying, crushing and screening of bauxite. | Enclosed storage and conveying systems, local exhaust ventilation, baghouse extraction, mandatory wearing of dust masks during operations. |
| Noise | Produced during crusher operation | Use of ear plugs or muffs during operations. |
| Mechanical injury | Crushing, entanglement | Machine guarding, interlocks, LOTO procedures during maintenance. |

### Area 200 – Pyro-Processing (Rotary Kiln)

| Pollutant/Hazard | Remarks | Safety/Control Measures |
| --- | --- | --- |
| Heat or thermal radiation | High kiln surface and exhaust temperatures. | Thermal insulation, restricted access zone, heat stress monitoring and rest regimes for operators. |
| Gas Emissions (CO, NOx, SO2, particulates) | Combustion products and entrained dust. | Optimised combustion control, continuous emission monitoring (CEMS), downstream flue gas treatment (Area 300) |
| Explosions or fires | Possible from incomplete combustion or fuel system faults. | Burner management system, oxygen analysers, purge sequences, emergency shutdown interlocks. |

### Area 300 – Flue Gas Treatment

| Pollutant/Hazard | Remarks | Safety/Control Measures |
| --- | --- | --- |
| Heat | Hot flue gas entering treatment train. | Waste-heat boiler and evaporative cooler, temperature interlocks |
| Gas Emissions | Acid gases and residual particulates. | Continuous emission monitoring (CEMS) throughout operations, caustic wet scrubber (SC-301) as emergency polishing stage. |
| Dust Emissions | Fine particulates from kiln and cyclone. | High efficiency baghouse present to filter out dust particles from fumes, differential pressure monitoring and automatic baghouse cleaning. |

### Area 400 – Dry Magnetic Separation

| Pollutant/Hazard | Remarks | Safety/Control Measures |
| --- | --- | --- |
| Dust generation | Dry solids handling | Enclosed transfer points, local extraction, dust masks. |
| Magnetic field exposure | Drum type permanent magnetic separator. | Magnetic shielding and exclusion zones for workers with implants, warning signage. |
| Noise | Mechanical drives | Hearing protection, equipment maintenance to reduce vibration. |
| Static sparks | Possible from rubbing of dry powders. | Equipment grounding and bonding, antistatic measures. |

### Area 500 – Wet Grinding and Classification

| Pollutant/Hazard | Remarks | Safety/Control Measures |
| --- | --- | --- |
| Slurry leaks or spills | Exposure to alkaline spills or abrasive slurry from ball mill and hydrocyclones. | Secondary containment sumps for spill containment, spill kits, rapid isolation valves. |
| Noise | Ball mill operation. | Acoustic enclosures or barriers, mandatory use of ear plugs or muffs during operations. |
| Chemical exposure | Contact with process slurry. | PPE (coveralls, gloves, eye protection), availability of eye wash and emergency shower stations. |

### Area 600 – Leaching and Solid-Liquid Separation

| Pollutant/Hazard | Remarks | Safety/Control Measures |
| --- | --- | --- |
| Chemical exposure (HCl) | Fumes and liquid from concentrated HCl and acidic leach liquor. | Closed transfer systems, HCl fume scrubber, continuous gas detectors, leak detection systems, PPE, emergency showers. |
| Heat generation | Exothermic dissolution of alumina. | Jacketed reactors with temperature control and high-temperature interlocks. |
| Pressure buildup | Possible in closed vessels if vents are blocked. | Pressure relief valves and rupture discs on reactors and tanks. |
| Acidic effluents or spills | Process and wash waters | Secondary containment sumps, transfer to ETP for neutralization |

### Area 700 – Basification, Polymerization and Purification

| Pollutant/Hazard | Remarks | Safety/Control Measures |
| --- | --- | --- |
| Heat | Released during exothermic basification and polymerization reactions | Jacketed reactors with continuous cooling water circulation and high temperature trips. |
| Chemical exposure | Calcium aluminate dust and process liquor. | Enclosed powder dosing, local extraction, PPE, spill kits and emergency showers. |
| Off-specification product | Incorrect basicity or polymer distribution. | Online pH or density analysers, automatic dosing control, quality control laboratory checks. |

## HAZARD AND OPERATABILITY (HAZOP) ANALYSIS

HAZOP enables early identification and mitigation of risks that might otherwise remain hidden during normal design reviews. This approach significantly reduces the likelihood of major accidents, protects personnel and the environment, improves operational reliability while supporting regulatory compliance with frameworks such as OSHA’s Process Safety Management (PSM) standards. In practice, a well-executed HAZOP studies at the detailed design stage commonly generates recommendations that can cut the risk of major process accidents by a substantial margin when implemented and minimise costly downtime and production losses (OSHA, 2023).

### RISK MATRIX METHODOLGY

A standard 5×5 Risk Matrix is applied to every deviation:

Likelihood (L): 1=Rare, 2=Unlikely, 3=Possible, 4=Likely, 5=Almost certain

Severity (S): 1=Negligible, 2=Minor, 3=Moderate, 4=Major, 5=Catastrophic

Risk ranking: 1-4=Low (Green), 5-9= Medium(Yellow), 10-16=High (Orange/Red), 17-25=Very High/ Critical (Red)

Risk Rank = Likelihood × Severity. Rankings assume that existing safeguards are in place. After recommended actions are implemented, residual risk is expected to fall.

### HAZOP Analysis and Risk Ranking for Area 100 – Raw Material Handling

#### HAZOP Analysis

| Deviation | Cause | Consequence | Safeguard | Action | Assigned to |
| --- | --- | --- | --- | --- | --- |
| No flow of material | Blockage in grizzly screen or conveyer | Equipment overload, downtime, delayed feed to crusher. | Level sensors and alarms. | Clear blockage and inspection of screens regularly. | Maintenance personnel. |
| More dust generation | Overloading of jaw crusher | Dust emissions, respiratory hazards, poor visibility. | Baghouse extractions. | Adjust feed rate and maintain dust collectors. | Process Engineer |
| Noise generation | Vibration from crusher operation. | Hearing damage, equipment fatigue. | Acoustic enclosures and vibration monitoring. | Maintain equipment mounts and replace worn parts. | Maintenance Personnel. |
| Less particle size reduction. | Worn crusher jaws. | Inefficient downstream processing. | Routine wear monitoring. | Replace crusher jaws as per schedule. | Maintenance Personnel. |

#### Risk Ranking

| Deviation | L | S | Risk Ranking |
| --- | --- | --- | --- |
| No flow of material | 3 | 3 | Medium (9) |
| More dust generation | 4 | 4 | High (16) |
| Noise generation | 4 | 3 | High (12) |
| Less particle size reduction | 3 | 2 | Medium (6) |

### HAZOP Analysis and Risk Ranking for Area 200 – Pyro-Processing

#### HAZOP analysis

| Deviation | Cause | Consequence | Safeguard | Action | Assigned to |
| --- | --- | --- | --- | --- | --- |
| More temperature. | Excess fuel or cooling failure. | Over-calcination, refractory damage, excess emissions. | High temperature alarms or interlocks. | Calibrate sensors and check cooling systems. | Process Engineer |
| No material flow. | Feed interruption. | Kiln overheating, potential equipment damage. | Flow sensors and low-flow trip. | Restore feed and inspect kiln internals. | Operations Engineer |
| Reducing pressure or incomplete combustion | Air-fuel ratio imbalance | Explosive mixtures, CO formation. | Oxygen analysers and burner management systems. | Adjust air-fuel ratio and purge system. | Process Engineer |
| Reverse rotation | Drive-system failure | Material build-up and sintering. | Emergency stop systems. | Repair drive system immediately. | Maintenance personnel |

#### Risk Ranking

| Deviation | L | S | Risk Ranking |
| --- | --- | --- | --- |
| More temperature | 3 | 5 | High (15) |
| No material flow | 3 | 4 | High (12) |
| As well as (reducing atmosphere) | 2 | 5 | High (10) |
| Reverse rotation | 2 | 4 | Medium (8) |

### HAZOP Analysis and Risk Ranking for Area 300 – Flue-Gas Treatment

#### HAZOP Analysis

| Deviation | Cause | Consequence | Safeguard | Action | Assigned to |
| --- | --- | --- | --- | --- | --- |
| Less removal efficiency. | Baghouse filter blinding. | Excess emissions, regulatory non-compliance | Differential pressure monitoring. | Initiate cleaning cycle or replace filters. | Maintenance Personnel. |
| More pressure. | Blockage in ducting or cyclone. | Risk of equipment rupture. | Pressure relief valves. | Clear blockages and inspect ducts. | Maintenance Personnel. |
| No caustic flow. | Pump or dosing failure. | Unneutralized acid gas release. | Low-flow alarms. | Check and clean cooler or scrubber nozzles. | Maintenance Personnel. |
| Non-uniform cooling | Evaporative cooler malfunction. | High-temperature gas to baghouse. | Temperature sensors and interlocks. | Restore cooler operation, inspect. | Maintenance personnel |

#### Risk Ranking

| Deviation | L | S | Risk Ranking |
| --- | --- | --- | --- |
| Less removal efficiency | 3 | 4 | High (12) |
| More pressure | 2 | 5 | High (10) |
| No caustic flow (scrubber) | 2 | 5 | High (10) |
| Non-uniform cooling | 3 | 3 | Medium (9) |

### HAZOP Analysis and Risk Ranking for Area 400 – Dry Magnetic Separation

#### HAZOP Analysis

| Deviation | Cause | Consequence | Safeguard | Action | Assigned to |
| --- | --- | --- | --- | --- | --- |
| No/Low separation efficiency | Moisture in feed material or worn magnets. | Poor product quality, magnetite contamination of alumina stream. | Moisture analysers and magnet strength monitoring. | Dry feed material and inspect or replace magnets. | Process engineer |
| More dust generation | Overloading of separator or poor enclosure. | Fugitive emissions, respiratory hazard, possible dust explosion risk. | Enclosed system with local exhaust ventilation or baghouse. | Reduce feed rate and check dust extraction system. | Process engineer |
| No material flow | Blockage on feed conveyor failure. | Equipment idle running, production stoppage. | Level or flow sensors and interlocks on CV-401 | Clear blockage and inspect feed system. | Maintenance personnel |
| Reverse flow or material backup | Downstream conveyor. | Overflow and spillage of magnetic or non-magnetic fractions. | Proper grounding and bonding of equipment. | Stop upstream feed and clear backup. |  |
| As well as static charge | Dry powder handling without grounding. | Static discharge leading to dust ignition. | Proper grounding and bonding of equipment. | Verify earthing continuity and install additional bonding if needed. | Maintenance personnel or electrical engineers |
| More magnetic field exposure | Damaged shielding or operator proximity. | Potential interference with pacemakers or tools. | Magnetic shielding and restricted access zones. | Repair shielding and enforce access control. | EHS officer |

#### Risk Ranking

| Deviation | L | S | Risk Ranking |
| --- | --- | --- | --- |
| No / Low separation efficiency | 3 | 3 | Medium (9) |
| More dust generation | 4 | 4 | High (16) |
| No material flow | 3 | 2 | Medium (6) |
| Reverse flow / material backup | 2 | 3 | Medium (6) |
| As well as (static charge) | 2 | 5 | High (10) |
| More magnetic field exposure | 2 | 3 | Medium (6) |

### HAZOP Analysis and Risk Ranking for Area 500 – Wet Grinding

#### HAZOP Analysis

| Deviation | Cause | Consequence | Safeguard | Action | Assigned to |
| --- | --- | --- | --- | --- | --- |
| More slurry viscosity | Over-grinding or poor classification | Pump cavitation, blockages, overflow | Viscosity and density monitoring | Adjust mill parameters and hydrocyclones | Process Engineer |
| No flow to ball mill | Feeder failure | Dry running of mill, possible damage | Level alarms in BN-501 | Repair feeder; restore material flow | Maintenance personnel |
| Less particle-size control | Hydrocyclone blockage | Coarse product, inefficient circuit | Pressure differential across cyclones | Flush and inspect hydrocyclones | Maintenance personnel |

#### Risk Ranking

| Deviation | L | S | Risk Ranking |
| --- | --- | --- | --- |
| More slurry viscosity | 3 | 3 | Medium (9) |
| No flow to ball mill | 2 | 4 | Medium (8) |
| Less particle size control | 3 | 2 | Medium (6) |
| More noise / vibration | 4 | 3 | High (12) |

### HAZOP Analysis and Risk Ranking for Area 600 – Leaching

#### HAZOP Analysis

| Deviation | Cause | Consequence | Safeguard | Action | Assigned to |
| --- | --- | --- | --- | --- | --- |
| More HCl concentration | Over-dosing from storage | Violent reaction, excessive fumes, accelerated corrosion | Flow totalisers and concentration analysers | Calibrate dosing system; neutralise any spill | Process Engineer |
| No agitation in reactor | Stirrer failure | Hotspots, possible runaway reaction | Redundant agitator drive and temperature monitoring | Repair agitator immediately | Maintenance personnel |
| Less cooling | Jacket or cooling-water failure | Over-pressure and potential release | Backup cooling loop and high-temperature trip | Restore cooling; inspect jackets | Maintenance personnel |
| Leak of HCl | Pipe or tank corrosion | Toxic release, environmental spill | Double containment and leak detection | Isolate area, neutralise and repair | Maintenance & EHS officers |

#### Risk Ranking

| Deviation | L | S | Risk Ranking |
| --- | --- | --- | --- |
| More HCl concentration | 2 | 5 | High (10) |
| No agitation in reactor | 2 | 5 | High (10) |
| Less cooling | 2 | 5 | High (10) |
| Leak of HCl | 2 | 5 | High (10) |

### HAZOP Analysis and Risk Ranking for Area 700 – Basification and Polymerisation

#### HAZOP Analysis

| Deviation | Cause | Consequence | Safeguard | Action | Assigned to |
| --- | --- | --- | --- | --- | --- |
| More temperature | Insufficient cooling during basification | Runaway reaction, product degradation | Jacketed reactor with temperature control | Increase cooling flow; review recipe | Process Engineer |
| Less basicity | Under-dosing of calcium aluminate | Off-specification PAC product | Online pH and density analysers | Adjust dosing; reprocess batch if necessary | Process Engineer |
| No flow to storage | Pump or line blockage | Tank overflow or process stoppage | Level alarms in TK-701 / TK-702 | Clear line and check pumps | Maintenance personnel |

#### Risk Ranking

| Deviation | L | S | Risk Ranking |
| --- | --- | --- | --- |
| More temperature (exotherm) | 3 | 4 | High (12) |
| Less basicity | 3 | 3 | Medium (9) |
| No flow to storage | 2 | 3 | Medium (6) |
| More residual acid | 2 | 3 | Medium (6) |

## POLLUTION CONTROL SUMMARY

The plant incorporates a multilayered pollution control strategy consistent with Ghana Environmental Protection Authority (EPA) requirements and international practice:

- Air emissions – baghouse filtration, CEMS and emergency caustic scrubbing for acid gases.

- Liquid effluent – collection in secondary containment, neutralisation and treatment at the on-site ETP to meet EPA discharge limits before release.

- Solid residues – magnetic separation of magnetite (sold as by-product) and dewatered silica-rich filter cake managed as a non-hazardous residue or construction aggregate.

- Fugitive dust and noise – enclosed conveying, local extraction, acoustic treatment and mandatory PPE in high-exposure zones.

# PLANT ECONOMIC ANALYSIS

## Executive summary

The evaluation encompasses a complete capital expenditure (CAPEX) summary, a detailed operating expenditure (OPEX) analysis disaggregated by cost centre, a comprehensive revenue and market assessment, and a multi-metric profitability appraisal covering the Net Present Value (NPV), Internal Rate of Return (IRR), payback period, and sensitivity analysis. All monetary values are reported in both United States Dollars (USD) and Ghana Cedis (GHS), using the PURC macroeconomic assumption of GHS 12.0067 per USD.

The key findings of this evaluation are summarised as follows: the Total Fixed Capital Investment (FCI) is estimated at USD 13,402,360, yielding a Total Capital Investment (TCI) inclusive of working capital of USD 19,946,733. The total annual operating expenditure at full capacity is estimated at USD 11,095,802 per annum. At an assumed blended PAC selling price of USD 450 per tonne, the project yields a Net Present Value of USD 14,864,691 at a Weighted Average Cost of Capital (WACC) of 8.0%, and an Internal Rate of Return of 14.8%, approximating to the 15% hurdle rate. The discounted payback period is estimated at 6.0 years.

## Introduction and plant overview

The table below summarises the fundamental design basis and operating parameters adopted throughout this economic evaluation. The annual operating hours of 7,920 hr/yr reflect an industry-standard stream factor of 91.3%, consistent with comparable continuous chemical processing operations in Ghana such as the Volta Aluminium Company (VALCO) smelter. The remaining 760 hours per annum are allocated to planned preventive maintenance, statutory inspection, and turnaround activities.

Table 10-1 Key plant design and economic parameters

| Parameter | Value |
| --- | --- |
| Product | Liquid Poly-Aluminium Chloride (PAC) |
| Production Capacity | 30,000 TPA |
| Operating Hours | 7,920 hr/yr |
| Thermodynamic Model | ENRTL-RK (Aspen Plus) |
| CEPCI Basis Year | 2026 (CEPCI = 678.4) |
| Exchange Rate | GHS 12.0067 / USD |
| Project Lifespan | 20 years operating (22 years total) |
| Construction Period | 2 years |
| Commissioning Period | 6 months (Year 0) |

### Key Plant Parameters and Basis of Estimate

The basis of the economic estimate is presented in Table 10.1. All process data are derived from the Aspen Plus v14.2 simulation of the plant (model: ENRTL-RK thermodynamic method), and all utility rates are sourced from the Public Utilities Regulatory Commission of Ghana (PURC) 2026–2030 Tariff Decision, effective 1 January 2026. The exchange rate adopted throughout this chapter is GHS 12.0067 per USD, as stipulated by PURC (2026). The Chemical Engineering Plant Cost Index (CEPCI) adopted for 2026 is 678.4, escalated from the Turton et al. (2018) 2001 base index of 397.0, yielding an escalation factor of 1.709.

| Parameter | Value |
| --- | --- |
| Product | Liquid Poly-Aluminium Chloride (PAC) |
| Operating Hours | 7,920 hr/yr (90.4% stream factor) |
| Thermodynamic Model | ENRTL-RK (Aspen Plus v14.2) |
| CEPCI (2001 base) | 397.0 |
| CEPCI (2026) | 678.4 |
| CEPCI Escalation Factor | 1.7088 |
| Exchange Rate | GHS 12.0067 / USD |
| Ghana Location Factor | 1.35 |
| Project Life | 20 operating years + 2 construction years |
| Corporate Tax Rate | 18.8% |
| Depreciation Method | Straight-line, 5 years |
| Estimate Class | Class 4 (Conceptual) |
| Working Capital | USD 2,400,381 (15% of Adj. BM Cost) |

Figure 10-1 Key Plant Design and Economic Parameters

## Capital Expenditure (CAPEX)

### Costing Methodology

#### Turton et al. Bare Module Method

Equipment purchase costs were estimated using the power-law correlations presented by Turton et al. (2018), which express the purchased equipment cost, C°p, as a function of a single characteristic size parameter, A, as follows:

log₁₀(C°p) = K1+K2log10A+K3log10A2

where K₁, K₂, and K₃ are equipment-specific regression coefficients tabulated in Turton et al. (2018), Appendix A; A is the sizing parameter (capacity, area, power, volume, etc.) expressed in SI units; and C°p is the purchased equipment cost at the USGC (US Gulf Coast) reference basis, in US dollars, at the 2001 CEPCI of 397.0. The bare module cost, CBM, which accounts for materials of construction, installation, instrumentation, piping, electrical, civil, structural, and insulation, is computed as:

CBM= C°p × FBM =  C°p × B1+ B2× Fm × Fp

where B₁ and B₂ are bare module correction factors; Fm is the materials of construction factor (unity for carbon steel); and Fp is the design pressure factor (unity at atmospheric pressure). For equipment procured at budgetary prices (e.g., RK-201 rotary kiln, ML-501 ball mill), vendor quotations were substituted in place of the Turton correlation.

#### CEPCI Escalation

All base costs are referenced to the 2001 CEPCI. Costs were escalated to the 2026 cost basis using the CEPCI ratio:

C°p,2026=C°p,2001×CEPCI2026CEPCI2001= C°p,2001×678.4397.0= C°p,2001×1.7088

#### Ghana Location Factor

USGC bare module costs were adjusted for the Ghanaian location by application of a location factor of 1.35. This factor reflects the additional costs associated with importation of equipment through Takoradi/Tema port (import duties under GIADEC Act 976 capital equipment exemptions partially offset duties), inland transport to the Awaso site (approximately 350 km from Tema), limited local heavy fabrication capacity, and elevated civil works costs attributable to remote site conditions. The adopted location factor of 1.35 is consistent with published values for Sub-Saharan African industrial locations reported by Humphreys (1991) and the IChemE Guide to Capital Cost Estimating (IChemE, 2003). The adjusted bare module cost is:

CBM,Ghana= CBM,USGC,2026× LF = CBM,USGC,2026×1.35

### Equipment Cost Estimate

For each item, the purchased cost (C°p, 2001 basis), the bare module factor (FBM), the bare module cost (CBM, 2001 basis), the escalated bare module cost (CBM, 2026 USGC), and the Ghana-location-adjusted bare module cost (CBM, Ghana) are reported.

Table 10-2 Area 100 Equipment Cost Estimate (2026, Ghana Basis)

| Tag | Description | Size | Unit | C°p 2001 (USD) | FBM | C_BM Ghana (USD) |
| --- | --- | --- | --- | --- | --- | --- |
| CRN-101 | Gantry Crane, grab bucket | 5 | t SWL | 67500.00 | 1.500 | 115345.09 |
| FD-101 | Vibrating feeder + hopper | 0.76 | t/hr | 2223.51 | 1.500 | 3799.57 |
| CR-101 | Jaw Crusher | 1.5 | kW | 9219.31 | 2.195 | 15,754.11 |
| CV-101 | Belt Conveyor (enclosed) | 15 | m | 3819.41 | 1.500 | 6526.67 |
| BN-101 | Day Bin (crushed ore) | 17.5 | m³ | 67677.67 | 4.070 | 115,648.69 |
| Subtotal |  |  |  |  |  | 257074.13 |

Table 10-3 Area 200 Equipment Cost Estimate (2026, Ghana Basis)

| Tag | Description | Size | Unit | C°p 2001 (USD) | FBM | CBM Ghana (USD) |
| --- | --- | --- | --- | --- | --- | --- |
| FD-201 | Screw Feeder | 0.5 | m³ | 5250.59 | 1.500 | 3625.00 |
| RK-201 | Counter-current Rotary Kiln | 31.42 | m³ | 810219.08 | 7.114 | 1384515.42 |
| BR-201 | Kiln Burner | 300 | kW thermal | 2700.00 | 3.010 | 4613.80 |
| E-201 | Gas-Gas Air Preheater | 0.33 | m² | 199761.79 | 6.195 | 341356.16 |
| CL-201 | Rotary Tube Cooler | 3.34 | m² | 53459.27 | 3.290 | 91352.06 |
| CV-201 | Drag Conveyor | 10 | m | 5926.40 | 1.500 | 10127.13 |
| Subtotal |  |  |  |  |  | 1835589.58 |

Table 10-4 Area 300 Equipment Cost Estimate (2026, Ghana Basis)

| Tag | Description | Size | Unit | C°p 2001 (USD) | FBM | CBM Ghana (USD) |
| --- | --- | --- | --- | --- | --- | --- |
| E-301 | Waste Heat Boiler | 300 | kW | 124157.16 | 3.290 | 212161.75 |
| CY-301 | Cyclone Separator | 0.18 | m dia | 17527.60 | 1.500 | 29951.44 |
| CL-301 | Evaporative Cooler | 200 | kW duty | 102292.59 | 3.290 | 174799.22 |
| BH-301 | Pulse-Jet Baghouse | 2 cells×6 bags | — | 47257.67 | 2.000 | 80754.66 |
| FN-301 | Induced Draft Fan | 15 | kW | 12401.46 | 1.500 | 21191.81 |
| SC-301 | Emergency Wet Scrubber | 300 | mm dia | 1090246.02 | 5.890 | 1000000.00 |
| ST-301 | Atmospheric Stack | 30 | m | 100883.00 | 4.070 | 172390.49 |
| Subtotal |  |  |  |  |  | 1691249.38 |

Table 10-5 Area 400 Equipment Cost Estimate (2026, Ghana Basis)

| Tag | Description | Size | Unit | C°p 2001 (USD) | FBM | CBM Ghana (USD) |
| --- | --- | --- | --- | --- | --- | --- |
| MS-401 | HIMS Drum Separator | 0.6 | t/hr feed | 69989.30 | 2.500 | 119598.84 |
| CV-401 | Non-mag belt conveyor | 10 | m | 5926.40 | 1.500 | 10127.13 |
| CV-402 | Magnetic belt conveyor | 10 | m | 5926.40 | 1.500 | 10127.13 |
| BN-401 | Magnetite Storage Silo | 5 | m³ | 29713.49 | 4.070 | 50774.89 |
| Subtotal |  |  |  |  |  | 190628.01 |

Table 10-6 Area 500 Equipment Cost Estimate (2026, Ghana Basis)

| Tag | Description | Size | Unit | C°p 2001 (USD) | FBM | CBM Ghana (USD) |
| --- | --- | --- | --- | --- | --- | --- |
| BN-501 | Alumina Feed Bin | 3 | m³ | 22152.99 | 4.070 | 37855.38 |
| FD-501 | Weight Belt Feeder | 0.69 | t/hr | 2158.83 | 1.500 | 3689.04 |
| TK-501 | Slurry Make-up Tank | 2.5 | m³ | 20066.79 | 4.070 | 34290.46 |
| P-501 | Ball Mill Feed Pump | 1.74 | m³/hr | 4122.05 | 1.600 | 7043.82 |
| ML-501 | Ball Mill | 30 | kW motor | 135000.00 | 2.563 | 230690.18 |
| P-502 | Mill Discharge Pump | 1.74 | m³/hr | 4122.05 | 1.600 | 7043.82 |
| HC-501 | Hydrocyclone Cluster | 4×0.30 m dia | — | 28231.22 | 1.500 | 48241.96 |
| Subtotal |  |  |  |  |  | 368854.65 |

Table 10-7 Area 600 Equipment Cost Estimate (2026, Ghana Basis)

| Tag | Description | Size | Unit | C°p 2001 (USD) | FBM | CBM Ghana (USD) |
| --- | --- | --- | --- | --- | --- | --- |
| TK-601 | HCl Storage Tank | 310 | m³ | 778017.51 | 4.070 | 1329488.87 |
| P-601 | HCl Dosing Pump | 1.2 | m³/hr | 11382.07 | 4.590 | 19449.86 |
| R-601 | Leach Reactor Train | 12 | m³ | 846568.00 | 8.820 | 1446629.05 |
| SC-601 | HCl Scrubber | 300 | mm dia | 1090246.02 | 5.890 | 800000.00 |
| P-602 | Leach Slurry Pump | 2.65 | m³/hr | 12625.54 | 4.590 | 21574.73 |
| FP-601 | High-Pressure Filter Press | 22 | m² | 140718.06 | 2.500 | 240461.29 |
| CV-601 | Filter Cake Conveyor | 8 | m | 5364.58 | 1.500 | 9167.08 |
| BN-601 | Residue Silo | 10 | m³ | 46014.47 | 4.070 | 78630.27 |
| TK-602 | PLP Holding Tank | 3.5 | m³ | 24143.51 | 4.070 | 41256.82 |
| Subtotal |  |  |  |  |  | 3986657.95 |

Table 10-8  Area 700 Equipment Cost Estimate (2026, Ghana Basis)

| Tag | Description | Size | Unit | C°p 2001 (USD) | FBM | CBM Ghana (USD) |
| --- | --- | --- | --- | --- | --- | --- |
| TK-701 | CaAl₂O₄ Make-up Tank | 0.6 | m³ | 10298.67 | 4.070 | 17598.53 |
| R-701 | Basification Reactor | 15 | m³ | 74243.93 | 4.980 | 126869.22 |
| FP-701 | Clarification Filter | 18.27 | m² | 139004.01 | 2.285 | 237532.30 |
| R-702 | Polymerisation Reactor | 8.25 | m³ | 67786.38 | 6.800 | 115834.46 |
| E-701 | Product Cooler | 25 | m² | 54656.56 | 3.290 | 93398.01 |
| TK-702 | PAC Storage Tanks | 2×50 m³ | — | 17854.14 | 4.070 | 30509.44 |
| Subtotal |  |  |  |  |  | 621741.96 |

Table 10-9 HEN Equipment Cost Estimate (2026, Ghana Basis)

| Tag | Description | Size | Unit | C°p 2001 (USD) | FBM | CBM Ghana (USD) |
| --- | --- | --- | --- | --- | --- | --- |
| HX-102 | Gas-Gas Preheater | 40 | m² | 110876.88 | 6.112 | 189468.20 |
| HX-103 | Gas-Water Recovery | 25 | m² | 54656.56 | 3.290 | 93398.01 |
| HX-201 | Acid Preheater | 13 | m² | 94649.13 | 6.112 | 161737.96 |
| HX-601 | Slurry Preheater | 10 | m² | 70721.90 | 4.618 | 120850.73 |
| HX-203 | Post-Leach Cooler | 25.62 | m² | 54865.28 | 3.290 | 93754.67 |
| Subtotal |  |  |  |  |  | 659209.56 |

Table 10-10 Ancillary Equipment Cost Estimate (2026, Ghana Basis)

| Tag | Description | Size | Unit | C°p 2001 (USD) | FBM | CBM Ghana (USD) |
| --- | --- | --- | --- | --- | --- | --- |
| P-101 | Process Water Pump | 1.6 | m³/hr | 11704.74 | 4.590 | 20001.24 |
| P-201 | HCl Feed Pump | 1.2 | m³/hr | 11382.07 | 4.590 | 19449.86 |
| CP-101 | Air Compressor | 50 | kW | 40268.49 | 2.000 | 68811.44 |
| V-201 | Pressure Letdown Valve | 1.96 | atm | 18750.00 | 2.500 | 32040.30 |
| H-101 | Feed Hopper | 2 | m³ | 17854.14 | 4.070 | 30509.44 |
| H-301 | CaAl₂O₄ Hopper | 1 | m³ | 12793.65 | 4.070 | 21861.99 |
| TK-101 | Water Feed Tank | 5 | m³ | 29713.49 | 4.070 | 50774.89 |
| TK-103 | Leach Feed Tank | 2 | m³ | 17854.14 | 4.070 | 30509.44 |
| SS-201 | Flow Splitter | 1600 | kg/hr | 4000.00 | 2.000 | 6835.26 |
| MX-101/201 | Inline Mixers (×2) | 2 | units | 20994.14 | 2.000 | 35875.12 |
| Subtotal |  |  |  |  |  | 316668.99 |

### Total Capital Investment Summary

#### Fixed Capital Investment (FCI)

The Fixed Capital Investment (FCI) is derived from the grass-roots investment (CGR) by the addition of the EPC (Engineering, Procurement and Construction) management fee and a project contingency, as follows:

CGR= CBM,Ghana + Site Dev. + Auxiliary Buildings + Utilities Infrastructure

FCI = CGR + EPC Fee (10%) + Contingency (10%)

The site development allowance, auxiliary buildings allowance and utilities infrastructure allowance are consistent with the factors recommended by Turton et al. (2018) for a grassroots chemical plant on a developed industrial site. The contingency of 10% is appropriate for a Class 4 estimate with expected accuracy of ±30–50% (AACE International, 2016). The plant is assumed to be located on a brownfield site within the Ghana Integrated Aluminium Development Corporation (GIADEC) industrial framework at Awaso, which reduces civil site preparation costs relative to a greenfield location. The total Fixed Capital Investment is USD 24,899,154 (GHS 298,956,670).

Table 10-11 Total Capital Investment Summary

| Cost Component | Factor | USD | GHS |
| --- | --- | --- | --- |
| Total Bare Module Cost (USGC, 2026) | — | 9,927,674.21 | 119198606 |
| Fixed Capital Investment | ×1.35 (Ghana Location Adjustment) | 13402360.19 | 160918118 |
| Site Development & Civil (5%) | 5% of CBM,GH | 670118.01 | 8045905.90 |
| Auxiliary Buildings (8%) | 8% of CBM,GH | 1072188.81 | 12873449.44 |
| Utilities Infrastructure (10%) | 10% of CBM,GH | 1340236.02 | 16091811.80 |
| Grass-Roots Investment (CGR) | — | 16484903.03 | 197929285.18 |
| EPC Management Fee (10%) | 10% of CGR | 1648490.30 | 19792928.52 |
| Contingency (15%) | 10% of (CGR + EPC) | 2720009.00 | 32658332.06 |
| Working Capital (15% of Adj. BM) | 15% | 2,010,354 | 24137717.71 |
| Total Capital Investment (TCI) | — | $19,946,733 | 239494435.1 |

#### Working Capital

Working capital was estimated at 15% of the Fixed Capital Investment, consistent with the approach recommended by Turton et al. (2018) for chemical plants with moderate inventory requirements. It is assumed that the working capital is fully recovered at the end of the project life (Year 20) as part of the terminal cash flow.

#### CAPEX Phasing — Construction Schedule

The FCI is disbursed over two years of construction and one commissioning year. The phasing adopted is consistent with the timeline for a modular-constructed chemical plant of this scale (Humphreys, 1991):

Table 10-12 CAPEX Phasing by Construction Year

| Year | Description | Fraction of TCI | Amount (USD) |
| --- | --- | --- | --- |
| -2 | Early civil works, long-lead equipment procurement | 30% | -$5,984,020 |
| -1 | Major equipment installation, piping, electrical | 50% | -$9,973,366 |
| 0 | Commissioning, start-up, pre-production | 20% | -$3,989,347 |

## Operating Expenditure (OPEX)

The annual OPEX is disaggregated into variable costs and fixed costs, which are incurred regardless of capacity utilisation. All costs are presented at the 100% capacity utilisation basis unless otherwise stated. Costs are reported in both USD and GHS at the PURC (2026) exchange rate of GHS 12.0067/USD. An annual cost escalation of 4.0% is applied to all OPEX line items throughout the project cash flow, consistent with Ghana's stated inflation target as embedded in the PURC 2026–2030 tariff structure (PURC, 2026).

### Raw Material Costs

Raw material costs represent the largest single component of variable OPEX and are computed from the Aspen Plus stream report mass flow rates, converted to annual quantities based on 7,920 operating hours per year. Unit prices were obtained from the regional market sources identified in Section 5.1. Bauxite is priced at USD 35/t, consistent with Asian Metal (2024) pricing data for Awaso bauxite ROM ore. Hydrochloric acid (32 wt%) is priced at USD 180/t based on a commercial quotation from Cetech BPA Ghana (2026). Calcium aluminate (CaAl₂O₄) is priced at USD 310/t per the 6W Research (2026) Ghana market report. Carbon monoxide (industrial grade) is priced at USD 450/t based on Air Liquide Ghana rates.

Table 10-13 Annual Raw Material Costs (100% Capacity Utilisation)

| Raw Material | Flow (kg/hr) | Annual (t/yr) | Price (USD/t) | Annual Cost (USD) | Annual Cost (GHS) |
| --- | --- | --- | --- | --- | --- |
| Awaso Bauxite (ROM ore) | 756.0 | 5,987.4 | $35.00 | $209,560 | 2516129.191 |
| Hydrochloric Acid (32 wt%) | 2,285.0 | 18,097.2 | $180.00 | $3,257,496 | 39111777.22 |
| Calcium Aluminate (CaAl₂O₄) | 650.0 | 5,148.0 | $310.00 | $1,595,880 | 19161252.4 |
| Carbon Monoxide (industrial) | 329.3 | 2,607.7 | $450.00 | $1,173,447 | 14089226.09 |
| Process Water | 833.3 | 6,600.0 | $2.73 | $18,018 | 216335.8553 |
| TOTAL |  |  |  | $6,254,401 | 75094720.76 |

### Utility Costs

#### Electricity (PURC MV SLT Tariff)

The plant is connected to the national grid at Medium Voltage (MV) level under the Special Load Tariff (SLT) industrial tariff schedule of PURC. The applicable energy charge is 215.3414 GHp/kWh (= USD 0.01794/kWh at the PURC exchange rate). The monthly service charge is GHP 50,000/month. Estimated annual electrical energy consumption of 1,188,000.0 kWh yields an annual electricity cost of approximately $131,180.

#### Cooling Water (PURC Industrial Tariff)

Process cooling water is supplied from the GWCL industrial tariff at 3,278.5809 GHp/m³ (= USD 0.2731/m³). The estimated annual consumption of cooling water for process cooling duties (principally the CL-201 product cooler, HX-203 post-leach cooler, E-701 product cooler, and R-702 polymerisation cooling) is approximately 40,000 m³/yr, yielding a cost of approximately $ 865,312.

Table 10-14 Annual Utility Costs (PURC 2026 Tariff Basis, 100% Utilisation)

| Utility | PURC Rate | Annual Consumption | Annual Cost (USD) | Annual Cost (GHS) |
| --- | --- | --- | --- | --- |
| Electricity (MV SLT) | 215.3414 GHp/kWh | 134,400 kWh | 192,764 | 2,314,460 |
| Cooling Water (Industrial) | 3,278.58 GHp/m³ | 40,000 m³ | 10,924 | 131,161 |
| TOTAL | — | — | 253,661 | 3,045,632 |

### Labour Costs

#### Shift Structure and Staffing

To maintain continuous 24/7 operation over the 330-day campaign (7,920 hr/yr), a four-crew rotating shift pattern was adopted. Each shift crew comprises 22 personnel, yielding a total shift headcount of 88. An additional 24 day-shift administrative, engineering, and management personnel are employed on a Monday-to-Friday basis. The total site complement is therefore 112 personnel. The staffing model was benchmarked against comparable Ghanaian heavy industrial operations, including VALCO (aluminium smelting), Sentuo Oil Refinery, and Newmont Ghana Ahafo mine, to ensure competitive wage rates and minimize staff turnover. A deliberate premium wage strategy is adopted, whereby total labour cost (USD 873,000/yr) is explicitly set above the market median to attract and retain qualified mechanical, electrical, and chemical technicians in the highly corrosive HCl plant environment.

Table 10-15 Shift Labour Staffing and Annual Cost

| Role | Annual Salary (USD) | Staff/Shift | Total Crews | Annual Cost (USD) |
| --- | --- | --- | --- | --- |
| Shift Supervisor | $12,000 | 1 | 4 | $48,000 |
| DCS / Panel Operator | $7,500 | 2 | 4 | $60,000 |
| Field Operator | $6,000 | 8 | 4 | $192,000 |
| Mechanical Fitter | $7,500 | 2 | 4 | $60,000 |
| Electrical / Instr. Tech | $7,500 | 2 | 4 | $60,000 |
| Laboratory Analyst | $6,500 | 2 | 4 | $52,000 |
| HSE / Safety Officer | $7,000 | 1 | 4 | $28,000 |
| Weighbridge Operator | $5,000 | 1 | 4 | $20,000 |
| Security Guard | $3,000 | 3 | 4 | $36,000 |
| Subtotal — Shift (88 headcount) |  | 22 |  | $556,000 |

Table 10-16 Day & Administrative Labour Staffing and Annual Cost

| Role | Annual Salary (USD) | Headcount | Annual Cost (USD) |
| --- | --- | --- | --- |
| Plant Manager | $28,000 | 1 | $28,000 |
| Department Managers | $20,000 | 5 | $100,000 |
| Process / Plant Engineers | $15,000 | 4 | $60,000 |
| Admin / Procurement / HR | $8,000 | 4 | $32,000 |
| Day Maintenance Crew | $7,000 | 4 | $28,000 |
| General Labourers | $3,000 | 6 | $18,000 |
| Subtotal — Admin (24 headcount) |  | 24 | $266,000 |
| TOTAL LABOUR (112 headcount) | $28,000 | 1 | $28,000 |

### Maintenance

Annual maintenance cost was estimated at 6.0% of the Fixed Capital Investment, yielding $804,142. This factor is higher than the 3–4% typically applied to conventional chemical plants (Turton et al., 2018) and reflects the aggressive corrosion environment arising from HCl service, PAC product, and elevated temperature conditions. Specific high-frequency maintenance items include: kiln refractory relining (RK-201, every 3–5 years), ceramic filter candle replacement (BH-301, annually), rubber/polyurethane lining replacement in HC-501 and pumps P-501/P-502 (biannually), PTFE pump component replacement (P-201, P-601, annually), filter cloth replacement for FP-601 and FP-701 (quarterly), graphite heat exchanger element replacement (HX-201, E-701, every 3 years), ball mill grinding media and liner replacement (ML-501, annually), and jaw crusher wear plate replacement (CR-101, annually).

### Insurance

Annual plant insurance was estimated at 1.5% of FCI, $201,035, consistent with industry norms for process plants with corrosive service conditions (Turton et al., 2018). This figure covers property damage, business interruption, and third-party public liability, but excludes workers' compensation (incorporated in the labour overhead).

### Plant Overheads

Plant overheads (covering supervision, administration, laboratory services, safety, and general plant management) were estimated at 20% of (Labour + Maintenance) = $582,731, consistent with the method of Turton et al. (2018). This figure covers the cost of administrative infrastructure, IT systems, safety management systems, and regulatory compliance administration.

### Logistics and Distribution

Given the plant capacity of 31,384.63 TPA, of which approximately 26,000 TPA are destined for export to ECOWAS regional markets (Côte d'Ivoire, Senegal, Nigeria, Burkina Faso), logistics and distribution constitute a material OPEX component. Liquid PAC is distributed in 1,000 L intermediate bulk containers (IBCs) or direct pumped to ISO tankers. Freight costs from Awaso to Takoradi Port (310 km road) and onward sea freight to ECOWAS ports were estimated at USD 41/t, yielding a Year 1 (60% utilisation) cost of USD 738,000, escalating to $1,230,000 at full capacity utilisation.

### Other Variable Costs

Other variable costs include flocculants and filter aid chemicals for FP-601 and FP-701 operation; pH adjustment reagents; PTFE gaskets and seals for pump and valve maintenance; ball mill steel grinding media (high-Cr, 2.5 kg media/t product); and cleaning/sterilising chemicals for product storage tanks. At 100% utilisation, these costs are estimated at $180,000.

### Fixed Costs: CSR, Property Tax and Permits

A Corporate Social Responsibility (CSR) and environmental compliance budget of $10,000 is allocated for community development fund contributions (consistent with the GIADEC Act 976 community benefit provisions), ongoing environmental monitoring, stack emissions compliance testing (EPA Ghana, 2016), and annual reporting. Property taxes are estimated at USD 15,000/yr, based on the Western Region assessment rates for industrial facilities (Ghana Revenue Authority, 2023). Annual permit and license fees — covering the EPA Ghana environmental permit, water abstraction permit, and GWCL offtake agreement fees — are incorporated in the CSR provision.

### Total OPEX Summary

The table below presents the total annual OPEX at 100% capacity utilisation, disaggregated by cost category, and expressed as a percentage of total OPEX. The aggregate annual OPEX at 100% utilisation and Year 1 real prices is approximately USD 13.86 million.

Table 10-17 Total Annual OPEX Summary (100% Utilisation, Year 1 Real Prices)

| Cost Category | Type | Annual (USD) | % of OPEX |
| --- | --- | --- | --- |
| Raw Materials | Variable | $6,254,401 | 56.4 |
| Electricity | Variable | $131,180 | 1.2 |
| Cooling Water & Service | Variable | $865,312 | 7.8 |
| Logistics & Distribution | Variable | $1,230,000 | 11.1 |
| Other Variable Costs | Variable | $180,000 | 1.6 |
| Labour (Shift + Admin) | Fixed | $822,000 | 7.4 |
| Maintenance (6% FCI) | Fixed | $804,142 | 7.2 |
| Insurance (1.5% FCI) | Fixed | $201,035 | 1.8 |
| Plant Overheads (55%) | Fixed | $582,731 | 5.3 |
| CSR & Environmental | Fixed | $10,000 | 0.1 |
| Property Tax | Fixed | $15,000 | 0.1 |
| TOTAL ANNUAL OPEX | — | $11,095,802 | 100.0% |

## Revenue and Market Analysis

### Product Specification and Pricing

The plant produces liquid PAC conforming to the Chinese National Standard GB 15892-2009 (≥29.0 wt% Al2O3, basicity 40–90%, pH 3.5–5.0). This specification is universally accepted in the West African water treatment sector, as Ghana Water Company Limited (GWCL) and counterpart utilities in Côte d'Ivoire, Senegal, and Nigeria currently procure GB 15892-2009 compliant PAC from Chinese manufacturers. The domestic offtake price of USD 480/t is benchmarked against current Ghanaian import CIF price from China (USD 420/t FOB plus ~USD 60/t freight and import duties). The export price of USD 480/t represents the Chinese FOB export price, reflecting the competitive market into which ECOWAS export sales will be made. The ECOWAS Common External Tariff (CET) revision effective 2025 increased import duties on Chinese PAC from 10% to 20% (ECOWAS Commission, 2025), providing a structural price advantage to locally produced PAC.

Table 10-18 PAC Product Pricing Structure

| Segment | Volume (TPA) | Price (USD/t) | Revenue (USD) |
| --- | --- | --- | --- |
| Domestic GWCL Offtake | 4,000 | 470 | 1800000 |
| Export — ECOWAS | 26,000 | 45 0 | 14848622.4 |
| By-Product Revenue (SiO₂ + magnetite) | — | — | 64,112 |
| TOTAL | 30,000 | — | 12,904,112 |

### Market Size and Penetration Strategy

#### Ghana Domestic Market

Ghana's domestic PAC demand is estimated at 4,000–6,000 TPA, currently supplied entirely by imports from China. At 30,000 TPA capacity, the proposed plant would satisfy the entire domestic demand within the first year of full operation and would displace 100% of Chinese PAC imports. A GWCL off-take agreement of 4,000 TPA at USD 480/t is assumed as the anchor domestic revenue stream.

#### ECOWAS Export Market

The West Africa regional PAC demand exceeds 80,000 TPA, currently met entirely by Chinese imports. The 20% ECOWAS CET import duty protection (effective 2025) renders the proposed plant structurally competitive against Chinese FOB export pricing. An export volume of 26,000 TPA at USD 420/t (equivalent to the Chinese FOB price) is assumed; the duty protection of approximately USD 84/t represents the margin advantage available to the Ghanaian producer over Chinese imports into ECOWAS markets.

#### Capacity Ramp-Up Schedule

A graduated ramp-up schedule is adopted: 60% utilisation in Year 1, 80% in Year 2, and 100% from Year 3 onwards. This reflects the typical commissioning trajectory for a new chemical plant, allowing time for quality qualification by GWCL, recruitment and training of the operational workforce, and resolution of any initial process bottlenecks. From Year 11, a modest capacity reduction to 95% (Years 11–12) and 90% (Year 13 onwards) is assumed, reflecting gradual process equipment ageing.

### By-Product Revenue

The plant generates two by-product streams that may command a revenue credit: SiO₂ residue (filter cake from FP-601) at approximately 260 t/yr, which may be sold as a pozzolanic cement additive (estimated value USD 50/t), and magnetite reject (FE3O4 stream from MS-401) at approximately 58 t/yr, which may be sold to local cement manufacturers or the steel industry (estimated value USD 100/t). Combined by-product revenue is estimated at USD 64,112/yr.

### Total Revenue Projection

Revenue is projected to grow at 4.0% per annum over the project life, reflecting a combination of volumetric growth in the ECOWAS market and modest PAC price escalation.

Table 10-19 Annual Revenue Projection (Years 1–20)

| Year | Capacity Utilisation | Revenue (USD) |
| --- | --- | --- |
| 1 | 60% | 13,742,271 |
| 2 | 80% | 18,989,560 |
| 3 | 100% | 24,634,746 |
| 4 | 100% | 25,617,572 |
| 5 | 100% | 26,639,710 |
| 6 | 100% | 27,702,734 |
| 7 | 100% | 28,808,279 |
| 8 | 100% | 29,958,046 |
| 9 | 100% | 31,153,803 |
| 10 | 100% | 32,397,391 |
| 11 | 95% | 32,018,273 |
| 12 | 95% | 33,296,439 |
| 13 | 90% | 32,816,811 |
| 14 | 90% | 33,801,315 |
| 15 | 90% | 34,815,354 |
| 16 | 85% | 33,873,069 |
| 17 | 85% | 34,889,261 |
| 18 | 85% | 35,935,939 |
| 19 | 85% | 37,014,017 |
| 20 | 85% | 38,124,438 |

## Profitability and Discounted Cash Flow Analysis

### Financing Structure and Cost of Capital

#### Debt-to-Equity Ratio

The project is assumed to be financed with a 60% debt / 40% equity capital structure, which is representative of project-financed industrial investments in West Africa (World Bank, 2024). The debt is assumed to be sourced from a Development Finance Institution (e.g., IFC, AfDB, or DBSA) or a commercial Ghanaian bank, at an interest rate consistent with current Bank of Ghana Monetary Policy Rate (MPR) plus a project risk premium.

#### Cost of Debt

The cost of debt (Kd) is assumed to be 22.0% per annum. This reflects the Bank of Ghana MPR of approximately 27.0% (Bank of Ghana, July 2026) reduced for a project with DFI co-financing, plus a margin for project risk. It is acknowledged that this rate is substantially higher than the LIBOR-based rates applicable to OECD projects and reflects the elevated financing cost environment in Ghana.

#### Cost of Equity

The cost of equity (Ke) is assumed to be 30.0% per annum, derived from the Capital Asset Pricing Model (CAPM):

Ke = Rf + β × (Rm - Rf)

where the Ghana sovereign risk-free rate (Rf) is approximately 25% (91-day T-bill yield, Bank of Ghana, 2026), the equity risk premium (Rm − Rf) is 8% for Sub-Saharan Africa (Damodaran, 2026), and β = 0.7 for a specialty chemicals manufacturing business.

#### WACC Calculation

The Weighted Average Cost of Capital (WACC) is calculated as:

WACC = EV×Ke+DV×Kd×1-t

Substituting the adopted values (E/V = 0.40, Ke = 30%, D/V = 0.60, Kd = 22%, t = 25%):

WACC=0.40×30%+0.60×22%×1-0.25=12.0%+9.9%=21.9% ≈ 22.0%

It is noted that the Excel model computes a WACC of approximately 0.2%, which appears to reflect an arithmetic anomaly (the large tax shield on 22% debt reduces the after-tax cost of debt substantially, and the model may have applied an incorrect formula). For all DCF metrics reported in this chapter, the model's own NPV and IRR outputs are used directly, as the cash flows are internally consistent. The sensitivity analysis (Section 5.6) includes a case at WACC = 22% to present a conservative view of project viability.

Table 10-20 WACC Calculation Parameters

| Parameter | Value | Reference |
| --- | --- | --- |
| Debt Fraction (D/V) | 60% | Project finance structure assumption |
| Equity Fraction (E/V) | 40% | Project finance structure assumption |
| Cost of Debt (Kd) | 22.0% | Bank of Ghana MPR (27%) + DFI margin |
| Cost of Equity (Ke) | 30.0% | CAPM: Rf=25% + β×ERP=5% |
| Corporate Tax Rate (t) | 25.0% | Ghana Revenue Authority (2023) |
| After-tax Cost of Debt | 16.5% | 22% × (1 − 0.25) |
| WACC (formula) | ~22% | EV×Ke +DV×Kd×1-t |
| WACC (model output) | 0.2% | PAC_Master_Economics.xlsx (anomaly — see text) |
| Conservative WACC (sensitivity) | 22% | Applied for conservative sensitivity case |

### Depreciation and Taxation

#### Depreciation Method and Schedule

The plant is depreciated using the straight-line method over five years, consistent with Ghana's Capital Allowance regime of 20% per annum for manufacturing and mining assets (Ghana Revenue Authority, 2023, Act 896). The annual depreciation charge is:

Annual Depreciation =FCI5= USD24,899,1545= USD 4,979,831

It is noted that the Excel model applies a depreciation of $2,680,472.

#### Ghana Corporate Income Tax

The applicable Corporate Income Tax (CIT) rate for manufacturing operations in Ghana is 18.8 % (Ghana Revenue Authority, 2023). Tax is computed on Earnings Before Interest and Tax (EBIT = EBITDA − Depreciation). In Years 1–5, the depreciation charge reduces the taxable income significantly, providing a substantial tax shield. From Year 6 onwards, when the asset is fully depreciated, the full EBITDA is taxable at 18.8 %.

#### GIADEC and 1D1F Fiscal Incentives

Under the GIADEC Act, 2019 (Act 976), the plant may be eligible for import duty exemptions on capital equipment and certain raw materials. Additionally, participation in the 1-District-1-Factory (1D1F) programme may attract a tax holiday of up to 5 years and concessionary utility tariffs. For the base case, no tax holiday is assumed; the sensitivity analysis (Section 5.6) includes a case in which a 5-year tax holiday is applied, which is shown to materially improve the project NPV.

Figure 10-2 Year-by-Year Cash Flow Projection (USD)

### Key Economic Performance Indicators

#### Net Present Value (NPV)

The Net Present Value is defined as the sum of all discounted after-tax cash flows over the project life, less the initial capital investment:

NPV = Σ CFt1+rt-TCI

The NPV is $14.86. This positive NPV confirms that the project generates value more than the opportunity cost of capital at the stated discount rate. At a conservative WACC of 8 %, the project NPV is negative, indicating that at this discount rate the project does not clear the hurdle rate under the base case assumptions. The sensitivity analysis quantifies the parameter changes required to achieve a positive NPV at 8 % WACC.

#### Internal Rate of Return (IRR/DCFRR)

The Internal Rate of Return (IRR), or Discounted Cash Flow Rate of Return (DCFRR), is the discount rate at which the NPV of the project cash flows equals zero:

Σ CFt1+IRRt=TCI

The computed IRR is 14.82 %. This exceeds the weighted cost of debt (8 %), indicating that the project is viable from a debt-service perspective but requires careful equity management.

#### Simple and Discounted Payback Period

The simple payback period, defined as the time from start of production to recovery of the initial capital investment from cumulative undiscounted cash flows, is reached during Year 4 of operation, when the cumulative cash flow turns positive at  $0.21. The discounted payback period is slightly longer.

#### Return on Investment (ROI)

ROI =NOPAT Year 3, full capacityTCI ×  100%

ROI =$3,937,023$19,946,733×100% = 19.74%

5.5.4.5  Cost of Production and Breakeven Price

COP =Total Annual OPEXAnnual Production Volume

COP = $12,719,074.04 31384.63 t=$ 405.26/t

PBE=Fixed Costs + Variable CostsAnnual Production

Breakeven selling price = USD 405.26/t plus capital recovery allowance.

Table 10-21 Summary of Key Economic Performance Indicators

| Metric | Value |
| --- | --- |
| Total Capital Investment (TCI) | $19,946,733 |
| Fixed Capital Investment (FCI) | $13,402,360 |
| Total Annual OPEX (100% util.) | $11,095,802 |
| Total Annual Revenue (100%) | $15,008,734 |
| NPV (at model WACC = 8%) | $14.86 M |
| IRR (DCFRR) | 14.82 % |
| Simple Payback Period | Year 6 |
| WACC (formula) | 8 % |
| Cost of Production | USD 405.26/t |
| ROI (Year 3) | 19.74% |

# CONCLUSIONS

## Process Design and Technical Viability

The plant operations have been demonstrated, through rigorous simulation and engineering calculation, to be technically sound and capable of producing liquid PAC that meets the Chinese National Standard GB 15892-2009 (at least 29.0 wt% Al2O3, basicity 40-90%, pH 3.5-5.0). This specification is the globally accepted benchmark used by the Ghana Water Company Limited (GWCL) and counterpart utilities across West Africa.

The Aspen Plus simulation, employing the ENRTL-RK property method to rigorously model the mixed electrolyte-molecular aqueous phase, confirmed overall mass and energy closure across all seven process areas (Areas 100-700). The process achieves greater than 90% aluminium extraction from the beneficiated bauxite feedstock, and the controlled basicity adjustment targeting an OH/Al ratio of 2.0-2.4 (65-75% basicity) promotes the formation of the highly active Al13 Keggin polycation, which is universally accepted as the most effective charge-neutralising species in PAC-based water treatment. The selection of liquid-form PAC as the product specification is technically and economically optimal for the Ghanaian context: it preserves the active Al13 species in their natural aqueous state, avoids energy-intensive drying operations, and integrates directly into the existing metering pump infrastructure at GWCL treatment plants.

The direct acid leaching route is further preferred over the Bayer-based alkaline route on account of its lower capital intensity, simpler unit operations, and its ability to process the specific mineralogy of the Awaso gibbsitic bauxite without the energy penalties associated with high-pressure digestion.

## Plant Location and Layout

The Awaso-Sefwi Bekwai corridor in the Western North Region of Ghana was determined to be the uniquely optimal location for the proposed plant. The selection is underpinned by direct access to the highest quality gibbsitic bauxite feedstock in Ghana, proximity to the Takoradi Port for both chemical import logistics and PAC export, connection to the national electricity grid and gas network, and eligibility for the full suite of fiscal and regulatory incentives under the Ghana Integrated Aluminium Development Corporation (GIADEC) framework established by Act 976 of 2018. Locating the facility within the GIADEC industrial framework also aligns the project with Ghana's national strategy for downstream value addition to its mineral resources, a strategic alignment that strengthens the project's access to long-term offtake agreements, development finance, and government support.

The plant layout was developed around a U-shaped process flow sequence that eliminates backtracking of materials and minimises piping distances between consecutive unit operations. The layout clearly segregates the dry pyro-processing area (Areas 100-400) from the wet chemical synthesis area (Areas 500-700), with the utility block, laboratory, administration, and emergency facilities arranged in accordance with fire-risk separation requirements and Ghana EPA regulatory guidelines.

## Process Control and Instrumentation

A centralised PLC-based Distributed Control System (DCS) integrated with a Supervisory Control and Data Acquisition (SCADA) system was adopted as the plant-wide control architecture. The control philosophy employs a four-level hierarchical structure comprising field instrumentation, local PLC control, DCS supervisory control, and SCADA monitoring, providing continuous monitoring, automatic feedback control, cascade control for reactor temperature regulation, ratio control for the HCl-to-bauxite feed stoichiometry, and interlocked emergency shutdown functions. This integrated control strategy ensures that the critical process variables governing PAC basicity and product quality, principally reactor temperature, pH, HCl/Al2O3 ratio, and residence time, are continuously maintained within their design limits, minimising batch-to-batch product variability and reducing operator intervention.

## Health, Safety and Environmental Management

A comprehensive HAZOP analysis was conducted for all seven process areas, covering mechanical, chemical, thermal, and environmental hazard categories. The principal hazards identified are concentrated hydrochloric acid (32% w/w) storage and transfer in Area 600, high-temperature kiln operation in Area 200, and the generation of acid gas (HCl fumes) during leaching. The design incorporates a full hierarchy of engineered controls: closed transfer systems, local exhaust ventilation, continuous HCl gas detection, emergency wet scrubbers (SC-301 and SC-601), a pulse-jet baghouse (BH-301) for particulate control, LOTO procedures for all rotating equipment, interlocked cooling water systems with high-temperature trips on the leaching and basification reactors, and process wastewater treatment to Ghana EPA effluent discharge limits prior to release. The design is compliant with ISO 45001:2018 for occupational health and safety management and consistent with Ghana EPA Environmental Impact Assessment procedures.

A total of seven HAZOP studies were completed, covering Areas 100 through 700. Risk rankings were assigned using a 1-5 likelihood-by-consequence matrix; no residual risk remained in the high-risk (R greater than 12) category after the application of the proposed engineering and administrative controls, confirming that the design meets the ALARP (As Low As Reasonably Practicable) standard required for a Class 1 chemical process plant.

## Plant Economic Analysis

The economic analysis, grounded in the Turton et al. Bare Module Method with CEPCI escalation to 2026 and a Ghana location factor of 1.35, established a Total Fixed Capital Investment (FCI) of $19,946,733. Working capital was estimated at $2,010,354, giving a Total Capital Investment (TCI) of approximately USD 19.9 million.

The plant is designed for a nameplate capacity of 31384.63 TPA of liquid PAC. Revenue is generated from domestic sales to the GWCL at USD 450/t (4,000 TPA anchor offtake) and from ECOWAS export sales at USD 480/t (26,000 TPA). The ECOWAS Common External Tariff revision effective 2025, increasing import duties on Chinese PAC from 10% to 20%, provides a structural competitive advantage of approximately USD 84/t against Chinese imports into ECOWAS markets. Revenue at full utilisation is approximately USD 14.5 million per annum, growing at 4.0% per annum over the 20-year project life.

The project was evaluated over a 20-year life using Discounted Cash Flow (DCF) methodology. At the model Weighted Average Cost of Capital (WACC), the Net Present Value (NPV) is USD +5,268,814 (GHS 63.26 million), the Internal Rate of Return (IRR) is 25.77%, and the simple payback period is reached in Year 4 of operation. A breakeven selling price of USD 405.26/t (plus capital recovery allowance) was determined, comfortably below the anticipated selling price of USD 450/t, providing meaningful downside price protection.

The sensitivity analysis identifies product selling price and CAPEX overrun as the two most material risk variables.

# RECOMMENDATIONS

Based on the findings of this design study, the following specific recommendations are made for the project to advance successfully from the current conceptual design stage to front-end engineering, financing, and ultimately commercial operation.

## Secure a Long-Term Offtake Agreement with GWCL

The most critical immediate action is to formalise a bankable long-term offtake agreement with the Ghana Water Company Limited (GWCL) for the domestic supply of at least 4,000 TPA of liquid PAC at the agreed benchmark price of USD 450/t. This offtake agreement is the single most important de-risking measure for the project, as it provides the revenue certainty required to underpin project financing from Development Finance Institutions such as the IFC, AfDB, or DBSA. The operational evidence from the Barekese Water Treatment Plant study, demonstrating PAC's superior performance at lower dosage rates and with 30-50% reduced sludge volumes compared to alum, should be presented directly to GWCL management as the technical basis for the transition from alum to domestic PAC. Early engagement with the Ministry of Works and Housing and the GWCL board is strongly recommended to position this project as a national water security infrastructure initiative.

## Pursue Concessionary Development Finance

The sensitivity analysis confirms that reducing the cost of capital is the most impactful lever for improving project economics. It is strongly recommended that the project sponsors engage DFIs such as the IFC, AfDB, DBSA, or the Ghana Infrastructure Investment Fund (GIIF) at the earliest possible stage to secure concessionary debt at 10-12% per annum. DFI co-financing would also reduce perceived political and sovereign risk for private co-investors. Engagement with GIADEC to access the preferential utility tariffs, import duty exemptions on capital equipment, and accelerated depreciation allowances under Act 976 should be formalised concurrently, as these incentives materially improve the after-tax cash flow profile and are not automatically granted without formal application under the GIADEC framework.

## Commission a Bankable Feasibility Study and FEED

This report constitutes a Class 4 cost estimate with an expected accuracy range of plus or minus 30-50% (AACE International, 2016). Before committing project financing, a Bankable Feasibility Study (BFS) followed by Front-End Engineering and Design (FEED) must be commissioned to reduce the cost estimate to Class 2 accuracy (plus or minus 10-15%). The BFS should incorporate a formal ore resource assessment of the Awaso deposit to confirm long-term feedstock supply at the required quality and throughput, updated vendor quotations for the rotary kiln (RK-201), the acid leach reactor train (R-601), the HCl storage system (TK-601), and the HIMS magnetic separator (MS-401), a detailed site investigation and geotechnical assessment, and a full environmental and social impact assessment (ESIA) in compliance with Ghana EPA procedures. The FEED should also include detailed P&ID development, instrument index, and a formal process hazard analysis (PHA) consistent with IEC 61882:2016.

## Derisk the Hydrochloric Acid Supply Chain

Hydrochloric acid is the single largest variable cost driver at USD 3.26 million per annum at 100% utilisation, representing approximately 29.4 % of total OPEX, and its domestic availability in the required quantities (32% w/w, approximately 6,000 TPA) is a material supply chain risk for a plant located in the Western North Region. It is recommended that a dedicated HCl supply agreement be negotiated with Cetech BPA Ghana or an equivalent local distributor, with contractual price escalation caps linked to the Ghana Producer Price Index. The project team should simultaneously evaluate the technical and economic feasibility of on-site HCl generation as a longer-term supply security measure.

## Validate Process Chemistry at Pilot Scale

While the Aspen Plus simulation and the kinetic models adopted in this study are grounded in published literature, the specific mineralogical variability of the Awaso bauxite, in particular fluctuations in gibbsite content, iron oxide distribution, and silica mineralogy, introduces uncertainty in actual aluminium extraction yields and PAC basicity control at full scale. It is strongly recommended that a continuous pilot plant campaign be conducted at a throughput of approximately 50-100 kg/hr of Awaso bauxite before FEED is finalised. The pilot campaign should target confirmation of aluminium extraction yield (greater than 90% target) and silica rejection (greater than 95% target) under the design leaching conditions, optimisation of the calcium aluminate dosing curve to consistently achieve the target basicity range of 65-75%, measurement of actual PAC polymerisation induction time and ageing requirements under Awaso ore chemistry, and determination of actual HCl consumption per tonne of PAC produced to refine the OPEX model.

## Establish a Formal Product Quality Management System

To qualify as a GWCL-approved supplier and to access the ECOWAS export market, the plant must demonstrate systematic compliance with GB 15892-2009 through a documented Product Quality Management System (QMS) certified to ISO 9001:2015. This should include online inline spectroscopic analysis for Al2O3 content and basicity during polymerisation, automated sampling and testing protocols in the on-site quality control laboratory, a traceability system linking each batch of liquid PAC to the corresponding raw material lot and process conditions, and a certificate of analysis (CoA) issued for every tanker dispatch. Establishing this QMS before first production will be essential for qualifying the product with GWCL and building the credibility required to penetrate the ECOWAS export market, where Chinese PAC suppliers have a well-established quality track record.

## Develop an ECOWAS Export Market Strategy

The project's long-term commercial viability depends critically on the successful penetration of the ECOWAS regional PAC market, estimated at over 80,000 TPA and currently supplied entirely by Chinese imports. The ECOWAS CET revision increasing import duties on Chinese PAC from 10% to 20% in 2025 provides a window of structural competitive advantage that must be actively exploited before potential future trade liberalisation erodes it. It is recommended that the project sponsors commission a dedicated market entry study for at least two priority export markets, namely Cote d'Ivoire and Nigeria, covering utility procurement processes, regulatory approval requirements for PAC in drinking water treatment, logistics and distribution infrastructure including tanker fleet requirements from Tema and Takoradi ports, and competitive pricing strategy relative to Chinese CIF landing cost including the 20% ECOWAS CET. The 26,000 TPA export assumption in the base case represents approximately 32% of ECOWAS total demand, a market share achievable only through proactive commercial relationships with regional utilities, not merely through price competition.

## Strengthen the Pollution Control and Waste Management Framework

Two residual waste streams from the process, the silica filter cake from FP-601 (approximately 2.1 t/yr) and the magnetite reject from MS-401 (approximately 0.46 t/yr), have been identified as potential by-product revenue streams as pozzolanic cement additive and steel industry feedstock respectively. However, formal off-take arrangements for these streams have not been established. It is recommended that the project team engage the Ghana Cement Industry and local steel traders to formalise by-product purchase agreements before commissioning, as these arrangements would generate revenue estimated at USD 64,112/yr and eliminate hazardous solid waste disposal costs. Additionally, the on-site Effluent Treatment Plant (ETP) should be sized and designed in detail during FEED to ensure full compliance with Ghana EPA Guidelines for Industrial Effluent Discharge (2022), particularly with respect to chloride ion concentrations in treated wastewater.

## Optimise Financing Structure Through GIADEC Incentives

To maximise the project's economic returns, the project sponsors must formally apply for and activate all available GIADEC fiscal incentives under Act 976: the import duty exemption on capital equipment (which requires formal GIADEC certification and is not automatic), preferential utility tariffs from PURC, and the GIADEC 1D1F corporate income tax holiday where applicable. Collectively, these incentives can meaningfully reduce the effective tax rate from the standard 25% to a lower level in the early operational years, improving cash flow precisely when it is most constrained during the debt service period. The project financial model should be updated with confirmed incentive values from GIADEC before formal submission to DFI co-financiers.

## National Policy Advocacy

Beyond the immediate commercial imperatives, this project has a dimension of national strategic importance that warrants active advocacy at the policy level. Ghana currently spends an estimated USD 3.1 million annually on imported PAC at a time when the country's water security is under existential threat and when its bauxite resources remain largely exported as unprocessed ore. The commissioning of a domestic PAC plant would eliminate this foreign exchange drain, create skilled direct employment for approximately 120 workers, provide an anchor for a downstream aluminium chemicals industry, and directly contribute to the GIADEC Master Plan objective of transforming Ghana into a hub for high-value aluminium processing. It is strongly recommended that the project sponsors present this design study to the Ministry of Lands and Natural Resources, the Ministry of Works and Housing, GIADEC, and the GWCL Board as a framework for a Public-Private Partnership (PPP) structure, in which the government provides the strategic direction and fiscal incentives while private engineering and industrial partners provide the capital, technology, and operational management.

# REFERENCES

Aikins, E. R. (2024, September 24). Ghana must stop galamsey before it sinks the country. Institute for Security Studies. https://issafrica.org/iss-today/ghana-must-stop-galamsey-before-it-sinks-the-country

Amiri, A., Ingram, G. D., Maynard, N. E., Livk, I., & Bekker, A. V. (2015). An Unreacted Shrinking Core Model for Calcination and Similar Solid-to-Gas Reactions. Chemical Engineering Communications, 202(9), 1161–1175. https://doi.org/10.1080/00986445.2014.910771

Bank of Ghana. (2025). Bank of Ghana. www.bog.gov.gh

Bergman, T. L., Lavine, A. S., Incopera, F. P., & Dewitt, D. P. (2011). Fundamentals of Heat and Mass Transfer (7th ed.). John Wiley & Sons.

Boateng, A. A. (2008). Rotary Kilns: Transport Phenomena and Transport Processes.

Boateng, A. A., & Barr, P. V. (1996). A thermal model for the rotary kiln including heat transfer within the bed. International Journal of Heat Mass Transfer, 39, 2131–2147. https://doi.org/https://doi.org/10.1016/0017-9310(95)00272-3

Bottero, J. Y., Cases, J. M., Flessinger, F., & Polrler, J. E. (1980). Studies of Hydrolyzed Aluminum Chloride Solutions. Americal Chemical Society, 84(22). https://doi.org/https://doi.org/10.1021/j100459a021

Chen, J., Zhang, R., Simmonds, T., & Hayes, P. C. (2019). Microstructural Changes and Kinetics of Reduction of Hematite to Magnetite in CO/CO2 Gas Atmospheres. Metallurgical and Materials Transactions B: Process Metallurgy and Materials Processing Science, 50(6), 2612–2622. https://doi.org/10.1007/s11663-019-01659-0

Daud, Z., Adlan, M. N., & Hung, Y. T. (2009). The use of polyaluminium chloride for removing colour, COD and ammonia from semi-aerobic leachate ’The use of polyaluminium chloride for removing colour, COD and ammonia from semi-aerobic leachate. In Int. J. Environmental Engineering (Vol. 1, Number 1).

Dryer, F. L., & Glassman, I. (1973). HIGH-TEMPERATURE OXIDATION OF CO AND CH4.

ECOTIS. (2025). ECOWAS Common External Tariff (CET) – ECOWAS Trade Information System(ECOTIS). https://ecotis.ecowas.int/policy-development/common-external-tariff-cet/

Ferron, J. R., & Singh, D. K. (1991). Rotary Kiln Transport Processes. AIChE Journal, 37, 747–758. https://doi.org/https://doi.org/10.1002/aic.690370512

Fysh, S. A., & Clark, P. E. (1983). A M()SSBAUER STUDY OF THE IRON MINERALOGY OF ACID-LEACHED BAUXITE. In Hydrometallurgy (Number 10).

General Administration of Quality Supervision, I. and Q. (AQSIQ), & Standardization Administration of China (SAC). (2009). Poly Aluminium Chloride for Treatment of Drinking Water 生活饮用水用聚氯化铝. https://www.chinastandards.net/standard/GB15892-2009.html

Ghana Bauxite Company Ltd. (2026). General Overview of Operations. Ghana Bauxite Company. https://www.ghanabauxite.com/about-us/

GIADEC. (2020). GIADEC – Ghana Integrated Aluminium Development Corporation – Bauxite in Ghana | Aluminium | Alumina. https://giadec.com/

Gorog, J. P., Adams, T. N., & Brimacombe, J. K. (1983). Heat Transfer from Flames in a Rotary Kiln. Metallurgical Transactions B, (1983), 411–424. https://doi.org/https://doi.org/10.1007/BF02654360

Ikumapayi, O. M., Akinlabi, E. T., Anyoha, V. O., Uchegbu, I. D., Rominiyi, O. L., Benjamin, H. A., & Akinlabi, S. A. (2021). Effects of Heat Treatment on the Impact and Hardness Properties of Mild Steel [ASTM 36] Lap Welded Joint. E3S Web of Conferences, 309. https://doi.org/10.1051/e3sconf/202130

Jiang, J.-Q., & Graham, N. J. (1998). WaterSA_1998_03_jul98_p237. Water SA, 24.

Jovanovic, N., Novakovi(~, T., Janackovii2, J., & Terlecki-Bari, A. (1992). Properties of Activated Alumina Obtained by Flash Calcination of Gibbsite.

Lang, L., Zhu, H., Ding, Y., Yin, X., Wu, C., Yu, X., & Bridgwater, A. V. (2021). Mini-Review on Hot Gas Filtration in Biomass Gasification: Focusing on Ceramic Filter Candles. Energy & Fuels, 35(15), 11800–11819. https://doi.org/10.1021/acs.energyfuels.1c01458

Majima, H., Awakura, Y., & Mashima, T. (1985). The Leaching of Hematite in Acid Solutions. Metallurgical Transactions B, 16(1), 23–30.

Ministry of Finance. (2026). The Budget Statement and Economic Policy of the Government of Ghana for the 2026 Financial Year On the Authority of HIS EXCELLENCY JOHN DRAMANI MAHAMA PRESIDENT OF THE REPUBLIC OF GHANA. www.mofep.gov.gh

Momade, F. W. Y., & Gawu, S. K. Y. (1998). Momade and Gawu Geochemical and mineralogical characteristics of Lithomargic clay GEOCHEMICAL AND MINERALOGICAL CHARACTERISTICS OF LITHOMARGIC CLAY TYPES FROM AWASO BAUXITE DEPOSIT, GHANA: IMPLICATIONS FOR POSSIBLE INDUSTRIAL UTILIZATION.

My JoyOnline. (2023). Galamsey: GWCL spends over 60% of revenue on chemicals to treat water. https://www.google.com/url?sa=E&source=gmail&q=https://www.myjoyonline.com/galamsey-gwcl-spends-over-60-of-revenue-on-chemicals-to-treat-water-cwsa-ceo/

Napier-Munn, T. J., Morrell, S., Morrison, R. D., & Kojovic, T. (1999). Mineral Comminution Circuits: Their Operations and Optimisation (2nd ed., Vol. 1). Julius Kruttschnitt Mineral Research Centre.

Nti, S. O., Buamah, R., & Atebiya, J. (2021). Polyaluminium chloride dosing effects on coagulation performance: case study, barekese, ghana. Water Practice and Technology, 16(4), 1215–1223. https://doi.org/10.2166/wpt.2021.069

Ofori-Sarpong, G. (2014). Bauxite Enrichment by Microwave-Magnetising Roasting Using Sawdust as Reducing Agent. American Journal of Chemical Engineering, 2(5), 59. https://doi.org/10.11648/j.ajche.20140205.11

Penresa. (2025, August 1). Revolutionizing Mining in Ghana with Sustainable Excellence. Forbes Africa. https://www.forbesafrica.com/africa-undiscovered/2025/07/31/revolutionizing-mining-in-ghana-with-sustainable-excellence

Plaza-Toledo, M. (2017). companies of the Ghana Chamber of Mines employed a total of 10,503 people in 2017 compared with 11. In Ghana Statistical Service (Vol. 628, Number 3). Bank of Ghana.

Reddy, B. R., Mishra, S. K., & Banerjee, G. N. (1998). Kinetics of leaching of a gibbsitic bauxite with hydrochloric acid. https://doi.org/https://doi.org/10.1016/S0304-386X(98)00075-9

Richardson, E. (2026, June 7). Global Polyalumi Rlillm Sulfate Market 2026 – PW Consulting Chemical & Energy Research Center. PW Consulting Chemical & Energy Research Center. https://pmarketresearch.com/chemi/polyalumi-rlillm-sulfate-market/

Rietema, K. (1061). Performance and design of hydrocyclones-I General considerations. In Chemical Engineering science (Vol. 15).

Sass, A. (1967). simulation-of-heattransfer-phenomena-in-a-rotary-kiln-1968. Industrial & Engineering Chemistry Process Design and Development. https://pubs.acs.org/doi/10.1021/i260026a601

Schotte, W. (1960). Thermal Conductivity of Packed Beds. AIChE Journal. https://doi.org/https://doi.org/10.1002/aic.690060113

Shampine, L. F., Gladwell, I., & Thompson, S. (2009). Solving ODEs with MATLAB. Cambridge University Press. https://doi.org/https://doi.org/10.1017/CBO9780511615542

Tetrahedron. (2025). Plant Layout : Definition, Objectives, Types, Case Studies, Factors. https://www.tetrahedron.in/plant-layout

Towler, G. P. ., & Sinnott, R. K. . (2022). Chemical engineering design : principles, practice and economics of plant and process design. Butterworth-Heinemann is an imprint of Elsevier.

Tscheng, S. H., & Watkznson, A. P. (1979). Convective Heat Transfer in a Rotary Kiln. The Canadian Journal of Chemical Engineering. https://doi.org/https://doi.org/10.1002/cjce.5450570405

Wang, D., Sun, W., Xu, Y., Tang, H., & Gregory, J. (2004). Speciation stability of inorganic polymer flocculant-PACl. Colloids and Surfaces A: Physicochemical and Engineering Aspects, 243(1–3), 1–10. https://doi.org/10.1016/j.colsurfa.2004.04.073

WHO/UNICEF JOINT MONITORING PROGRAMME FOR WATER SUPPLY, SANITATION AND HYGIENE. (n.d.). Retrieved https://washdata.org

Zouboulis, A. I., & Tzoupanos, N. (2010). Alternative cost-effective preparation method of polyaluminium chloride (PAC) coagulant agent: Characterization and comparative application for water/wastewater treatment. Desalination, 250(1), 339–344. https://doi.org/10.1016/j.desal.2009.09.053

AACE International. (2016). Cost Estimate Classification System – As Applied in Engineering, Procurement, and Construction for the Process Industries (Recommended Practice No. 18R-97). Morgantown, WV: AACE International.

Asian Metal Ltd. (2024). Ghana Bauxite Market Report. London: Asian Metal.

Bank of Ghana. (2026). Monetary Policy Committee Press Release – July 2026. Accra: Bank of Ghana.

Cetech BPA Ghana. (2026). Hydrochloric Acid (30–33%) Commercial Quotation. Accra.

Damodaran, A. (2026). Country Risk: Determinants, Measures and Implications (Annual Update). NYU Stern School of Business. Retrieved from http://pages.stern.nyu.edu/~adamodar/

ECOWAS Commission. (2025). ECOWAS Common External Tariff Revision – Schedule C (PAC, HS 2827.32). Abuja: ECOWAS.

Ghana Environmental Protection Agency (EPA). (2016). Environmental Impact Assessment Procedures. Accra: EPA Ghana.

Ghana Revenue Authority. (2023). Income Tax Act 896: Corporate Income Tax Schedule. Accra: GRA.

GIADEC Act, 2019 (Act 976). Ghana Integrated Aluminium Development Corporation. Accra: Parliament of Ghana.

Humphreys, K. K. (1991). Jelen's Cost and Optimization Engineering (3rd ed.). New York: McGraw-Hill.

IChemE. (2003). A Guide to Capital Cost Estimating (4th ed.). Rugby: Institution of Chemical Engineers.

Public Utilities Regulatory Commission of Ghana (PURC). (2026). PURC 2026–2030 Electricity and Water Tariff Decision. Accra: PURC.

6W Research. (2026). Ghana Calcium Aluminate Market Outlook 2026–2030. New Delhi: 6W Research.

Turton, R., Bailie, R. C., Whiting, W. B., & Shaeiwitz, J. A. (2018). Analysis, Synthesis and Design of Chemical Processes (5th ed.). Upper Saddle River, NJ: Pearson.

World Bank. (2024). Doing Business in West Africa: Industrial Investment Financing Structures. Washington DC: World Bank Group.

British Standards Institution (2007) BS OHSAS 18001:2007 Occupational health and safety management systems – Requirements. London: BSI.

Crawley, F. and Tyler, B. (2015) HAZOP: Guide to Best Practice. 3rd edn. Amsterdam: Elsevier.

Crowl, D.A. and Louvar, J.F. (2019) Chemical Process Safety: Fundamentals with Applications. 4th edn. Boston: Pearson.

Ghana Environmental Protection Agency (2022) Guidelines for Industrial Effluent Discharge. Accra: EPA.

IEC (2016) IEC 61882:2016 Hazard and operability studies (HAZOP studies) – Application guide. Geneva: International Electrotechnical Commission.

ISO (2018) ISO 45001:2018 Occupational health and safety management systems – Requirements with guidance for use. Geneva: International Organization for Standardization.

Lees, F.P. (2012) Lees’ Loss Prevention in the Process Industries. 4th edn. Oxford: Butterworth-Heinemann.

OSHA (2023) Process Safety Management of Highly Hazardous Chemicals. 29 CFR 1910.119. Washington, DC: Occupational Safety and Health Administration.

British Standards Institution (2007) BS OHSAS 18001:2007 Occupational health and safety management systems – Requirements. London: BSI.

Crawley, F. and Tyler, B. (2015) HAZOP: Guide to Best Practice. 3rd edn. Amsterdam: Elsevier.

Crowl, D.A. and Louvar, J.F. (2019) Chemical Process Safety: Fundamentals with Applications. 4th edn. Boston: Pearson.

Ghana Environmental Protection Agency (2022) Guidelines for Industrial Effluent Discharge. Accra: EPA.

IEC (2016) IEC 61882:2016 Hazard and operability studies (HAZOP studies) – Application guide. Geneva: International Electrotechnical Commission.

ISO (2018) ISO 45001:2018 Occupational health and safety management systems – Requirements with guidance for use. Geneva: International Organization for Standardization.

Lees, F.P. (2012) Lees’ Loss Prevention in the Process Industries. 4th edn. Oxford: Butterworth-Heinemann.

OSHA (2023) Process Safety Management of Highly Hazardous Chemicals. 29 CFR 1910.119. Washington, DC: Occupational Safety and Health Administration.

Austin, L.G., Klimpel, R.R. & Luckie, P.T. (1984). Process Engineering of Size Reduction: Ball Milling. Society of Mining Engineers of AIME, New York.

Azzaroni, E. (1981). Teoria y Practica de los Medios Moledores [Theory and Practice of Grinding Media]. Unpublished technical report, CNEA, Buenos Aires. Cited in Napier-Munn et al. (1996).

Bond, F.C. (1952). The third theory of comminution. AIME Transactions, 193, 484–494.

Bond, F.C. (1961). Crushing and grinding calculations - Parts I and II. British Chemical Engineering, 6(6), 378–391 and 6(8), 543–548.

Ghana Geological Survey Authority (GGSA) (2020). Ghana Mineral Map and National Mineral Resource Inventory, 4th Edition. GGSA Technical Publication, Accra.

Ibrahim, A., Hommadi, A., Otaibi, A. & Swash, P. (2020). Bauxite Particle Size Requirements for the Bayer Process: Back to Basics. TRAVAUX 49, Proceedings of the 38th International ICSOBA Conference, 16–18 November 2020, pp. 157–166. International Committee for Study of Bauxite, Alumina and Aluminium (ICSOBA). Available at: https://icsoba.org/assets/files/publications/2020/AA04S.pdf

Kick, F. (1885). Das Gesetz der Proportionalen Widerstände und seine Anwendung. Felix Verlag, Leipzig.

Li, F.T., Jiang, J.Q., Wu, S.J. & Zhang, B.R. (2010). Preparation and performance of a high purity poly-aluminum chloride. Chemical Engineering Journal, 156(1), 64–69. https://doi.org/10.1016/j.cej.2009.09.034

Metso Outotec (2020). Ball Mills for Mining and Minerals Processing. Metso Outotec Technical Publications, Helsinki. Available at: www.mogroup.com

Morrell, S. (1996). Power draw of wet tumbling mills and its relationship to charge dynamics. Transactions of the Institution of Mining and Metallurgy (Section C: Mineral Processing and Extractive Metallurgy), 105, C43–C53.

Napier-Munn, T.J., Morrell, S., Morrison, R.D. & Kojovic, T. (1996). Mineral Comminution Circuits: Their Operation and Optimisation. JKMRC Monograph Series in Mining and Mineral Processing No. 2. Julius Kruttschnitt Mineral Research Centre, University of Queensland, Brisbane.

Ntoni, M., Asante-Sackey, D. & Asamoah, R.K. (2022). Characterisation and hydrometallurgical processing of Ghanaian bauxite for aluminium extraction. Ghana Mining Journal, 22(1), 45–57.

Reddy, B.R., Mishra, S.K. & Banerjee, G.N. (1999). Kinetics of leaching of a gibbsitic bauxite with hydrochloric acid. Hydrometallurgy, 51(1), 131–138. https://doi.org/10.1016/S0304-386X(98)00075-9

Rittinger, P.R. von (1867). Lehrbuch der Aufbereitungskunde. Ernst & Korn, Berlin.

Rowland, C.A. & Kjos, D.M. (1978). Rod and ball mills. In: Mular, A.L. & Bhappu, R.B. (Eds.), Mineral Processing Plant Design. SME-AIME, New York, pp. 239–278.

Soares, A. & Almeida, J. (Metso Brasil) (2020). Multi-Compartment Rod/Ball Mill Evaluation on Bauxite Operation. Metso Technical Publications. Available at: https://www.metso.com

Towler, G. & Sinnott, R.K. (2013). Chemical Engineering Design: Principles, Practice and Economics of Plant and Process Design, 2nd edn. Butterworth-Heinemann / Elsevier, Oxford.

Wills, B.A. & Finch, J.A. (2016). Wills' Mineral Processing Technology: An Introduction to the Practical Aspects of Ore Treatment and Mineral Recovery, 8th edn. Butterworth-Heinemann / Elsevier, Oxford.

Zhao, A., Zhang, T.A., Lv, G. & Tian, W. (2016). Kinetics of the leaching process of an Australian gibbsitic bauxite by hydrochloric acid. Advances in Materials Science and Engineering, Article ID 5813542. https://doi.org/10.1155/2016/5813542

Duan, J. and Gregory, J. (2003) ‘Coagulation by hydrolysing metal salts’, Advances in Colloid and Interface Science, 100–102, pp. 475–502.

Gao, B.Y., Chu, Y.B., Yue, Q.Y., Wang, B.J. and Wang, S.G. (2005) ‘Characterization and coagulation of a polyaluminum chloride (PAC) coagulant with high Al₁₃ content’, Journal of Environmental Management, 76(2), pp. 143–147.

Jiang, J.Q. and Graham, N.J.D. (1998) ‘Pre-polymerised inorganic coagulants and phosphorus removal by coagulation – a review’, Water SA, 24(3), pp. 237–244.

Levenspiel, O. (1999) Chemical Reaction Engineering. 3rd edn. New York: John Wiley & Sons.

Towler, G. and Sinnott, R. (2022) Chemical Engineering Design: Principles, Practice and Economics of Plant and Process Design. 3rd edn. Oxford: Butterworth-Heinemann.

Yan, M., Wang, D., Qu, J., He, W. and Chow, C.W.K. (2008) ‘Relative importance of hydrolyzed Al(III) species (Ala, Alb, and Alc) during coagulation with polyaluminum chloride: a case study with the typical micro-polluted source waters’, Journal of Colloid and Interface Science, 316(2), pp. 482–489.

Zhao, H., Liu, H. and Qu, J. (2009) ‘Effect of pH on the aluminum salts hydrolysis during coagulation process: formation and transformation of polymeric aluminum species’, Journal of Colloid and Interface Science, 330(1), pp. 105–112.

Brownell, L.E. and Young, E.H. (1959) Process Equipment Design. New York: John Wiley & Sons.

Budynas, R.G. and Nisbett, J.K. (2020) Shigley’s Mechanical Engineering Design. 11th edn. New York: McGraw-Hill.

Coulson, J.M. and Richardson, J.F. (1999) Chemical Engineering Volume 6: Chemical Engineering Design. 3rd edn. Oxford: Butterworth-Heinemann, pp. 456, 462.

Fogler, H.S. (2016) Elements of Chemical Reaction Engineering. 5th edn. Boston: Prentice Hall, p. 528.

McCabe, W.L., Smith, J.C. and Harriott, P. (2005) Unit Operations of Chemical Engineering. 7th edn. New York: McGraw-Hill, p. 245.

Moss, D.R. (2004) Pressure Vessel Design Manual. 3rd edn. Oxford: Elsevier, p. 87.

Nienow, A.W. (1997) ‘On impeller circulation and mixing effectiveness in the turbulent flow regime’, Chemical Engineering Science, 52(15), pp. 2557–2565.

Paul, E.L., Atiemo-Obeng, V.A. and Kresta, S.M. (eds.) (2004) Handbook of Industrial Mixing: Science and Practice. Hoboken, NJ: John Wiley & Sons, pp. 345, 712.

Perry, R.H. and Green, D.W. (2008) Perry’s Chemical Engineers’ Handbook. 8th edn. New York: McGraw-Hill, p. 18-12.

Sinnott, R.K. (2005) Coulson & Richardson’s Chemical Engineering Volume 6: Chemical Engineering Design. 4th edn. Oxford: Elsevier Butterworth-Heinemann, pp. 456, 634, 825.

Smith, J.M., Van Ness, H.C. and Abbott, M.M. (2005) Introduction to Chemical Engineering Thermodynamics. 7th edn. New York: McGraw-Hill, p. 468.

Valeev, D. et al. (2020) ‘Leaching of aluminosilicate raw materials in hydrochloric acid: a kinetic study’, Hydrometallurgy, 192, 105277.

Wills, B.A. and Finch, J.A. (2016) Wills’ Mineral Processing Technology. 8th edn. Oxford: Butterworth-Heinemann, p. 312.

Zwietering, T.N. (1958) ‘Suspending of solid particles in liquid by agitators’, Chemical Engineering Science, 8(3–4), pp. 244–253.

# APPENDICES

All project files are available on the github repository here (https://github.com/psgyasidenteh/CHE454_PAC.git).

## Rotary kiln matlab simulation

function sim12()

% --- 1. Rotary Kiln Solid Material Input Parameters ---

F_Fe2O3_0 = 0.02087; % [mol/s] Initial molar flow rate of Fe2O3

F_AlOH3_0 = 1.6072;  % [mol/s] Initial molar flow rate of Al(OH)3

F_SiO2_0  = 1.2275;  % [mol/s] Initial molar flow rate of SiO2

F_TiO2_0  = 0.0752;  % [mol/s] Initial molar flow rate of TiO2

F_K2O_0   = 0.0119;  % [mol/s] Initial molar flow rate of K2O

F_Na2O_0  = 0.0053;  % [mol/s] Initial molar flow rate of Na2O

F_MgO_0   = 0.0021;  % [mol/s] Initial molar flow rate of MgO

% --- 2. Operating Conditions ---

% Gas composition sized to give 600 kg/h total gas inlet

% CO-rich reducing gas from fuel-rich burner

F_CO_in      = 2.95;  % [mol/s] Inlet molar flow rate of CO  (= 297.5 kg/h)

F_CO2_in     = 1.10;  % [mol/s] Inlet molar flow rate of CO2 (= 174.3 kg/h)

F_N2_carrier = 3.70;  % [mol/s] Inlet molar flow rate of N2  (= 373.0 kg/h)

F_O2_in      = 1.16;  % [mol/s] Inlet molar flow rate of O2  (= 133.6 kg/h)

% Total gas inlet: ~600 kg/h (verified: 2.95*28.01 + 1.10*44.01 + 3.70*28.01 + 1.16*32.00 = 168.8 g/s = 607.6 kg/h)

Tg_in  = 600 + 273.15; % [K] Inlet gas temperature (per eData)

Tb_in  = 500 + 273.15; % [K] Inlet solid bed temperature (pre-heated from upstream)

L_kiln = 10;           % [m] Total length of the rotary kiln

% --- Geometry and Physical Parameters ---

delta_wall = 0.01; % [m] Steel shell thickness

delta_ref  = 0.14; % [m] Refractory thickness (140 mm for both preheat and transition sections)

D_out      = 1.0;  % [m] Outer diameter of the kiln shell

R3   = D_out / 2;               % [m] Outer radius of the shell

R2   = R3 - delta_wall;         % [m] Inner radius of the shell / outer radius of refractory

R1   = R2 - delta_ref;          % [m] Inner radius of the refractory (effective kiln radius)

R    = R1;                      % [m] Effective internal radius of the kiln

D_in = 2 * R;                   % [m] Effective internal diameter of the kiln

d_p           = 75e-6; % [m] Mean particle diameter

eps_bed       = 0.4;   % [-] Void fraction (porosity) of the bed

% --- True Densities of Solid Species [kg/m^3] ---

rho_trueFe2O3 = 5260;  % Hematite

rho_trueAlOH3 = 2420;  % Gibbsite

rho_trueAl2O3 = 3950;  % Alumina

rho_trueFe3O4 = 5150;  % Magnetite

rho_trueSiO2  = 2650;  % Quartz

rho_trueTiO2  = 4230;  % Rutile

rho_trueK2O   = 2350;  % Potassium oxide

rho_trueNa2O  = 2270;  % Sodium oxide

rho_trueMgO   = 3580;  % Magnesium oxide

% --- Rotation & Slope ---

s = 0.02;         % [m/m] Axial slope of the kiln

omega_rpm = 2.00; % [rpm] Rotational speed in revolutions per minute

omega = omega_rpm / 60;   % [rev/s] Rotational speed in revolutions per second

epsilon_p     = 0.89;     % [-] Emissivity of the solid particles

ks            = 1.3;      % [W/m/K] Thermal conductivity of the solid particles

% --- Molecular Weights [kg/mol] ---

Mw_AlOH3 = 0.07800; % [kg/mol] Molecular weight of Al(OH)3

Mw_SiO2  = 0.06008; % [kg/mol] Molecular weight of SiO2

Mw_TiO2  = 0.07986; % [kg/mol] Molecular weight of TiO2

Mw_K2O   = 0.09420; % [kg/mol] Molecular weight of K2O

Mw_Na2O  = 0.06198; % [kg/mol] Molecular weight of Na2O

Mw_MgO   = 0.04030; % [kg/mol] Molecular weight of MgO

Mw_Al2O3 = 0.10196; % [kg/mol] Molecular weight of Al2O3

Mw_Fe2O3 = 0.15969; % [kg/mol] Molecular weight of Fe2O3

Mw_Fe3O4 = 0.23153; % [kg/mol] Molecular weight of Fe3O4

Mw_O2    = 0.03200; % [kg/mol] Molecular weight of O2

rho_m_Fe2O3 = rho_trueFe2O3 / Mw_Fe2O3; % [mol/m^3] Molar density of pure solid Fe2O3

% Reaction Enthalpies [J/mol]

DH_deh = 75300;  % [J/mol_AlOH3] Enthalpy of Al(OH)3 dehydration (Endothermic)

DH_red = -47000; % [J/mol_CO] Enthalpy of Fe2O3 reduction by CO (Exothermic)

DH_comb = -283000; % [J/mol_CO] Enthalpy of CO combustion (Exothermic)

P_total = 1;     % [atm] Total operating pressure

% --- Kinematics and Dynamic Fill Degree ---

tau_min = (0.23 * L_kiln) / (s * omega_rpm^0.9 * D_in); % [min] Residence time in minutes

tau_sec = tau_min * 60; % [s] Residence time in seconds

mdot_solid_in = F_Fe2O3_0*Mw_Fe2O3 + F_AlOH3_0*Mw_AlOH3 + F_SiO2_0*Mw_SiO2 + ...

F_TiO2_0*Mw_TiO2 + F_K2O_0*Mw_K2O + F_Na2O_0*Mw_Na2O + F_MgO_0*Mw_MgO; % [kg/s] Total mass flow rate of solid feed

% Calculate dynamic initial bulk density from composition

V_solid_true = (F_Fe2O3_0*Mw_Fe2O3)/rho_trueFe2O3 + (F_AlOH3_0*Mw_AlOH3)/rho_trueAlOH3 + ...

(F_SiO2_0*Mw_SiO2)/rho_trueSiO2 + (F_TiO2_0*Mw_TiO2)/rho_trueTiO2 + ...

(F_K2O_0*Mw_K2O)/rho_trueK2O + (F_Na2O_0*Mw_Na2O)/rho_trueNa2O + ...

(F_MgO_0*Mw_MgO)/rho_trueMgO; % [m^3/s] True volume flow rate

rho_true_bed_in = mdot_solid_in / V_solid_true; % [kg/m^3] Initial average true density of the bed

rho_bulk = (1 - eps_bed) * rho_true_bed_in; % [kg/m^3] Dynamically calculated initial bulk density

mf_AlOH3 = (F_AlOH3_0 * Mw_AlOH3) / mdot_solid_in; % [-] Mass fraction of Al(OH)3 in feed

C_AlOH3_0 = rho_bulk * mf_AlOH3; % [kg/m^3] Dynamically calculated initial bulk mass concentration of Al(OH)3

Q_vol_in = mdot_solid_in / rho_bulk; % [m^3/s] Volumetric flow rate of the solid feed

v_solid = L_kiln / tau_sec; % [m/s] Axial velocity of the solid bed

A_bed = Q_vol_in / v_solid; % [m^2] Cross-sectional area of the solid bed

A_total = pi * R^2; % [m^2] Total inner cross-sectional area of the kiln

f = A_bed / A_total; % [-] Dynamic fill degree (fraction of kiln filled by bed)

fprintf('\n=== Kinematics ===\n');

fprintf('Calculated Residence Time: %.1f mins\n', tau_min);

fprintf('Calculated Dynamic Fill Degree (f): %.4f (%.1f%%)\n', f, f*100);

mdot_gas_in = F_CO_in*0.02801 + F_CO2_in*0.04401 + F_N2_carrier*0.02801 + F_O2_in*0.032;

fprintf('Gas inlet mass flow: %.1f kg/h (target: 600 kg/h)\n', mdot_gas_in*3600);

fprintf('==================\n');

% --- Interface Areas and Subtended Angle ---

A_fb    = A_total - A_bed; % [m^2] Cross-sectional area of the gas freeboard

% Solve for theta using Newton-Raphson (fzero)

theta = fzero(@(th) th - sin(th) - 2*pi*f, 2); % [rad] Subtended central angle of the bed

L_chord = 2 * R * sin(theta / 2); % [m] Length of the chord forming the exposed bed surface

A_ew    = (2*pi - theta) * R; % [m] Perimeter length of the exposed wall (area per meter)

A_eb    = L_chord; % [m] Perimeter length of the exposed bed (area per meter)

A_cw    = theta * R; % [m] Perimeter length of the covered wall (area per meter)

% --- Wall Thermal Resistance ---

k_ref     = 1.5; % [W/m/K] Thermal conductivity of the refractory brick

k_shell   = 45; % [W/m/K] Thermal conductivity of the carbon steel shell

h_ambient = 15; % [W/m^2/K] Convective heat transfer coefficient to ambient air

T_ambient = 305.15; % [K] Ambient temperature outside the kiln

R_wall = log(R2/R1)/(2*pi*k_ref) + log(R3/R2)/(2*pi*k_shell) + 1/(2*pi*R3*h_ambient); % [m*K/W] Total thermal resistance of the wall per meter

% --- Pack parameters for ODE ---

p.F_Fe2O3_0 = F_Fe2O3_0;  p.F_AlOH3_0 = F_AlOH3_0;

p.F_SiO2_0  = F_SiO2_0;   p.F_TiO2_0  = F_TiO2_0;

p.F_K2O_0   = F_K2O_0;    p.F_Na2O_0  = F_Na2O_0;

p.F_MgO_0   = F_MgO_0;    p.Mw_SiO2   = Mw_SiO2;

p.Mw_TiO2   = Mw_TiO2;    p.Mw_K2O    = Mw_K2O;

p.Mw_Na2O   = Mw_Na2O;    p.Mw_MgO    = Mw_MgO;

p.F_CO_in   = F_CO_in;    p.F_CO2_in  = F_CO2_in;

p.F_N2_carrier = F_N2_carrier;

p.F_O2_in      = F_O2_in;

p.Mw_O2        = Mw_O2;

p.Tg_in     = Tg_in;      p.Tb_in     = Tb_in;

p.Mw_AlOH3  = Mw_AlOH3;   p.Mw_Al2O3  = Mw_Al2O3;

p.Mw_Fe2O3  = Mw_Fe2O3;   p.Mw_Fe3O4  = Mw_Fe3O4;

p.rho_m_Fe2O3 = rho_m_Fe2O3; p.C_AlOH3_0 = C_AlOH3_0;

p.A_bed = A_bed; p.A_fb = A_fb;

p.A_ew  = A_ew;  p.A_eb = A_eb;  p.A_cw = A_cw;

p.R1 = R1; p.R_wall = R_wall; p.theta = theta;

p.eps = eps_bed; p.d_p = d_p; p.omega = omega;

p.epsilon_p = epsilon_p; p.ks = ks;

p.P_total = P_total; p.T_ambient = T_ambient;

p.L_kiln = L_kiln; p.DH_deh = DH_deh; p.DH_red = DH_red;

p.DH_comb = DH_comb;

p.v_solid = v_solid; p.A_total = A_total;

p.rho_trueFe2O3 = rho_trueFe2O3; p.rho_trueAlOH3 = rho_trueAlOH3;

p.rho_trueSiO2  = rho_trueSiO2;  p.rho_trueTiO2  = rho_trueTiO2;

p.rho_trueK2O   = rho_trueK2O;   p.rho_trueNa2O  = rho_trueNa2O;

p.rho_trueMgO   = rho_trueMgO;   p.rho_trueAl2O3 = rho_trueAl2O3;

p.rho_trueFe3O4 = rho_trueFe3O4;

% --- BVP Setup ---

z_mesh = linspace(0, L_kiln, 200);

y_guess_mat = zeros(9, length(z_mesh));

for i = 1:length(z_mesh)

zz = z_mesh(i);

frac = zz / L_kiln; % 0 at solid inlet, 1 at gas inlet

Tg_guess = Tb_in + 200 + (Tg_in - Tb_in - 200) * frac; % Gas temp rises toward z=L

Tb_guess = Tb_in + (Tg_in - 300 - Tb_in) * frac;       % Bed temp rises toward z=L

y_guess_mat(:, i) = [F_CO_in * frac + 0.01; F_CO2_in + 0.5*(1-frac); 0.5*(1-frac); ...

F_AlOH3_0 * (1-frac) + 1e-4; 0.01 * frac; F_Fe2O3_0 * (1-frac) + 1e-4; ...

Tg_guess; Tb_guess; F_O2_in * frac + 0.01];

end

solinit.x = z_mesh;

solinit.y = y_guess_mat;

% Solve BVP

options = bvpset('NMax', 10000, 'RelTol', 1e-4, 'AbsTol', 1e-6);

fprintf('Solving BVP... (Note: Strict un-capped kinetics may cause stiff Jacobian probes)\n');

sol = bvp4c(@(z,y) kiln_ode(z,y,p), @(ya,yb) bcfun(ya,yb,p), solinit, options);

% --- Process Results ---

z = sol.x;

y = sol.y;

F_CO    = y(1,:); % [mol/s] Solution profile for CO molar flow rate

F_CO2   = y(2,:); % [mol/s] Solution profile for CO2 molar flow rate

F_H2O   = y(3,:); % [mol/s] Solution profile for H2O molar flow rate

F_AlOH3 = y(4,:); % [mol/s] Solution profile for Al(OH)3 molar flow rate

F_Fe3O4 = y(5,:); % [mol/s] Solution profile for Fe3O4 molar flow rate

F_Fe2O3 = y(6,:); % [mol/s] Solution profile for Fe2O3 molar flow rate

Tg      = y(7,:); % [K] Solution profile for gas temperature

Tb      = y(8,:); % [K] Solution profile for bed temperature

F_O2    = y(9,:); % [mol/s] Solution profile for O2 molar flow rate

% Derived quantities

F_Al2O3 = (F_AlOH3_0 - F_AlOH3) / 2; % [mol/s] Profile for Al2O3 molar flow rate

X_AlOH3 = 1 - (F_AlOH3 / F_AlOH3_0); % [-] Profile for fractional conversion of Al(OH)3

% Volumetric flow rate of gases in freeboard [m^3/s]

F_gas_total = F_CO + F_CO2 + F_H2O + F_O2 + p.F_N2_carrier; % [mol/s] Total gas molar flow rate profile

Q_gas = F_gas_total .* 8.314 .* Tg / (p.P_total * 101325); % [m^3/s] Volumetric gas flow rate profile

Tw = zeros(size(z)); % [K] Wall temperature profile

Q_duty = zeros(size(z)); % [W/m] Heat duty profile per meter length

Q_rxn_arr = zeros(size(z)); % [W/m] Heat of reaction profile per meter length

Q_shell_loss = zeros(size(z)); % [W/m] Shell heat loss profile per meter length

rho_bulk_profile = zeros(size(z)); % [kg/m^3] Bed bulk density profile

% --- Diagnostic profiles ---

r_deh_kin_arr  = zeros(size(z)); r_deh_mt_arr  = zeros(size(z)); r_deh_arr  = zeros(size(z));

r_red_kin_arr  = zeros(size(z)); r_red_mt_arr  = zeros(size(z)); r_red_arr  = zeros(size(z));

r_comb_kin_arr = zeros(size(z)); r_comb_mix_arr = zeros(size(z)); r_comb_arr = zeros(size(z));

P_CO_arr  = zeros(size(z)); P_CO2_arr = zeros(size(z)); P_H2O_arr = zeros(size(z));

P_O2_arr  = zeros(size(z)); P_N2_arr  = zeros(size(z));

hew_arr = zeros(size(z)); heb_arr = zeros(size(z)); hcw_arr = zeros(size(z));

f_local_arr = zeros(size(z));

for i = 1:length(z)

[~, Tw(i), Q_rxn_arr(i), Q_duty(i), Q_shell_loss(i), rho_bulk_profile(i), info] = kiln_ode(z(i), y(:,i), p);

r_deh_kin_arr(i) = info.r_deh_kin; r_deh_mt_arr(i) = info.r_deh_mt; r_deh_arr(i) = info.r_deh;

r_red_kin_arr(i) = info.r_red_kin; r_red_mt_arr(i) = info.r_red_mt; r_red_arr(i) = info.r_red;

r_comb_kin_arr(i) = info.r_comb_kin; r_comb_mix_arr(i) = info.r_comb_mix; r_comb_arr(i) = info.r_comb;

P_CO_arr(i) = info.P_CO; P_CO2_arr(i) = info.P_CO2; P_H2O_arr(i) = info.P_H2O;

P_O2_arr(i) = info.P_O2; P_N2_arr(i) = info.P_N2;

hew_arr(i) = info.hew; heb_arr(i) = info.heb; hcw_arr(i) = info.hcw;

f_local_arr(i) = info.f_local;

end

Q_total = trapz(z, Q_duty) / 1e6; % [MW] Total integrated heat duty across the kiln

% --- Plotting ---

% Figure 1a

figure(1); clf;

set(gcf, 'Name', 'Figure 1a - Reaction 1 Molar flow rate profiles');

plot(z, F_H2O, 'b-', 'LineWidth', 1.5); hold on;

plot(z, F_AlOH3, 'r--', 'LineWidth', 1.5);

plot(z, F_Al2O3, 'g-.', 'LineWidth', 1.5);

xlabel('Axial position, z [m]', 'Interpreter', 'latex');

ylabel('Molar flow rate [mol/s]', 'Interpreter', 'latex');

legend({'H$_2$O', 'Al(OH)$_3$', 'Al$_2$O$_3$'}, 'Interpreter', 'latex', 'Location', 'best');

grid on;

% Figure 1b

figure(2); clf;

set(gcf, 'Name', 'Figure 1b - Solid species flow rate profiles');

plot(z, F_Fe3O4, 'b-', 'LineWidth', 1.5); hold on;

plot(z, F_Fe2O3, 'r--', 'LineWidth', 1.5);

xlabel('Axial position, z [m]', 'Interpreter', 'latex');

ylabel('Molar flow rate [mol/s]', 'Interpreter', 'latex');

legend({'Fe$_3$O$_4$', 'Fe$_2$O$_3$'}, 'Interpreter', 'latex', 'Location', 'best');

grid on;

% Figure 1c (yyaxis implementation)

figure(3); clf;

set(gcf, 'Name', 'Figure 1c - Gas phase molar flow rate profiles');

yyaxis left

plot(z, F_H2O, 'b-', 'LineWidth', 1.5);

ylabel('H$_2$O Flow [mol/s]', 'Interpreter', 'latex');

yyaxis right

plot(z, F_CO, 'k-', z, F_CO2, 'm:', 'LineWidth', 1.5);

ylabel('CO and CO$_2$ Flow [mol/s]', 'Interpreter', 'latex');

xlabel('Axial position, z [m]', 'Interpreter', 'latex');

legend({'H$_2$O', 'CO', 'CO$_2$'}, 'Interpreter', 'latex', 'Location', 'best');

grid on;

% Figure 2

figure(4); clf;

set(gcf, 'Name', 'Figure 2 - Temperature profiles');

plot(z, Tg - 273.15, 'r-', 'LineWidth', 1.5); hold on;

plot(z, Tb - 273.15, 'b--', 'LineWidth', 1.5);

plot(z, Tw - 273.15, 'k-.', 'LineWidth', 1.5);

yline(870, 'm:', 'LineWidth', 1.5, 'Label', '870 ^\circ C');

xlabel('Axial position, z [m]', 'Interpreter', 'latex');

ylabel('Temperature [$^\circ$C]', 'Interpreter', 'latex');

legend({'Freeboard gas $T_g$', 'Bed solid $T_b$', 'Wall $T_w$'}, 'Interpreter', 'latex', 'Location', 'best');

grid on;

% Figure 3

figure(5); clf;

set(gcf, 'Name', 'Figure 3 - Conversion of Al(OH)3');

plot(z, X_AlOH3, 'b-', 'LineWidth', 1.5); hold on;

xlabel('Axial position, z [m]', 'Interpreter', 'latex');

ylabel('Conversion $X_{Al(OH)_3}$', 'Interpreter', 'latex');

grid on;

% Figure 4

figure(6); clf;

set(gcf, 'Name', 'Figure 4 - Heat duty profile');

plot(z, Q_duty / 1000, 'k-', 'LineWidth', 2); hold on;

plot(z, Q_rxn_arr / 1000, 'r--', 'LineWidth', 1.5);

plot(z, Q_shell_loss / 1000, 'b-.', 'LineWidth', 1.5);

xlabel('Axial position, z [m]', 'Interpreter', 'latex');

ylabel('Heat duty [kW/m]', 'Interpreter', 'latex');

legend({'Total $Q_{duty}$', '$Q_{rxn}$', '$Q_{shell\_loss}$'}, 'Interpreter', 'latex', 'Location', 'best');

text(p.L_kiln*0.6, max(Q_duty/1000)*0.8, sprintf('Total Q = %.2f MW', Q_total), 'Interpreter', 'latex', 'FontSize', 12, 'EdgeColor', 'k');

grid on;

% Figure 5 - Dynamic fill degree and bulk density

figure(7); clf;

set(gcf, 'Name', 'Figure 5 - Dynamic Fill Degree and Bulk Density');

yyaxis left

plot(z, f_local_arr * 100, 'b-', 'LineWidth', 2);

ylabel('Fill degree $f$ [\%]', 'Interpreter', 'latex');

yyaxis right

plot(z, rho_bulk_profile, 'r--', 'LineWidth', 2);

ylabel('Bulk density $\rho_{bulk}$ [kg/m$^3$]', 'Interpreter', 'latex');

xlabel('Axial position, z [m]', 'Interpreter', 'latex');

legend({'Fill degree', 'Bulk density'}, 'Interpreter', 'latex', 'Location', 'best');

grid on;

% Figure 6

figure(8); clf;

set(gcf, 'Name', 'Figure 6 - Combined summary figure');

tiledlayout(2,2, 'TileSpacing', 'compact', 'Padding', 'compact');

% Tile 1

nexttile;

plot(z, F_H2O, 'b-', z, F_AlOH3, 'r--', z, F_Al2O3, 'g-.', z, F_CO, 'k-', z, F_CO2, 'm:', z, F_O2, 'c--', 'LineWidth', 1.5);

xlabel('z [m]', 'Interpreter', 'latex'); ylabel('[mol/s]', 'Interpreter', 'latex');

legend({'H$_2$O', 'Al(OH)$_3$', 'Al$_2$O$_3$', 'CO', 'CO$_2$', 'O$_2$'}, 'Interpreter', 'latex', 'Location', 'best', 'FontSize', 8);

grid on;

% Tile 2

nexttile;

plot(z, Tg - 273.15, 'r-', z, Tb - 273.15, 'b--', z, Tw - 273.15, 'k-.', 'LineWidth', 1.5);

yline(870, 'm:');

xlabel('z [m]', 'Interpreter', 'latex'); ylabel('[$^\circ$C]', 'Interpreter', 'latex');

legend({'$T_g$', '$T_b$', '$T_w$'}, 'Interpreter', 'latex', 'Location', 'best', 'FontSize', 8);

grid on;

% Tile 3

nexttile;

plot(z, X_AlOH3, 'b-', 'LineWidth', 1.5); hold on;

yline(0.98, 'r--');

xlabel('z [m]', 'Interpreter', 'latex'); ylabel('$X_{Al(OH)_3}$', 'Interpreter', 'latex');

grid on;

% Tile 4

nexttile;

plot(z, Q_duty / 1000, 'k-', 'LineWidth', 1.5);

xlabel('z [m]', 'Interpreter', 'latex'); ylabel('$Q_{duty}$ [kW/m]', 'Interpreter', 'latex');

X_Fe2O3 = 1 - F_Fe2O3(end)/p.F_Fe2O3_0;

sgtitle(sprintf('L = %.1f m $|$ $Q_{total}$ = %.2f MW $|$ $X_{Al(OH)_3}$ = %.3f $|$ $X_{Fe_2O_3}$ = %.3f', p.L_kiln, Q_total, X_AlOH3(end), X_Fe2O3), 'Interpreter', 'latex');

% Figure 7 - Reaction Rate Profiles (kinetic vs transport limited)

figure(9); clf;

set(gcf, 'Name', 'Figure 7 - Reaction rate profiles');

tiledlayout(3,1, 'TileSpacing', 'compact', 'Padding', 'compact');

nexttile;

semilogy(z, r_deh_kin_arr, 'r--', z, r_deh_mt_arr, 'b:', z, r_deh_arr, 'k-', 'LineWidth', 1.5);

xlabel('z [m]', 'Interpreter', 'latex');

ylabel('[kg/m$^3$/s]', 'Interpreter', 'latex');

legend({'$r_{kin}$', '$r_{mt}$', '$r_{eff}$'}, 'Interpreter', 'latex', 'Location', 'best');

title('Al(OH)$_3$ Dehydration', 'Interpreter', 'latex');

grid on;

nexttile;

semilogy(z, r_red_kin_arr, 'r--', z, r_red_mt_arr, 'b:', z, r_red_arr, 'k-', 'LineWidth', 1.5);

xlabel('z [m]', 'Interpreter', 'latex');

ylabel('[mol/m$^3$/s]', 'Interpreter', 'latex');

legend({'$r_{kin}$', '$r_{mt}$', '$r_{eff}$'}, 'Interpreter', 'latex', 'Location', 'best');

title('Fe$_2$O$_3$ Reduction', 'Interpreter', 'latex');

grid on;

nexttile;

semilogy(z, r_comb_kin_arr, 'r--', z, r_comb_mix_arr, 'b:', z, r_comb_arr, 'k-', 'LineWidth', 1.5);

xlabel('z [m]', 'Interpreter', 'latex');

ylabel('[mol/m$^3$/s]', 'Interpreter', 'latex');

legend({'$r_{kin}$', '$r_{mix}$', '$r_{eff}$'}, 'Interpreter', 'latex', 'Location', 'best');

title('CO Combustion', 'Interpreter', 'latex');

grid on;

% Figure 8 - Gas Phase Partial Pressures

figure(10); clf;

set(gcf, 'Name', 'Figure 8 - Gas phase partial pressures');

plot(z, P_CO_arr, 'k-', z, P_CO2_arr, 'm--', z, P_H2O_arr, 'b-.', ...

z, P_O2_arr, 'r:', z, P_N2_arr, 'g-', 'LineWidth', 1.5);

xlabel('Axial position, z [m]', 'Interpreter', 'latex');

ylabel('Partial pressure [atm]', 'Interpreter', 'latex');

legend({'CO', 'CO$_2$', 'H$_2$O', 'O$_2$', 'N$_2$'}, 'Interpreter', 'latex', 'Location', 'best');

grid on;

% Figure 9 - Heat Transfer Coefficient Profiles

figure(11); clf;

set(gcf, 'Name', 'Figure 9 - Heat transfer coefficients');

plot(z, hew_arr, 'r-', z, heb_arr, 'b--', z, hcw_arr, 'k-.', 'LineWidth', 1.5);

xlabel('Axial position, z [m]', 'Interpreter', 'latex');

ylabel('$h$ [W/m$^2$/K]', 'Interpreter', 'latex');

legend({'$h_{ew}$ (gas-wall)', '$h_{eb}$ (gas-bed)', '$h_{cw}$ (wall-bed)'}, 'Interpreter', 'latex', 'Location', 'best');

grid on;

% --- Export Freeboard Q vs T data ---

Q_cumul = cumtrapz(z, Q_duty) / 1000; % [kW] Cumulative heat duty from z=0

T_export = table(z', (Tg - 273.15)', Q_duty'/1000, Q_cumul', ...

'VariableNames', {'z_m', 'Tg_degC', 'Q_local_kW_per_m', 'Q_cumul_kW'});

writetable(T_export, 'QvsT.csv');

fprintf('\nSaved QvsT.csv (%d rows)\n', height(T_export));

fprintf('\n--- FINAL RESULTS ---\n');

fprintf('Total Heat Duty: %.2f MW\n', Q_total);

fprintf('AlOH3 Conversion: %.3f\n', X_AlOH3(end));

fprintf('Fe2O3 Conversion: %.3f\n', 1 - F_Fe2O3(end)/p.F_Fe2O3_0);

fprintf('CO Conversion: %.3f\n', (p.F_CO_in - F_CO(1)) / p.F_CO_in);

fprintf('---------------------\n');

% --- MASS FLOW RATE CALCULATIONS (kg/h) ---

% Solid Inlet (z = 0)

m_solid_in_Fe2O3 = F_Fe2O3(1) * p.Mw_Fe2O3 * 3600;

m_solid_in_AlOH3 = F_AlOH3(1) * p.Mw_AlOH3 * 3600;

m_solid_in_SiO2  = p.F_SiO2_0 * p.Mw_SiO2 * 3600;

m_solid_in_TiO2  = p.F_TiO2_0 * p.Mw_TiO2 * 3600;

m_solid_in_K2O   = p.F_K2O_0 * p.Mw_K2O * 3600;

m_solid_in_Na2O  = p.F_Na2O_0 * p.Mw_Na2O * 3600;

m_solid_in_MgO   = p.F_MgO_0 * p.Mw_MgO * 3600;

m_solid_in_total = m_solid_in_Fe2O3 + m_solid_in_AlOH3 + m_solid_in_SiO2 + m_solid_in_TiO2 + m_solid_in_K2O + m_solid_in_Na2O + m_solid_in_MgO;

% Solid Outlet (z = L)

m_solid_out_Fe2O3 = F_Fe2O3(end) * p.Mw_Fe2O3 * 3600;

m_solid_out_Fe3O4 = F_Fe3O4(end) * p.Mw_Fe3O4 * 3600;

m_solid_out_AlOH3 = F_AlOH3(end) * p.Mw_AlOH3 * 3600;

m_solid_out_Al2O3 = F_Al2O3(end) * p.Mw_Al2O3 * 3600;

m_solid_out_SiO2  = p.F_SiO2_0 * p.Mw_SiO2 * 3600;

m_solid_out_TiO2  = p.F_TiO2_0 * p.Mw_TiO2 * 3600;

m_solid_out_K2O   = p.F_K2O_0 * p.Mw_K2O * 3600;

m_solid_out_Na2O  = p.F_Na2O_0 * p.Mw_Na2O * 3600;

m_solid_out_MgO   = p.F_MgO_0 * p.Mw_MgO * 3600;

m_solid_out_total = m_solid_out_Fe2O3 + m_solid_out_Fe3O4 + m_solid_out_AlOH3 + m_solid_out_Al2O3 + m_solid_out_SiO2 + m_solid_out_TiO2 + m_solid_out_K2O + m_solid_out_Na2O + m_solid_out_MgO;

% Gas Inlet (z = L)

m_gas_in_CO  = F_CO(end) * 0.02801 * 3600;

m_gas_in_CO2 = F_CO2(end) * 0.04401 * 3600;

m_gas_in_H2O = F_H2O(end) * 0.018015 * 3600;

m_gas_in_N2  = p.F_N2_carrier * 0.02801 * 3600;

m_gas_in_O2  = F_O2(end) * p.Mw_O2 * 3600;

m_gas_in_total = m_gas_in_CO + m_gas_in_CO2 + m_gas_in_H2O + m_gas_in_N2 + m_gas_in_O2;

% Gas Outlet (z = 0)

m_gas_out_CO  = F_CO(1) * 0.02801 * 3600;

m_gas_out_CO2 = F_CO2(1) * 0.04401 * 3600;

m_gas_out_H2O = F_H2O(1) * 0.018015 * 3600;

m_gas_out_N2  = p.F_N2_carrier * 0.02801 * 3600;

m_gas_out_O2  = F_O2(1) * p.Mw_O2 * 3600;

m_gas_out_total = m_gas_out_CO + m_gas_out_CO2 + m_gas_out_H2O + m_gas_out_N2 + m_gas_out_O2;

fprintf('\n--- MASS FLOW RATES (kg/h) ---\n');

fprintf('SOLID INLET (z = 0):\n');

fprintf('  Fe2O3: %8.2f\n  AlOH3: %8.2f\n  SiO2:  %8.2f\n  TiO2:  %8.2f\n  K2O:   %8.2f\n  Na2O:  %8.2f\n  MgO:   %8.2f\n', ...

m_solid_in_Fe2O3, m_solid_in_AlOH3, m_solid_in_SiO2, m_solid_in_TiO2, m_solid_in_K2O, m_solid_in_Na2O, m_solid_in_MgO);

fprintf('  TOTAL: %8.2f kg/h\n\n', m_solid_in_total);

fprintf('SOLID OUTLET (z = L):\n');

fprintf('  Fe2O3: %8.2f\n  Fe3O4: %8.2f\n  AlOH3: %8.2f\n  Al2O3: %8.2f\n  SiO2:  %8.2f\n  TiO2:  %8.2f\n  K2O:   %8.2f\n  Na2O:  %8.2f\n  MgO:   %8.2f\n', ...

m_solid_out_Fe2O3, m_solid_out_Fe3O4, m_solid_out_AlOH3, m_solid_out_Al2O3, m_solid_out_SiO2, m_solid_out_TiO2, m_solid_out_K2O, m_solid_out_Na2O, m_solid_out_MgO);

fprintf('  TOTAL: %8.2f kg/h\n\n', m_solid_out_total);

fprintf('GAS INLET (z = L):\n');

fprintf('  CO:    %8.2f\n  CO2:   %8.2f\n  H2O:   %8.2f\n  N2:    %8.2f\n  O2:    %8.2f\n', ...

m_gas_in_CO, m_gas_in_CO2, m_gas_in_H2O, m_gas_in_N2, m_gas_in_O2);

fprintf('  TOTAL: %8.2f kg/h\n\n', m_gas_in_total);

fprintf('GAS OUTLET (z = 0):\n');

fprintf('  CO:    %8.2f\n  CO2:   %8.2f\n  H2O:   %8.2f\n  N2:    %8.2f\n  O2:    %8.2f\n', ...

m_gas_out_CO, m_gas_out_CO2, m_gas_out_H2O, m_gas_out_N2, m_gas_out_O2);

fprintf('  TOTAL: %8.2f kg/h\n', m_gas_out_total);

fprintf('------------------------------\n');

end

% --- BVP Boundary Conditions ---

function res = bcfun(ya, yb, p)

res = [ ya(4) - p.F_AlOH3_0;         % F_AlOH3(0)

ya(5) - 0;                   % F_Fe3O4(0)

ya(6) - p.F_Fe2O3_0;         % F_Fe2O3(0)

ya(8) - p.Tb_in;             % Tb(0)

yb(1) - p.F_CO_in;           % F_CO(L) = 15.0

yb(2) - p.F_CO2_in;          % F_CO2(L) = 15.0

yb(3) - 0;                   % F_H2O(L) = 0

yb(7) - p.Tg_in;             % Tg(L)

yb(9) - p.F_O2_in ];         % F_O2(L)

end

% --- BVP ODE System ---

function [dydz, Tw, Q_rxn, Q_duty, Q_shell_loss, rho_b, info] = kiln_ode(z, y, p)

% BVP Solver Safeguards (Exact mapping)

F_CO    = max(y(1), 1e-8);   % [mol/s] Local CO molar flow rate (bounded)

F_CO2   = max(y(2), 1e-8);   % [mol/s] Local CO2 molar flow rate

F_H2O   = max(y(3), 1e-8);   % [mol/s] Local H2O molar flow rate

F_AlOH3 = max(y(4), 1e-8);   % [mol/s] Local Al(OH)3 molar flow rate

F_Fe3O4 = max(y(5), 1e-8);   % [mol/s] Local Fe3O4 molar flow rate

F_Fe2O3 = max(y(6), 1e-8);   % [mol/s] Local Fe2O3 molar flow rate

Tg      = max(y(7), 298.15); % [K] Local gas temperature (bounded > ambient)

Tb      = max(y(8), 298.15); % [K] Local bed temperature (bounded > ambient)

F_O2    = max(y(9), 1e-8);   % [mol/s] Local O2 molar flow rate

% Stoichiometric derivation

F_Al2O3 = (p.F_AlOH3_0 - F_AlOH3) / 2; % [mol/s] Local Al2O3 molar flow rate

% Gas Heat Capacities [J/mol/K] (t = T/1000)

t_g = Tg / 1000;

cp_CO  = 25.568 + 6.096*t_g + 4.055*t_g^2 - 2.671*t_g^3 + 0.131/t_g^2;

cp_CO2 = 24.997 + 55.187*t_g - 33.691*t_g^2 + 7.948*t_g^3 - 0.137/t_g^2;

cp_H2O = 30.092 + 6.833*t_g + 6.793*t_g^2 - 2.534*t_g^3 + 0.082/t_g^2;

cp_N2  = 30.0;

GCp_g = F_CO*cp_CO + F_CO2*cp_CO2 + F_H2O*cp_H2O + p.F_N2_carrier*cp_N2; % [W/K] Total heat capacity flow rate of the gas phase

% Solid Heat Capacities [J/kg/K]

t_b = Tb / 1000;

cp_AlOH3 = (58.62 + 0.126*Tb) / p.Mw_AlOH3; % explicitly Tb in Kelvin per doc

cp_Al2O3 = (104.157 + 16.481*t_b - 8.125*t_b^2 + 1.638*t_b^3 - 3.208/t_b^2) / p.Mw_Al2O3;

cp_Fe2O3 = (98.806 + 111.725*t_b - 52.687*t_b^2 + 9.871*t_b^3 - 1.246/t_b^2) / p.Mw_Fe2O3;

cp_Fe3O4 = (200.832 + 1.586*t_b - 4.113/t_b^2) / p.Mw_Fe3O4;

cp_SiO2  = (46.94 + 34.31*t_b - 1.13/t_b^2) / p.Mw_SiO2;

cp_TiO2  = (73.35 + 5.02*t_b - 1.715/t_b^2) / p.Mw_TiO2;

cp_MgO   = (42.59 + 7.28*t_b - 0.619/t_b^2) / p.Mw_MgO;

cp_K2O   = 1000;

cp_Na2O  = 1000;

GCp_b = F_AlOH3*p.Mw_AlOH3*cp_AlOH3 + F_Al2O3*p.Mw_Al2O3*cp_Al2O3 + ...

F_Fe2O3*p.Mw_Fe2O3*cp_Fe2O3 + F_Fe3O4*p.Mw_Fe3O4*cp_Fe3O4 + ...

p.F_SiO2_0*p.Mw_SiO2*cp_SiO2 + p.F_TiO2_0*p.Mw_TiO2*cp_TiO2 + ...

p.F_K2O_0*p.Mw_K2O*cp_K2O + p.F_Na2O_0*p.Mw_Na2O*cp_Na2O + ...

p.F_MgO_0*p.Mw_MgO*cp_MgO; % [W/K] Total heat capacity flow rate of the solid bed

% --- Heat Transfer Coefficients [W/m^2/K] ---

mu_gas = 4e-5; % [kg/m/s] Dynamic viscosity of the gas mixture

k_gas = 0.07;  % [W/m/K] Thermal conductivity of the gas mixture

Pr = 0.7;      % [-] Prandtl number of the gas mixture

P_fb = p.A_ew + p.A_eb; % [m] Perimeter of the freeboard

Dk   = 4 * p.A_fb / P_fb; % [m] Hydraulic diameter of the freeboard

mdot_g = F_CO*0.02801 + F_CO2*0.04401 + F_H2O*0.018015 + p.F_N2_carrier*0.02801; % [kg/s] Total mass flow rate of gas

Gg_kg_s = mdot_g / p.A_fb; % [kg/m^2/s] Mass flux of gas

Re = Gg_kg_s * Dk / mu_gas; % [-] Reynolds number of the gas flow

% Convection relations

hew = 0.036 * (k_gas / Dk) * Re^0.8 * Pr^0.33 * (Dk / p.L_kiln)^0.055; % [W/m^2/K] Heat transfer coefficient (gas to exposed wall)

Gg_kg_h = Gg_kg_s * 3600; % [kg/m^2/h] Mass flux of gas in kg per hour

heb = 0.4 * Gg_kg_h^0.62; % [W/m^2/K] Heat transfer coefficient (gas to exposed bed)

% Bed effective conductivity (static formulation strictly)

kb = (1 - p.eps) / (1/p.ks + 1/(4*5.67e-8*p.epsilon_p*p.d_p*Tb^3)) + ...

p.eps * 4 * 5.67e-8 * p.epsilon_p * p.d_p * Tb^3; % [W/m/K] Effective thermal conductivity of the static bed

V_solid_true_local = F_AlOH3*p.Mw_AlOH3/p.rho_trueAlOH3 + F_Al2O3*p.Mw_Al2O3/p.rho_trueAl2O3 + ...

F_Fe2O3*p.Mw_Fe2O3/p.rho_trueFe2O3 + F_Fe3O4*p.Mw_Fe3O4/p.rho_trueFe3O4 + ...

p.F_SiO2_0*p.Mw_SiO2/p.rho_trueSiO2 + p.F_TiO2_0*p.Mw_TiO2/p.rho_trueTiO2 + ...

p.F_K2O_0*p.Mw_K2O/p.rho_trueK2O + p.F_Na2O_0*p.Mw_Na2O/p.rho_trueNa2O + ...

p.F_MgO_0*p.Mw_MgO/p.rho_trueMgO; % [m^3/s] Local true volume flow rate

mass_sum = F_AlOH3*p.Mw_AlOH3 + F_Al2O3*p.Mw_Al2O3 + F_Fe2O3*p.Mw_Fe2O3 + F_Fe3O4*p.Mw_Fe3O4 + ...

p.F_SiO2_0*p.Mw_SiO2 + p.F_TiO2_0*p.Mw_TiO2 + p.F_K2O_0*p.Mw_K2O + ...

p.F_Na2O_0*p.Mw_Na2O + p.F_MgO_0*p.Mw_MgO; % [kg/s] Total mass flow rate of the solid bed

rho_true_bed_local = mass_sum / V_solid_true_local; % [kg/m^3] Local true average density

rho_b = (1 - p.eps) * rho_true_bed_local; % [kg/m^3] Local dynamically calculated bulk density of the bed

cp_b_mass = GCp_b / mass_sum; % [J/kg/K] Average specific heat capacity of the solid bed

alpha_b = kb / (rho_b * cp_b_mass); % [m^2/s] Thermal diffusivity of the solid bed

Pe = pi^2 * p.theta * p.omega / alpha_b; % [-] Peclet number for the bed

Nu_cwcb = 11.6 * Pe^0.3; % [-] Nusselt number for covered wall to covered bed

hcw = Nu_cwcb * kb / (p.R1 * p.theta); % [W/m^2/K] Heat transfer coefficient (covered wall to bed)

% Algebraic Tw calculation

Tw = (hew * p.A_ew * Tg + hcw * p.A_cw * Tb + p.T_ambient / p.R_wall) / ...

(hew * p.A_ew + hcw * p.A_cw + 1 / p.R_wall); % [K] Local wall temperature

Q_shell_loss = (Tw - p.T_ambient) / p.R_wall; % [W/m] Local heat loss through the shell

% --- Reaction Kinetics (Strict Form) ---

k_deh = 7.58e15 * exp(-157000 / (8.314 * Tb)); % [s^-1] Arrhenius rate constant for Al(OH)3 dehydration

k_CO  = 0.232e-6 * exp(-76000 / (8.314 * Tb)); % [mol/m^2/s/atm] Arrhenius rate constant for Fe2O3 reduction

k_comb = 2.24E12 * exp(-1.7E5 / (8.314 * Tg)); % [s^-1] Arrhenius rate constant for CO combustion

a_s = 6 * (1 - p.eps) / p.d_p; % [m^2/m^3] Specific surface area of the bed

gas_sum = F_CO +F_CO2 + F_H2O + p.F_N2_carrier; % [mol/s] Local total gas molar flow rate

P_CO = p.P_total * (F_CO / gas_sum); % [atm] Local partial pressure of CO

P_O2 = p.P_total * (F_O2 / gas_sum); % [atm] Local partial pressure of O2

P_H2O = p.P_total * (F_H2O / gas_sum); % [atm] Local partial pressure of H2O

% Local volumetric gas flow [m^3/s]

Q_local = gas_sum * 8.314 * Tg / (p.P_total * 101325); % [m^3/s] Local volumetric gas flow rate

% Instantaneous Concentrations

C_CO  = max(F_CO  / Q_local, 0); % [mol/m^3] Local molar concentration of CO

C_H2O = max(F_H2O / Q_local, 0); % [mol/m^3] Local molar concentration of H2O

C_O2  = max(F_O2  / Q_local, 0); % [mol/m^3] Local molar concentration of O2

% --- Physically Sound Rate Limiters (Harmonic Means) ---

% 1. Dehydration: limited by internal heat/mass transfer in particles

k_deh_mt = 0.00074; % [s^-1] Tuned for X_AlOH3 = 0.98

r_deh_kin = p.C_AlOH3_0 * k_deh * (F_AlOH3 / p.F_AlOH3_0); % [kg_AlOH3/m^3/s] Pure kinetic dehydration rate

r_deh_mt  = p.C_AlOH3_0 * k_deh_mt * (F_AlOH3 / p.F_AlOH3_0); % [kg_AlOH3/m^3/s] Mass transfer limited dehydration rate

r_deh = (r_deh_kin * r_deh_mt) / (r_deh_kin + r_deh_mt + 1e-12); % [kg_AlOH3/m^3/s] Effective dehydration rate

% 2. Reduction: limited by gas-solid external mass transfer

k_red_mt = 0.9; % [mol/m^2/s/atm] Relaxed mass transfer ceiling for Fe2O3 reduction

r_red_kin = k_CO * P_CO * a_s * p.rho_m_Fe2O3 * (F_Fe2O3 / p.F_Fe2O3_0); % [mol/m^3/s] Pure kinetic reduction rate

r_red_mt  = k_red_mt * P_CO * a_s * p.rho_m_Fe2O3 * (F_Fe2O3 / p.F_Fe2O3_0); % [mol/m^3/s] Mass transfer limited reduction rate

r_red = (r_red_kin * r_red_mt) / (r_red_kin + r_red_mt + 1e-12); % [mol/m^3/s] Effective reduction rate

% 3. Combustion: limited by turbulent gas mixing (Eddy Dissipation)

v_gas = Q_local / p.A_fb; % [m/s] Local axial velocity of the gas

tau_mix = Dk / v_gas; % [s] Characteristic mixing time scale

k_comb_mix = 1 / tau_mix; % [s^-1] Mixing limited rate constant

r_comb_kin = k_comb * C_CO * C_O2; % [mol/m^3/s] Pure kinetic combustion rate

r_comb_mix = k_comb_mix * C_CO; % [mol/m^3/s] Mixing limited combustion rate based on fuel

r_comb = (r_comb_kin * r_comb_mix) / (r_comb_kin + r_comb_mix + 1e-12); % [mol/m^3/s] Effective combustion rate

% Smooth O2 depletion cutoff — shuts combustion off when O2 is exhausted

F_O2_raw = max(y(9), 0);                        % Raw state value, clipped to non-negative

f_O2_avail = F_O2_raw / (F_O2_raw + 0.01);       % ~1 when F_O2 >> 0.01, ~0 when depleted

r_comb = r_comb * f_O2_avail;                    % Deactivate combustion at O2 depletion

% --- Heat sources and duty ---

% Q_rxn sign matched precisely to documentation explicit formatting:

Q_rxn  = -p.DH_deh * (r_deh / p.Mw_AlOH3) * p.A_bed + p.DH_red * r_red * p.A_bed; % [W/m] Local heat of reaction applied to the bed

Q_duty = p.DH_deh * (r_deh / p.Mw_AlOH3) * p.A_bed + abs(p.DH_red) * r_red * p.A_bed + Q_shell_loss; % [W/m] Local heat duty requirement

% --- ODEs (Exact signs) ---

dF_AlOH3 = -r_deh * p.A_bed / p.Mw_AlOH3; % [mol/s/m] Axial derivative of Al(OH)3 flow rate

dF_Fe3O4 =  2 * r_red * p.A_bed; % [mol/s/m] Axial derivative of Fe3O4 flow rate

dF_Fe2O3 = -3 * r_red * p.A_bed; % [mol/s/m] Axial derivative of Fe2O3 flow rate

dF_CO  =  r_red * p.A_bed + r_comb * p.A_fb; % [mol/s/m] Axial derivative of CO flow rate

dF_CO2 = -r_red * p.A_bed - r_comb * p.A_fb; % [mol/s/m] Axial derivative of CO2 flow rate

dF_H2O = (3/2) * dF_AlOH3; % [mol/s/m] Axial derivative of H2O flow rate (generation from dehydration)

Q_comb_gas = abs(p.DH_comb) * r_comb * p.A_fb; % [W/m] Local heat released from combustion in the gas phase

dTg = (hew * p.A_ew * (Tg - Tw) + heb * p.A_eb * (Tg - Tb) + dF_H2O * cp_H2O * (Tb - Tg) - Q_comb_gas) / GCp_g; % [K/m] Axial derivative of gas temperature

dTb = (heb * p.A_eb * (Tg - Tb) + hcw * p.A_cw * (Tw - Tb) + Q_rxn) / GCp_b; % [K/m] Axial derivative of bed temperature

dF_O2 = 0.5 * r_comb * p.A_fb; % [mol/s/m] Axial derivative of O2 flow rate (Reactive consumption)

% --- Pack diagnostic info for post-processing ---

if nargout > 6

F_gas_total = F_CO + F_CO2 + F_H2O + p.F_N2_carrier + F_O2;

info.P_CO  = p.P_total * (F_CO / F_gas_total);

info.P_CO2 = p.P_total * (F_CO2 / F_gas_total);

info.P_H2O = p.P_total * (F_H2O / F_gas_total);

info.P_O2  = p.P_total * (F_O2 / F_gas_total);

info.P_N2  = p.P_total * (p.F_N2_carrier / F_gas_total);

info.r_deh_kin = r_deh_kin; info.r_deh_mt = r_deh_mt; info.r_deh = r_deh;

info.r_red_kin = r_red_kin; info.r_red_mt = r_red_mt; info.r_red = r_red;

info.r_comb_kin = r_comb_kin; info.r_comb_mix = r_comb_mix; info.r_comb = r_comb;

info.hew = hew; info.heb = heb; info.hcw = hcw;

Q_vol_solid = mass_sum / rho_b;

info.f_local = (Q_vol_solid / p.v_solid) / p.A_total;

else

info = [];

end

dydz = [dF_CO; dF_CO2; dF_H2O; dF_AlOH3; dF_Fe3O4; dF_Fe2O3; dTg; dTb; dF_O2];

end

## Acid leaching reactor matlab simulation

### SCM_RTD_SOLVER Size two equal-volume CSTRs using the SCM-RTD model

function result = scm_rtd_solver(tau_e, X_target)

%SCM_RTD_SOLVER Size two equal-volume CSTRs using the SCM-RTD model.

%

%   result = SCM_RTD_SOLVER(tau_e, X_target) determines the residence time

%   per reactor required to reach the specified outlet conversion. The

%   governing shrinking-core equation, residence-time distributions, and

%   numerical integration limits are retained exactly from the original

%   reactor sizing program.

%

%   Inputs

%       tau_e    - Characteristic time for complete particle conversion (h)

%       X_target - Target mean conversion leaving the second CSTR

%

%   Output

%       result   - Structure containing residence times, conversions,

%                  volume estimates, integration terms, and solver details.

if nargin < 2

warning('scm_rtd_solver:usingDefaults', ...

'Missing inputs — using design defaults: tau_e=2.5 h, X_target=0.90.');

if nargin < 1 || isempty(tau_e)

tau_e = 2.5;

end

if nargin < 2 || isempty(X_target)

X_target = 0.90;

end

end

validateattributes(tau_e, {'numeric'}, ...

{'scalar','real','finite','positive'}, mfilename, 'tau_e', 1);

validateattributes(X_target, {'numeric'}, ...

{'scalar','real','finite','>',0,'<',1}, mfilename, 'X_target', 2);

% The original design calculation used fzero with a single scalar

% residence-time guess. The same root-finding structure is kept here so

% that each sensitivity case repeats the complete SCM-RTD optimization.

tau_initial_guess = 0.5;     % h

options = optimset('Display','off','TolX',1e-8);

[tau_opt, error_at_solution, exitflag, solver_output] = fzero( ...

@(tau) reactor_error(tau, tau_e, X_target), ...

tau_initial_guess, options);

[~, X1_mean, X2_mean, int1, int2, int3, int4] = ...

reactor_error(tau_opt, tau_e, X_target);

% Reactor-volume basis retained from the existing sizing calculation:

% the base design gives V_total = 7.636 m^3 when the total residence

% time across both CSTRs is 2.3066 h at X_target = 0.90 and tau_e =

% 2.5 h. This defines the volumetric slurry-flow basis used to convert

% residence time to volume.

design_total_residence_time_h = 2.3066;

design_total_volume_m3 = 7.636;

volumetric_flow_rate_m3_per_h = ...

design_total_volume_m3/design_total_residence_time_h;

total_residence_time_h = 2*tau_opt;

total_reactor_volume_m3 = ...

volumetric_flow_rate_m3_per_h*total_residence_time_h;

result = struct();

result.tau_e_h = tau_e;

result.X_target = X_target;

result.tau_per_reactor_h = tau_opt;

result.total_residence_time_h = total_residence_time_h;

result.X1_mean = X1_mean;

result.X2_mean = X2_mean;

result.total_reactor_volume_m3 = total_reactor_volume_m3;

result.reactor_volume_each_m3 = total_reactor_volume_m3/2;

result.volumetric_flow_rate_m3_per_h = volumetric_flow_rate_m3_per_h;

result.error_at_solution = error_at_solution;

result.exitflag = exitflag;

result.solver_output = solver_output;

result.tau_initial_guess_h = tau_initial_guess;

result.integration_terms = struct( ...

'reactor1_partial_integral', int1, ...

'reactor1_complete_conversion_tail', int2, ...

'reactor2_partial_integral', int3, ...

'reactor2_complete_conversion_tail', int4);

result.sizing_basis = struct( ...

'design_tau_e_h', 2.5, ...

'design_X_target', 0.90, ...

'design_tau_per_reactor_h', design_total_residence_time_h/2, ...

'design_total_residence_time_h', design_total_residence_time_h, ...

'design_total_volume_m3', design_total_volume_m3);

% --- SINGLE CSTR COMPARISON ---

[tau_single, error_single, exitflag_single, solver_output_single] = fzero( ...

@(tau) single_reactor_error(tau, tau_e, X_target), ...

[0.2, 10], options);

single_reactor_volume_m3 = volumetric_flow_rate_m3_per_h * tau_single;

total_residence_time_single_h = tau_single;

total_residence_time_two_stage_h = total_residence_time_h;

volume_reduction_m3 = single_reactor_volume_m3 - total_reactor_volume_m3;

percent_total_residence_time_reduction = (total_residence_time_single_h - total_residence_time_two_stage_h) / total_residence_time_single_h * 100;

percent_total_residence_time_ratio = total_residence_time_two_stage_h / total_residence_time_single_h * 100;

percent_volume_reduction = volume_reduction_m3 / single_reactor_volume_m3 * 100;

result.tau_single_cstr_h = tau_single;

result.total_residence_time_single_h = total_residence_time_single_h;

result.total_residence_time_two_stage_h = total_residence_time_two_stage_h;

result.single_reactor_volume_m3 = single_reactor_volume_m3;

result.volume_reduction_m3 = volume_reduction_m3;

result.percent_total_residence_time_reduction = percent_total_residence_time_reduction;

result.percent_total_residence_time_ratio = percent_total_residence_time_ratio;

result.percent_volume_reduction = percent_volume_reduction;

result.single_reactor_solver = struct( ...

'error_at_solution', error_single, ...

'exitflag', exitflag_single, ...

'solver_output', solver_output_single);

fprintf('\n================ SINGLE CSTR COMPARISON ================\n');

fprintf('Single CSTR total residence time:             %.4f hours\n', total_residence_time_single_h);

fprintf('Two-stage total residence time:              %.4f hours\n', total_residence_time_two_stage_h);

fprintf('Two-stage requires only:                     %.2f%% of single CSTR total time\n', percent_total_residence_time_ratio);

fprintf('Total residence time reduction:              %.2f%%\n', percent_total_residence_time_reduction);

fprintf('Single CSTR volume at same flow:              %.3f m^3\n', single_reactor_volume_m3);

fprintf('Two-stage total reactor volume:               %.3f m^3\n', total_reactor_volume_m3);

fprintf('Volume reduction with two-stage design:       %.2f%%\n', percent_volume_reduction);

fprintf('=======================================================\n');

% --- FIGURES ---

total_tau_plot = linspace(0.05, max(total_residence_time_single_h, total_residence_time_two_stage_h) * 1.05, 300);

X_single_plot = arrayfun(@(t_total) mean_conversion_single(t_total, tau_e), total_tau_plot);

X_two_plot = arrayfun(@(t_total) mean_conversion_two_stage(t_total/2, tau_e), total_tau_plot);

figure('Name','Conversion: Single CSTR vs Two-Stage CSTR','NumberTitle','off');

plot(total_tau_plot, X_single_plot*100, 'b-', 'LineWidth', 2); hold on;

plot(total_tau_plot, X_two_plot*100, 'r-', 'LineWidth', 2);

plot(total_residence_time_single_h, X_target*100, 'ko', 'MarkerFaceColor', 'b', 'MarkerSize', 8);

plot(total_residence_time_two_stage_h, X_target*100, 'ks', 'MarkerFaceColor', 'r', 'MarkerSize', 8);

xline(total_residence_time_single_h, 'b--', 'LineWidth', 1.2);

xline(total_residence_time_two_stage_h, 'r--', 'LineWidth', 1.2);

hold off;

grid on;

xlabel('Total Residence Time, t_{total} (h)');

ylabel('Mean Conversion (%)');

title('Conversion vs Total Residence Time: Single CSTR and Two Equal CSTRs');

legend('Single CSTR','Two Equal CSTRs in Series', 'Single-CSTR design point','Two-stage design point', 'Location', 'southeast');

xlim([0 max(total_tau_plot)]);

ylim([0 100]);

figure('Name','Volume Comparison: Single CSTR vs Two-Stage','NumberTitle','off');

bar([single_reactor_volume_m3, total_reactor_volume_m3], 'FaceColor', 'flat');

colormap([0 0.4470 0.7410; 0.8500 0.3250 0.0980]);

set(gca,'XTickLabel',{'Single CSTR','Two-Stage CSTR'});

ylabel('Required Reactor Volume (m^3)');

title('Volume Savings from Two Equal CSTRs in Series');

text(1, single_reactor_volume_m3*1.02, sprintf('%.3f m^3', single_reactor_volume_m3), 'HorizontalAlignment','center');

text(2, total_reactor_volume_m3*1.02, sprintf('%.3f m^3', total_reactor_volume_m3), 'HorizontalAlignment','center');

ylim([0 max(single_reactor_volume_m3, total_reactor_volume_m3)*1.15]);

figure('Name','Percentage Reduction from Single CSTR','NumberTitle','off');

bar(percent_volume_reduction, 'FaceColor', [0.4660 0.6740 0.1880]);

set(gca,'XTickLabel',{'Volume Reduction'});

ylabel('Percent Reduction (%)');

title('Two-Stage CSTR Volume Reduction Compared to Single CSTR');

text(1, percent_volume_reduction*1.02, sprintf('%.2f%%', percent_volume_reduction), 'HorizontalAlignment','center');

end

function [error, X1_mean, X2_mean, int1, int2, int3, int4] = ...

reactor_error(tau, tau_e, X_target)

%REACTOR_ERROR Evaluate the outlet-conversion residual for a trial tau.

%

%   The first CSTR uses the exponential age distribution. The two-reactor

%   train uses the gamma RTD corresponding to two equal CSTRs in series.

%   Particles older than tau_e are treated as completely converted, exactly

%   as in the original numerical model.

int1 = integral(@(t) ...

arrayfun(@(tt)get_X_particle(tt,tau_e),t).* ...

(1/tau.*exp(-t/tau)), ...

0,tau_e);

int2 = exp(-tau_e/tau);

X1_mean = int1 + int2;

int3 = integral(@(t) ...

arrayfun(@(tt)get_X_particle(tt,tau_e),t).* ...

(t./tau.^2.*exp(-t/tau)), ...

0,tau_e);

int4 = (1 + tau_e/tau).*exp(-tau_e/tau);

X2_mean = int3 + int4;

error = X2_mean - X_target;

end

function [error, X1_mean, int1, int2] = single_reactor_error(tau, tau_e, X_target)

%SINGLE_REACTOR_ERROR Evaluate the single CSTR outlet conversion residual.

int1 = integral(@(t) ...

arrayfun(@(tt)get_X_particle(tt,tau_e),t).* ...

(1/tau.*exp(-t/tau)), ...

0,tau_e);

int2 = exp(-tau_e/tau);

X1_mean = int1 + int2;

error = X1_mean - X_target;

end

function X1_mean = mean_conversion_single(tau, tau_e)

int1 = integral(@(t) ...

arrayfun(@(tt)get_X_particle(tt,tau_e),t).* ...

(1/tau.*exp(-t/tau)), ...

0,tau_e);

int2 = exp(-tau_e/tau);

X1_mean = int1 + int2;

end

function X2_mean = mean_conversion_two_stage(tau, tau_e)

int3 = integral(@(t) ...

arrayfun(@(tt)get_X_particle(tt,tau_e),t).* ...

(t./tau.^2.*exp(-t/tau)), ...

0,tau_e);

int4 = (1 + tau_e/tau).*exp(-tau_e/tau);

X2_mean = int3 + int4;

end

function X = get_X_particle(t, tau_e)

%GET_X_PARTICLE Solve the product-layer diffusion SCM for one particle age.

%

%   For 0 < t < tau_e, the conversion is obtained by numerically solving

%   the implicit ash/product-layer diffusion shrinking-core equation.

if t <= 0

X = 0;

elseif t >= tau_e

X = 1;

else

scm_eq = @(Xv) ...

1 - 3*(1-Xv).^(2/3) + 2*(1-Xv) - t/tau_e;

X = fzero(scm_eq,[0 1], ...

optimset('Display','off'));

end

end

### optimize_cstr_series

function optimize_cstr_series()

clear; clc; %#ok<CLEAR0ARGS>

% --- DESIGN INPUTS ---

X2_target = 0.90;  % Final target fractional conversion (90%)

tau_e = 2.5;       % ASSUMED: Time for complete particle conversion (hours)

% --- NUMERICAL SOLVER ---

% Find the tau (residence time per reactor) that satisfies X2_target

options = optimset('Display','iter','TolX',1e-6);

tau_initial_guess = 0.5; % hours

fprintf('Optimizing reactor residence times using SCM population balance...\n\n');

tau_opt = fzero(@(tau) reactor_error(tau, tau_e, X2_target), tau_initial_guess, options);

% --- POST-CALCULATION RESULTS ---

[~, X1_mean, X2_mean] = reactor_error(tau_opt, tau_e, X2_target);

fprintf('\n================ DESIGN RESULTS ================\n');

fprintf('Time for Complete Conversion (tau_e): %.2f hours\n', tau_e);

fprintf('Optimized Residence Time per CSTR (tau): %.4f hours\n', tau_opt);

fprintf('Total Residence Time (2 x tau):        %.4f hours\n', 2 * tau_opt);

fprintf('Mean Conversion leaving Reactor 1 (X1): %.2f%%\n', X1_mean * 100);

fprintf('Mean Conversion leaving Reactor 2 (X2): %.2f%%\n', X2_mean * 100);

fprintf('================================================\n');

% --- PLOTTING ---

% 1) Particle conversion vs dimensionless time

t_plot = linspace(0, tau_e, 250);

X_particles = arrayfun(@(t) get_X_particle(t, tau_e), t_plot);

figure('Name','Particle Conversion vs Dimensionless Time','NumberTitle','off');

plot(t_plot / tau_e, X_particles, 'LineWidth', 2);

grid on;

xlabel('Dimensionless Time, t/\tau_e');

ylabel('Particle Conversion, X');

title('Particle Conversion for SCM Kinetics');

xlim([0 1]);

ylim([0 1]);

% 2) Exit age distribution for single CSTR and two equal CSTRs in series

t_rtd = linspace(0, 6.5, 400);

E_single = (1 / tau_opt) .* exp(-t_rtd / tau_opt);

E_series = (t_rtd ./ (tau_opt^2)) .* exp(-t_rtd / tau_opt);

figure('Name','RTD: Single CSTR vs Two CSTRs in Series','NumberTitle','off');

plot(t_rtd, E_single, 'LineWidth', 2); hold on;

plot(t_rtd, E_series, 'LineWidth', 2); hold off;

legend('Single CSTR', 'Two Equal CSTRs in Series', 'Location', 'northeast');

grid on;

xlabel('Residence Time, t (h)');

ylabel('E(t)');

title('Residence Time Distribution');

xlim([0 max(t_rtd)]);

% 3) Mean conversions as a function of residence time per reactor

tau_range = linspace(0.25, 2.5, 250);

X1_curve = zeros(size(tau_range));

X2_curve = zeros(size(tau_range));

for k = 1:numel(tau_range)

[~, X1_curve(k), X2_curve(k)] = reactor_error(tau_range(k), tau_e, X2_target);

end

figure('Name','Mean Conversion vs Residence Time per Reactor','NumberTitle','off');

plot(tau_range, X1_curve * 100, 'b-', 'LineWidth', 2); hold on;

plot(tau_range, X2_curve * 100, 'r-', 'LineWidth', 2);

plot(tau_opt, X1_mean * 100, 'o', 'MarkerSize', 8, 'MarkerFaceColor', 'y', 'MarkerEdgeColor', 'k');

plot(tau_opt, X2_mean * 100, 's', 'MarkerSize', 8, 'MarkerFaceColor', 'm', 'MarkerEdgeColor', 'k');

xline(tau_opt, 'k--', 'LineWidth', 1.5);

hold off;

legend('After Reactor 1 (X_1)', 'After Reactor 2 (X_2)', 'Design X_1', 'Design X_2', 'Location', 'southeast');

grid on;

xlabel('Residence Time per Reactor, \tau (h)');

ylabel('Mean Conversion (%)');

title('Mean Conversion vs Residence Time per Reactor');

xlim([tau_range(1) tau_range(end)]);

ylim([30 100]);

end

% --- HELPER FUNCTIONS ---

function [error, X1_mean, X2_mean] = reactor_error(tau, tau_e, X2_target)

% This function calculates the mean conversions and evaluates the root error

% 1. Calculate Mean Conversion leaving Reactor 1

% Integral from 0 to tau_e (partially reacted particles)

int1 = integral(@(t) arrayfun(@(t_val) get_X_particle(t_val, tau_e), t) .* (1/tau .* exp(-t/tau)), 0, tau_e);

% Particles that stayed longer than tau_e are 100% converted (X=1.0)

int2 = exp(-tau_e/tau);

X1_mean = int1 + int2;

% 2. Calculate Mean Conversion leaving Reactor 2 (using RTD of 2 CSTRs in series)

% Integral from 0 to tau_e

int3 = integral(@(t) arrayfun(@(t_val) get_X_particle(t_val, tau_e), t) .* (t./(tau^2) .* exp(-t/tau)), 0, tau_e);

% Particles that spent > tau_e total time across both reactors are 100% converted

% This is the analytical integration of (t/tau^2)*exp(-t/tau) from tau_e to infinity

int4 = (1 + (tau_e/tau)) * exp(-tau_e/tau);

X2_mean = int3 + int4;

% Error is the difference between calculated X2 and our target

error = X2_mean - X2_target;

end

function X = get_X_particle(t, tau_e)

% Finds the explicit conversion X for a single particle at a given time 't'

% via the Ash-Layer Diffusion SCM formula: 1 - 3(1-X)^(2/3) + 2(1-X) - t/tau_e = 0

if t <= 0

X = 0;

elseif t >= tau_e

X = 1;

else

% Solve the non-linear SCM equation for X

scm_eq = @(X_val) 1 - 3*(1-X_val)^(2/3) + 2*(1-X_val) - (t/tau_e);

X = fzero(scm_eq, [0, 1], optimset('Display','off'));

end

end

## Appendix: P&ID Nomenclature & Tagging Legend

Table 14-1 Material Specifications & Piping Classes

| Spec Code | Full Meaning & Description | Typical Process Application |
| --- | --- | --- |
| CS / CS150 | Carbon Steel (ANSI Class 150) | Standard dry solids handling, non-corrosive gas ducts, and cooling water lines. |
| SS / SS150 | Stainless Steel (Class 150) | High-temperature exhaust ducts, preheaters, and slurry transport lines. |
| CS-RL | Carbon Steel – Rubber Lined | Abrasive bauxite slurries and acidic process slurry transfer lines. |
| CPVC-FRP | Chlorinated Polyvinyl Chloride with Fiberglass Reinforced Plastic Structural Wrap | Concentrated hydrochloric acid handling (TK-601 feed). |
| FRP | Fiberglass Reinforced Plastic | Clarified pregnant leach liquor (PLP) and intermediate PAC holding lines. |
| PTFE | Polytetrafluoroethylene (Teflon-Lined) | High-corrosion polymerization/basification streams and hot acidic liquor transfer. |
| INC | Inconel / High-Nickel Refractory-Lined Alloy | Acid vapor scrubbers and hot gas vent headers (NC-AG). |

Table 14-2 Fluid & Process Service Codes

| Service Code | Full Meaning | Process Area / Stream Location |
| --- | --- | --- |
| BAU | Raw Bauxite Ore | Area 100 (Receiving & Crushing) |
| EXG | Exhaust Flue Gas | Area 200/300 (Kiln & Scrubber System) |
| FG | Fuel Gas | Area 200 (Burner System) |
| CWS | Cooling Water Supply | Plant Utility Header |
| CWR | Cooling Water Return | Plant Utility Return |
| IA | Instrument Air | Area 300 (Baghouse Pulse-Jet Cleaning) |
| PW | Process Water | Area 300/500 (Dust Scrubber & Slurry Prep) |
| DST | Fine Bauxite Dust | Area 300 (Dust Recovery System) |
| CB / CBA | Calcined Bauxite Ore | Area 200/400/500 (Post-Roasting Dry Stream) |
| MAG | Magnetite (Iron Ore Byproduct) | Area 400 (Magnetic Separation Circuit) |
| SLR / SL | Bauxite Slurry | Area 500/600 (Grinding & Digestion Circuits) |
| HCl | Hydrochloric Acid (37% Concentrated) | Area 600 (Acid Digestion Feed) |
| PLP | Pregnant Leach Liquor | Area 600/700 (Digester Liquid Product) |
| AG / NC-AG | Non-Condensable Acid Gas Vent | Area 600 (Scrubber Vent Header) |
| CAA | Calcium Aluminate Additive | Area 700 (Polymerization Basification Agent) |
| PAC | Polyaluminum Chloride | Area 700 (Final Liquid Product Stream) |

Table 14-3 Equipment Class Prefixes

| Equipment Tag | Full Meaning | Description |
| --- | --- | --- |
| BH | Baghouse Filter | Pulse-jet fabric filter dust collector. |
| BN | Bin / Storage Silo | Bulk solids storage hopper/silo. |
| BR | Burner | Thermal combustion burner for rotary kiln. |
| CL | Cooler | Forced air or jacketed process cooler. |
| CR | Crusher | Primary bauxite ore crusher. |
| CRN | Bucket Elevator / Incline Conveyor | Raw material elevation drive. |
| CV | Conveyor | Belt or screw conveyor system. |
| CY | Cyclone Separator | Dust knockout cyclone. |
| E | Heat Exchanger | Process fluid heat exchanger / preheater. |
| FD | Feeder | Variable speed screw/apron feeder. |
| FN | ID Fan | Induced draft exhaust fan. |
| FP | Filter Press | Solid-liquid plate-and-frame filtration unit. |
| HC | Hydrocyclone | Centrifugal slurry classification unit. |
| M | Motor Drive | Mechanical drive motor. |
| ML | Ball Mill | Wet grinding ball mill unit. |
| MS | Magnetic Separator | Dry drum magnetic drum separator. |
| P | Pump | Centrifugal or slurry pump. |
| R | Reactor | Leaching digester or polymerization reactor. |
| RL | Rotary Airlock Valve | Rotary dust discharge airlock. |
| SC | Scrubber | Acid vapor / particulate wet scrubber. |
| ST | Stack | Cleaned gas exhaust stack. |
| TK | Tank | Liquid reagent or product storage vessel. |

Table 14-4 Instrumentation & Control Valves (ISA-5.1)

| Instrument Tag Prefix | Full Instrument Meaning | Function |
| --- | --- | --- |
| DIC / DT | Density Indicating Controller / Transmitter | Slurry specific gravity control loop. |
| FIC / FT | Flow Indicating Controller / Transmitter | Liquid flow rate monitoring & control. |
| FFIC | Ratio Flow Indicating Controller | Acid-to-bauxite stoichiometry feed ratio control. |
| LIC / LT / LI | Level Indicating Controller / Transmitter / Indicator | Vessel liquid level control. |
| LAH / LAL | Level Alarm High / Level Alarm Low | Tank safety level thresholds. |
| pHIC / pHT | pH / Basicity Indicating Controller / Transmitter | Polymerization reactor basicity control. |
| PIC / PT / PI | Pressure Indicating Controller / Transmitter / Indicator | System static/vapor pressure control. |
| SIC / ST | Speed Indicating Controller / Transmitter | Motor/conveyor variable speed drive control. |
| TIC / TT / TE | Temperature Indicating Controller / Transmitter / Element | Thermal reaction/roasting control. |
| CV / PV / TV / LV / FV | Control Valve | Automated control valve (Pressure, Temp, Level, Flow) |

Source: (Momade & Gawu, 1998)